Pseudokick 1.3  1991-1996 by OR

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

INTRODUCTION

It's a Kickstart loader.

It allows you to boot your A500 or A500 Plus from your harddisk

into Kickstart R2.0-R3.1.

It's freeware.

REQUIREMENTS:

A500 or A500 Plus

A590 hard drive

2MB expansion memory

512KB chip memory

Kickstart image file

TESTED AT THESE SYSTEMS:

A500, KS1.3, 512KB chip / 512KB fast and 1MB chip memory via A501.

A500 Plus.

USER GUIDE

- It doesnt need to patch or reloc tables like other kickers.

The only needed thing is the ROM image of the Kickstart.

Dont use patched/relocated etc images.

You can obtain the image from the address $F80000-$1000000 (512KB).

While testing this utility, i tried KS1.3,3.0,3.1 (v40.68).

KS1.3 didnt support booting from harddisk and this version of Kickstart

is both obsolete and unreliable. Because the program is written

to load KS2.0+. KS3.1 didnt work. Because it was the

68020+ version taken from an A4000. If you have an image of

KS3.1 for A500 you can try it.

- The image file must be found in 'L:' directory named as 'kickfile'.

So as to run the program, you have to include the line;

Pseudokick

at the very beginning of your Startup-Sequence (exactly before the

command Setpatch). When you first power on your Amiga it will boot

using its own version of Kickstart first. This old Kickstart will

run the program since it is the very first thing to be executed in the

Startup-Sequence. Then, your computer will reset and boot using the

Kickstart image file in your L: . This new (strange, it might also

be an older one,...anyway lets not go deeper into this) Kickstart

will execute your Startup-Sequence. Pseudokick, when it's executed,

will first check whether the ROM is in RAM or not. If it finds a

version of Kickstart ROM in RAM, the program will exit. But if there

is something going wrong it will exit with an error code.

- If you see white lines during kicking, it means that Pseudokick

has found an error. After a soft-reset you can learn what happened

from the given guru message. Here are the messages that can

appear and their meaning:

80000002.000ACABA  Internal error

80000004.000ACABA  Expansion memory size less than 2MB

80000007.000ACABA  Expansion memory not available

80000008.000ACABA  Expansion memory not configurable

OTHER FEATURES

Prevents floppy drives clicks and sets retry count 1.

You can use "*" instead of AmigaDos wildcard chars "#?".

Important: If you use "*" alone it means the default window,

so that things change a little. Because of that you are adviced

to use double stars. For example use LIST ** instead of LIST *.

'Copy console: file' command writes the informatin taken from the console

to the specified file. For EOF press ctrl-backslash.

Email address:

Bitnet:   prohst03@trearn

Internet: prohst03@vm.ege.edu.tr

*<>*