$VER: Terminal documation 3.5

*****************************************************************************

**                                                                         **

** ___________                                                             **

** \        */                                                             **

**  /-----------     #######     #######    ########                       **

**  |    ^ |        ##      ##  ##     ##   ##     ##                      **

**  |      _\       ##          ##     ##   ##     ##  Version 1.58        **

**  |      |        ##          ##     ##   ########                       **

**  |    --|        ##          ##     ##   ##                             **

**  +-----/         ##      ##  ##     ##   ##                             **

**   |   |           #######     #######    ## (c)1993-96 THOR-Software    **

**                                                                         **

**              A Low Level Debugger for all Amiga Systems                 **

**-------------------------------------------------------------------------**

**                 __                                                      **

**               ___/|                                                     **

**              /  |:|  Type 3 Terminal (COP/DASB) Specifiactions          **

**       ====== |  |:|  Type 4 Terminal (COP/DASB 4.2)                     **

**       ====== |  |:|  Type 5 Terminal (COP)                              **

**       ====== |  |:|  (c) 1994,95 THOR-Software                          **

**              \__|/                                                      **

**                                                                         **

**      Release 3.3             14th May 1994                              **

**      Release 3.4             29th May 1994 - added type 4 terminals     **

**      Release 3.5             31th May 1995 - added type 5 terminals     **

*****************************************************************************

This documentations describes the character and control codes used by the

COP System debugger, copyrighted by THOR.

Terminology

-----------

Orientation of a "COP SCREEN"

first column                                   last column

|                                           |

+-------------------------------------------+ <- top line

|                                           |

|                                           |

|                                           | <- top window

|                                           |    (this one is fixed and

|                                           |     does not scroll)

+-------------------------------------------+

|                                           |

|*******************************************| <- marked line (type 4

|                                           |    terminals text

|                                           |    blinking or high

|                                           |    intensity)

|                                           |

|                                           | <- bottom window

|           # (cursor)                      |       (scrollable)

|                                           |

|                                           |

+-------------------------------------------+ <- bottom line

^

Screen  |       (the whole thing)

Lines are counted from top bottom with the top line beeing number 0.

Columns are counted from left to right with the first column beeing 0.

Screen Specifications

---------------------

columns:        80 (fixed)

rows:           betweeen 24 and 64

Line marking (Type 4 terminals only)

------------

The characters in a given line share a common attribute, selectable by

an escape sequence (see below). The following attributes are defined and

will be send by the highlight escape sequence :

00:             Normal  (as for type 3 and ascii terminals)

01:             Highlight or white background

02:             Blink or red background

03:             undefined, terminal specific and not used by COP

Notes:

i)      It is not possible to mark SINGLE characters, only whole lines.

Character marking might be included in a later release.

ii)     Line marking is not compatible with scrolling. Disable the

marks first, if you want to scroll.

Some terminals scroll the marks together with the characters,

others won't.

Serial Transfer Specifications (if any)

---------------------------------------

baud rate:      9600

start bits:     1

stop bits:      1

parity:         none

Character Map

-------------

i)control characters (in HEX)

00      :       NUL     ignore

08      :       BS      non-erasing backspace

do nothing at start of line

09      :       TAB     Jump to next tab stop

tab stops are set every eigth

columns. Jump to next line and

scroll if beyond last tab stop.

0A      :       LF      Move cursor down one line. Scroll

if end of display reached. Perform

NO CR, don't set cursor to start of

line. (No auto CR)

The effect of scrolling marked

lines is undefined.

0C      :       FF      Clear screen. Move cursor to top of

screen. Set window size to zero

(=disable splitting).

On type 4 terminals : clear all line

marks.

0D      :       CR      Set cursor to start of line. Perform

NO LF, do not scroll, move cursor

NOT downwards. (No auto LF)

1B      :       ESC     introduce ESC-sequence (see below)

1C      :       UP      move cursor upwards one line. If top

of screen reached, move to

bottommost line, do no reverse scroll

1D      :       DOWN    Move cursor downwards one line. If

bottom reached, jump to first line,

do not scroll.

1E      :       LEFT    move cursor one character to the

left, if first column passed, move

to last column of the same line

(do not move cursor upwards)

1F      :       RIGHT   move cursor one character to the

right, if last column passed, move

to first column of the same line

(do not move cursor downwards)

20-7E   :               standard american ASCII.

7F      :       DEL     Reserved, not used by COP.

You should not use this character.

9B      :       EOL     Move cursor down one row, then move

it to the first column (=LF & CR)

Note:   The term "scrolling" is used as "scrolling the scrollable bottom

window", the top window (if any) is never scrolled.

All other codes are ignored, especially no 8-bit ASCII-codes are supported

and interpretation of this codes are terminal-dependent. (National languague

characters as defined by ISO, IBM or MAC characters or inverse characters).

They are not used by COP.

ESC-Sequences

-------------

All arguments to ESC-sequences are 7 bit binary numbers. I decided not to

use decimal numbers to keep the thing simple and fast....

The answer values of an external terminal, if any, is also a 7 bit number

and is noted here by "->":

1B 01           :       Send ID

-> id           The terminal should send within 1/2 sec.

its version number as binary code.

All terminals with codes >=3 are

believed to support type 3 terminal

codes. If the terminal does not repond

in this period, COP assumes this beeing

an ASCII terminal.

All terminals >=4 support type 4 codes.

If this is a type 5 terminal, it should

disable the printer output, similar to

the disable command described below.

1B 02 xx yy     :       Set cursor to column xx, line yy, the

first column/line is number 0.

xx and yy are transmitted as binary

codes, NOT AS DECIMALS.

1B 03 yy        :       Draw horizontal line at line yy, set

cursor to line yy,column 0. The drawing

should be done by inverting the screen,

so drawing the line again undoes it.

1B 04           :       Draw horizontal line at current cursor

position, reset cursor to column 0.

1B 05 yy        :       Set the size of the top window in

lines, no top window if yy=0.

With the top window activated, scolling

takes place only in the bottom window,

starting with line yy to the bottom line.

No graphics are printed to make the window

visible, use the draw line command in the

top window to do this.

1B 06           :       Send cursor position. The terminal must

-> xx yy        repond with two binary numbers beeing

the column number/line number of the

current cursor position resp.

The first column/line is number 0.

1B 07 y1 y2     :       Clear region. The lines from y1 inclusive

and y2 exclusive are cleared. The cursor

position is not affected.

(Don't forget to redraw the cursor !)

1B 08           :       Clear this line. The line at current cursor

position is cleared, the cursor is not

affected.

(Don't forget to redraw the cursor !)

1B 09           :       Scroll bottom window, do not affect cursor.

The window should be scrolled under the

cursor, so the cursor position won't

change in respect to the upper left edge

of the screen, but will change in respect

to the text scrolled under the cursor.

The effect of scrolling marked lines

is undefined and COP does not send codes

to do so.

1B 0A           :       Send screen size. The terminal must respond

-> yy           with the size of the active display in

lines, coded as a binary number.

The following sequences are defined for type 4 terminals only and won't

be send to type 3 terminals:

1B 0B yy cc     :       Mark line yy with color cc. Colorcodes

00      :       normal background

01      :       white background or

high intensity

02      :       red background or

blinking mode

03      :       terminal specific

(not sent by COP)

1B 0C cc        :       Mark cursor line with color cc.

1B 0D           :       Remove all marks.

1B 0E           :       Clear until end of line

the cursor position won't change by this

command

...and now the type 5 terminal sequences (won't be sent to types 3 and 4):

1B 10           :       Start printer output

All data send to the terminal should be

printed to a local printing device,

except ESC-sequences

1B 11           :       Stop printer output

Display data only on the screen

Both calls should not nest, this is done

by COP

Implementations

---------------

Currently only two terminal programs support COP control sequences. The

first is the "Terminal" program found in the COP archive, for amiga computers.

If you own a second amiga, run this program and use a null modem cable for

connection (GND,TxD and RxD crossed is enough for COP).

The second program is "DASB" for the antique Atari 8 bit series. It requires

an 850 or 850XL interface box and a disk drive. It is also available in the

COP archive, in the drawer "DASB_German".

It would be nice if somebody could write more COP terminals for other

computers. Contact me in this case, the programs will be included in the

COP archive next time.

Thomas