DOCUMENT FILE FOR PCBTODXF.EXE  Version 2.01

(c) 1991  PROTEL TECHNOLOGY PTY LTD

This program  converts an  AUTOTRAX PCB  file  to  the  AUTOCAD  Drawing

Interchange file (DXF) format.

Because of  the space  intensive nature  of the  DXF format, the program

generates either "draft" mode or "full" mode output.

Running PCBtoDXF.

Type PCBtoDXF

The following will appear

AUTOTRAX FILE to AUTOCAD FILE PROGRAM  Version 2.01

---------------------------------------------------

Copyright (c) 1988,1991 Protel Technology Pty. Ltd.

Input File Name >

Enter the AUTOTRAX PCB file name (the extension will be forced to .PCB)

Output File Name >

Enter the DXF output file name.(the extension will be forced to .DXF)

If the  output file  already exists,  then you  will be asked to confirm

overwriting it.

Output File SOMEFILE.DXF Exists, Overwrite Y/N >

Press the Y or N key.

You can  also supply  these filenames  from the  command line,  but  the

overwrite prompt will still appear.

If you  supply incorrect number of  command  line  parameters  then  the

following help message will appear.

Correct Command Line Usage

PCBtoDXF <AUTOTRAX File Name> <DXF File Name>

For Example

PCBtoDXF DEMO TEST

Will Generate The File TEST.DXF

If No File Names Are Supplied, The Program Will Prompt For Them.

If the  input file  is not found, then the following message will appear

and you will be returned to the DOS prompt.

Input File Not Found.  PCBtoDXF Program Aborted

Otherwise the following message will be displayed

Do You Want Full Quality Output in the DXF file (Big File)

Y:es, Full Quality

N:o, Generate Only Draft Quality

Press the  Y or N key to continue. The following message will appear and

the DXF file will be generated.

Processing SOMEFILE.PCB

The sizes  of the  DXF file  will vary  dramatically  depending  on  the

contents of  the PCB file, but as a rough guide, the draft mode DXF file

will be  3 to  4 times  the size  of the PCB file, and the full mode DXF

file will be 8 to 12 times the size of the PCB file.

Notes on storage of data in the DXF file

----------------------------------------

The program  will place  all of  the PCB  data from  all PCB layers onto

equivilent AutoCAD layers. To keep the space to a minimum, single letter

layer designators are used.

Top Layer        T

Mid Layer 1      1

Mid Layer 2      2

Mid Layer 3      3

Mid Layer 4      4

Bottom Layer     B

Top Overlay      O

Bottom Overlay   M

Ground Layer     G

Power Layer      P

Board Layer      D

Keep-Out Layer   K

Multi-Layer      U

All items  (both free  and component primitives) are saved as individual

entities in the DXF file.

Tracks

In draft mode, tracks are stored as simple lines.

In full  mode, tracks  are stored  as a  single poly-line  with a  solid

circle on  each end.  Each solid  circle  consists  of  two  180  degree

polyarcs.

Round Pads and Vias

In draft mode, round pads and vias are stored as simple circles.

In full mode they are stored as two 180 degree polyarcs.

Round Rectangular Pads

In Draft  mode, these consist of two simple arcs, with two simple lines.

In Full  mode rounded  rectangular pads  are stored  in the  same way as

tracks.

Rectangular Pads and Fills

These are  always drawn  as solid  fills. Although this is inconsistant,

(ie fills  are solid  when  all  others  are  wire-frame),  the  storage

requirements for  rectangular solids  are less  than for 4 lines. If you

wish to  display a  draft mode  DXF file as all wire-frame (ie including

area-fills), then  use the  AutoCAD FILL  command to  display SOLIDS  as

wire-frames.

Octagonal Pads

In Draft mode these are stored as eight lines.

In full  mode, they  are stored  as three  horizontally oriented AutoCAD

solids.

Targets

In draft mode, stored as 2 lines and 2 arcs.

In full mode, stored as 2 polylines and 2 polyarcs.

Moires

Stored as either arcs in draft mode or polyarcs in full mode.

Text

Always stored  as scaled  and rotated  AutoCAD text  using  the  default

AutoCAD font. Line thickness is always 1 pixel.

Arcs

In draft mode, stored as a single AutoCAD arc.

In full  mode stored  as 1 or 2 polyarcs (since polyarcs can only be 180

degrees max).

(rev 3.91)

(end)