LANtastic (tm) LANCHECK Program Documentation

------------------------------------------

(C) Copyright 1990 ARTISOFT Inc.

Once the NETBIOS has been installed, you can be sure that

the adapter is functioning.  You cannot, however, be assured

that the adapter can communicate with the network.

There may be several reasons why an adapter is not

communicating with the rest of the network.  The most

obvious cause is that the cabling is faulty or not connected

correctly.  Another reason may be that there has been a

hardware failure on the adapter in the circuitry that is

responsible for communication on the network.

The distribution floppy disk that comes with your LANtastic

Starter Kit contains a LAN diagnostic program (LANCHECK)

which may be used to make sure that all adapter cards are

functioning and connected together on the network.  The

LANCHECK program appears as LANCHECK.EXE on the distribution

diskette.

To run the program type:

LANCHECK a-unique-name  [/MONO]

where a-unique-name is a unique name (up to 16 characters)

that you wish your PC to be known by on the network.  This

name will be displayed on each computer running LANCHECK and

allows you to monitor which computers are actively running

LANCHECK.

It is recommended that you bring up all your network nodes

simultaneously to ensure that they are evaluated for

performance during the same period.   LANCHECK evaluates

each node's performance from when it is first brought up on

the network, not just from the time LANCHECK is executed.

LANCHECK will test six adapter cards per computer (adapters

0 through 5) and can display a maximum of 1000 nodes.  You

can run LANCHECK any time after bringing up the NETBIOS, even

after other LAN software is running.

The most important test for the adapters is whether they all

appear on the screen when LANCHECK is run.  If an adapter is

not listed on the LANCHECK screen, it is probably not

communicating with the rest of the network.

The optional switch /MONO forces the screen to run

the LANCHECK program in monochrome (2 color) mode.

Testing a Network Adapter

After the NETBIOS software has been installed, type the

following commands:

LANCHECK a-unique-name

For example,

LANCHECK JOAN

or

LANCHECK BOB

would be typical unique names.

After an initialization message and a short pause (Note: the

pause will be longer if you have multiple adapter cards

attached),  the LANCHECK screen will appear as shown:

LANtastic (tm) LANCHECK  Version 3.01  (C) Copyright 1990 Artisoft Inc.

���������������������������������������������������������������������������ͻ

� A#  NAME             NODE NUMBER   MINUTES RUNNING   STATUS   ERROR INDEX �

���������������������������������������������������������������������������͹

� 0   BOB              00006EA00029        9           (local)     2%       �

� 0   JOAN             00006EA00045        9           active      2%       �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

���������������������������������������������������������������������������ͼ

Enter - Select Entry,  Esc - Exit Lancheck,  Space - Update Screen,  F1 - Help

At any time while the LANCHECK program is running you may

press the F1 key to receive more information about the

fields presented in the screen.  You can make the help

window double in size by pressing the letter Z on the

keyboard.  Pressing Z a second time will change the help

screen back to its original size. Use the arrow keys to

scroll the text up and down, or use the PgUp or PgDn  keys

to move the text up or down a page.

The screen updates itself every ten seconds, or when you

press the SPACE BAR.

"A#" tells you which adapter you are receiving information

on .  The first adapter is numbered "0," the second "1" and

so on.  Up to six adapters per computer may be tested.

"NAME" refers to  the unique node name you specified with

the LANCHECK command.   Check to make sure that all the

adapter cards you are running LANCHECK on are listed in this

table.  If any are missing, you may have a cabling problem

or a faulty adapter.

"NODE NUMBER" denotes the unique network number assigned to

the adapter. "Minutes Running"  shows how long the adapter

has been running since you installed the NETBIOS software.  The

"Status"  field tells you whether the adapter being tested

is active or inactive, and which adapters are "local,"

(located on your computer).  If an adapter that is supposed

to be active lists as inactive, there may be a problem with

the adapter or the cabling to that adapter.  If you see an

adapter with a status that keeps switching between "Active"

and "Inactive"  there is probably an intermittent problem

with the cabling.

The Field "ERROR INDEX"  presents you with a statistic to

help you deduce the relative performance of each adapter in

the sending and receiving of network data.  The values given

in the example above should not be considered an average or

ideal value for the adapters you are evaluating.  Look

instead for adapters that have ERROR INDEX  values that are

either much higher or lower than the other adapters in your

network.  An adapter with a much higher ERROR INDEX  might have a

cabling problem or other hardware problem.  Low values might

indicate the node is able to receive data correctly, but is

not properly transmitting data to the rest of the network.

This will increase the other nodes' ERROR INDEX  scores

because they will be receiving bad transmissions from the

faulty adapter.

To further evaluate the efficiency of a network node, use

the arrow keys to move the highlight bar to the desired node

and press ENTER. This window will appear:

LANtastic (tm) LANCHECK  Version 3.01  (C) Copyright 1990 Artisoft Inc.

���������������������������������������������������������������������������ͻ

� A#  ��������������������������������������������������������������ͻ INDEX�

�����ͺ               Adapter Status of: JOAN                        ������͹

� 0   ��������������������������������������������������������������͹      �

� 0   �                  Adapter Number: 0                           �      �

�     �                     Node Number: 00006EA00045                �      �

�     �                Software Version: 2.10                        �      �

�     ��������������������������������������������������������������ͺ      �

�     �  TRAFFIC AND ERROR STATISTICS  � ADAPTER RESOURCE STATISTICS �      �

�     ��������������������������������������������������������������ĺ      �

�     �    Minutes Running: 9          �     NCBs Available: 26      �      �

�     �         CRC Errors: 222        �     NCBs Allocated: 32      �      �

�     �   Alignment Errors: 320        �  Max NCBs Possible: 128     �      �

�     �         Collisions: 9          �    Active Sessions: 4       �      �

�     �  Bad Transmissions: 65         � Sessions Allocated: 32      �      �

�     � Good Transmissions: 3928       �  Sessions Possible: 128     �      �

�     �      Good Receives: 23103      �    Max Packet Size: 2461    �      �

�     �    Retransmissions: 65         �    Number of Names: 8       �      �

�     �   Resource Exhaust: 838        �                             �      �

�     ��������������������������������������������������������������ͼ      �

�  									    �                                                                           �

���������������������������������������������������������������������������ͼ

Esc - Exit Status Window,  Space - Update Data, F1 - Help

The information at the top of the window tells you the name

of the node, adapter number and node number being tested as

well as the version of the LANCHECK  software you are using.

The bottom left portion of the screen provides you with

"Traffic and Error Statistics"  which may help you diagnose

a hardware problem with you network.  To the right of this

are the "Adapter Resource Statistics"  which provide

information concerning your NETBIOS configuration.

Once again, do not assume that the values presented in the

example above are the same values that you should see.  What

you should look for instead are values that are out of the

average range for the rest of your adapters.

Traffic and Error Statistics

The field "Minutes Running"  refers to the length of time

since the NETBIOS was run.

Cyclical Redundancy Check (CRC) errors occur when a packet

of data arrives at a node in a corrupt form.  Before a

computer sends a packet,  it mathematically evaluates the

contents of the packet and places this information in it.

The receiving node performs the same calculation on the data

as it receives it then matches it against the results of the

sending node's evaluation.  If the results don't match, the

receiving node asks the sending node to retransmit the

packet.

A high value in the "CRC" field could indicate that an

adapter is not correctly transmitting data.  It might also

indicate faulty cabling or electrical noise is corrupting

the data in transit.

Alignment errors occur when bits of data arrive out of the

8-bit group they were sent in.  These bits must then be

regrouped into their original bytes by the receiving node.

While this does not usually degrade network performance,  a

high value in this field could indicate cabling or hardware

problems.

Network collisions occur when two or more nodes attempt to

access the network simultaneously.  A high value here will

usually indicate a busy network.  In rare cases, however, a

malfunctioning adapter will continuously transmit noise

resulting a higher number of collisions.

The fields, "Good Transmissions"  and "Bad Transmissions"

provide you with statistics for successful and unsuccessful

transmissions.  Busier networks will have more unsuccessful

transmissions than networks with less frequent use.  A high

number of Bad transmissions could indicate a faulty adapter.

"Retransmissions"  occur after unsuccessful transmissions.

Once again,  look for values that are significantly higher

than the rest of the nodes.

A low value for "Good Receives" could indicate that an

adapter is not receiving network data well.  In such cases,

it is best to test this node with only a few other adapters

and make sure this low value is not a result of poor

transmissions from an adjoining network node.

When a node exhausts its supply of buffers, it can no longer

receive data from the network.  The field "Resource Exhaust"

keeps count of each time this happens.  If your node is

continually running out of buffers, it may be advisable to

set your buffers higher to better  accommodate your network

traffic.

If you suspect that there is a hardware problem and an

adapter has a higher than average value in one or more of

the above categories, remove the node from the network and

see how network performance changes.  If performance goes up

dramatically the adapter could be at fault.

Adapter Resource Statistics

The values  given in  this field  can help  you evaluate how

well your  software configuration  meets your network needs.

For example, if you don't have enough Network Control Blocks

(NCBs), to  process data  and commands  in and  out  of  the

adapter, network  performance will  suffer, or  some network

software will not run.

If the field "NCBs Available"  has a value less than eight

you should consider raising the number of NCBs allowed for

this adapter.  You can set this with the NETBIOS command

line switches.  A good way to estimate the number of NCBs you

need is to take the number of log ins you will need and add

eight.

The "Max NCBs Allocated" field informs you how many are

allocated in the NETBIOS command line.  The maximum

number of NCBs available depends on the NETBIOS.

A session is created any time a connection between two nodes

is made or attempted to be made.  If the number given in the

"Active Sessions"  seems out of proportion with what you

feel it should be, check your software to make sure it is

configured for the proper number of sessions.  The maximum

number of sessions available depends on the NETBIOS.

"Max Packet Size"   refers to the largest size data packet

that this adapter can either send or receive.   The Maximum

size of a data packet depends on the size of the buffers set

for the adapter.

The field "Number of Names"  refers to the number of network

names currently being used.  An adapter with a much larger

value than the other adapters may be running a program that

is erroneously creating a large number of names.

To check a second node, press Esc to exit to the previous

window then select the node you want to test and press

ENTER.

You can also check the adapter status of any network node by

pressing Ins while in the first LANCHECK window.  A screen

will appear asking you for the name of the node you want to

test.  Type in the name exactly as it used by the node in

question, then press ENTER.  Normally you will use all

capital letters. The "Adapter Status"   window for the

desired node will appear.  Repeat this process for each

adapter you want to evaluate.  To exit the LANCHECK program

from the main window, press the  Esc key.

#