TESTDISK

THE NEXT GENERATION OF DISK THROUGHPUT BENCHMARKS

(c) COPYRIGHT 1988 BY

COLUMBIA DATA PRODUCTS, INC.

851 WEST S.R. 436, SUITE 1061

P.O. BOX 2584

ALTAMONTE SPRINGS, FL  32714-2584

(407) 869-6700

Welcome to the world of hard disk performance measurement!  Until

recently, measuring  performance of  disk subsystems  for the IBM

PC/XT/AT/PS2 and compatibles has been somewhat over simplified by

the  industry.   Now,  with  the  introduction  of   SST-equipped

controllers, contemporary  benchmarks are  no longer  adequate to

predict  disk  subsystem  performance  in  actual  everyday  use.

Benchmarks for the  next generation of  mass storage systems  for

the  IBM  PC/XT/AT/PS2  and  compatibles  now  require  that more

variables  be  taken  into  account  to  accurately  measure  the

throughput a system can achieve.

ACCESS TIMES

Until now the only  major variable that generally  was considered

was the Average  Disk Head Seek  Time, expressed in  milliseconds

(commonly  referred  to  as  Average  Access  Time).   From   the

standpoint of a disk drive  manufacturer, this is the major  item

which is used to market their products and they are  consistently

touting faster and faster access times.  But the REALLY important

aspects of overall disk system performance are just not shown.

Let's take a disk that has an access time of 20 milliseconds, for

example.   Does  that  mean  that  just  because  it  has  a   20

millisecond access time that it  is fast?  Well, yes and  no.  It

means that it can move the disk head from one spot on the disk to

another  spot  on   the  disk  in   an  overall  average   of  20

milliseconds.   As  far  as  access  times  go,  a 20 millisecond

average access time disk drive is relatively fast.  But it  tells

you nothing of  how fast it  will deliver data  to your computer.

So you  ask, "Well,  then, how  can I  tell when  a disk drive is

fast?"  Read on!!

Disk performance really starts at your hard disk  controller--not

at your disk.  Disk performance is a function of how rapidly data

can be transferred from the hard disk into your computer.  If you

have a disk controller which  can move data at 29,000  characters

per second (which is not uncommon), then it stands to reason that

it  will  take  you  six  seconds  to load your 174,000 character

spreadsheet.  So, whether you have a 20 millsecond hard disk or a

65 millisecond hard disk, it  still takes six seconds to  recover

your spreadsheet!!

DATA RATES

As you  can probably  tell, disk  performance measurement,  which

tells you how long  you are going to  wait for something to  load

into your computer from your hard disk, can best be expressed  in

characters per second.  Since one character is equal to one byte,

and  most  disk  systems  move  data  in  thousands  of bytes (or

characters)  per  second,  the  standard  term  for  data rate is

kilobytes (1024 bytes) per second (KBS).

Some benchmark tests  actually DO report  the data transfer  rate

(in KBS) of  the controller.  They  measure the movement  of data

from the disk to memory.  They only report, however, the  highest

data rate your disk drive can achieve on your computer.  Now,  if

you read closely,  you will notice  that I said  the HIGHEST data

rate your  disk drive  can achieve  on your  computer.  Does this

mean that there is more than one data rate expressed in KBS which

can be achieved by your disk system?  The answer is YES!

WHICH DATA RATES?

Whenever the  disk system  is asked  to move  data from  the hard

drive  into  memory  or  vice  versa,  it takes time for the disk

system to figure out what you are asking for, where it is on  the

disk, and how to move the data.  This time lag is called  command

processing latency.  It is this  initial time lag before data  is

actually moved  to or  from the  disk that  is the  evil of  disk

performance.  Since this time lag  is pretty much a constant,  it

is wise to get  as much data off  of the disk at  one time rather

then to repeatedly ask for data in small blocks.  For example, if

you wanted  to move  327,680 bytes  off the  disk and the initial

command  processing  latency  was  70  milliseconds (70/1000 of a

second) and the 327,680 bytes  was moved in 512 byte  blocks, you

would be asking the disk to move data 640 times and your  command

processing  latency  alone  would  be  44.8  seconds  (640 X .070

seconds)!!

If, however,  you moved  the 327,680  bytes off  the disk  65,536

bytes (64 KB)  at a time,  you would be  asking the disk  to move

data only 5  times and your  command processing latency  would be

35/100's of a second (5 X .070 seconds).  That is 992%  faster!!!

44.8 seconds compared  to a fraction  of a second  and that's not

even counting the time to move the data!!  This is extremely  eye

opening isn't it?

So, you see, it  doesn't appear to be  very wise to ask  the disk

for data in small blocks, now does it?  But this is precisely the

way most application programs (such as Lotus, Dbase, etc) DO move

their  data--512  bytes  at  a  time.   Now,  these  other   disk

performance  testers  I  mentioned  test  the disk by moving data

65,536 bytes at a time.  So, you have absolutely no idea how fast

Lotus,  Dbase,  or  any  of  the  other  application programs can

retrieve data.  All you know, is that IF YOUR PROGRAM MOVED  DATA

64K AT A TIME  OFF THE DISK, THIS  IS HOW FAST IT  WOULD BE!!  In

essence,  then,  to  measure  disk  performance  in 64K blocks is

utterly useless to you if you  really want to know how fast  your

application program can load data.

Your Columbia  Data Products  disk performance  tester, TESTDISK,

doesn't just move data off the disk in 64K blocks.  It moves  the

data in 8 different size blocks:  1/2K, 1K, 2K, 4K, 8k, 16K,  32K

and 64K.  This will give you  a clear idea of how fast  your disk

system really is.  When  you run this on  a disk system which  is

650 KBS at 64K  blocks, you may be  surprised to find that  it is

only 120 KBS (or less) at the 1/2K block rate.  You may even find

that you paid a  premium for a "fast"  disk system and you  never

will see the "speed" because the programs you run cannot  utilize

it.

One last word  about average access  times.  They are  important.

We at Columbia Data Products  do not dispute that.  They  account

for a sizeable portion  of your command processing  latency.  You

should not, however,  pay a premium  for a "fast"  access time if

you  can't  take  advantage  of  it.   This  test  will  help you

determine that.  The last example I wish to give will compare two

hard drives.  The  first hard drive  has a 1  millisecond average

access time and  the other has  a 100 millisecond  average access

time.  The 1 millisecond access time hard drive can move data off

itself and  into the  computer at  100 thousand  bytes per second

(100 KBS).  The 100 millisecond  access time hard drive can  move

data off itself and into  the computer at 900 thousand  bytes per

second (900 KBS).

To move 900  thousand bytes, the  1 millisecond access  time hard

drive takes 9 seconds while the 100 millisecond access time  hard

drive takes only 1 second.  Therefore, the 100 millisecond access

time hard drive is 9 times  faster moving data even though it  is

100 times slower moving its head!  It probably costs a lot  less,

too.  I think you are beginning to see the picture.

RUNNING TESTDISK

To start testdisk, simply type TESTDISK and press  enter.  First,

you must choose whether or not you want to run testdisk in color.

Next, you will be asked whether or not your printer can print the

two  characters  which  are  displayed  on the screen.  A printer

which  can  print  the  IBM  character  set  will have no trouble

printing the displayed characters.  The printer will print out  a

graph of the  test results, if  you desire, at  the completion of

the test.

You will then  be asked for  a DOS drive  letter (C:, D:  etc) to

test.  Next  you will  be asked  how much  data in  Kilobytes you

would like to read.  We recommend a minimum of at least 1000K for

an  accurate  test.   The   test  then  proceeds  to   perform  a

verification  pass  to  insure  that  the  area  being  tested is

error-free.  If it is not, an error will occur and the test  will

stop.  If it is error-free, the test will continue.

The data is then read by two methods--SEQUENTIAL and  REPETITIVE.

The SEQUENTIAL test  will read data  from the drive  the same way

that your application program  will.  The REPETITIVE test,  which

reads data the way most "disk test" programs do, will not reflect

real world performance.  We have included the REPETITIVE test for

comparitive purposes only.

TESTDISK creates a  graph of 16  individual tests, 8  SEQUENTIAL,

and 8 REPETITIVE.   Each individual test  reads data in  only one

block size.  The tests are  run in pairs (one SEQUENTIAL  and one

REPETITIVE) for block sizes of 1/2K, 1K, 2K, 4K, 8K, 16K, 32K and

64K.  In other  words, if you  are reading 1000K  in 1/2K blocks,

you will  access the  disk drive  2000 times  for each individual

test.  For the 1K test, you will access the disk 1000 times, then

500 times for the 2K test, 250 times for the 4K test, and so on.

The  SEQUENTIAL  test  reads  the  disk one sector after another,

beginning with data in sector 1 on the hard drive.  To read  data

sequentially, the hard  drive head reads  all the sectors  on the

first track, then is moved from that track to the next track, and

so on, until the  amount of  data requested  is read.   The first

sequential test shows the  rate for reading the  requested amount

of data in  1/2K blocks, starting  in sector 1.   The second test

also begins in sector 1 and reads in 1K blocks, the third  begins

once again in sector 1 and reads in 2K blocks, and so on.

The REPETITIVE read test is included to show how most other  disk

test  programs  read  their  data--repetitively, 64K  at  a time.

Rather than reading  the data sequentially,  as required by  most

application programs,  data from  the same  spot on  the disk  is

repeatedly read over  and over in  64K blocks.  Reading  the same

data repeatedly and in such large blocks does not accurately test

disk  performance   for  an   application  program.    Our  first

repetitive test  represents the  requested amount  of data  being

read from one spot  on the disk in  1/2K blocks, the second  test

shows the rate for reading the same data from the same spot in 1K

blocks, and so on, up to 64K.

IN SUMMARY

The sequential test results  will indicate your disk  performance

on user programs which call for  data from the disk.  Use of  the

repetitive test will show only what other test programs show.

HAPPY TESTING!!