S C A N C O D E

A Scan Code Display Program

Version 2.2

28 December 1987

SCANCODE allows you to see the codes that are sent to the computer

from the keyboard as the keys are pressed and released.  The codes

are displayed in hexadecimal in the lower window and are converted

by this program and displayed as ASCII in the upper window.  The

BIOS is not used for the ASCII display, so if it is wrong, blame

this program, not the BIOS.

Pressing the ESC key activates a one line menu in the middle of the

display.  You are allowed to toggle the keyboard microprocessor

scan code translation on or off (if you have an AT or compatible),

clear both of the display windows, toggle keystroke capture mode or

exit to DOS.  If you don't want to do any of those things, press the

Enter key.

A word of explanation is necessary about the 8042 toggle function.

Normally, an AT keyboard sends AT scan codes to the computer.  The

8042 keyboard microprocessor translates these scan codes into PC

scan codes and sends them to whoever needs them (normally the BIOS,

but in our case, the SCANCODE program).  In order to see the real

scan codes coming from the keyboard, we must turn off the 8042

translation.  At the same time, the ASCII display is also disabled,

as it will not be the same as for PC scan codes.  Note that

depression of any key with an associated LED will not result in the

LED turning on, as the BIOS (who ultimately gets what comes from the

keyboard) can't recognize AT scan codes.  In summary, you can verify

the AT scan codes, but nothing else.

No matter what mode you leave the 8042 in when you exit the program,

SCANCODE will set translation on.

Capture mode is a method of capturing keystrokes for later analysis.

When it is on, all scan codes are saved in a file called

CAPTURE.DTA.  If the file already exists, it will be appended to,

otherwise it will be created and then appended to.

SCANCODE consists of four executable file; GETSCANS.COM,

RESETINT.COM, SCANCODE.COM and SCNCODES.BAT.  ALWAYS use the

SCNCODES batch file to run SCANCODE, as the routine GETSCANS must be

present before the program will work, and the routine RESETINT must

be run to remove GETSCANS.

DO NOT RUN THIS PROGRAM WITH ANY RAM-RESIDENT SOFTWARE such as

SuperKey, as it will hang up the computer.  When GETSCANS is

running, CTL-ALT-DEL will no longer reset the computer.

SCANCODE is a Shareware program, which is an attempt to provide good

software at a low cost.  You are free to copy the program for all

your friends, but if you find yourself using it on a regular basis,

we ask that you make an $8 contribution to:

Michael Burton Software

15540 Boot Hill Rd.

Hayden Lake, ID  83835

You will receive the latest update of SCANCODE and will be placed on

our mailing list.  If you have any questions about SCANCODE, call

(208)772-9347 after 6 p.m. Pacific time.