TURBO PASCAL JOYSTICK ROUTINES

The IBM joystick port is located at hex 201.  All the information

about both joysticks can be read in the byte at that port.  Each bit reports

on a specific thing, namely:

Bit                 Reports on

0            Joystick A X-Axis Coordinate

1            Joystick A Y-Axis Coordinate

2            Joystick B X-Axis Coordinate

3            Joystick B Y-Axis Coordinate

4            Joystick A Button #1 (this is usually the top button)

5            Joystick A Button #2

6            Joystick B Button #1

7            Joystick B Button #2

To read the status of a button, you simply read the relevant bit.  If

the bit is 1, the button is not pressed; if it is 0, the button is pressed.

Aside from the fact that this relationship seems counterintuitive, reading the

buttons is a simple task, easily accomplished with the Turbo Pascal port array

and a little bit masking.

The stick itself is a different story.  How is it possible to get

enough information from one bit?  The answer is that you get it by repeated

reads of the relevant bit.  The process works like this:  when you output

something--anything--to port 201, the lower four bits are all set to 1

automatically.  (I think of this as "nudging" the port.)  The length of time

it takes each bit to return to 0 depends on the position of the stick.  For

example, if Joystick A is all the way to the left, Bit 0 will bounce back to 0

immediately after a nudge.  If it is all the way to the right, it will take

longer to return to 0.  If it is centered, it will take about half as long to

return to 0 as it does when it is all the way to the right.

You can use the Turbo Pascal port array in a loop to make successive

reads of port 201, but that's very slow.  The successive reads are too far

apart to be very reliable as a direction indicator, and they are woefully

inadequate to use as a position indicator.  (If you don't understand the

distinction between a direction indicator and a position indicator, compare

JOYDEMO2.PAS with JOYDEMO3.PAS.)

Only machine code is fast enough to get a good reading of the joystick

port.  And, of course, Turbo Pascal makes that easy (at least as easy as it

ever can be) with the inline procedure.  READJOY.INC is an inline procedure

written by me to do the job.  I hope you find it useful.

The ARC file that includes this file comprises seven others.  This may

be a bit of overkill, but I thought some might find it helpful to have as much

information as possible.  The files have comments where I thought a bit of

exegesis might be in order.  The eight files are:

JOYSTICK.DOC    This file.

READJOY.INC     The key file, with the inline procedure.

BUTTONS.INC     Functions to read the joystick buttons.

JOYPRSNT.INC    Function to determine whether a joystick is installed.

JOYDEMO1.PAS

JOYDEMO2.PAS

JOYDEMO3.PAS

JOYDEMO4.PAS

I would appreciate any comments or suggestions.

David Howorth

CIS User ID: 71600,521

March 29,1987