58 PRESS KIT

SEPTEMBER 1993

PUBLIC AFFAIRS CONTACTS

For Information on the Space Shuttle

Ed Campion

Policy/Management

Headquarters, Wash., D.C.

James Hartsfield

Mission Operations/EVA

Astronauts

Johnson Space Center, Houston

Bruce Buckingham

Launch Processing

KSC Landing Information

Kennedy Space Center, Fla.

June Malone

External Tank/SRBs/SSMEs

Marshall Space Flight Center, Huntsville, Ala.

Nancy Lovato

DFRF Landing Information

Dryden Flight Facility, Edwards, Calif.

For Information on NASA-Sponsored STS-58 Experiments

Michael Braukus

SLS-2 Program

Headquarters, Wash., D.C.

Kari Fluegel

JSC Experiments

Johnson Space Center, Houston

Jane Hutchison

Animal Experiments

Ames Research Center, Mountain View, Calif.

Catharine Schauer

OARE Experiments

Langley Research Center, Hampton, Va.

Terri Sindelar

SAREX-II

Headquarters, Wash., D.C.

CONTENTS

GENERAL BACKGROUND

General Release

Media Services Information

Quick-Look Facts

Shuttle Abort Modes

Summary Timeline

Orbital Events Summary

Payload and Vehicle Weights

Crew Responsibilities

CARGO BAY PAYLOADS & ACTIVITIES

Spacelab Life Sciences-2 (SLS-2)

Cardiovascular System Experiments

Regulatory System Experiments

Neurovestibular System Experiments

Musculoskeletal System Experiments

EDO Medical Project DSOs

Orbital Acceleration Research Experiment (OARE)

IN-CABIN PAYLOADS

Shuttle Amateur Radio Experiment-II (SAREX-II)

DTO-667 Pilot Inflight Landing Operations Trainer (PILOT)

STS-58 CREW BIOGRAPHIES

John Blaha, Commander (CDR)

Rick Searfoss, Pilot (PLT)

Rhea Seddon,  Mission Specialist 1 (MS1)

Bill McArthur, Mission Specialist 2 (MS2)

David Wolf, Mission Specialist 3 (MS3)

Shannon Lucid, Mission Specialist 4 (MS4)

Martin Fettman, Payload Specialist  (PS)

RELEASE:  93-135

STS-58 SECOND SPACELAB MISSION DEDICATED TO LIFE SCIENCES

The Space Shuttle Columbia mission STS-58 will be the second spacelab

flight dedicated to life sciences research.  Columbia's seven crewmembers will

perform a series of experiments to gain more knowledge on how the human body

adapts to the weightless environment of space.

While in Earth orbit, almost every human physiological system undergoes

some form of adaptation.  Understanding the causes of these changes will aid

NASA in the effort to fly longer missions as well as give researchers insight

into medical problems experienced by indiviudals on Earth.

The STS-58 crew will perform experiments focusing on the cardiovascular,

regulatory, neurovestibular and musculoskeletal systems of the body.  The

experiments performed on Columbia's crew and on laboratory animals, along with

data collected on the SLS-1 mission in June 1991, will provide the most

detailed and interrelated physiological measurements acquired in the space

environment since the Skylab program in 1973 and 1974.

The SLS-2 mission currently is scheduled for launch no earlier than Sept.

10, 1993.  The mission will be flown at an altitude of 153 nautical miles and

at an inclination of 39 degrees to the Equator.

The mission is planned to last 14 days concluding with a landing at

Edwards Air Force Base, Calif. Commander for the flight is John Blaha, Col.,

USAF. Rick Searfoss, Lt. Col., USAF will serve as Pilot. Mission specialists

are Rhea Seddon, M.D.; Bill MacArthur, Lt. Col., USA; David Wolf, M.D.; and

Shannon Lucid, Ph.D. The payload specialist for STS-58 is Martin Fettman,

D.V.M., Ph.D.

Following the STS-58 mission, Columbia will be returned to Kennedy Space

Center, Fla., where the spacelab payload will be removed.  Work then will begin

to prepare Columbia for its next flight, STS-62 -- the United States

Microgravity Payload-2 mission scheduled to fly in February1994.

- end of general release -

MEDIA SERVICES INFORMATION

NASA Select Television Transmission

NASA Select television is available on Satcom F-2R, Transponder 13,

located at 72 degrees west longitude; frequency 3960.0 MHz, audio 6.8 MHz.

The schedule for television transmissions from the orbiter and for mission

briefings will be available during the mission at Kennedy Space Center, Fla;

Marshall Space Flight Center, Huntsville, Ala.; Ames-Dryden Flight Research

Facility, Edwards, Calif.; Johnson Space Center, Houston and NASA Headquarters,

Washington, D.C. The television schedule will be updated to reflect changes

dictated by mission operations.

Television schedules also may be obtained by calling COMSTOR 713/483-

5817.  COMSTOR is a computer data base service requiring the use of a telephone

modem.  A voice recording of the television schedule is updated daily at noon

eastern time.

Status Reports

Status reports on countdown and mission progress, on-orbit activities and

landing operations will be produced by the appropriate NASA newscenter.

Briefings

A mission press briefing schedule will be issued prior to launch.  During

the mission, status briefings by a flight director or mission operations

representative and when appropriate, representatives from the science team,

will occur at least once per day.  The updated NASA Select television schedule

will indicate when mission briefings are planned.

STS-58 Quick Look

Launch Date/Site:	September 1993/Kennedy Space Center - Pad 39B

Launch Time:		TBD

Orbiter:		Columbia (OV-102) - 15th Flight

Orbit/Inclination:	153 nautical miles/39 degrees

Mission Duration:	14 days, 23 minutes

Landing Time/Date:	TBD

Primary Landing Site:	Edwards Air Force Base, Calif.

Abort Landing Sites:	Return to Launch Site - KSC, Fla.

TransAtlantic Abort landing - Ben Guerir, Morroco

Moron, Spain   Zaragoza, Spain

Abort Once Around -  Edwards AFB, Calif.

Crew: 			John Blaha, Commander (CDR)

Rick Searfoss, Pilot (PLT)

Rhea Seddon,  Mission Specialist 1 (MS1)

Bill MacArthur, Mission Specialist 2 (MS2)

David Wolf, Mission Specialist 3 (MS3)

Shannon Lucid, Mission Specialist 4 (MS4)

Martin Fettman, Payload Specialist (PS)

Cargo Bay Payloads:	SLS-2 (Spacelab Life Sciences-2)

In-Cabin Payloads:	SAREX-II (Shuttle Amateur Radio Experiment-II)

DTOs/DSOs:

DTO 312:	ET TPS Performance

DTO 623: 	Cabin Air Monitoring

DTO 663: 	Acoustical Noise Dosimeter Data

DTO 665: 	Acoustical Noise Sound Level Data

DTO 667: 	Pilot In-Flight Landing Operations Trainer

DTO 910: 	Orbital Acceleration Research Experiment

DSO 314: 	Acceleration Data to Support Microgravity Disturbances

DSO 485: 	InterMars Tissue Equivalent Proportional Counter

DSO 603B:   	Orthostatic Function During Entry, Landing and Egress

DSO 611: 	Air Monitoring Instrument Evaluation and Atmosphere

Characterization

DSO 612: 	Energy Utilization

DSO 617: 	Evaluation of Functional Skeletal Muscle Performance

Following Space Flight

DSO 620: 	Physiological Evaluation of Astronaut Seat Egress

Ability at Wheel Stop

DSO 623: 	Lower Body Negative Pressure Countermeasures

DSO 624: 	Cardiorespiratory Responses to Submaximal Exercise

DSO 802: 	Education Activities

DSO 904: 	Assessment of Human Factors

DSO 901: 	Documentary Television

DSO 902: 	Documentary Motion Picture Photography

DSO 903: 	Documentary Still Photography

STS-58 SUMMARY TIMELINE

Flight Day One..........................Ascent

OMS-2 (153 n.m. x 153 n.m)

Spacelab activation

Spacelab Life Sciences-2 activities

Flight Day Two..........................Spacelab Life Sciences-2 activities

to Flight Day Thirteen

Flight Day Fourteen................	Flight Control Systems checkout

Reaction Control System hot-fire

Spacelab stow

Cabin stow

Spacelab deactivation -- first half

Flight Day Fifteen....................	Spacelab final deactivation

Deorbit

Entry

Landing

STS-58 Orbital Events Summary

EVENT		START TIME	VELOCITY CHANGE			ORBIT

(dd/hh:mm:ss)	 (feet per second)		(n.m.)

OMS-2		00/00:40:00		198 			153 x 153

Deorbit 	13/23:23:00		TBD			N/A

Landing		14/00:23:00		N/A			N/A

STS-58 Vehicle and Payload Weights

Vehicle/Payload						Pounds

Orbiter (Columbia) empty and 3 SSMEs			168,713

Spacelab Life Sciences-2				 21,840

Extended Duration Orbiter Pallet			  3,588

SAREX-II						     46

DSOs/DTOs						    547

Total Vehicle at SRB Ignition			      4,519,358

Orbiter Landing Weight					227,395

SPACE SHUTTLE ABORT MODES

Space Shuttle launch abort philosophy aims toward safe and intact

recovery of the flight crew, orbiter and its payload.  Abort modes include:

* Abort-To-Orbit (ATO) -- Partial loss of main engine thrust late

enough to permit reaching a minimal 105-nautical mile orbit with orbital

maneuvering system engines.

* Abort-Once-Around (AOA) -- Earlier main engine shutdown with the

capability to allow one orbit around before landing at Edwards Air Force Base,

Calif.

* TransAtlantic Abort Landing (TAL) -- Loss of one or more main engines

midway through powered flight would force a landing at either Ben Guerir,

Morocco; Moron, Spain; or Zaragoza, Spain.

* Return-To-Launch-Site (RTLS) -- Early shutdown of one or more engines

without enough energy to reach Banjul would result in a pitch around and thrust

back toward KSC until within gliding distance of the Shuttle Landing Facility.

STS-58 contingency landing sites are the Kennedy Space Center, Edwards

Air Force Base, Ben Guerir, Moron or Zaragoza.

STS-51 CREW RESPONSIBILITIES

TASK/PAYLOAD		PRIMARY		BACKUP

SLS-2			Seddon		Lucid, Wolf, Fettman

Spacelab activation	Seddon		Wolf

Spacelab deactivation	Wolf		Seddon

Spacelab systems	Blaha		Searfoss

Experiments:

SLS-2			Seddon		Lucid, Wolf, Fettman

SAREX			McArthur	Searfoss

DTO's/DSO's:

ET Photo		Searfoss

Acoustic Noise		Searfoss

Orbiter Acceleration	Searfoss

Landing Trainer		Blaha

Cabin Air		McArthur

LBNP			Searfoss

Seat Egress/Landing	Fettman

Education Activities	Fettman

Human Factors		Lucid, Fettman

Other Responsiblities:

Photography/TV		Searfoss		McArthur

Earth observations	Searfoss		McArthur

Maintenance-Orbiter	McArthur		Searfoss

Maintenance-Spacelab	Wolf			McArthur

Medic			Wolf			Seddon

EVA			Lucid (EVI)		Wolf (EV2), Searfoss (IV)

SPACELAB LIFE SCIENCES-2 (SLS-2)

In the past 32 years, Americans have been rocketed into space 87 times.

Each time the astronauts' bodies adapted to the unique weightlessness

environment of space and then readapted to Earth's gravity, but still the

mechanisms responsible for that adaptation remain a mystery.

The Spacelab Life Sciences missions seek to solve the mystery.  A series

of comprehensive experiments will fly for the second time on STS-58 and will

give researchers from across the nation access to the most unique laboratory

available to science -- the microgravity environment of space.

In weightlessness, virtually every human physiological system undergoes

some form of adaptation.  The capacity of the cardiovascular system is

diminished.  Muscle and bone density also begin to decrease.  A shifting of the

body's fluids affect the renal and endocrine systems as well as the way the

blood system operates.  And the balance and position sensing organs of the

neurovestibular system must readapt to an environment where up and down no

longer matter.

Spacelab Life Sciences 2 consists of 14 experiments focusing on the

cardiovascular, regulatory, neurovestibular and musculoskeletal systems of the

body.  Eight of the experiments will use the astronaut crew as subjects and six

will use rats.  A broad range of instruments -- some, unique hardware and

others, standard equipment -- will be used for the human subjects throughout

the mission.  Equipment items will include a Gas Analyzer Mass Spectrometer,

rotating dome and a rotating chair, a Body Mass Measuring Device, Inflight

Blood Collection System, Urine Monitoring System, strip chart recorders,

incubators, refrigerator/freezers, a low-gravity centrifuge and an

echocardiograph.

Activities involved with the human experiments on-board Columbia are

managed by the Lyndon B. Johnson Space Center (JSC), Houston. Preflight

baseline data collection with the astronauts will be performed primarily at JSC

with several tests scheduled at the Kennedy Space Center, Fla., just prior to

launch.  Investigators also will perform post-flight tests at the Ames- Dryden

Flight Research Facility, Edwards. Calif.

The rodent experiments are managed by the Ames Research Center, Moffett

Field, Calif. In addition to inflight data and tissue collection, data will be

collected preflight and post-flight for comparison with flight results.  Both

ground-control and flight animals will be part of post-flight studies.

The primary goal of the SLS-2 mission is to conduct experiments in a

variety of disciplines to address important biomedical questions related to

physiological responses to microgravity and subsequent re-adaptation to

gravity.  The science also is constructed to ensure crew health and safety on

missions of up to 16 days in duration.  A third goal of SLS-2 is to demonstrate

the effectiveness of hardware standardization in experiment- to-rack interfaces

for future applications on Space Station.

CARDIOVASCULAR

Throughout the space program, cardiovascular "deconditioning" has often

been observed in spaceflight crews.  This diminished capacity of the

cardiovascular system is evidenced by decreased orthostatic tolerance, or

lightheadedness, upon return to Earth's gravity and is usually accompanied by

increases in resting heart rate and decreases in pulse pressure post- flight.

Measurements of body fluids in microgravity reveal a redistribution of

circulating blood and body water toward the head and neck area.  The fluid

redistribution fools the body into thinking there is too much fluid and results

in a reduction of fluid volume.  This overall shift may influence

cardiovascular parameters such as cardiac output, arterial and venous pressure

and stroke volume.  Upon return to Earth, the cardiovascular system must

readapt rapidly.  This challenges the space-adapted cardiovascular system,

which contains less blood volume than normal and sometimes results in

orthostatic intolerance.

Scientists also believe that microgravity may alter lung function in orbit

and are investigating the effect that weightlessness has on the pulmonary

system, particularly on respiration, blood flow and gas exchange.

The SLS-2 cardiovascular/cardiopulmonary experiments seek to understand

and quantify these changes that occur on orbit and focus both on the acute

fluid shift and the long-term adaptation of the heart and lungs.

Inflight Study of Cardiovascular Deconditioning

Principal Investigator:  Leon E. Farhi, M.D.

State University of New York at Buffalo

Buffalo, New York

Quantifying and identifying the mechanisms and time course of how

astronauts adapt to microgravity and then readjust to the normal gravitational

forces on Earth is the focus of Experiment 066 -- "Inflight Study of

Cardiovascular Deconditioning."

Data from SLS-1 suggest that the cardiovascular adjustment is seen in

total peripheral resistance.  In other words, the resistance of blood flow

through the body and not cardiac output.  Cardiac output stayed elevated

throughout the SLS-1 mission, but total peripheral resistance adapted.  Early

inflight data was not collected on that flight and is needed to enhance the

investigation's findings.

To do this, STS-58 crew members will be measuring cardiac output or the

amount of blood pumped by the heart with each beat at rest and during exercise.

Both activities begin with the subject performing a calibration with the Gas

Analyzer Mass Spectrometer followed by the collection of heart rate, blood

pressure and respiratory gas volumes and concentrations.  Changes to these

parameters will be measured during the exercise portion of the investigation.

Many of the inflight procedures are being performed jointly with

Experiment 294 to efficiently use mission resources and optimize science

return.

Cardiovascular Adaptation to Zero Gravity

Principal Investigator:  C. Gunnar Blomqvist, M.D.

University of Texas Southwestern Medical Center

Dallas, Texas

Experiment 294, "Cardiovascular Adaptation to Zero Gravity," will study

the changes in the cardiovascular system to increase understanding of

microgravity-induced changes in the heart's structure and function in space.

According to Experiment 294 investigators, the experiment's central venous

pressure results from SLS-1 were surprising.  Central venous pressure decreased

in space, refuting the hypothesis that the pressure increases due to the

microgravity-induced fluid shift.  Despite that decrease, heart size increased

as did stroke volume, the amount of blood pumped with each heart beat, and

overall cardiac output when compared to preflight supine values.  These changes

were not predicted by ground-based models and could indicate that there is a

general opening of blood vessels on orbit.

Many of the inflight procedures are being performed jointly with

Experiment 066 to efficiently use mission resources and optimize science

return.  In addition to cardiovascular measurements -- heart rate, blood

pressure and respiratory gas volumes taken during rest and exercise, the

central venous pressure will be measured during the first few hours of flight

by a venous catheter inserted into the arm of a crew member preflight.

Echocardiograph data also will be taken, and leg blood flow and volume will be

measured using a leg cuff device.

Pulmonary Function During Weightlessness

Principal Investigator:  John B. West, M.D., Ph.D.

University of California

San Diego, Calif.

Investigating the effects of microgravity on the various aspects of

pulmonary function is the goal of Experiment 198, "Cardiovascular Adaptation to

Zero Gravity."

Prior to SLS-1, scientists believed that lung ventilation would be much

more even in space than on Earth. Data from the first mission, however,

revealed that the lung function improved only by about half as much as was

expected.  The investigators hope to identify the mechanisms that are

responsible for this unexpected trend on SLS-2.

The experiment protocol calls for astronauts to participate in a series of

breathing tests designed to examine the distribution and movement of blood and

gas within the pulmonary system and how these measurements compare to normal

respiration in Earth's gravity.

By measuring gas concentrations, the flow of gas through the lungs into

the blood stream and rate of blood flow into the lungs in space, investigators

hope to isolate irregularities seen on Earth. Non-invasive breathing exercises

involve the inhalation of gas mixtures including oxygen, nitrogen and other

trace gases.

Experiment 198 is a reserve experiment and will be conducted in flight if

crew time and resources become available.

REGULATORY SYSTEM

Investigations of regulatory physiology in space include studies of both

the renal/endocrine and hematological systems.

The amount of fluids and the pressures inside veins and arteries is well-

regulated by the kidneys and hormones of the renal/endocrine system.  On Earth,

gravity affects the distribution of fluids inside the body by pulling the

various body fluids down toward the feet.  But in space, fluids redistribute

upwards toward the chest and the head.  This perceived increase causes multiple

physiological changes in the kidneys and associated fluid regulating hormones

in the cardiovascular system and in the blood system.

The SLS-2 regulatory physiology experiments investigate the theory that

the kidneys and endocrine glands adjust the body's fluid regulating hormones to

stimulate an increase in fluid to be excreted.  Over a longer period of time,

the kidneys and hormones establish new levels of salts, minerals and hormones

appropriate for the reduced fluid volume.  The fluid shift also impacts the

blood system initially by a decrease in the plasma volume.

Another effect of spaceflight is a decrease in red blood cells which are

responsible for carrying oxygen to the tissues.  Investigators hope to better

understand the mechanisms behind these changes after SLS-2.

Fluid-Electrolyte Regulation During Spaceflight

Principal Investigator:  Carolyn Leach, Ph.D.

Lyndon B. Johnson Space Center

Houston

Experiment 192, "Fluid-Electrolyte Regulation During Spaceflight," makes

detailed measurements before, during and after Shuttle missions to

determine immediate and long-term changes in kidney function; changes in water,

salt and mineral balance; shifts in body fluids from cells and tissues; and

immediate and long-term changes in levels of hormones which effect kidney

function and circulation.

Both the acute (short-term) and adaptive (long-term) phases of the body's

adjustment to spaceflight will be studied for the experiment.  Investigators

will focus on the acute phase during the immediate inflight period by placing

emphasis on the circulatory system, hormonal and kidney involvement in the

initial loss of fluids.

For the adaptive phase, the specific mechanisms related to the body

developing a new stable level in response to these fluid shifts will be

examined.

The test protocols require crew members to collect urine samples

throughout the flight.  Body weight is measured daily and a log is kept of all

food, fluids and medication taken in flight.  Chemical tracers will be used to

measure the rate and loss of body water and to determine changes in blood

plasma volume and extracellular fluid.

Regulation Of Blood Volume During Spaceflight

Principal Investigator:  Clarence P. Alfrey, M.D.

Baylor College of Medicine

Houston

Experiment 141, "Regulation of Blood Volume During Spaceflight," will

study changes in the volume of blood in response to spaceflight.  Blood in the

body is contained in the heart, arteries, veins and capillaries.  The amount of

blood contained within specific spaces is determined in part by gravity.  Since

the force of gravity during spaceflight is greatly reduced, the space available

to contain blood is decreased.  In response, the body senses that the amount of

blood is too great.

Previous spaceflight crews have consistently exhibited decreases in the

volume of plasma, the liquid part of blood.  An experiment on SLS-1 showed a

similar decrease in rats.

Because less space is available to blood during spaceflight, the body

slows down the rate at which it produces red blood cells.  This flight

experiment should provide important information about the role of growth

factors and hormones in the maturation and release of red blood cells from the

bone marrow.

Radioactive tracers will be injected into the rats to permit monitoring of

changes in the blood when blood samples are taken preflight, inflight and

post-flight.  From these samples scientists can determine the volume of red

blood cells and plasma, the length of survival of the red blood cells, the

number of red blood cells being made during spaceflight and the number released

into the blood.

Understanding how the hormone erythropoietin controls the formation and

maturation of red blood cells has important implications on Earth.

Erythropoietin is a very expensive treatment for anemia -- a common consequence

of AIDS and kidney failure.  With increased knowledge about how this hormone

works, more cost-effective erythropoietin treatment schedules could be

developed.  Spaceflight affords an opportunity to study the altered rate of

production and release of red blood cells from the bone marrow in non-disease

states.

Regulation Of Erythropoiesis In Rats During Space Flight

Principal Investigator:  Albert Ichiki, Ph.D.

University of Tennessee Medical Center

Knoxville, Tenn.

Astronauts on previous Space Shuttle missions have exhibited anemia or

decreased red blood cell mass.  The causes of this change, which are not known,

are the subject of Experiment 012.  A decrease in red blood cell production may

play a role.

A decrease in red blood cell mass so far has not been life-threatening

during short duration flights.  There are concerns, however, about what might

happen on long-term flights and how this change may affect crew performance.

This experiment, along with the hematology experiments (rat and human) of

Dr. Clarence Alfrey, will determine what processes are occurring in space

anemia.  They also will determine whether the changes in the blood system of

the rat during spaceflight are the same as in humans.

For the first time, scientists will be able to study blood samples

collected at various points during the flight.  Other samples will be taken

before and after flight.  SLS-2 will provide the opportunity to investigate the

consequences of the body's adaptation to microgravity and subsequent

readjustment to normal Earth gravity.

By studying the mechanisms of space anemia, scientists can learn the basic

regulatory physiology of red blood cell production in a controlled environment.

The principal investigator will compare measurements of red blood cells, white

cells, platelets and hemoglobin from flight rats with those from crew members

and from ground-control rats.  Post-flight analyses of the bone marrow, spleen

and thymus from flight and ground-control rats will provide additional

information about how microgravity affects the blood system.

The health care industry will be the largest benefactor of information

from these experiments.  Information may be useful in learning to control or

prevent harm to people on Earth with anemia or other blood diseases.

Influence of Spaceflight on Erythrokinetics in Man

Principal Investigator:  Clarence Alfrey, M.D.

Baylor College of Medicine

Houston, Texas

One of the most consistent findings from spaceflight research is the

decrease in circulating red blood cells, or erythrocytes, and the subsequent

reduction in the blood's oxygen carrying capacity.  Experiment 261, "Influence

of Spaceflight on Erythrokinetics in Man," studies the mechanisms which may be

responsible for this decrease, including the effect of spaceflight on red blood

cell production rate and the role of changes in body weight and plasma volume

on red blood cell production.

The experiment will measure specific factors of red blood cell function,

particularly erythropoiesis or the production of red blood cells, which may be

altered during weightlessness.

During the mission, crew members will measure iron uptake by injecting

tracers followed by timed blood draws.  The blood then will be centrifuged and

stored in the Spacelab freezer for post-flight analysis.

During SLS-1, investigators saw a decrease in red blood cell mass, plasma

volume and erythropoietin.  The life span of the red blood cells did not change

and hemolysis, or the disintegration of red blood cells, did not occur.  This

indicates that the decrease in red blood cell mass was do to a suppressed red

blood cell production in flight.  The SLS-2 subjects will help verify the

preliminary findings of the study.

NEUROVESTIBULAR

Neurovestibular changes related to equilibrium and body orientation affect

astronauts early in flight probably more than any other physiological changes.

The awareness of body orientation on Earth is attributed, in part, to the

detection of gravity by the otolith organs in the inner ear.  Gravity sensors

in the joints and touch sensors in the skin also are involved, and the eyes

contribute by sensing the body's relationship to other objects.  In space,

however, the weightless environment no longer corresponds with the visual and

sensual cues set to the brain, causing disorientation.

Space motion sickness may result from this disorientation, and although

astronauts adapt within a few days, investigators are working to better

understand and counter these negative effects.  A similar disorientation of the

balancing organs can occur when crew members readapt to Earth's gravity after

landing.

The SLS-2 neuroscience investigations seek to document both physical

vestibular changes and perception changes and to investigate the mechanisms

involved.  Investigators also hope to identify countermeasures to alleviate the

effects of space motion sickness.

A Study Of The Effects Of Space Travel On Mammalian Gravity Receptors

Principal Investigator:  Muriel Ross, Ph.D.

NASA Ames Research Center

Moffett Field, Calif.

The neurovestibular system, which helps animals maintain balance, is very

sensitive to gravity.  Experiment 238 will continue research begun on SLS-1.

This flight experiment showed that gravity sensors in adult rats adapt to a

novel environment by changing the number, type and groups of synapses.

Synapses are gaps between nerve cells.  The SLS-2 flight experiment should

uncover the precise nature of this adaptation and the structural changes that

may occur within the inner ear in response to the microgravity of space.

This experiment will examine how gravity influences the development,

organization and maintenance of the mammalian gravity receptors.  It also will

study how these gravity-sensing endorgans can adapt to an altered gravitational

environment provided the change falls within a range that is non-lethal to the

functioning system and how rapid is the speed of readaptation to Earths

gravitational field.

If we are to learn the effects of near-zero gravity on gravity receptors

and the recovery from these effects, it is essential that sample collection and

fixation of the inner ear tissue be completed during flight.  Scientists will

look at the implications of spaceflight on the structure of gravity receptors

using the rat maculas (inner ear sensory hair cells) as the model for human

sensors.  Gravity receptors are sensitive to linear accelerations (movements)

and gravitational force.  Complete and accurate knowledge of the systems

anatomy is essential to understanding the functional basis of vestibular

involvement in disequilibrium, including space motion sickness.

The results of this study should have clinical applications for people

suffering from motion sickness or from vestibular disorders, which may lead to

falls and dizziness.  This health-related spin-off is in addition to the study

of adaptation to microgravity and the possibility of developing countermeasures

based on anatomical, physiological and behavioral findings.

These results also will help shape the future of physiological experimentation

on gravity receptors in space.

Vestibular Experiments in Spacelab

Principal Investigator:  Laurence R. Young, Sc.D.

Massachusetts Institute of Technology

Cambridge, Mass.

A battery of closely-related tests will serve as the basis of a joint

U.S./Canadian research program to study space motion sickness and changes in

the vestibular system in space.

Experiment 072, "Vestibular Experiments in Spacelab," consists of six

different tests to assess sensory-motor adaptation.  A rotating chair will be

used to test ocular deviation and vestibulo-ocular reflex, in other words the

changes in reflexive eye motions.  One chair protocol requires that the test

subject be rotated about a vertical axis and stopped suddenly.  A second

procedure requires the subject to pitch his or her head forward after the chair

is stopped.

A rotating dome is used in another test.  When a crew member sees the

dot-patterned dome rotating in one direction, he/she senses that he/she is

moving in the opposite direction.  The subject uses a joystick to indicate the

perceived direction and velocity of rotation.

In the awareness of position task, crew members will view various targets

on a visual screen and then, with their eyes closed, point to them with a light

pointer.  This will allow investigators to quantify differences in crew

members' perceived relationships of their bodies to their environments in

spaceflight versus on Earth.

The otolith-spinal reflex, or the reflex that causes one to catch oneself

when sensing a fall, will be tested in the "drop" experiment.  For the study, a

crew member will don a harness with bungee cords attached to the floor and hang

from a suspended T-handle. When the T-handle automatically releases and drops

the crew member, electromyogram electrodes will measure the muscle activity to

determine the relationship between the nervous system and muscle response in

space.

Crew members also will wear an Acceleration Recording Unit to measure head

movements, both natural and exaggerated, and record any space motion sickness

symptoms on a pocket voice recorder.

A variety of pre- and post-flight tests will enhance the information

collected on orbit.

Astronaut Science Advisor

The Astronaut Science Advisor (ASA) is a computer-based intelligent

assistant designed to help astronauts work more efficiently and improve the

quality of space science.  The first flight prototype will be operational on

SLS-2.

The ASA will support the Rotating Dome Experiment, which measures how the

visual and vestibular (balance) systems interact and how this interaction is

affected as humans adapt to microgravity.

Developed by NASA's Ames Research Center and the Massachusetts Institute

of Technology, the ASA addresses many of the problems of conducting experiments

in space.  It uses a Macintosh PowerBook computer and a combination of

commercial and NASA-developed software.

Two critical resources in flight experiments are crew time and the limited

ability of an Earth-bound scientist to correct problems or follow new leads as

the experiment unfolds in space.

The ASA can help with both of these problems.  It helps astronauts

increase their productivity and improve the scientific quality of the data they

collect.  The ASA uses detailed knowledge of the experiment to provide the

astronaut with observations about the quality and importance of the data as it

is collected inflight.

The ASA performs four major functions: diagnosis and trouble-shooting of

experiment equipment, data collection, protocol management and detection of

interesting data.  It keeps track of the time spent on the experiment and can

generate potential new protocols that could be used to optimize the course of

the experiment.

The system should significantly enhance the crews ability to perform

science experiments.  It also should reduce their reliance on air-to-ground

communications.

MUSCULOSKELETAL

In microgravity, the body's bones and muscles are not used as extensively

as they are on Earth. As a result, researchers have seen a decrease in the mass

of both during spaceflight.

Human muscle atrophy has been noted frequently among returning astronauts

and can be characterized by a loss of lean body mass, decreased muscle mass in

the calves and decreased muscle strength.  Despite an adequate protein intake,

the effects of spaceflight appear analogous to those of the fasting state when

muscle protein is broken down into its constituent amino acids.

Researchers also have identified a progressive loss of skeletal mass in

microgravity.  This is associated with changes of calcium homeostasis as is

evidenced by increased urinary and fecal excretion of calcium.  Efforts to

avoid the loss of skeletal density through exercise have been only partially

successful, and researchers have not been able to reverse calcium and nitrogen

loss.

On return to Earth after short-duration missions, these responses are

shown to be reversible, but the effects on muscles and bones during long-

duration missions yet are not well known.  The SLS-2 studies will provide more

information about this complex system.

Protein Metabolism During Spaceflight

Principal Investigator:  T. Peter Stein, Ph.D.

University of Medicine and Dentistry of New Jersey

Camden, N.J.

Experiment 120, "Protein Metabolism During Spaceflight," will investigate

changes in protein metabolism which lead to a decrease in muscle mass on orbit.

Previous studies of nitrogen balance during spaceflight have reported a

persistent negative nitrogen balance.  Normal metabolic processes establish a

balance between protein synthesis (build up) and protein catabolism (break

down).  To understand the protein metabolic changes, it is necessary to measure

both the synthetic and catabolic rates to determine how each contributes to the

decrease in muscle mass.

Data collected during SLS-1 suggests that the decrease of muscle mass is

similar to a stress response which is characterized by an increase in both

protein synthesis and protein breakdown, with a larger increase in the

breakdown rate.  SLS-2 tests will focus on measuring the levels of fibrinogen,

a blood protein essential to coagulation.

Inflight procedures require crew members to ingest an amino acid labeled

with a non-radioactive isotope of nitrogen which can be used to track protein

metabolism in the body.  Approximately 10 hours later, urine, saliva, and blood

samples will be collected and frozen for post-flight analysis.  These samples

will be compared to baseline samples to determine the rates of protein

synthesis and catabolism.

Effects of Zero Gravity on the Functional and Biochemical Properties of

Antigravity Skeletal Muscle

Principal Investigator:  Kenneth M. Baldwin, Ph.D.

University of California

Irvine, Calif.

Experiment 127 will examine how the function and biochemistry of skeletal

muscles changes when exposed to the microgravity of space.  Previous research

has shown that exposure to microgravity causes a loss of muscle mass or size.

This occurs chiefly in muscles used primarily for normal activities such as

standing and walking.  The goal of this flight experiment is to determine how

this atrophy process impacts the strength and endurance of various muscles used

by rodents for standing and walking.

One explanation is that in microgravity there is not enough stress or

tension on the muscles to maintain adequate levels of certain proteins and

enzymes.  These proteins and enzymes enable cells to use oxygen to convert

nutrients into energy.  Under conditions of less gravitational stress, protein

activity also decreases.  Muscles become more dependent on glycogen stored in

the liver and muscles for energy.  As the body metabolizes glycogen, muscle

endurance decreases.

The principal investigator hypothesizes that the muscle will lose its

strength and ability to perform repetitive contractions when tested after

spaceflight.  The principal investigator will measure and compare muscle

strength, muscle power, muscle performance capabilities and the ability of

muscle to sustain work.  He will compare results from flight animals with those

from ground-control animals.

The results of this project should provide insight into the extent that

muscle atrophy induces altered muscle performance.  The experiment can be

extended in the future to determine how this altered function can be prevented

or corrected.

The problem of muscle atrophy associated with spaceflight is important

because it may represent an accelerated process similar to what is seen in

individuals as they get older.  Aging involves a gradual atrophying process in

the muscles, due largely to progressive inactivity.  By identifying the causes

and extent of atrophy and by developing appropriate counter measures for

preventing muscle atrophy, scientists may be able to help a broad spectrum of

population to live more productive lives with greater vitality.

The Effects Of Microgravity On The Electron Microscopy, Histochemistry

and Protease Activities Of Rat Hindlimb Muscles

Principal Investigator:  Danny A. Riley, Ph.D.

Medical College of Wisconsin

Milwaukee, Wis.

Humans exposed to long-duration spaceflight can experience muscle

weakness, fatigue, post-flight soreness and faulty coordination, despite

strenuous inflight exercise.  Experiment 303 will analyze the effects of

spaceflight on the muscles from rat hindlimbs.  Previous spaceflight missions

have shown that rat skeletal muscles exhibit pathological changes, as well as

the simple atrophy typically associated with decreased use.  The changes of

greatest concern include tears in muscle fibers, blood clots in capillaries and

abnormal tissue swelling.

After almost 2 weeks in space, rats showed a nearly 40 percent shrinkage

in the fibers of skeletal leg muscles.  Previous studies on rats also suggest

that muscle fiber shrinkage and death are progressive.  Similar changes in

humans could mean that with longer periods of living in space, astronauts could

potentially risk irreversible loss of muscle strength.

This flight experiment will compare the atrophy rates of muscles used

primarily to oppose gravity with muscles used for movement.  It also will

examine tissues for physical and chemical changes that may be related to the

stress of launch, microgravity, re-entry and readaptation to Earth's gravity.

Returning to Earth's gravity force probably produces muscle fiber tearing,

blood clotting, tissue swelling and increased tissue necrosis.

These results will aid the development of inflight countermeasures.  They

may help prevent damage of the muscular system during adaptation to

microgravity and following return to Earth. These findings also will help

humans on Earth who are confined to bed for extended periods.  Such people

frequently experience muscle deterioration and slow recovery once they resume

movement.

Pathophysiology of Mineral Loss During Spaceflight

Principal Investigator:  Claude D. Arnaud, M.D.

University of California

San Francisco, Calif.

Experiment 305, " Pathophysiology of Mineral Loss During Spaceflight,"

will help researchers identify mechanisms causing changes in calcium

homeostasis, or function, in microgravity.  Those changes include increase

calcium excretions and bone mineral loss.

According to the SLS-1 results, bone-dissolving cells called osteoclasts

work at a higher rate than bone-building cells called osteoblasts.  This trend

causes bone and mineral abnormalities and revealed that ionized calcium in the

blood reached levels that are considered clinically abnormal on Earth. The

SLS-2 activities will include a study of the role of active vitamin D

metabolites and calciotropic hormones, such as parathyroid hormones, by

measuring their circulating levels.  A calcium absorption study examines the

role of intestinal calcium handling.

Blood and urine samples will be drawn for both studies.  The absorption

study also will require crew members to ingest a calcium isotope tracer

followed by an injection of a different calcium isotope tracer.  The blood

samples following the tracer ingestion will be refrigerated, centrifuged and

then frozen for post-flight analysis.

Bone, Calcium and Space Flight

Principal Investigator:  Emily R. Morey-Holton, Ph.D.

NASA-Ames Research Center

Moffett Field, Calif.

On Earth, the force imposed by gravity causes bone in growing animals to

increase simultaneously in mass and strength.  As a result, larger animals have

larger and stronger bones.  During spaceflight, however, bone strength does not

increase in proportion to bone size as it would on Earth. The purpose of

Experiment 194 is to delineate the early changes that occur in both weight

bearing and non-weight bearing bone tissues in growing rats and to relate these

changes to alterations in calcium metabolism.  The experiment allows more

precise calculations of the length of flight time required to significantly

inhibit bone formation in juvenile rats.

Natural calcium in the diet will be replaced with a single stable non-

radioactive isotope of calcium (40Ca).  Since diet and bone are the major

sources of calcium in the body, any calcium other than 40Ca found in blood,

urine or feces must come from bone.

This study also will determine whether the production of new bone cells

decreases or ceases entirely and whether bone mineralization is restored to

preflight levels following spaceflight.  Finally, it will determine the total

amount of bone formed and the total amount of bone lost at intervals during the

flight.  Bone turnover also will be measured in the crew during this mission.

In space or during restricted exercise, changes in bone structure, coupled

with the decrease in the mass of the gravity-dependent muscles, make movement

difficult.  Because of this instability, individuals may be prone to bone

fractures when exercise or structural loading is increased (i.e., return to

Earth from space).

We need to find out what components of the bone structure are changed, the

extent to which they change, the impact of the changes on bone strength and if

it is necessary to prevent the changes from occurring.  Only then can we

develop techniques to inhibit potentially damaging changes in bone structure

during restricted physical use.

Research Animal Holding Facility

NASA Ames Research Center,

Moffett Field, Calif.

The rodent Research Animal Holding Facility (RAHF) is a general-use

facility for housing rodents in life sciences experiments in the Spacelab. It

is a self-contained unit providing food, water, temperature and air-flow

control, waste management and lighting for the animals on board.  It can

accommodate 24 400-gram rodents.

The rodent RAHF contains 12 cages that are removable for easy access to

the animals.  A cage can contain up to two animals, one in each of two

compartments measuring 4 by 4-1/4 by 10 inches.  Each cage contains a waste

management system plus individual feeders and watering lixits.  Food and water

are available ad lib.

Additional control can be exercised over temperature and light/dark

cycles.  Protection against cross-contamination between crew and animal is

provided through bacteriological isolation.  An environmental control system is

mounted on the back of each cage module to circulate conditioned air through

the cages.

Cage temperature, animal activity, lighting, humidity and water

consumption can be monitored by the ground crew and by the astronaut crew

onboard.  Food consumption on orbit is monitored by the crew.  The rodent RAHF

flew successfully on the 1991 Spacelab Life Sciences -1 mission.  Two rodent

RAHFs will fly on the SLS-2 mission.

General Purpose Work Station

NASA Ames Research Center,

Moffett Field, Calif.

The General Purpose Work Station (GPWS) is an enclosed, retractable

cabinet providing broad-range support for general laboratory operations in

isolation from the Spacelab environment.  The GPWS can support animal

experiments, biological sampling and microbiological experimentation.  It also

can serve as a closed environment for containment while routine equipment

repair or other inflight operations are performed.

The GPWS provides the working space and accommodates the laboratory

equipment and instruments for many life sciences investigations.  The unit is

self-contained, except for power, data and cooling interfaces with the

Spacelab. Laboratory workbench accommodations, including airflow, power and

lighting, are provided in a rack-mounted retractable cabinet.  The cabinet has

a full-size front door, which allows large experimental hardware to be mounted

in the cabinet interior during flight.  In addition, two crew members may

simultaneously perform tasks inside the cabinet through ports on the front and

side of the cabinet.

The GPWS was flown successfully on the first Spacelab Life Sciences

mission in 1991 and on the Spacelab-J mission in 1992.

EDO Medical Project DSOs

Supplementing the Spacelab Life Sciences 2 Investigations, a series of

detailed supplementary objectives (DSO) will provide additional information for

on-going medical studies that support the Extended Duration Orbiter (EDO)

Medical Project.

The EDO Medical Project is designed to assess the impact of long duration

spaceflight (10 or more days) on astronaut health, identify any operational

medical concerns and test countermeasures for the adverse affects of

weightlessness on human physiology.

For STS-58, the Medical Sciences Division of the Johnson Space Center,

Houston, is sponsoring seven DSOs that support the project.  Most of the

studies have been flown on previous Shuttle missions.

Three of the tests will take place inflight - DSO 611, "Air Monitoring

Instrument Evaluation and Atmosphere Characterization;" DSO 612, "Energy

Utilization;" and DSO 623, "Lower Body Negative Pressure (LBNP)

Countermeasures." The others will occur before and/or after the mission.

The LBNP activity employs a bag in which a vacuum can be created.  The bag

encases the lower body and seals at the waist.  By lowering the pressure within

the bag, the subject's body fluids are drawn into his lower extremities,

mimicking the natural fluid distribution that occurs on Earth. This conditions

the cardiovascular system for the fluid shift that occurs upon re-entry and

improves orthostatic tolerance.

For the LBNP protocols, Commander John Blaha and Mission Specialist Bill

McArthur will serve as the subjects and Pilot Rick Searfoss will serve as the

operator.  Short 45-minute ramp protocols are scheduled for both subjects on

Flight Days 3, 8, and 11, followed by a 4-hour soak protocol on Flight Day 14.

DSO 611 is designed to evaluate and verify equipment for collecting the

microbial contaminate level of the orbiter air.  This is done several times

during the mission using a device that resembles a large flashlight.  DSO 612

will assist researchers in determining the actual caloric requirements for

spaceflight.  Crew members will collect urine and saliva samples as well as

keep a log of all fluid and food intake.  Measurements also will be taken on

astronaut's blood glucose levels.

The post-flight DSOs include 603B, "Orthostatic Function During Entry,

Landing and Egress;" 617, "Evaluation of Functional Skeletal Performance

Following Space flight;" 620, Physiological Evaluation of Astronaut Seat Egress

Ability at Wheel Stop;" and DSO 624, "Cardiorespiratory Responses to Submaximal

Exercise."

DSO 603B documents the relationship between mission duration and changes

in orthostatic function of crew members during the actual stresses of landing

and egress from the seat and crew cabin.  This requires crew members to

instrument themselves with Holter monitors and automated blood pressure

monitors prior to donning their launch and entry suits.  Data from the monitors

will be recorded as will comments from the crew members during the operations.

DSO 620 also looks at changes in crew members' ability to stand following

landing but uses a video camera attached to the middeck lockers to document

changes in sway and balance.

Determining the change in a crew member's response to submaximal exercise

(about 85 percent of the individual's predetermined maximal exercise range) is

the goal of DSO 624, while DSO 617 uses isokinetic equipment and

electromyography to determine skeletal muscle strength, endurance and power.

In both studies, the post-flight data will be compared to baseline measurements

taken before the mission.

ORBITAL ACCELERATION RESEARCH EXPERIMENT (OARE)

The main goal of the experiment is to measure the Shuttle's aerodynamic

forces (drag) in orbit and during the early stages of reentry.  The OARE sensor

is capable of discerning accelerations as small as one-billionth of the Earth's

surface gravitational acceleration (i.e. 1:109).

OARE is designed to calibrate itself on-orbit so that absolute values of

these low accelerations can be measured.  All previous accelerometers onboard

the Shuttle depended upon ground calibrations.  This, of course, is done in a

1-g field on Earth and past experience has shown that, for the level of

precision required for the OARE objectives, ground calibrations are not

adequate.

The OARE sensor is mounted on top of a calibration-table platform which is

free to rotate.  The calibration system allows small drifts and offsets to be

measured directly in flight.  For example, the table can be spun at a precision

rate to generate a known artificial gravity in orbit from which the output

signals can be scaled.

The OARE equipment maiden flight was on STS-40 in June 1991.  OARE was

operated for about 3.5 days on orbit.  However, some equipment problems were

present and although much information was obtained, the flight data was

partially compromised by an equipment malfunction.  The problems were isolated

after the flight, and the equipment has been repaired and has flown a second

time on STS-50 (June 1992) where the instrument operated continuously for 14

days in orbit.

This was the first time insitu accelerometer calibrations were performed

on the orbiter, and the data are of excellent quality.  Although the entire 14

days of data has not been fully analyzed, the data has been sought by other

investigators involved with microgravity experimentations on the flight.  The

overall flight plan for OARE on STS-50 is about the same as on STS- 58.  The

plan is to operate the equipment over the entire 13 days in orbit.  This means

the OARE flight computer is pre-programmed to take into account the duration in

orbit so that the flight data would fit into its 4 megabyte memory storage.

The OARE pitch, yaw and roll maneuvers, performed on STS-40 and STS-50,

also will be performed on this flight.  These maneuvers are an important source

of information about the Shuttle physical characteristics such as vehicle

center of gravity.  In addition, the maneuvers can be used to check the

automatic calibration procedures.  For STS-58, the maneuvers will be performed

on flight days 2, 7 and 13.  In addition, the gravity gradient, turn-drag

maneuver also will be performed on flight day 2.

The OARE flight hardware consists of 4 electronics boxes and a table

assembly with a container mounted on its surface.  This container houses the

electrostatic-suspended proof-mass accelerometer sensor.  The whole system

weighs about 107 lbs and is 17x13x41 inches and requires about 110 watts of

power.

The OARE is manifested as a complex DTO and is mounted on a special keel

bridge which spans bay 11 of the orbiter.  This is essentially the floor of the

orbiter payload bay, near the aft end of the bay.

The Principal Investigator for OARE is R.C. Blanchard, NASA Langley

Research Center, Hampton, Va., and the Project Manager is R. Giesecke, NASA

Johnson Space Center, Houston.

Shuttle Amateur Radio EXperiment (SAREX)

Students in the United States and France will have a chance to speak

via amateur radio with astronauts aboard the Space Shuttle Columbia during

STS-58. Ground-based amateur radio operators ("hams") will be able to contact

the Shuttle through automated computer-to-computer amateur (packet) radio link.

There also will be voice contacts with the general ham community as time

permits.

Shuttle Pilot Richard A. Searfoss (license pending) and mission

specialist William S. McArthur, Jr. (KC5ACR) and payload specialist Martin J.

Fettman (KC5AXA) will talk with students in 16 schools in the United States and

in France using "ham radio."

Students in the following schools will have the opportunity to talk

directly with orbiting astronauts for approximately 4 to 8 minutes:

*  Russellville H.S., Russellville, Ariz. (K5PXP)

*  Lloyd Ferguson Elementary, League City, Texas (KB5UFJ)

*  Eastern Heights Jr. H.S., Elyria, Ohio (N8AM)

*  Bloomfield Elementary, Bloomfield, Mo. (N0UOP)

*  Carl Hayden Community H.S., Phoenix, Aruz. (N7UJJ)

*  Sycamore Middle School, Pleasant View, Tenn. (AC9R)

*  Alamo Heights Junior School, San Antonio, Texas (WA5FRF)

*  Nashua H. S., Nashua, N.H. (N1NHS)

*  Meyzeek Middle School, Louisville, Ky. (N4OKX)

*  Webber Junior H.S., Fort Collins, Colo. (N0LHW)

*  Red Springs H.S., Red Springs, N.C. (W4MZP)

*  Ernest Elliott School, Munster, Ind. (AJ9N)

*  Space Center Intermediate School, Houston (KA5GLX)

*  St. Barnabas Episcopal School, Houston (N5NYD)

*  Gardens Elementary School, Pasadena, Texas  (N5VSP)

*  Lycee Gaston Febus, Pau, France (FE1OBV)

The radio contacts are part of the SAREX project, a joint effort by

NASA, the American Radio Relay League (ARRL), and the Amateur Radio Satellite

Corp. (AMSAT).

The project, which has flown on 10 previous Shuttle missions, is

designed to encourage public participation in the space program and support the

conduct of educational initiatives through a program to demonstrate the

effectiveness of communications between the Shuttle and low-cost ground

stations using amateur radio voice and digital techniques.

Information about orbital elements, contact times, frequencies and crew

operating schedules will be available during the mission from NASA, ARRL (Steve

Mansfield, 203/666-1541) and AMSAT (Frank Bauer, 301/ 286-8421).  AMSAT will

provide information bulletins for interested parties on INTERNET and amateur

packet radio.

The ham radio club at the Johnson Space Center(JSC), (W5RRR), will be

operating on amateur short wave frequencies, and the ARRL station (W1AW) will

include SAREX information in its regular voice and teletype bulletins.

There will be a SAREX information desk during the mission in the JSC

newsroom.  Mission information will be available on the computer bulletin

board.  To reach the bulletin board, use JSC BBS (8 N 1 1200 baud): dial

713/483-2500, then type 62511.

The amateur radio station at the Goddard Space Flight Center,

Greenbelt, Md. (WA3NAN), will operate around the clock during the mission,

providing SAREX information and retransmitting live Shuttle air- to-ground

audio.

STS-58 SAREX Frequencies

Routine SAREX transmissions from the Space Shuttle may be monitored on

a downlink frequency of 145.55 MHz. The voice uplink frequencies are:

144.91 MHz

144.93

144.95

144.97

144.99

Note: The astronauts will not favor any one of the above frequencies.

Therefore, the ability to talk with an astronaut depends on selecting one of

the above frequencies chosen by the astronaut.

The amateur packet frequencies are:

Packet downlink  		145.55 MHz

Packet uplink 			144.49

The Goddard Space Flight Center amateur radio club planned HF operating

frequencies:

3.860 MHz		7.185 MHz

14.295 			21.395

28.650

DTO-667:  PILOT INFLIGHT LANDING OPERATIONS TRAINER (PILOT)

One of the challenges of flying long duration Shuttle missions is the

issue of orbiter landing tasks.  These tasks require a high level of skill and

proficiency yet data shows that a pilot's landing skills degrade after an

extended absence from a landing trainer such as the Shuttle Training Aircraft.

During Shuttle Mission STS-58, a portable scientific workstation designed

to aid the Shuttle commander and pilot in maintaining those landing skills will

be demonstrated for the first time.

The PILOT system hardware consists of a portable scientific workstation, a

high resolution color display and a hand controller with orbiter look and feel.

The software used in the system was transferred from the Shuttle Engineering

Simulator software used to validate Shuttle flight software.  This provides

PILOT with orbiter handling and guidance characteristics.

The PILOT system is stowed in lockers on the flight deck and middeck areas

of the Space Shuttle. When a member of the crew wants to use the system, the

workstation is mounted on a console directly in front of the pilot's seat on

the flight deck and the PILOT system hand controller is attached to the

orbiter's hand controller.

In addition to evaluating the ability to maintain landing skills of a

Shuttle crew in Earth-orbit, the PILOT system may be integrated into the

standard training activities of all Shuttle crews at the Johnson Space Center,

Houston.

STS-58 CREW BIOGRAPHIES

John E. Blaha, 51, Col., USAF, is Commander (CDR) of STS-58. Selected as

an astronaut in May 1980, Blaha considers San Antonio, Texas, his hometown and

will be making his fourth spaceflight.

Blaha graduated from Granby High School in Norfolk, Va., in 1960; received

a bachelors degree in engineering science from the USAF Academy in 1965; and

received a masters degree in astronautical engineering from Purdue University

in 1966.

Blaha first flew as Pilot for Shuttle mission STS-29 in March 1989.  On

his second flight, he was Pilot for STS-33 in November 1989.  On his most

recent flight, he was Commander for STS-43 in August 1991.  Blaha has logged

more than 453 hours in space.

Richard A. Searfoss, 37, Lt. Col., USAF, serves as Pilot (PLT).  Selected

as an astronaut in January 1990, Searfoss was born in Mount Clemens, Mich., but

considers Portsmouth, N.H., his hometown.  He will be making his first

spaceflight.

Searfoss graduated from Portsmouth Senior High School in 1974; received a

bachelors degree in aeronautical engineering from the USAF Academy in 1978; and

received a masters degree in aeronautics from the California Institute of

Technology on a National Science Foundation Fellowship in 1979.

Searfoss graduated in 1980 from Undergraduate Pilot Training at Williams

AFB, Ariz. He flew the F-111F aircraft at RAF Lakenheath, England, followed by

a tour at Mountain Home AFB, Idaho. In 1988, he attended the Naval Test Pilot

School, Patuxent River, Md., as a USAF exchange officer.  He was a flight

instructor at the Air Force Test Pilot School at Edwards AFB, Calif., when

selected for the astronaut program.  Searfoss has logged over 2,500 hours

flying time in 54 different types of aircraft.

M.  Rhea Seddon, M.D., 45, serves as Payload Commander and Mission

Specialist 1 (MS1).  Selected as an astronaut in 1978, Seddon is from

Murfreesboro, Tenn., and will be making her third spaceflight.

Seddon graduated from Central High School in Murfreesboro in 1965;

received a bachelors degree in physiology from the University of California-

Berkeley in 1970; and received a doctorate of medicine from the University of

Tennessee College of Medicine in 1973.  She completed a surgical internship and

3 years of general surgery residency in Memphis following graduation.

Seddon served as a mission specialist aboard Discovery on STS-51D in April

1985.  Her next flight was as a mission specialist on STS-40 in May 1991.  She

has logged more than 218 hours in space.

William S. McArthur, Jr., 42, Lt. Col., USA, serves as Mission Specialist

2 (MS2).  Selected as an astronaut in January 1990, McArthur considers Wakulla,

N.C., his hometown and will be making his first spaceflight.

McArthur graduated from Red Springs High School, Red Springs, N.C., in

1969; received a bachelors degree in applied science and engineering from the

U.S. Military Academy in 1973; and received a masters degree in aerospace

engineering from the Georgia Institute of Technology in 1983.

After West Point and following a tour with the 82nd Airborne Division,

McArthur entered the U.S. Army Aviation School in 1975 and was designated an

Army Aviator in June 1976.  He served with the 2nd Infantry Division in Korea

and with the 24th Combat Aviation Battalion in Savannah, Ga., and later was an

assistant professor in the Department of Mechanics at West Point.

In June 1987, he graduated from the Naval Test Pilot School. At the time

of his selection as an astronaut, he was assigned to NASA at the Johnson Space

Center as a Space Shuttle Integration Test Engineer. McArthur has logged over

2,700 flight hours in 36 different aircraft.

David A. Wolf, M.D., 37, serves as Mission Specialist 3 (MS3).  Selected

as an astronaut in January 1990, Wolf considers Indianapolis, Ind., his

hometown and will be making his first spaceflight.

Wolf graduated from North Central High School in Indianapolis in 1974;

received a bachelors degree in electrical engineering from Purdue University in

1978; and received a doctorate in medicine from Indiana University in 1982.  He

completed his medical internship at Methodist Hospital in Indianapolis and

later completed Air Force flight surgeon primary training at Brooks AFB, San

Antonio, Texas.

Wolf worked as a research scientist at the Indianapolis Center for

Advanced Research from 1980 to 1983.  In 1983, he joined NASA in the Medical

Sciences Division of the Johnson Space Center, where he was responsible for

development of the American Flight Echocardiograph.

In 1986, he was assigned to direct development of the Space Bioreactor and

associated cancer research and tissue culture applications which use controlled

gravitational conditions.  He is a flight surgeon in the Air National Guard and

has logged over 500 hours in the F4 Phantom jet.

Shannon W. Lucid, Ph.D., 50, serves as Mission Specialist 4 (MS4).

Selected as an astronaut in January 1978, Lucid considers Bethany, Okla., her

hometown and will be making her fourth spaceflight.

Lucid graduated from Bethany High School in 1960 and received a bachelors

degree in chemistry, a masters degree and a doctorate in biochemistry from the

University of Oklahoma in 1963, 1970 and 1973, respectively.

Lucid first flew as a mission specialist on STS-51G in June 1985.  Her

next flight was as a mission specialist on STS-34 in October 1989.  Her third

flight was as a mission specialist on STS-43 in August 1991.  Lucid has logged

over 502 hours in space.

Martin J. Fettman, D.V.M., Ph.D., 36, serves as payload specialist.

Fettman considers Brooklyn, N.Y., his hometown and will be making his first

spaceflight.

Fettman graduated from Midwood High School in Brooklyn in 1973; received a

bachelors degree in animal nutrition from Cornell University in 1976; received

a masters degree in nutrition and a doctor of veterinary medicine from Cornell

University in 1980; and received a doctorate in physiology from Colorado State

University in 1982.  He is a diplomate of the American College of Veterinary

Pathologists.

Fettman served in the Department of Pathology of the College of Veterinary

Medicine and Biomedical Sciences at Colorado State University as an assistant

professor of clinical pathology from 1982 to 1986.

From 1983 to the present, he has held a joint appointment in the

Department of Physiology at Colorado State University. His research and

teaching interests have focused on selected aspects of the pathophysiology of

nutritional and metabolic diseases.

In 1988, Fettman assumed the duties of Section Chief of Clinical Pathology

in the Veterinary Teaching Hospital, Colorado State University. From 1989 to

1990, Fettman took a sabbatical leave as a visiting professor of medicine at

The Queen Elizabeth Hospital and the University of Adelaide in Australia. He

was named a Professor of Pathology at Colorado State in 1992.