IBM PS/2 Model 25SX Graphics Driver VESA TSR

(C) 1991,92 International Business Machines Corporation

developed for IBM by Tyler Software

Version 1.0

The graphics adapter on your PS/2 Model 25SX is actually an advanced

video adapter with 512K of on-board video memory that supports "SuperVGA"

modes beyond that of earlier VGA adapters.

VESA.EXE is a TSR ("Terminate-and-Stay-Resident") program that gives

VESA-aware application programs access to these SuperVGA video modes

through the industry-standard VESA interface.  Once VESA.EXE has been

loaded on your system, VESA-aware application programs will be able to

use the following additional VESA modes on your PS/2:

640x400 256-color    graphics mode

640x480 256-color    graphics mode

132-column x 25-row  text mode

To run this program, simply install VESA.EXE somewhere along your DOS

path and type 'vesa' at the DOS prompt.  Alternatively, VESA.EXE can be

called from a batch file, such as AUTOEXEC.BAT.

VESA.EXE may also be used on other IBM PS/2 systems, such as the PS/2

Model 40, 35, 56, and 57, that use the same advanced graphics adapter

as that on the PS/2 Model 25SX.  Note that many of these other systems

are limited to 256K of on-board video memory.  When run on those systems,

VESA.EXE does not activate the 640x480x256 mode.

The easiest way to find out if your particular PS/2 has the appropriate

graphics adapter is to run VESA.EXE on it - VESA.EXE will politely refuse

to load if it can't locate an adapter compatible with that on the PS/2

Model 25SX.