TABLE OF CONTENTS

pronet.device/--Overview--

pronet.device/CloseDevice

pronet.device/CMD_WRITE

pronet.device/OpenDevice

pronet.device/ReceivedData

pronet.device/--Overview--                         pronet.device/--Overview--

>> GENERAL INFORMATION <<

The device is able to handle several Units, which are defined in the

`devs:pronet/.config' file. Each Unit corresponds to a certain driver

and interface. The Units are subdivided into Ports, which are channels

between two sides of one Unit. This way two application systems may work

through the same connection without interfering each other.

pronet.device is *not* a SANA-II compliant networking device, and it is not

compatible to any other exec device, because it's got some very strange

behaviour regarding incoming data.  Sometimes it is useful, sometimes it is

not, but I will not change this anyway:  CMD_READ is not supported, instead

of that, when opening the device, you specify a MsgPort to which all

incoming data will be sent automatically, if you want it or not!

pronet.device v37 is incompatible to ProNET applications written

earlier, see below. I'm sorry for that, but I've not heard from people

using this device anyway ;-)

>> ERROR CODES <<

pronet.device v37 can return one of the following error codes after

OpenDevice():

PNDERR_PORTEXISTS

You've tried to open up a Port which is already in use.

PNDERR_DRIVERTROUBLE

The driver of the Unit you requested can not be started.

You can request a more detailed error message by setting the

according bit in OpenDevice()/Flags. pnr_Data must then point to

a buffer provided by you. The string will not exceed 63 characters

plus the terminating zero. If no error occurs, the buffer will

not be changed!

PNDERR_UNIT_NOT_DEFINED

The Unit you requested is missing a definition in the config file.

>> CHANGES SINCE V3 <<

v37: Write doesn't support two chunks anymore, Write can fail

with PNDERR_DESTINATION_GONE now, no more #?ConfigString() in order

to keep the configuration file tidy. Structure of received data

has changed. PNB_ERRORSTRING.

>> WRITING DRIVERS <<

You find information on this subject in the other autodoc file supplied

with the ProNET distribution.

pronet.device/CloseDevice                           pronet.device/CloseDevice

NAME

CloseDevice -- Close a port of a certain ProNET Unit.

SYNOPSIS

CloseDevice(PNRequest);

A1

FUNCTION

This is an exec.library call.

This function terminates access to the corresponding ProNET Unit.

Since the Unit is not shut down automatically, even if you were

the last user of it, a RemDevice() would be the right thing to

do after closing pronet.device.

INPUTS

PNRequest - A pointer to a previously opened ProNET IO Request.

EXAMPLE

This code can force a specified device to try and expunge.

Of course, if the device is in use nothing will happen:

void FlushDevice(name)

char  *name;

{

struct Device *result;

Forbid();

if(result=(struct Device *)FindName(&SysBase->DeviceList,name))

RemDevice(result);

Permit();

}

SEE ALSO

exec.library/CloseDevice(), exec.library/RemDevice()

pronet.device/CMD_WRITE                               pronet.device/CMD_WRITE

NAME

Write -- send output to ProNET Port.

FUNCTION

This command causes a packet of data to be written out the ProNET

Port/Unit. The number of characters is specified in pnr_Length.

IO REQUEST

io_Command - CMD_WRITE

pnr_Data - pointer to block of data to transmit

pnr_Length - number of characters to transmit. MUST BE EVEN AND MUST

NOT BE GREATER THAN 0x4000!

RESULTS

io_Error - if the Write succeeded, then io_Error will be zero.

The only other possible error code is PNDERR_DESTINATION_GONE.

pronet.device/OpenDevice                             pronet.device/OpenDevice

NAME

OpenDevice -- Open up a new port of a certain ProNET Unit.

SYNOPSIS

error = OpenDevice("pronet.device", unit, ioRequest, flags);

D0                  A0              D0    A1         D1

BYTE OpenDevice(STRPTR, ULONG, struct PNRequest*, ULONG);

FUNCTION

This is an exec.library call.

Hey, what do you think it does?!??

INPUTS

unit - This number must be defined in the `DEVS:ProNET/.config' file,

otherwise OpenDevice() will return PNDERR_UNIT_NOT_DEFINED.

ioRequest - A pointer to an initialized (see below) PNRequest block.

flags - As described in the overview page, you have the option

to get extensive error messages when setting a bit here.

The bit is defined as PNB_ERRORSTRING (the PNF_ definition also

exists).

Two components of the PNRequest must be initialized before calling

OpenDevice():

pnr_MsgPort - pointer to a MsgPort where incoming data is sent to.

pnr_NetSourcePort - The port number you want to open.

RESULT

error - zero if everything went o.k., otherwise an error code

as defined in exec/errors.h or devices/pronet.h.

NOTES

The *one and only* valid port numbers are 0x0001 to 0x7fff. If you

want a special number above 0x7fff reserved for your application,

please contact me!

If you don't have any special preferences for your port number,

you can use PNP_NEXTFREE to let pronet.device assign you the next

free one (it will be put into pnr_NetSourcePort then).

pronet.device/ReceivedData                         pronet.device/ReceivedData

Received data will be sent to the MsgPort you specified in pnr_MsgPort

when opening the device. You can not change it after opening.

mn_Length, as opposed to pre-v37, contains the length of the whole Message

structure. The first word of mn_Node.ln_Name contains the source ProNET

Port, the second word contains the destination port. The data comes behind

the Message structure.

This Message must be replied as soon as there is no use for it any more.

If there is incoming data but pronet.device can't allocate the Message

structure, the packet is lost!