romable/romable 					      romable/romable

ROMABLE.DOC

GENERATING ROMABLE FIRMWARE

REFERENCES

Please refer to the following DCC options:

-pi	generate position independant code.  All code references

forced to PC-relative

-pr	generate residentable position independant code.  Unless

-mw is specified A4 relative addressing is used for data

items.	All code references forced  to PC-relative.

-ma	specify that initialized data + bss will be hardwired to

a known address

-mw	specify that initialized data + bss will be hardwired to

a known address and all of data + bss occurs somewhere

within the first 32KBytes of the address space

(0x0000-0x7FFF).  Also allows DICE to use A4 as a register

variable.

-ms	const items go into EPROM in local cases

-mS	const items go into EPROM in local and external cases

The following documentation may appear confusing, but only because so

many possibilities exist when one intends to put code in ROM.  When you

are in doubt, it is usually a good idea to write a small test source

file, compile it, link it with symbols, and then look at the resulting

executable with DOBJ.

MAKING ROMABLE PROGRAMS

DICE has the capability to generate ROMABLE programs.  That is, program

binaries that you can use to directly burn an EPROM.  When generating

ROMABLE code you have more data model options available to you.  The

basic idea of ROMd code is as follows:

________________

FIXED ADDRESS = EPROM | 	       |

| 68000 vectors  |	const data objects

|----------------|

|     CODE       |

|----------------|

|  const DATA    |	string constants and other

| 	       |	const items, stays in eprom.

|----------------|

| read-only copy |

| of initialized |	this is copied to a working

| data	       |	RAM copy by the reset code

|________________|

________________

FIXED ADDRESS = RAM   | run-time copy  |    (copied from EPROM by startup)

| of initialized |

| data	       |

|----------------|    (cleared by EPROM startup)

| run-time BSS   |

|________________|

From the above you have two basic choices in terms of where your

RUN-TIME DATA is based.  If your RAM exists in low-memory

(0x0000-0x7FFF) you may choose to compile your code to use the

ABSOLUTE-WORD addressing mode.  This modes takes the same space as the

A4-RELATIVE addressing mode yet does not require use of the A4 register

and, in fact, the A4 register will then be available for use as a

register variable.

If you have minimal RAM in low-memory or your DATA+BSS is larger than

32KBytes (yes, that's 32KBytes people, not 64KBytes), then you will

have to choose either the formal large-data model or the A4-relative

small data model.  If you initialized data is over 64KBytes you can

still use the small-data model for most objects by using the __far

keyword judiciously.

NOTE: in all cases you can allocate or generate points for any ram

location, this just refers to accesses of declared variables!  Pointers

are always 32 bits.

model		options 	advantages	      disadvantages

absolute-word	-mw	-ms	no need for A4	      low 32KBytes only

A4 available as

reg variable

small-data		-ma	-ms	RAM can be anywhere   requires A4

must use __geta4

large-data		-ma -mD -ms	RAM can be anywhere   code is larger

libs must all be

large-data

CODE MODEL

The CODE MODEL is normally left at PC-relative and should not present a

problem.

CONST DATA

You want to use the -ms option or even the -mS option (see DCC.DOC for

the differences).  This will place all string constants in EPROM and

IN THE CODE SEGMENT, thus this initialized data will NOT take up space

in RAM.  Any 'const' initialized data will be placed in the code segment

and thus in EPROM.	Any non-const initialized data will be copied from

EPROM into RAM by your reset-startup code.

Any tables that you declare that you will never modify should be

declared const so they reside in EPROM and do not take up space in RAM.

On the otherhand, if your EPROM is running 6 wait states and your RAM

is running 0 you might consider not using 'const'.... assuming you have

a lot of RAM.

When your code is less than 32KBytes big you can use -pi or -pr and

should definitely use -mS.	When your code is larger than 32KBytes you

cannot use -mS unless you are absolutely positive pc-relative const

data references do not exceed the word-relative range.

RESET VECTOR, DATA MODEL SETUP

If your EPROM is mapped into low memory on boot you can declare a vector

table by declaring a CONST LONG array as the first data object in the

first object module that you link to create the executable:

const long ROMVectorTable[] = {

InitialSSP,

ResetVector,

.

.

.

};

Remember, this must be the first data declaration in the first object

module!!!  If you are using the A4-RELATIVE small-data model you must

qualify your ResetVector with the __geta4 type qualifier to set up A4.

In most cases you will want ResetVector to point to some assembly which

initializes your RAM DATA & BSS before calling C code.  The assembly

normally looks like this (this would be the second object in the link):

;   RESET exception, copy initialized data to RAM and clear

;   BSS area before calling rom_main()

;

;   Assumes less than 256KBytes of data and 256KBytes of BSS

xref	__DATA_BAS	    ; linker generated symbols

xref	__ABSOLUTE_BAS

xref	__DATA_LEN

xref	__BSS_LEN

xref	_rom_main

_ResetVector

lea	__DATA_BAS,A0	    ; ROM data

lea	__ABSOLUTE_BAS,A1   ; RAM data

move.w	#__DATA_LEN,D0	    ; long words of data

bra	trentry

trloop		move.l	(A0)+,(A1)+         ; copy EPROM copy to run-time

trentry 	dbf	D0,trloop	    ; RAM copy.

move.w	#__BSS_LEN,D0	    ; long words of BSS (follows data)

moveq.l #0,D1

bra	trbentry

trbloop 	move.l	D1,(A1)+            ; clear run-time RAM

trbentry	dbf	D0,trbloop

jmp	_rom_main(pc)

Where rom_main() is your C main routine qualified with __geta4 (if you

are using the A4-RELATIVE data model).  Any special items, such as

the EPROM getting mapped at reset, must be dealt with as well, usually

before the reset code sets up the run-time enviroment.

If you are using the ABSOLUTE-WORD data model DO NOT USE __GETA4!

Specifically, the -mw option TELLS DICE THAT A4 IS FREE FOR USE AS A

NORMAL REGISTER VARIABLE.  This gives you an extra register variable if

you haven't guessed!

EXAMPLE PROGRAM

Assume the above assembly module has been assembled and is called

'romc.o'.  The following C program is called 'test.c'.  In all cases

the code may start anywhere.

Also, in all cases note that ROMC.O is specified AFTER test in the

DCC line (that also serves as the link line).  This is because for

a standalone product we want our CONST data object that is the

ROM VECTOR TABLE to be first.  Of course, if you put the ROM VECTOR

TABLE in ROMC.A instead of TEST.C then ROMC.A would go first.  To

put the ROM VECTOR TABLE in ROMC.A it should be the first data

object declarations (dc.l's) in the CODE segment.

(1) SMALL-DATA-MODEL

extern void TrapReset();

extern char SSp[512];

const long RomVectors[] = { (long)SSp, (long)TrapReset };

char SSp[512];	    /*	our supervisor stack in BSS */

__geta4 void

rom_main()

{

short i;

for (;;) {

for (i = 0; i < 10000; ++i) {

<do something here with  i>

}

}

}

note, ram can be beyond 64K

v

COMPILE:    dcc test.c romc.o -ma 0x10000 -o t:test -r -v -lrom

romable t:test -o t:test.bin -D 0x10000 -C 0x0

burn eprom using test.bin

(2) ABSOLUTE WORD DATA MODEL

The source code is the same except you do not need __geta4.  Here

I am assuming RAM starts at, say, 0x2000.

void

rom_main()

{

...

}

COMPILE:    dcc test.c romc.o -mw 0x2000 -o t:test -r -lrom

romable t:test -o t:test.bin -D 0x2000 -C 0x0

burn eprom using test.bin

(3) ABSOLUTE LONG DATA MODEL

The source code is the same as for (2).  Here the RAM may start

anywhere (also true in (1)).

COMPILE:    dcc test.c romc.o -ma 0x10080 -o t:test -r -lrom

romable t:test -o t:test.bin -D 0x10080 -C 0

burn eprom using test.bin

(4) EXAMPLE OF EPROM INITIALLY MAPPED AT 0x0000 BUT REMAPPABLE TO

SOMEWHERE ELSE

In this case the assembly, ROMC.A, must relocate the EPROM back

to where it should be before initializing the data segment.  Here

your initial CONST data is the EPROM VECTOR TABLE, and your initial

non-CONST INITIALIZED data (because BSS gets stuck after all

initialized data) will be your RAM VECTOR TABLE.

In this example I assume that RAM will be mapped into 0x0000-

by the time rom_main() gets called.  Compilation depends on the

data model.  The example below uses the ABSOLUTE WORD data model.

extern void TrapReset();

extern char SSp[512];

const long RomVectors[] = { (long)SSp, (long)TrapReset };

long RamVectors[] = { (long)SSp, (long)TrapReset, ... };

char SSp[512];	    /*	our supervisor stack in BSS */

void

rom_main()

{

...

}

In this compilation example I am assuming the RAM will be at 0x0000 and

the ROM, say, at 0x20000 (above the 64KBmark)

COMPILE:    dcc test.c romc.o -mw 0 -o t:test -r -lrom

romable t:test -o t:test.bin -D 0 -C 0x20000

burn eprom using test.bin

USING ROM.LIB

ROM.LIB is generated from LIB/FILES.ROM3LIB and contains those modules

from C.LIB that do not declare any variables and are position

independant.  Modules in the library are entirely position independant.

If you use library calls instead of macros in <ctype.h> for ctype

functions those are position independant as well ... they use

pc-relative to access the lookup tables (which are declared const).

Nothing in ROM.LIB declares any data variables.  The idea is to link

your code with ROM.LIB instead of C.LIB to ensure that incompatible

routines are never included by mistake.

Warning: If you want position independant code you must compile with

-pi or -mS, especially when accessing <ctype.h> macros.  Accessing

<ctype.h> macros and not using either the above options will result in

absolute-code references to ctype's lookup tables.

------------------------------------------------------------------------

MAKING RAM MODULES

Generating a binary image for an executable that is meant to run from

RAM instead EPROM is relatively easy.  In many cases you want only a

single copy of the data (instead of two copies as in EPROM modules..

the permanent initialized data in EPROM and the run-time data+bss in

RAM).  This means that the program is not-reentrant... once an

initialized data variable is modified it stays modified.  If this is

not desireable you can use the same methods as were described for

making EPROM code combined with methods (see LIB/AMIGA/C_PR.A) to

allocate the data space run-time.

If you want to generate resident style modules use the small-data model

version in the previous section 'MAKING ROMABLE PROGRAMS' and simply

apply the concept of downloading such a module into RAM instead of ROM.

It is especially easy when the code is completely position independant

because your firmware does not have to deal with relocation at all.

This section describes standalone modules with UNsharable code because

the code is completely position independant and references only one set

of data.

________________

ANY ADDRESS IN RAM   |startup	       |

|     CODE       |

|----------------|

| initialized    |

|    data        |

|----------------|

| 	       |

|     BSS        |

|----------------|

The code uses a single PC-relative reference to find the start of the

data, then generates the A4 register to point to the data section.

NOT A SINGLE 32-BIT REFERENCE EXISTS.

WARNING:	The module has only one set of data.  Once initialized data

is modified it stays modified through multiple invocations of the module

The advantage of using a position independant module is obvious... you

can copy it to any point in ram as-is and then call it!  For small

68000 boxes with limited memory this is definite plus as it greatly

simplifies the loading mechanism.

To create a 100% relocatable module, the code must be less than 32K

and the data less than 64K.  You must use the -pi option when compiling

all source modules, when running dlink, and when running ROMABLE.

The -pi option, amoung other things, puts the __ABSOLUTE_BAS and

__DATA_BAS linker symbols in the code section.  Actually, dlink puts

the entire program into a single CODE hunk!

dcc -c test.c -pi

dlink -pi rampi.o test.o -lrom -o test

romable test -o test.bin -pi

RAMPI.A is something you have to write, but you do have a template.

Take a look at LIB/AMIGA/C_PI.A as an example (also note that

LIB/AMIGA/X.A is also required).  Remember that the autoinit sections

must be entirely supported though not necessarily the autoexit sections,

it depends on your application.

USING ROM.LIB

ROM.LIB is generates from LIB/FILES.ROM3LIB and contains compiler

support routines that do not require any RAM.  Modules in the library

are entirely position independant.	If you use library calls instead

of macros in <ctype.h> for ctype functions those are position

independant as well ... they use pc-relative to access the lookup

tables (which are declared const).

Nothing in ROM.LIB declares any data variables.  The idea is to link

your code with ROM.LIB instead of C.LIB to ensure that incompatible

routines are never included (instead you get an undefined symbol error

from the linker when you make a mistake).

Warning: If you want position independant code you must compile with

-pi or -mS, especially when accessing <ctype.h> macros.

------------------------------------------------------------------------

ROMABLE exeFile -o outFile [-o outFile2] -C addr -D addr -pi

addr:   decimal, 0octal, or 0xHEX

Romable generates a binary image for an executable compiled under DICE

and is normally used to generate a ROMABLE raw binary output file.

exeFile 	input executable linked with dlink

-o outFile	output binary (unformatted -- raw).  If TWO -o options

are specified the two output files will have even bytes

and odd bytes respectively, which is what you need when

you must program two eproms (one on the LSB data lines

and one on the MSB data lines)

example: -o out1		result: 01 02 03 04

example: -o out1 -o out2	result (out1):  01 03

result (out2):  02 04

-C addr 	code start address, 0octal, decimal, or 0xHEX

-D addr 	data start address, 0octal, decimal, or 0xHEX

-DC		place actual data+bss just after code (i.e. the

result is intended to be downloaded into RAM, there

is no duplicate data in this case).

'-D addr' is not specified in this case

-pi		generate a position independant module.  Neither

-C or -D are specified in this case, and ROMABLE

will warn you have any absolute references.

Note that your custom startup code determines how much of __autoinit

and __autoexit is to be supported.	Note especially that __autoinit0

MUST BE SUPPORTED because DICE will generate __autoinit0 sections to

handle 32 bit data relocations run-time.

ROMABLE generates a raw output file or files with the EPROM code first,

and initialized data after the main code (still in EPROM) which, as

has already been described, will be copied to RAM on reset by your

startup routine.

This startup-copying of initialized data and clearing of BSS makes it

extremely easy to use DICE to generate ROMED applications without

having to deal with major porting considerations.