#[1mTABLE OF CONTENTS#[m

Gemini-10X

#[1m#[3mGemini-10X #[m                                                     #[1m#[3mGemini-10X #[m

#[1mNAME#[m

Gemini-10X V35.1 - Driver for Star Gemini-10X printers - Requires V37

#[1mAUTHOR#[m

Michael B�hnisch                       Internet: billy@uni-paderborn.de

L�her Str. 2

FRG-4790 Paderborn

#[1mLEGALITIES#[m

This  driver is put in the public domain.  Use or misuse as you like to

but  don't make me responsible for any damage resulting through the use

of it.

Send suggestions, comments, flattery to my E-Mail address.

#[1mDESCRIPTION#[m

The  Workbench  2.04 kit offers "EpsonXOld" for printing to the archaic

Star  Gemini printers 10X and 15X.  The capabilities of this driver are

limited.   While  "EpsonX" offers "interleaved" printing to other Epson

compatible printers, this option was omitted for EpsonXOld.

Gemini-10X  provides  a  better  set  of  graphic  resolutions, special

abilities like downloadable fonts are likely to be supported in a later

release.

Table of preferences selectable GFX resolutions:

No. | x-dpi | y-dpi | xy-dpi | comment

-----+-------+-------+--------+--------------------------------

0  |   60  |   72  |   4320 | very draft but usable

1  |  120  |   72  |   8640 | double speed hi-res

2  |  120  |   72  |   8640 | normal speed hi-res

3  |  240  |   72  |  17280 | quad densitiy

4  |   60  |  144  |   8640 | draft interleave

5  |  120  |  144  |  17280 | double speed hi-res interleave

6  |  120  |  144  |  17280 | normal speed hi-res interleave

7  |  240  |  144  |  34560 | quad density interleave

#[1mINSTALLATION#[m

Copy   the   file   "Gemini-10X"  and  it's  associated  icon  file  to

DEVS:Printers/.   Then  use  Preferences to select the driver.  You may

want  to  select  density  7  and  give  "Multiprint"  by John Matthews

(tribble@gphs.vuw.ac.nz)  a try.  MultiPrint can be downloaded from any

AmiNet site.

#[1mBUGS#[m

Hopefully none.

#[1mTODO#[m

-  Some  of  the characters from the extended char set look ugly or are

poorly replaced (e.g.  'r' for '�').

-  Downloadable fonts.

-  Suggestions welcome!

#[1mHISTORY#[m

Date     | Version | Comment

---------+---------+---------------------------------------------------

06/21/93 |  35.1   | Initial release