File Maven (tm) 3.0

Technical/Misc Information

System Requirements

-------------------

File Maven will run on any 8088/286/386/486/Pentium compatible PC that has

640K of conventional RAM (460K free) and DOS 3.0 or higher.

Serial Link Null Modem Cable

----------------------------

There are basically two types of null modem cables: 3-wire and 7-wire.  The

3-wire cable contains only transmit, receive, and ground lines.  The

7-wire cable has four additional handshaking lines.  File Maven supports

both cables but users should get a 7-wire cable if possible for faster file

transfers.  A 7-wire cable can be used in 3-wire link mode, but the reverse

is not true.  Null modem cables are available in most computer supply stores.

If possible, look for a 4-headed null-modem cable that can support both 9-pin

and 25-pin serial ports at either end.

DB25 DB9            DB9  DB25

---- ---            ---  ----

2    3   TX -- RX   2    3

3    2   RX -- TX   3    2

4    7  RTS -- CTS  8    5    <-- 7-wire only

5    8  CTS -- RTS  7    4    <-- 7-wire only

6    6  DSR -- DTR  4   20    <-- 7-wire only

7    5   SG -- SG   5    7

20    4  DTR -- DSR  6    6    <-- 7-wire only

3-wire/7-Wire Null Modem Pin Configuration

Parallel Link Cable

-------------------

The parallel link mode is only available in the fully-licensed, registered

version of File Maven.  A parallel link cable is included when you purchase

the registered version.  There are basically two types of parallel link

cables: 4-bit and 8-bit.  The 8-bit cable contains more lines and uses

an additional register on the parallel port for faster data transfer.

File Maven supports both cables.  An 8-bit cable can be used in 4-bit mode

but the reverse is not true.  The parallel link has three advantages over

the serial link:

1. The 8-bit parallel link is up to 3 times faster than the 7-wire serial

link at the fastest baud rate.

2. All standard parallel ports are 25-pin, so 4-headed cables are never

needed.

3. There are no baud rate settings with the parallel link.

DB25M            DB25M

-----            -----

1 �������������  1     <-- 8-bit only

2 ������������� 15

3 ������������� 13

4 ������������� 12

5 ������������� 10

6 ������������� 11

10 �������������  5

11 �������������  6

12 �������������  4

13 �������������  3

14 ������������� 14     <-- 8-bit only

15 �������������  2

16 ������������� 16     <-- 8-bit only

17 ������������� 17     <-- 8-bit only

25 ������������� 25

4-bit/8-bit Parallel Pin Configuration

Ports and Interrupts

--------------------

File Maven reads the serial and parallel ports addresses from a 14-byte

array in your computer's BIOS data segment starting at 0040:0000.  The four

possible COM port addresses are contained in the first 8 bytes and the three

possible LPT port addresses are contained in the next 6 bytes.  File Maven

uses direct polling instead of interrupt driven communications for the

fastest possible connections.  Therefore, there no are IRQ settings to

adjust.

Blowfish File Encryption

------------------------

File Maven uses Blowfish for file encryption.  Blowfish, a variable size

key block-encryption algorithm, was developed by Bruce Schneier, author of

"Applied Cryptography" (Wiley), and presented in the April 1994 edition of

Dr. Dobb's Journal.  I thank Mr. Schneier for placing the algorithm in the

public domain, unpatented, so that it can be freely used by anyone.

File Maven's implementation differs from the original specification in two

areas.  First, instead of initializing the s-boxes with the hexadecimal

digits of pi, File Maven uses the output of a pseudo-random number generator.

This eliminates the need for a separate 4k data file.  The PRNG equation

used is: rng(i)=rng(i-1)*134775813+1 where rng is a 32-bit variable

initialized to ffffffff hex.  Second, PCs with Intel CPUs store data in

little endian format (least significant byte first in a multi-byte word).

The original specification converts each block to big endian format before

encrypting and back to little endian format before saving.  Since this only

slows encryption and does not change the security of the algorithm, File

Maven skips the conversions.

File Maven uses the Secure Hash Algorithm (SHA) to generate the session

key.  The SHA was developed by The National Institute of Standards and

Technology (NIST), along with the National Security Agency (NSA).  The

SHA is considered by many cryptography experts to be one of the most secure

one-way hash algorithms available.  File Maven takes the user's pass

phrase, converts it to uppercase, and runs it through the SHA.  This

results in a 160-bit message digest, or hash value.  When using the

Casual option, File Maven only uses the 32 most significant bits for the

session key.  When using the Secure option, all 160 bits are used.

Because of the efficient design of the Blowfish algorithm, there is no

speed penalty when using the Secure option.

Unfortunately, because the U.S. government classifies strong encryption

software as "munitions" and restricts its export, the Secure option is only

available in the Registered version sent to citizens of the U.S. and Canada.

Registered users in other countries receive the International Registered

version that only has the Casual option.  The 32-bit key still provides over

4 billion combinations that should keep your friends out of your private

files, assuming you did not use a password or pass phrase that could be

easily guessed.  On the other hand, the 160-bit key has over 10^48 (10

raised to the 48th power) combinations.  Assuming a brute force attack is

the best way to break it, a trillion supercomputers, each trying a trillion

keys per second, would take about 463 trillion centuries to try all

combinations.  In either case, don't call me if you forget a password.  I

have no method of recovering encrypted files without the password.  File

Maven leaves the original plaintext file in place so if you want it

permanently erased, use the Wipe command, not the Delete command.

Programming Language

--------------------

File Maven 3.0 was written using the Borland Pascal 7.01 compiler.  Version

3.0 represents a complete rewrite since version 2.2 and took over 7 months

to complete, outside of my regular full-time job.  File Maven now contains

over 15,000 lines of code.  The low level Link functions were written with

in-line assembly code.  All code is the original work of the author except

for the compiler's run-time library.  No external libraries were used.

Special thanks goes to my chief beta tester, Gerald Stump, and to my other

testers: Paul Bookout, Ed Simpson, Joe Branch, Ron Briggs, and Joe Stump.

Contacting the Author

---------------------

For technical support or other inquiries, send a note to:

Kent Briggs

109A Romana Circle

Hewitt, TX  76643

or e-mail:

kbriggs@execpc.com  (Internet)

72124,3234          (CompuServe)

Internet users can "finger kbriggs@execpc.com" or visit my World Wide Web

(WWW) site at http://execpc.com/~kbriggs to get the latest info on my

shareware and freeware products.  Also check the IBMSYS forum on CompuServe,

SimTel's mirror site at oak.oakland.edu, and Exec-PC BBS for the latest

versions.  Do you have suggestions for improvement?  Would you like to see a

Windows/Windows 95 version?  Drop me a note and let me know.