PS Ed-480

Yamaha PSS-480 Tone Bank Editor

for the IBM-PC and MPU-401 MIDI interface

Version 1.1

Copyright (c) 1989,1990  by David Rutkowski

2521 Riverside Dr.

Grand Rapids, MI 49505

1. Overview:

The Yamaha PortaSound PSS-480 Music Station is probably the

most sophisticated little synthesizer available for the money

(street price about $100-$120 U.S.) I have yet to hear of an

owner of one who isn't intrigued by, if not in love with, this

ultra low-end workstation.

PS Ed-480 is a program which allows access to all the internal

settings of the PSS-480's digital synthesizer (the 5 custom banks.)

The synthesizer's control panel allows access to 8 of the available

parameters.  Using a "patch editor" such as PS Ed-480, it is possible

to elicit a much greater variety of sounds from the tone generators.

2. PS Ed-480 Features:

--Allows access to all 36 PSS-480 tone parameters.

--Parameters can be accesed and modified with a minimum of

keystrokes.

--Save and load tone banks from disk files in standard MIDI System

Exclusive (SYSEX) data format.

--Individual patches can be named for easy identification.

--Has simple librarian functions--new tone banks can be built out

of existing tone banks on disk.

--Auto backup of a patch before editing allows comparison with what

patch sounded like before editing. Previous patch can be restored

at any time.

3. Shareware License:

This program, PS Ed-480, is being distributed as shareware. This

means PS Ed-480 is copyrighted material--it is not in the public

domain. The author retains all rights to the software, and in no

way assumes liability for any consequences resulting from the use

of this program. The software may be copied and distributed freely,

as long as neither the program nor documentation is modified. The

program may not be sold for profit. You are granted the right to

use the software for evaluation purposes, and are expected to

register as a user if you continue to use the software.

For the latest version of the software PLUS one future update, then

send $25.00 (an order form is at the end of this file.) to register as

a user. Your comments, suggestions, and financial support will be

greatly appreciated!

4. Getting Started:

The editor can be run from either the hard or floppy disk. It will

look for data files in the current directory.

For PS Ed-480 to work properly, you will have to put the PSS-480 in

MIDI MODE 00. To be able to test the tone banks in the entire range

of the keyboard, make sure AUTO ACCOMPANIMENT is set to NORMAL.

Also set VOICE/STYLE area of the LED window to VOICE.

Start the editor by entering PSED at the DOS prompt. The editor will

prompt:

Load Data from (f)ile or (s)ynth? _

If you want to a load previously saved tone bank, type 'f'. You

will be prompted for a file name. Either type in the name of the

desired tone bank, or press ENTER for a list of files in the

current directory from which to choose.

To load the current tone banks from the PSS-480 type 's' and press

ENTER. You will be prompted to start the Bulk Data Dump. This is

done by hitting the TRANSMIT CH/MEMORY BULK DUMP key twice so the

left side of the LED window flashes. Hitting the '+' key under the

flashing window will start the data dump. PS Ed-480 should

display the messages "Data being transferred..." and "Data Received."

If these messages do not occur, check out your MIDI connections.

When the data is loaded, the values for the various tone parameters

are displayed and the "Enter: " prompt appears. To change any para-

meter, type the number which appears in parantheses near that para-

meter. PS Ed-480 is programmed to recognize two character numeric

entries. If such an entry is made, it is automatically accepted

without the need to hit the enter key. This means the all editing

can be done using ONLY the numeric keypad.

The message area will display the valid range of values for any

parameter you select. Type in the desired value, or use the "+" or "-"

keys.  The tone generator in the synthesizer is updated with the new

value, so you can hear the effect of your changes immediately.

Repeated use of the "+" and "-" keys (even at the "Enter: " prompt)

will raise or lower the selected value one unit at a time.

In a similar way, you can invoke the menu functions at the bottom

of the screen by entering the number next to that function.

One of the best ways to learn how the sounds are created is to

study and make changes to the 100 "factory" sounds that are built

into the PSS-480. To aid in this, I have included several sets of

factory sounds in various catagories saved to the diskfiles,

STRINGS.PSS, HORNS.PSS, KEYBOARD.PSS, etc.

NOTE: When quickly cycling through values using the "+" or "-" key,

the data may occasionally get out of step and the PSS-480 will

display the "Err" error message. The tone bank will temporarily

revert to the default settings. The patch you are editing will

be downloaded to the PSS-480 when the next change to it is made.

The PSS-480's error message can be cleared by hitting the PSS-

480 MIDI MODE key.

5. PS Ed-480 Commands.

(80) Load from File: This command will load all 5 tone bank

voices  the tone banks from the file you specify.  If no

file extension is given, the extension ".PSS" will be added

automatically. If you press the ENTER key before entering

a file name, a window containing the files with the ".PSS"

extension will appear. Use the cursor keys to highlight the

file you wish to load then press enter.

(81) Load from Synth: This command will load all 5 tone

bank voices from the PSS-480 through a Memory Bulk Dump.

(82) Save to File: This command will save all 5 tone bank

voices to the file you specify. If no file extension is

given, the extension ".PSS" will be added automatically.

(83) Patch Name: This command allows you to add or edit a

name for the current tone bank (patch.) When editing, the

editor will only accept a limited number of characters,

including trailing spaces. If you are unable to enter

characters even though there appears to be room in the

field, it is because spaces at the end of the name are

taking up space. Move the cursor to the end of the text and

hit the delete key several times to clear out the extra

space characters.

(84) Read Patch: This command allows you to read one tone

bank from a file on disk into the currently selected tone

bank of the PS Ed-480. If no file extension is given, the

extension ".PSS" is added automatically. Using this

facility, it is possible to build up custom tone bank sets

from previously saved groups. Such custom sets can allow

you to instrument your recorded sequences or performance

splits with any combination of custom patches desired.

(85) Swap Backup: Whenever you change tone banks, PS Ed-480

makes a backup copy of the bank you change to. At any time,

this backup copy can be recalled with the current bank

settings taking its place as a backup. This allows you to

compare the sound of your changes to the original patch by

using this command once, playing some notes with the backup

settings, then using this command again to get back to your

edited version of the patch.  Once you switch to another

bank, the changes becomes permanent. So, if you wish too

abandon the changes made, use the Swap Backup command, then

switch to another bank. Switching back will give you a new

backup copy of the original and a chance to try another set

of changes.

(90) DOS: This command allows you to execute DOS commands

from within PS Ed-480. The editor will present you with

last DOS command executed. Just press enter to re-execute

the previous command, or Backspace over it to enter a new

command.

(99) EXIT: Leave the PS Ed-480 program.

6. PSS-480 Parameters.

Part III of the PSS-480 manual does a good job of explaining the

basic sound generation capabilities of the PSS-480. I urge you to

read it carefully if you haven't already.

PS Ed-480 sound generation parameters are explained in this

section with emphasis on the parameters not covered in the PSS-480

manual. These Parameters are grouped here, as on the PS Ed-480 screen,

by function:

ENVELOPE parameters control the ADSR (Attack, Decay, Sustain,

Release) envelope.

LEVEL parameters controls the various oscillator volume

levels.

TIMBRE controls the basic sound processed by ENVELOPE and

LEVEL settings.

A last group of parameters consists of various settings for

Vibrato and Tremelo effects.

Each parameter in this section is identified by the PS Ed-480 name

and number for that parameter followed by Yamaha's abbreviation as

appears in Addendum-1 in the back of the PSS-480's owners manual.

For a graphic representation of several of the following

parameters, see Addendum-1 (Add-1) in the back of the PSS-480

manual.

Bank

----------------------------- ENVELOPE -----------------------------

Attack     01,02   (AR): Attack Rate. This parameter controls

the way a sound begins. A high value will cause a sharp attack,

a lower value a slower, swelling of sound. When the attack

has reached its peak, the decay parameters determine what

happens next.

Decay1     03,04  (D1R): This is the first stage of the decay

portion of the Envelope. A lower value here will lengthen

the time it takes to reach the point where the settings for

the second portion of the decay envelope take over.

DecLevel   05,06  (D1L): This is Decay 1 Level. When the initial

rate of decay (Decay1) reaches this level, a second rate of

decay (Decay2) takes over. A lower value here means a higher

level of the cutoff point.

Decay2     07,08  (D2R): This is the rate of decay which takes

over once the Decay 1 cutoff level is reached. This is the

"DECAY RATE" parameter on PSS-480 control panel. The lower this

value, the longer the time this portion of the decay will be.

Release    09,10   (RR): Release Rate. This is how long the

note will endure after it is turned off (either by lifting the

key on the keyboard or through a MIDI Note Off command.) This

is similar to sustain, but does not require the sustain effect

be on. The lower this value, the longer it will take the down

to die down after it is released.

Sustain    11,12  (SRR): The Sustain Release rate which is in

effect when the sustain button is enabled. A lower value here

means a longer sustain.

----------------------------  LEVEL  -----------------------------

TotLevel   13,14  (TL): Total Level. This parameter is given two

different names on the PSS-480 control panel--MOD. LEVEL for the

Modulator, and TOTAL LEVEL for the Carrier. The higher the level

of the modulator, the greater its "sharpening" effect will be on

the tone of the carrier. The greater the level of the carrier,

the louder the sound will be overall.

ScaleLow   15,16  (LKS low): Level Key Scaling, low.

This parameter controls the relative volume between notes near

the low end of the keyboard and notes near the middle of the

keyboard. A value of 0 means there will be no change made between

low and middl notes.  Values between 1 and 15 represent the

loudest to softest settings for notes in the bass register.

ScaleHi    17,18  (LKS hi): This parameter is identical to the

ScaleLow parameter, except that it affects the notes at the

high end of the keyboard.

RateScale  19,20  (RKS): Rate Key Scaling. This parameter varies

the rate at which the envelope parameters are applied. A low

value will cause the entire envelope to be lengthened on the

lower notes. This mimics the response of aucustic instruments

(such as cellos) which take longer to reach a fully articulated

sound in the low registers.

AmpModEn   21,22  (AMEN): Amplitude Modulation Enable. This

parameter controls whether or not Tremolo (a rapid and repeated

rising and falling of volume level) is enabled for the Modulator

and/or Carrier oscillators. A value of 1 here enables Tremelo,

a value of 0 disables it.

FeedBack   23  (FB): This setting varies the amount of the modulator

output which is routed back into the input of the modulator. The

effect is a reinforcement of the modulator frequencies, with a bit

of additional complexity of sound. The higher this setting, the more

sound is being fed back.

----------------------------  TIMBRE  -----------------------------

SinTable   24,25  (SIN TBL): This parameter selects one of 4

wave forms for an oscillator. The four values and forms are:

0 : Sine Wave

1 : Squared Sine Wave

2 : Sine Half Wave

3 : Squared Sine Half Wave

Detune     28,29  (DT1): This parameter allows you to finely

"detune" an oscillator--that is, raise or lower it's pitch a

small amount. This result of this is a more complex result

from the interaction of the two oscillators, since they are

slightly out of step. Another effect can be a "beating" or

vibrato type pulsing of the sound due as the oscillators

interact. A value of 0 represents no detuning, 1..7

increasingly sharpen (raise) the tone and  9..15 increasingly

flatten (lower) the tone.

Coarse     30,31  (DT2): Coarse detuning. This parameter

detunes an oscillator by a large amount. The effect is to

produce a less musical, bell or gong type sound.

----------------------------(various)-----------------------------

Vibrato Level  32  (PMS): Pitch Modulation Sensitivity. This

parameter increases the intensity of the vibrato (rapid rising

and falling of pitch) effect, with 0 being the least vibrato,

and 3 the greatest.

Tremolo Level  33  (AMS): Amplitude Modulation Sensitivity.

This parameter increases the intensity of the tremolo (rapid

rising and falling of volume) effect.

Vibrato Enable 34  (V): Determines whether the vibrato key on

the PSS-480 will have any effect, with 1 permitting vibrato and

disabling it. (note: This parameter does not seem to have any

effect, although it is documented in the PSS-480 manual.)

Sustain Enable 35  (S): Similar to Vibrato Enable, only for the

sustain effect.

Vibrato Delay 36   (VBT): Adjusts the length of time before

the vibrato effect begins, with 0 being the shortest time and

99 being the longest.

6. Additional Features

PS Ed-480 has the following command line options:

--If the name of a patch file occurs on the command line PS

Ed-480 will load that file automatically, bypassing the

"Load from (f)ile or (s)ynth(?) " prompt. For example:

C> psed axes

will automatically load the file AXES.PSS for editing.

--A command line flag of "/t" for "transmit" will download

the specified file to the Yamaha PSS-480 without going

into editing mode, but rather terminating after the file

is downloaded.  For example:

C> psed axes /t

will download the file AXES.PSS to the synthesizer, and

immediately return you to the DOS prompt.

--The command line flag "/p" for "program" will allow you to

change parameters in PS Ed-480 by using the VOICE/STYLE

select keys on the PSS-480 control panel. To do this, the

VOICE/STYLE selector must be set to STYLE. PSED will read

program changes on channel 15 as keyboard input. In this

mode the numbers 44 and 66 are maped to the "-" and "+"

keys respectively. In some instances, editing in this way

facilitate more elaborate (two-handed) testing of changes.

--The command line flags /i and /a map the PS Ed-480 MPU-401

inteface routines to an alternate IRQ (/i) or port address

(/a). For example, if the MPU-401 you are using has been

configured to use interrupt 3, you would start PS Ed-480

as follows:

C> psed /i3

If, in addition, your MPU-401 was installed at port address

338H (hexidecimal) you would use the command line:

C> psed /i3 /a338

Please note that the port address MUST be in hexidecimal

format.

IRQ numbers and port addresses for the MPU-480 can also be

specified in the PSS.CFG text file, as detailed below.

The PSS.CFG file for Screen Colors and MPU-401 configuration:

The configuration file PSS.CFG can be modified to change

the screen colors displayed by PS Ed-480 as well as specify

the IRQ and port address of the MPU-401 interface. The file

is a standard ASCII file which contains one or more of the

following lines:

screen  = hh

fields  = hh

irq     = hh

address = hh

All numbers are in hexidecimal format. Please look at the

example PSS.CFG file included with PS Ed-480 for further

information.

PS Ed-480 was written in Turbo C 2.0.

7. The Future of PS Ed-480.

--Interface Refinements. User defined commands which will change

many parameters at once. This will allow such operations as

intensifying/softening the envelope with a single command.

Windowing for disk file lists, tone bank lists, etc.

--Enhanced Librarian Features. A single file will hold all

patches and bank sets and allow any combination of patches

to be downloaded to the PSS-480 at any time. The librarian

will also handle PSS-480 Chord and Song Memory Sequences.

--A Simplified Accompaniament feature which will allow easy

"step entry" of chord sequences directly from sheet music chord

symbols, with orchestration changes. (Prototype included in

the separate program, PSCHORD.EXE)

--Integration of the features of the PERF480 performance program

(included with this distribution.)

--Random Generation of patches and performances within user

defined ranges for quickly testing new ideas.

--User Suggestions -- what would you like the program to do? If

it sounds interesting and can be done...

Registered users of PSS-480 will be entitled to the latest version

of the program plus one automatic update (If it is the latest

version of the program you already have, then you will get

automatic updates for the next TWO versions.) Any subsequent

updates can be obtained by registered users for shipping and

handling fee of $3.00.

8. Registration Form.

------------------------------------------------------------------------

To register, please fill in this form and enclose $25.00 in

check or money order. You will be sent the latest version of

the program and one automatic update (or TWO automatic

updates if you already have the latest version.)

Name ______________________________________________________

Address ______________________________________________________

City ______________________________State ____ Zip _________

Computer __________________     MIDI Interface ________________

Version of PSS Ed-480 you currently have:   Version ___________

Mail to:  David Rutkowski

2521 Riverside Dr. N.E.

Grand Rapids, MI  49505

Thank You!