Parallel

���������������������������������������������������������������������

���������������������������������������������������������������������

Technologies, Inc.

Pipx

User's Guide

The Parallel Port Driver for ODI Workstations

Version 1.00

( Test Drive Release )

30 May 1993

�������������������������������������������������������������������������ͻ

�                                                                         �

�                   The Parallel Port Networking System                   �

�                                                                         �

�             comprehensive parallel port networking solutions            �

�                         for today's most popular                        �

�                        network operating systems                        �

�                                                                         �

�������������������������������������������������������������������������ͼ

(C) Copyright 1993 Jay Lowe, Parallel Technologies, Inc.

All Rights Reserved.

The use and copying of this product is subject to a License Agreement.

Federal copyright law permits you to make a backup copy of this

software for archival purposes only. Any other duplication of this

software, including copies offered through sale, loan, rental, or gift

is a violation of law, and subject to both criminal and civil

penalities.

No part of this publication may be reproduced, transmitted, stored in

a retrieval system or translated into any language in any form by any

means with the prior written permission of Parallel Technologies, Inc.

The information given in this manual is subject to change without

notice and does not represent a commitment on the part of the vendor.

Various trademarks used in this document are the property of the

companies which own them.

Parallel Technologies, Inc.               Parallel Technologies, Inc.

Corporate Address                         Shipping Address

Parallel Technologies, Inc.               Parallel Technologies, Inc.

PO Box 7                                  18080 NE 68th, Unit A-100

Redmond, Wa 98073-0007                    Redmond, Wa 98052

(206) 869-1136

(206) 869-9767 (fax)                      ( for UPS, FEDEX, etc )

Compuserve ID# 76640,203

InterNet ID# 76640.203@COMPUSERVE.COM

Page ii

Preface

This is the User's Guide for Pipx, an ODI driver which is part of the

family of Parallel Port Networking drivers from Parallel Technologies.

Expert Users

If you are an expert user, you may jump ahead to Quick Pipx                                                        Quick Pipx                                                        Quick Pipx.

New Users

If you are new to networking or parallel port technology, please

examine the manual before you install Pipx.

This manual assumes that you have already installed your network

operating system software and want to modify your configuration to

use the Pipx driver for Parallel Port Networking.

Your Personal Satisfaction ...

We would like you to know that we at Parallel Technologies are

committed to your satisfaction with our products and services.

We appreciate your comments, suggestions, and recommendations for

improvements and features you would like to see in Pipx, and if you

have any problems with using Pipx on your systems, please don't

hesitate to use our Technical Support Service. Please read the

section on Technical Support before calling. This will enable us to

get you up and running as fast as possible.

Thank you for purchasing Pipx.

Page iii

Software License Agreement

Read this agreement before installing or using the Software.

Installing and/or using the Software on your computer indicates your

acceptance of these terms. If you do not agree to these terms and

conditions, delete the Software and all it's associated files from

your computer. This Software License Agreement gives you ONLY

RESTRICTED RIGHTS to use the Software (and it's associated files and

documentation) as follows:

YOU MAY

1.  Use the Software on at most TWO SYSTEMS at the same time

for a period of 15 days for evaluation purposes only,

2.  Make backup copies of the Software, so long as you place our

copyright notice on the backup copies,

3.  Request and receive technical support as provided in the

Technical Support Policy.

YOU MAY NOT

1.  Use the Software in any way except as provided in this

agreement,

2.  Modify, transfer, or reproduce the Software in any way,

except as expressly allowed in this agreement,

This license is effective from the date of first use of the Software

and shall remain in force until terminated. You may terminate this

agreement by destroying any and all copies of the Software and its

documentation. Parallel Technologies, Inc. may terminate this agreement

if you fail to comply with any of the terms and conditions of it. You

agree to destroy any and all copies of the Software and its

documentation, in whatever form, if we terminate this license

agreement, and you agree to provide us written certification of such

destruction upon our request.

Page iv

Statement of Exclusion of Warranty

ALL PRODUCTS DELIVERED PURSUANT TO THIS AGREEMENT ARE DELIVERED ON AN

"AS-IS" BASIS AND PARALLEL TECHNOLOGIES, ITS DEALERS AND DISTRIBUTORS,

EXPRESSLY DISCLAIM ANY AND ALL WARRANTIES OR CONDITIONS OF

MERCHANTABLE QUALITY OR FITNESS FOR A PARTICULAR PURPOSE.

Limitation of Liability

PARALLEL TECHNOLOGIES DISCLAIMS ALL WARRANTIES RELATING TO THIS

SOFTWARE, WHETHER EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE, AND ALL SUCH WARRANTIES ARE EXPRESSLY AND SPECIFICALLY

DISCLAIMED.  NEITHER PARALLEL TECHNOLOGIES NOR ANYONE ELSE WHO HAS

BEEN INVOLVED IN THE CREATION, PRODUCTION, OR DELIVERY OF THIS

SOFTWARE SHALL BE LIABLE FOR ANY INDIRECT, CONSEQUENTIAL, SPECIAL,

PUNITIVE, OR INCIDENTAL DAMAGES OR DAMAGES DUE TO LOST PROFITS, DATA,

OR INFORMATION ARISING OUT OF THE USE OR INABILITY TO USE SUCH

SOFTWARE EVEN IF PARALLEL TECHNOLOGIES HAS BEEN ADVISED OF THE

POSSIBILITY OF SUCH DAMAGES OR CLAIMS.  IN NO EVENT SHALL PARALLEL

TECHNOLOGIES' LIABILITY FOR ANY DAMAGES EVER EXCEED THE PRICE PAID FOR

THE LICENSE TO USE THE SOFTWARE, REGARDLESS OF THE FORM OF CLAIM.  THE

PERSON USING THE SOFTWARE BEARS ALL RISKS AS TO THE QUALITY AND

PERFORMANCE OF THE SOFTWARE.  This warranty gives you specific rights

and you may have other rights.

This agreement shall be governed by the laws of the State of Washington

and shall inure to the benefit of Parallel Technologies, Inc. and any

successors, administrators, heirs and assigns.  Any action or

proceeding brought by either party against the other arising out of or

related to this agreement shall be brought only in a STATE or FEDERAL

COURT of competent jurisdiction located in King County, Washington.

Page v

Table of Contents

Welcome to Parallel Port Networking......          1

Pipx - The Parallel Port ODI Driver......          5

Pipx Features ..........................          6

Pipx Applications ......................          8

The Pipx Family of Drivers .............          10

Pipx Requirements ......................          11

Pipx Cable Type Chart ..................          12

Pipx Installation........................          13

Quick Pipx for Expert Users..............          15

Configuring Pipx.........................          16

Pipx Parameter List ....................          20

Using Pipx with Your Network Software....          21

Netware Lite ...........................          22

Windows Workgroups .....................          23

FTP Software's PC/TCP ..................          24

Map Assist .............................          25

Procedure Reference......................          26

Directories and your PATH ..............          27

Editing NET.CFG ........................          28

Copying Files ..........................          29

Comparing Files ........................          30

Using PARALLEL.EXE .....................          31

Changing to a different Cable ..........          32

Changing to another Port ...............          33

Networking and Printing on the Same Port          34

Using an Extension Cable ...............          35

Fun with Lights and Sounds .............          36

Measuring Pipx's Performance ...........          37

Improving Pipx's Performance on Your System       38

Pipx Error Messages......................          39

Troubleshooting Tips.....................          41

Appendix A.  Technical Support Policy....          42

Appendix B.  Making Your Own 4BIT Cable..          43

Appendix C.  Other Parallel Technologies Products  44

[ Documentation is abbreviated in the Test Drive Version ]

Page vi

Welcome to Parallel Port Networking

Welcome to the convenience and simplicity of parallel port networking!

For more than 30 years, computer networks have been an excellent way

for users to share information, programs, and system resources such as

printers, modems, and other peripherals.

Networking has evolved to a high art and networks have become a common

feature of the workplace. But all is not sweetness and light. Networks

are still arcane, expensive, and often inaccessible.

To network a workstation, one must still have a network card, a

special network cable, a point of access to the network, and a certain

amount of expertise, not to mention patience and a screwdriver! All

these tedious complications add up to a simple fact... networks are

often too costly, to troublesome, or simply impossible.

Parallel Port Networking comes to the rescue  --  by allowing you to

network together any two PCs with just a inexpensive cable.  You don't

have to open any cases or understand any strange hardware details. The

secret is the driver software, which makes the parallel port

connection look like a network adapter to your favorite networking

software. This means you can add the unique convenience and

portability of parallel networking to the network software environment

you already have.

The benefits and limitations of Parallel Port Networking compliment

the capabilities of traditional networking to offer you a endless

array of exciting new applications and adds new value to your existing

network.

Anywhere you go, you can have a network in minutes or... just seconds!

Page 1

General Specifications for Parallel Port Networking Products

Parallel Port MAC Drivers:

ODI Workstation        Pipx      available now           69.95

NDIS 2.01              Pndis     available April 93      69.95

NDIS 3.0               Pndis3    est available May 93    69.95

Packet Driver          Ppacket   available April 93      69.95

IPX (old monolithic)   ParaIpx   under consideration     69.95

Parallel Port Cables:

4BIT Cable   10 feet                                     29.95

Actual performance up to 50 Kbyte/sec

laplink-compatible, supports all port types

8BIT Cable   10 feet                                     29.95

Actual performance up to 75 Kbytes/sec

supports only 8BIT true bidirectional ports

Turbo Cable  Turbo Module + 2 6-foot cables             149.95

Actual performance up to 250 Kbytes/sec

Raw transfer rate up to 800 Kbytes/sec

Supports all parallel port types

Turbo Port   EPP Parallel Port for ISA systems          149.95

Actual performance up to 250 Kbytes/sec

Raw transfer rate up to 800 Kbytes/sec

Supports all parallel port types

Supported Parallel Ports (99% of all ports are supported):

4BIT ports   Unidirectional ports typical on ISA expansion cards

8BIT ports   True bidirectional ports

EPP ports    Intel SL style EPP ports are currently supported

Additional EPP ports will be supported as they appear

ECP ports    ECP ports will be supported when they enter the market

Ports not supported    Ports with inverted IRQ

Ports with non-functioning IRQ

Ports at nonstandard addresses or IRQs

(These limitations may be lifted in future releases)

Supported PC Systems:    PC, XT, AT, ISA clones (8086, 286, 386, 486)

Page 4

Features and Benefits of Parallel Port Networking

Features                Benefits               Applications

-------------------    -------------------     ---------------------

Uses Parallel Port     Completely External     Temporary Connections

No Screwdrivers       Changable Connections

No System Impacts      Transient Connections

Quick, Easy         Immediate Connections

Installation

Supports All Port           Universal            Connect to any PC

Types                Connectivity

Uses just a Cable         Very Low Cost         Entry Level Networks

Very Portable         Travelling Networks

Standardized Drivers        Emulates NIC             Netware Lite

(ODI, NDIS, packet)       Supports Popular      Windows for Workgroups

Networks

many, many others

Range of Cable Types       4BIT - laplink         30 - 50 Kbytes/sec

compatible

8BIT - 50% faster        40 - 75 Kbytes/sec

TURBO - fast         50 - 250 Kbytes/sec

universal

Scaleable         Covers range from low    Buy the speed you need

Performance                 cost

to high performance        Upgrade anytime

Menu Configuration        Plug and Play              For casual

applications

No HW expertise        or for novice users

required

Practical Limitations of Parallel Port Networking

Limitation                Impact                 Significance

--------------------   ------------------------    -------------------

Limited Cable Length         10' typical            PCs must be close

50' possible

Limited Bandwidth      4BIT/8BIT slower than        Useable but not

NIC                     blazing

TURBO cable rivals NIC        Use TURBO Cable

for high performance

4BIT and 8BIT modes      Server PC may drag        Use TURBO Cable to

are CPU intensive      under very heavy load      reduce CPU loading

Page 6

Pipx - The Parallel Port ODI Driver

Pipx is the ODI member of the Parallel Port Networking driver family.

Pipx is an ODI workstation driver.  ODI is Novell's Open Data Link

interface used with Netware products, including especially Netware Lite.

Pipx is mainly intended for use with Netware Lite but it may be used

with a variety of other network operating systems that support ODI:

Microsoft Windows for Workgroups (via ODINSUP)

Microsoft Lan Manager (via ODINSUP)

FTP Software's PC/TCP

Microtest's MAP ASSIST

NCSA Telnet

and other ODI-compatible software

Pipx comes to you from the same engineers who invented the first

parallel port network adapters, the first parallel SCSI hard disk, the

first high-speed parallel port, and the first daisychained and

multiplexed parallel devices, and more than 12 parallel port devices

altogether. Now we've brought that expertise to bear in offering

parallel port networking solutions for popular network operating

systems. Based on many years of experience in developing parallel port

devices and driver software, Pipx includes many advanced features

which we hope you'll enjoy.

We appreciate your comments and suggestions about Pipx. They help us

to continue improving and refining the product to better serve you.

This chapter describes these features, explains what you need to run

Pipx, and discusses how Pipx can be used in various applications.

Page 7

Pipx Features

Convenience

All you need to network any two PCs is a cable.

No network adapters, no screwdrivers, no slots, no opening cases,

no power supplies, no fancy network wiring. Just a simple parallel

cable which you may already have. And since almost all PCs have

parallel ports, anywhere you go you're network ready.

Portability

Pipx provides network capability wherever you go. On the road, at

home, in the office, throughout the lab, Pipx allows you to make

temporary or permanent network connections quickly and easily.

For the travelling professional, Pipx's truly portable networking can

save time and money while increasing your capabilities in the field.

Intelligence

Pipx let's you make connections between any two parallel ports.

Pipx understands various types of parallel ports and supports

several kinds of cables. It adaptively selects the best available

IO mode for your ports. Old ports, new ports, slow ports, fast

ports, no problemo.

Compatibility

Because Pipx is an ODI driver, it can be used with quite a

selection of network software. Since the ODI interface is fairly

popular, various shims exist for NDIS and packet driver

interoperability.

In some configurations, Pipx can run along with other network

interfaces (Ethernet, Token Ring, etc) in the same machine.  This

feature allows you to have connectivity clusters which integrate

the LAN, your desktop, and your laptop into a seamless environment.

Page 8

Universal Connectivity

Pipx can be used to network any two PCs which have parallel ports.

Pipx understands understands a wide variety of different parallel

port behaviors and adapts to the peculiarities of most ports.

Across the installed base of PCs, Pipx is actually a more universal

data transfer method than even floppy disks!

Low Cost

Pipx's only hardware requirement is the parallel port cable itself,

which you may already have at your disposal. Pipx supports various

types of popular cables, including those shipped with laplink-style

products. Pipx also supports an advanced high speed TurboCable that

has a raw transfer rate of up to 800 Kbytes/sec.

To keep Pipx inexpensive, it is available with or without a cable.

Scaleable performance

Pipx allows you to buy the level of performance that fit's your

needs. Pipx's performance ranges from about 40 Kbytes/sec to over

250 Kbytes/sec, depending on the type of ports you have, the cable

connecting them, and on the performance of the network software.

At the low end, Pipx's performance is quite usable for many

applications under both DOS and Windows and is, at the very least,

a great improvement over laplink-style connections. At the high

end, Pipx rivals network cards in actual performance. With the

increasing popularity of fast EPP and ECP parallel ports, Pipx

brings true NIC level performance to more and more systems.

Scaleable performance means that you can buy the performance level

you want while keeping universal connectivity to all PCs.

Page 9

Pipx Applications

Temporary Connections

Pipx is great for hooking two system together temporarily,

to transfer files, install software, perform backups, and so forth.

Transient Connections

We find Pipx expecially useful for transient connections in our

environment where "who's connected to who" changes minute by

minute.  It's handy to have one server running Pipx so we can just

plug the cable into any other machine and mount the server drives

to transfer files or perform tests. Pipx is extremely tolerant of

this sort of transient connection, where clients come and go as

needed.

Alternatively, you could carry a portable server running Pipx from

client to client and connect to them in seconds, without even

rebooting, to perform backups, distribute software upgrades, etc.

Desktop/Laptop Integration

Many laptops travel from home to office and usually there is a

desktop system waiting at either or both ends. My laptop is never

alone. it has friends at work and a close, meaningful relationship

with my home system. When I get home, I just plug in a cable, run a

batch file, and my day's work is available on my desktop.

Pipx cures the file-shuffling blues and cures "terminal versionitis".

Connectivity Clusters

Some network software, like Windows for Workgroups, knows how to

handle multiple networks. This is excellent for Pipx, since it

allows you to keep a permanent Ethernet connection while having

another "sideways" connection using Pipx. This configuration brings

your main network resources, your desktop, and your laptop together

into one seamless environment.

Page 10

Binary Networks

It's amazing how many networks consist of just two PCs. If you're

considered networking, then Pipx is your opportunity to get started

with a small two system network at very low cost. Whether at home,

in the office, or in the lab, Pipx gives you the capability to

network two systems in the easiest possible way.  Such networks

make great learning tools and have many practical applications as

well.

Low Cost Connections

Pipx is an extremely cost-effective way to connect two systems. It

gives you full networking capabilities, reasonable transfer rates,

lots of convenient features, and it does so at a very low cost.

When you figure Pipx's performance per dollar ratio, you'll be

pleasantly surprised. Compared to other networking technologies, it

offers more bang for the buck across a wide performance range.

If you compare kilobytes per second per dollar of cost, Pipx costs

about $1 per KBPS whereas Ethernet costs about twice as much for

desktops and more than 6 times as much for laptops.

Page 11

The Pipx Family of Drivers

Pipx is part of a family of network drivers for the parallel port. All

of these driver have the same features as Pipx including: support for

all types of parallel ports, multiple cable types, adaptive IO mode

selection, and so forth.

Pipx ... Parallel Port ODI driver

The ODI workstation driver covered by this manual.

Pndis ... Parallel Port NDIS driver

Pipx for the NDIS environment native to Windows for Workgroups,

Microsoft Lan Manager, and other systems. Artisoft has announced

forthcoming Lantastic support for NDIS drivers.

The NDIS enviroment offers the capability of accessing multiple

networks from a single workstation. This feature is a good fit for

Pndis, since it allows you to have a parallel port connection and

an Ethernet or Token Ring connection operating simultaneously.

Pndis will be released in the very near future.

Ppacket ... Parallel Port Packet driver

Pipx for the Packet Driver interface used with various commercial

and public domain network software, notably TCP/IP applications.

Ppacket is under development. Call for details on availability.

Contact Parallel Technologues regarding the availability of parallel

port networking for other driver interface types.

We will be pleased to provide compatible parallel port networking for

other environments with all the advanced port and cable features of Pipx.

Page 12

Pipx Requirements

To use Pipx, all you need is two PCs, your network software, and an

appropriate parallel port cable.  Your choices are listed below:

Supported Network Software

Novell Netware Lite

Microsoft Windows for Workgroups (through ODINSUP)

Microsoft Lan Manager (through ODINSUP)

FTP Software's PC/TCP

other ODI-compatible software may be supported

Supported Computers

for 286 and better systems use PIPX.COM

for 8086-based IBM/PCs and XTs use PIPX86.COM

Supported Parallel Ports

Virtually all parallel ports are supported

4BIT ports ... often called "unidirectional ports"

8BIT ports ... may be called "bidirectional ports"

EPP ports ... Intel 386SL and 486SL Enhanced Parallel Ports

ECP ports ... will be supported when available

Currently the port must have normal interrupt capability

Use PARALLEL.EXE to determine your port type

Supported Cable Types

Advanced design supports a range of cables and performance

4BIT ... compatible with laplink-style cables for all ports

8BIT ... for 8BIT ports only, 50% faster than 4BIT

TURBOcable ... high speed cable compatible with all ports

TURBOport   ... EPP-capable ISA add-in card

See the Pipx Cable Type Chart on the next page

Page 13

Pipx Cable Type Chart

Pipx supports various of cable types offering a range of performance

and compatibility with different parallel ports.

Cable Type      Compatibility      Performance                Cost

actual max KBps

through NW Lite

-------------------------------------------------------------------

4BIT            all ports         30 - 50 KBps              29.95

8BIT          8BIT ports only     50 - 75 KBps              29.95

TurboCable        all ports         50 - 250 KBps            149.95

TurboPort         all ports         50 - 250 KBps            149.95

-------------------------------------------------------------------

The 4BIT Cable

This cable works with all parallel ports and is compatible with the

cables used with many laplink-style systems. This is the most

universal "dumb" (wire-only) cable. Only 4BIT and TURBO cables will

work with all parallel ports.

The 8BIT Cable

This cable type offers 50% better performance than the 4BIT cable

but can be used only between two true 8BIT parallel ports.

The Turbo Cable

The Turbo Cable is actually a small module containing logic which

allows both ports to function independently at their maximum speed.

The Turbo Cable's 800 Kbyte/sec transfer rate achieves NIC speeds

between EPP ports and speeds up other modes as well

The Turbo Port

The Turbo Port is a ISA add-in EPP parallel port card which

provides the same high-speed capability as the Turbo Cable and can

also be used as an EPP port for fast printing in DOS and Windows.

Page 14

Pipx Installation

Overview

Install Pipx by copying all the files contained on the Driver

Diskette to your hard disk (or floppy) in the desired directory. We

will assume that you will install Pipx into C:\NWLITE. In any case,

instructions given later in the manual require only that the PIPX

components be available in a directory which is on your path.

Installation Procedure

1. Connect to the desired directory:

C:\> cd \nwlite

2. Copy the contents of the Driver Diskette to C:\NWLITE:

C:\> copy a:*.* c:\nwlite

3. Use TYPE, PRINT, or your editor to examine the README file

C:\> type README.DOC | more

or,  C:\> print README.DOC

or,  C:\> edit README.DOC

Page 16

Contents of the Driver Diskette

PIPX.COM          the Pipx ODI workstation driver

PIPX86.COM        the Pipx ODI workstation driver for PC/XT's

PIPX.DOC          the User's guide in ASCII text format

README.DOC        the latest information on Pipx

PARALLEL.EXE      the Parallel Port Information Utility

PARALLEL.DOC      the User's Guide for Parallel.Exe

PIPXLITE.BAT      sample batch file for using Pipx with NW Lite

PIPX_NET.CFG      sample NET.CFG file for Pipx

Page 17

Quick Pipx for Expert Users

If you are an expert user, you can proceed simply by installing Pipx

and using it instead of your existing ODI driver.

1. Copy the contents of the Driver Diskette to the desired directory.

2. Run PARALLEL to determine your parallel port configuration.

3. Configure PIPX by adding the following NET.CFG section:

Link driver Pipx

Port             [ parallel port address: 3BC, 378, 278 ]

Int              [ parallel port IRQ level: 7 or 5 ]

Cable            [ cable type: 4BIT, 8BIT, TURBO ]

Node Address     [ arbitrary node address, 12 hex digits ]

Lights           [ optional: Persistent or Transient ]

Sounds           [ optional: Transmit or Receive ]

Be sure you use different Node Addresses on each system.

4. Modify your network load procedure to use Pipx, for example:

[ sample Netware Lite STARTNET.BAT file ]

SHARE

LSL

PIPX      <--  load the Pipx ODI driver instead

IPXODI

SERVER

CLIENT

5. Repeat the installation on the second system.

6. Connect the systems with the appropriate cable.

7. Start the network on both systems.

Page 18

Configuring Pipx

Overview

The v1.00 release of Pipx requires manual configuration of the

parallel port parameters and cable parameters. We have provided a

copy of PARALLEL, the Parallel Port Information Utility, which can

be used to determine the required parameters for your systems.

It is only necessary to perform this manual configuration procedure

(Steps 1 through 3 below) once on each system where you'll be using

Pipx. We will be adding automatic configuration in a future release.

These instructions assume you have already installed your network

software for some other network adapter using an ODI driver and now

want to run that software between two systems using Pipx.

The following steps are required to configure Pipx on each system:

Step 1 -  Determine your parallel port configuration

Step 2 -  Edit your NET.CFG file

Step 3 -  Revise your network loading procedure

Step 4 -  Install the appropriate parallel port cable

Step 5 -  Run your network software

Step 1  -  Determine your parallel port configuration

Decide which parallel port(s) you want to use with Pipx.

Use the PARALLEL program provided on the Driver Diskette to

determine the Address and IRQ level of the selected parallel ports.

Page 19

Read the PARALLEL.DOC file if you need to use the IRQ loopback

procedure in order to determine the IRQ level. This file also

contains useful information on configuring your ports for best

performance.

Step 2  -  Edit your NET.CFG file

Pipx looks in your NET.CFG file for it's configuration parameters

when it is loaded. This file must be in a directory on your path.

Edit your NET.CFG file to add the necessary Pipx parameters, in the

"Link driver Pipx" section:

Link driver Pipx

Port         3BC          [ the base address of the port ]

Int          7            [ the IRQ level of the port ]

Cable        4BIT         [ the cable type you are using ]

Node Address babeface     [ arbitrary 12 hex digits ]

Lights       Transient    [ optional display lites ]

Sounds       Transmit     [ optional sound effects ]

These parameters are discussed in detail in the Pipx Parameters

section, but a few immediate comments are appropriate:

The Node Address may be any unique IEEE Ethernet node address

consisting of 12 hex (0-9, A-F) digits.

Be sure you use a different Node Address for each system.

The Node Address is present to provide for a future routing

feature which will allow Pipx packets to be routed onto other

network segments: Ethernet, Token Ring, other Pipx.

We suggest that you enable the Lights Transient feature

initially because it provides immediate feedback that Pipx is

working properly. You can remove it from NET.CFG later.

Step 3  -  Revise your network loading procedure

Revise your batch file (or manual loading procedure) to use PIPX.

Page 20

We suggest you first copy your existing batch file to a new file:

C:\NWLITE> copy startnet.bat startpip.bat

and then edit the STARTPIP.BAT to use PIPX instead of the previous

ODI driver.

You may also use the PIPXLITE.BAT batch file on the Driver Diskette

which load and unloads Netware Lite with Pipx. This batch file is

handy as it allows you to easily load and unload the network.

Here is an example using the Netware Lite STARTNET.BAT file:

STARTNET.BAT for NE2000  STARTPIP.BAT for Pipx

SHARE                                 SHARE

LSL                                   LSL

NE2000  <- replace this line ->       PIPX

IPXODI                                IPXODI

SERVER                                SERVER

CLIENT                                CLIENT

Step 4  -  Install the appropriate cable

Attach the parallel port cable to the parallel ports of both systems.

4BIT cables may be used with any parallel ports

8BIT cables require that both ports have 8BIT capability

TURBO cables may be used with any parallel ports

Step 5  -  Load your network software

Run your network software using the STARTPIP.BAT file or your own

custom procedure.

Pipx will display the configuration information when it runs, and,

if you selected the Lights feature, Pipx will begin to display it's

current status in the upper right hand corner of the screen

immediately. This display, which appears only in text modes, has an

extremely small performance impact and should not affect the

operation of programs.

Page 21

If Pipx encounters an error during loading, it will display an

error message and an ordered list of steps which may correct the

error.  Refer to the Pipx Error Messages section for more

information.

You may load the network software on either station first. Pipx

does not require that the second station be running when it starts.

Network operations will simply not start happening until the second

Pipx station is ready to listen.

Page 22

Pipx NET.CFG Parameter List

Port         Required: [ 3BC | 378 | 278 ]

The base address of the parallel port being used.

Int          Required: [ 7 | 5 ]

The IRQ level of the parallel port being used.

NOTE: Polled operation and Inverted IRQ is not

supported in this release. Please contact us if you

require support for non-standard ports.

Cable        Required: [ 4BIT | 8BIT | TURBO ]

The type of parallel port cable being used.

Node Address Required: 12 hex digits

A locally administered network address. May be any 12

hex (0-9, A-F) digits. Be sure you use a different node

address on each Pipx station.

Lights       Optional: [ Persistent | Transient ]

Persistent  ... enables persistent Display Lites

Transient   ... enables transient Display Lites

Sounds       Optional: [ Transmit | Receive ]

Transmit    ...  sound effects on Tx complete

Receive     ...  sound effects on Rx complete

Frame        Optional: one or more of the following:

Ethernet_802.3 (default)

Ethernet_802.2

Ethernet_II

Ethernet_SNAP

Page 23

Pipx Error Messages

Pipx may display the following error messages when it loads.

FATAL: You must specify the PORT, INT, and CABLE type in NET.CFG

Cause: One or more missing NET.CFG parameters

Fix  : Please configure the Link Driver PIPX section of NET.CFG

: You may refer to PIPX.DOC for configuration instructions

One or more of the required NET.CFG parameters is not present. You

should examine your NET.CFG file to see that the "Link driver Pipx"

section is properly configured. Be sure that NET.CFG is on the path.

FATAL: The PORT specified in NET.CFG does not respond, see Pipx.Doc

Cause: The specified PORT does not act like a parallel port

Fix  : Check your PORT parameter. Is it one of 3BC, 378, 278?

: Verify that the port works. Run PARALLEL.EXE to test your ports

: Make sure you are using the right type of cable for your port

: If possible, try a different parallel port

The port type detection has reported that the specified parallel port

is not operating properly.  You should check your Port parameter in

NET.CFG.  Use PARALLEL to test your port for correct operation. Try to

isolate the problem by using Pipx on a different parallel port.

Page 42

FATAL: Unrecognized CABLE type in NET.CFG, see Pipx.Doc

Cause: The CABLE parameter is not one of 4BIT, 8BIT, or TURBO

Fix  : Check your CABLE parameter

The Cable type given in NET.CFG is not valid. It must be one of the

following: 4BIT, 8BIT, or Turbo. Only the first character is

significant. Check your Cable parameter in NET.CFG. Check that NET.CFG

is on your path.

FATAL: The specified CABLE will not work on the selected PORT

Cause: You have specified a cable which is incompatible with your port

Fix  : Refer to Pipx.Doc for information on cable and port

compatibility

: This is usually caused by using an 8BIT cable on a 4BIT port

: 4BIT ports only support 4BIT and TURBO cables

Pipx has determined that the Cable you have specified is incompatible

with your parallel port.  This is usually caused by trying to use an

8BIT cable on a 4BIT port. The 8BIT cable only works between 8BIT

ports. You should use a 4BIT or TURBO cable with 4BIT ports.

Page 43

FATAL: The PORT setting in NET.CFG is not a valid port address

Cause: PORT parameter is not one of 3BC, 378, or 278

Fix  : Set the PORT parameter to the address of the port being used

: You may use PARALLEL.EXE to examine your ports if necessary

: Valid PORTs are 3BC, 378, and 278

: Refer to Pipx.Doc for example configurations

You have specified a PORT address which is not a valid parallel port.

Valid address are 3BC, 378, and 278. Check the PORT parameter in your

NET.CFG file.  Be sure that NET.CFG in on the path.

FATAL: The INT setting in NET.CFG is not a valid interrupt level

Cause: INT parameter is not one of 7 or 5

Fix  : Set the INT parameter to the IRQ level of the port being used

: You may use PARALLEL.EXE to determine the IRQ level if necessary

: Valid INTs are 7 and 5

: Refer to Pipx.Doc for example configurations

The INT parameter in NET.CFG is not a valid interrupt level for the

parallel port. The INT parameter must be 7 or 5.  Check the INT

parameter in your NET.CFG file.  Be sure NET.CFG is on the path.

FATAL: This version of Pipx cannot run on this computer

Cause: PIPX.COM requires a 286 or better CPU

Fix  : Use PIPX86.COM which supports 8086 processors

You are trying to run PIPX.COM on an 8086-based computer. You must use

PIPX86.COM instead on 8086-based systems.

Page 44

Technical Support Policy

With your purchase of Pipx, you receive up to 4 hours of technical

support for a period of one year. Technical support is offered via

Compuserve Email, via fax, and by telephone at our office during the

hours of 1:00pm - 5:00 pm Pacific Time.

When requesting technical support, please be prepared with the

following information in printed form.  If your problem cannot be

resolved on the spot, we will ask you to fax this information to us:

1. Your parallel port configuration as reported by PARALLEL

2. The manufacturer and model numbers of your systems

3. The type of cable you are using

4. Your CONFIG.SYS, AUTOEXEC.BAT, NET.CFG files

5. Your PROTOCOL.INI file if you are using NDIS software

Parallel Technologies, Inc.               Parallel Technologies, Inc.

Corporate Address                         Shipping Address

---------------------------               ---------------------------

Parallel Technologies, Inc.               Parallel Technologies, Inc.

PO Box 7                                  18080 NE 68th, Unit A-100

Redmond, Wa 98073-0007                    Redmond, Wa 98052

(206) 869-1136

(206) 869-9767 (fax)                      ( for UPS, FEDEX, etc )

Compuserve ID# 76640,203

Internet ID# 76640.203@COMPUSERVE.COM

Page 47

Making Your Own 4BIT Cable

If you wish, you may make your own 4BIT cable for use with Pipx.

the cable wiring diagram is given below:

*******************************

* 4-BIT CABLE WIRING DIAGRAM  *

*******************************

DB-25M                                               DB-25M

Male Connector                                    Male Connector

1--------------------------------1

2--------------------------------15

3--------------------------------13

4--------------------------------12

5--------------------------------10

6--------------------------------11

7                                7

8                                8

9                                9

10--------------------------------5

11--------------------------------6

12--------------------------------4

13--------------------------------3

14--------------------------------14

15--------------------------------2

16--------------------------------16

17--------------------------------17

18--*--------------------------*--18

|                          |

19--*                          *--19

|                          |

20--*                          *--20

|                          |

21--*                          *--21

|                          |

22--*                          *--22

|                          |

23--*                          *--23

|                          |

24--*                          *--24

|                          |

25--*                          *--25

Sheild--------------------------------Sheild

Page 50

---------------------------------------------

Other Products from Parallel Technologies

---------------------------------------------

We at Parallel Technologies' have been creating innovative parallel

port devices and software since 1989. Here are a few highlights:

o  1st ... network adapter for the parallel port

o  1st ... built-in passthrough printing port

o  1st ... automatic self-configuring drivers

o  1st ... universal support for all parallel ports

o  1st ... network adapters with multiple media support

o  1st ... Windows 3.1 multimedia driver for parallel adapter

o  1st ... Multiple Parallel Device API for shared devices

o  1st ... intelligent parasitic power for self-powered devices

o  1st ... daisychained parallel port peripherals

o  1st ... multiplexed parallel port peripherals

o  1st ... Parallel Port Expansion System

o  1st ... product development tools for OEMs and developers

o  PC MAG Editor's Choice for Ethernet parallel port NIC

o  most total network adapters  --  6 and counting...

o  most total parallel port peripherals  --  12 and counting...

We have in-depth experience with the wonderful and sometimes bizarre

world of the PC parallel port and serve on both the EPP and IEEE 1284

standards committees. Having created more than a dozen parallel port

devices with their associated drivers and software utilities, we are

the most experienced source for off-the-shelf technology, developer

tools and components, and contract services.

Here are a few of our current and forthcoming products that you may

find interesting.

Parallel -- the Parallel Port Information Utility

-------------------------------------------------

A handy utility which examines your system's parallel ports and

reports the Port Type, IO address, IRQ level, BIOS name, and an

assortment of informative notes and warnings in a compact and easy

to read display. The output may be redirected to a file for tech

support purposes. Parallel uses very sophisticated techniques for

port and IRQ detection and is aware of a broad range of quirky port

features.

Parallel is the definitive parallel port Tech Support tool.

Parallel is Freeware, subject to a restricted license agreement.

It may be used free of charge for any personal, non-commercial

purpose. Vendors should contact us regarding commercial licenses.

A forthcoming commercial version of Parallel will use an active

loopback connector enabling many new features including detailed

resolution of port IO behavior and effective throughput testing.

Parallel Port Networking

------------------------

Now you can network two PCs using your favorite network software

with just a parallel port cable between the systems.

Our family of parallel port network drivers (PIPX, PNDIS, PPACKET,

and others TBA) work with standard network software to turn your

parallel port into a network adapter.  All types of ports are

supported with a very few specific exceptions.

Supports: Netware Lite, Workgroups for Windows, MS Lan Manager,

various TCP/IP systems, and other network operating systems that

use the ODI, NDIS, or packet driver interfaces.

Performance is better than you might expect, ranging from a low

end of 40 KBps between 4BIT ports all the way up to an effective

200 KBps between EPP ports using our TURBO cable. This upper range

is better than many ethernet ISA cards.

The drivers are available with or without cables, and existing cable

types typically used with laplink type products are also supported.

The MultiPort Parallel Port Expansion System

--------------------------------------------

The MultiPort 4-port multiplexor turns your parallel port into

a general purpose expansion system, capable of supporting up to

four simultaneously operating parallel port peripherals including

network adapters, portable hard drives, tape backup units, CDROMs,

floppies, and so forth. For example, you can use the MultiPort Mux

to attach a network adapter, a hard disk, a CD-ROM, and a printer,

all operating together through a single parallel port.

Now your laptop (and desktop) can enjoy a full range of peripherals

with all the portability, sharability, and convenience of parallel

port devices.

Parallel Port Developer's Kit

-----------------------------

Pipx was constructed with Parallel Technologies' advanced parallel

port software, which has been used in over 10 parallel port devices

including network adapters, parallel port disks, parallel port

wireless IR adapters, parallel port MIDI interfaces, and so forth.

This software includes support for all types of ports, IO code

that is extremely resistant to port idiosyncrasies, and full

autodetection of all port hardware features. The code is very well

documented. A selection of sample drivers for parallel port devices

is also available.

OEM Products

------------

Most of our products are available for OEM licensing, including

network adapters, EPP-capable hard disks, parallel-SCSI interface,

parallel MIDI adapter, and the MultiPort parallel port expander.

Development Services

--------------------

We specialize in contract development of parallel port peripherals

and associated software including DOS and Windows device drivers.

Our broad and deep experience with parallel port devices and our

commitment to specialization allows us to design, implement, test,

and support them very efficiently.

You might be interested in our 30/90 development offer. We propose

to convert any ISA peripheral into a parallel port device in only

30 days to prototype, 90 days to production.

We also offer more limited development support, such as developing

additional devices drivers for existing peripherals.

We are quite experienced with EPP ports and are ready to upgrade

your existing parallel port devices to full EPP performance.

Consulting and Training

-----------------------

In support of in-house development, you will find our consulting

and training services invaluable in avoiding the pitfalls of

parallel port development while achieving the highest possible

product performance and quality.