TECHNICAL INFORMATION FOR PROGRAMERS

------------------------------------

Perhaps you need know how to format 2M diskettes from your high-level

applications.  2M access always directly to NEC765 diskette controller chip

and 8237 DMA controller. But you have'nt to do the same!!.

When 2M is installed on memory,  INT 13h  service 5 becomes increased

its functionality. To find 2M resident on memory:

Load AL with 0.

Load ES:DI with 1492h:1992h.

Load AH with a value from 0C0h to 0FFh.

Call INT 2Fh.

When INT 2Fh returns, if AL=0FFh and ES:DI points a "CiriSOFT:2M:1.3"

null terminated string...  2M is in memory!.   Note that ':' is a delimiter

and not always CiriSOFT will be  the  manufacturer  (I recomend search only

for the ":2M:" string).  You  can  begin scanning with AH=0C0h and,  if not

found 2M there, continue with AH=0C1h and so on.

When you had detected 2M on memory,  you can call to INT 13h function

5 to format a diskette track.  This  is easiest than standard BIOS service:

you don't need to select disk density or media.  Of  course,  standard BIOS

function is still available, but if you can call INT 13h as bellow:

Load AH with 5.

Load AL with 7Fh.

Load CH with track number.

Load DH with head number.

Load DL with drive number.

Load SI with 324Dh signature ("2M").

Load ES:BX pointing to boot sector of future 2M diskette.

Note that in standard BIOS call, AL sets the number of sectors (never

is 7Fh) and SI is not neccesary equal to 324Dh. This lets 2M to detect your

intention. You have not to fill big tables on ES:BX, simply point to a boot

sector  of  future diskette. This boot sector can be obtained from standard

2M diskette already formatted. New INT 13h format function return the sames

codes in AH than the BIOS standard routines.

You don't need to build your own boot  sectors,  buf if you decide to

modify something, you must know:

- 2M diskettes have always  TWO FAT  copies,  for hardware reasons. Do

not try to alter this specification (yes, for *hardware* reasons).

- Modifing system ID probably makes 2M not able to detect the diskette,

and standard BIOS will report sector-not-found errors.

- Yoy can modify the jump instruction at the begin  of  boot,  and the

boot code located after destination of this jump.  But  data located

from offset 63 to last byte before boot code is covered by checksum,

and you have to compute it if you modify something (the arithmetic 8

bits add off all bytes must be zero, use checksum byte to force it).

- Resuming: you can only modify offsets from 0 to 2, from 11 to 61 (it

is not recommended) and the boot code.

- If you try to write a boot sector modifing  system ID,  or  with  an

incorrect  checksum,  2M  resident  code  will  not really write the

sector on target disk and you will lose your time and your patience.

If you uninstall 2M to save a boot with  bad  checksum,  the  sector

will be saved,  but...  with 2M installed you'll get 'seek error' on

future access.

Here are the format of 2M diskettes, in order to meet your curiosity;

2M is not a secret or magic program!.  See source code for more information

(if you don't understand the source code comments,  you  may  come to Spain

next summer :) ).

* BOOT SECTOR OF 2M DISKETTE (COMMON TO ALL FORMATS).

Assembler                        Comment                 Offset

---------------------  ----------------------------------  ----------

JMP   SHORT BootP     ; 2 bytes                                0

NOP                   ; 1 byte                                 2

DB    "2M-STV04"      ; system ID                              3

DW    512             ; bytes per sector                      11

DB    1               ; sectors per cluster                   13

DW    1               ; sectors reserved at beginning         14

DB    2               ; number of FAT copies                  16

DW    224             ; root directory entries                17

DW    3772            ; total sectors on disk                 19

DB    0F0h            ; media descriptor                      21

DW    11              ; sectors ocupied by each FAT           22

DW    23              ; sectors per track                     24

DW    2               ; number of heads                       26

DD    0               ; special sectors reserved              28

DD    0               ; number of sectors (32 bit drives)     32

DB    0               ; physical drive                        36

DB    0               ; reserved                              37

DB    29h             ; DOS > 4.0  diskette                   38

DD    4B368A0Eh       ; serial number (random)                39

DB    "NO NAME    "   ; disk label                            43

DB    "FAT12   "      ; FAT type                              54

DB    0               ; reserved by 2M                        62

DB    0               ; 2M checksum (if used)                 63

DB    6               ; 2MF format version                    64

DB    1               ; 1 if write track after format         65

DB    0               ; data transfer rate on track 0         66

DB    0               ; data transfer rate on track<>0        67

DW    BootP           ; offset to boot program                68

DW    Infp0           ; T1: information for track 0           70

DW    InfpX           ; T2: information for track<>0          72

DW    InfTm           ; T3: tracks sectors size table         74

* NORMAL FORMATS (820/1476/984/1804K). FOR EXAMPLE: 1804K.

Infp0 DB    18, 108         ; number of sectors / GAP 3 for format

DB    1,2,3,4,5,6,7,8 ; sectors ordered (19..20 does not exist)

DB    9,10,11,12,13

DB    14,15,16,17,18

InfpX DB    11, 40          ; number of sectors / GAP3 for format

DB    3               ; sector size

DB    1, 2            ; sector slidding (override by 2MF /X /Y)

InfTm DB    3,3,3,3,3,3     ; size of sectors 1, 2, 3,...

DB    3,3,3,3,3

BootP:...                   ; boot program

*  MAXIMUM CAPACITY FORMATS (902/1558/1066/1886K). FOR EXAMPLE: 1886K.

Infp0 DB    18, 108         ; number of sectors, GAP3 for format

DB    1,2,3,4,5,6,7,8 ; sectors ordered (19..23 does not exist)

DB    9,10,11,12,13

DB    14,15,16,17,18

InfpX DB    64, 3           ; number of sectors / GAP3 for preformat

DB    7               ; number of sectors to renum

DB    128+1,  4, 4    ; renum table:

DB    128+12, 1, 4    ; number of sector, new number, size

DB    128+23, 5, 4

DB    128+34, 2, 4

DB    128+45, 6, 3

DB    128+51, 3, 4

DB    128+62, 7, 2

InfTm DB    4,4,4,4,4,3,2   ; size of sectors 1, 2, 3,...

BootP:...                   ; boot program