This archive includes the following files:

LER      SHX	smooth leroy font

LERQ     SHX	quick leroy font (straight line segments)

DOT      SHX	ghost leroy font

MONOLER  SHX	mono-spaced smooth leroy font

MONOLERQ SHX	mono-spaced quick leroy font

MONODOT  SHX	mono-spaced ghost leroy font

LEROY    SCR	sample leroy script

LEROYQ   SCR	sample leroyq(uick) script

READ     DOC	This document

This is an update of an earlier upload which did not include the mono-spaced

or the ghost leroy fonts.  After looking at the text I was embarrassed at how

poorly I explained the setup.  It still isn't very good but you'll get the

idea and we all have different ways of setting up the styles sizes to suit our

particular drawing technique.

When we automated our plat drafting we needed to continue using leroy style

lettering; we are bound to by a Manual of Instructions.  To satisfy the

requirements Tom Noble of our Alaska office wrote the shape file for a Leroy

font.  We soon found out that regens took forever with the smooth leroy font

because of the arcs in the lettering, so Tom wrote a shape file called Leroy

Q(uick) which occupies the same space as the leroy font but is composed of

straight line segments.  The quick font is much faster and we use it during

the construction phase of the drawing.  It wasn't long before our draftsmen

asked for a ghost lettering style in leroy so this shape file has been added

to the system.  The ghost lettering was originally based on dots or points but

we found that it beat the heck out of the plotter so the fonts are now based

on circles with small diameters; a little slower but the plotter (not to

mention the pens) is much happier.

We create our drawings in real units, 1=1, therefore the height and size of

the lettering styles, V80, S80, V100, S100, etc., have to scaled up

proportionally.  To accomplish this we multiply the true leroy template size

(for V80, 0.08) times the final plot scale, which is stored in the USERR1

drawing variable.   This keeps everything in proper spatial relationship; at

1"=1,320 feet (or 1/4 mile) 80 leroy lettering is 105.6 feet tall.  When the

drawing is plotted to the predefined scale the lettering will be scaled back

down to the correct height.

We begin a drawing by running Leroyq.scr and prior to final plotting we run

leroy.scr to substitute the smooth leroy font.

This is our leroy script file:

regenauto off

style S50 LER (* 0.05 sc) 1 22.5 N N N

style S60 LER (* 0.06 sc) 1 22.5 N N N

style S80 LER (* 0.08 sc) 1 22.5 N N N

style S100 LER (* 0.1 sc) 1 22.5 N N N

style S120 LER (* 0.12 sc) 1 22.5 N N N

style S140 LER (* 0.14 sc) 1 22.5 N N N

style S175 LER (* 0.175 sc) 1 22.5 N N N

style S240 LER (* 0.24 sc) 1 22.5 N N N

style S350 LER (* 0.35 sc) 1 22.5 N N N

style V50 LER (* 0.05 sc) 1 0.0 N N N

style V60 LER (* 0.06 sc) 1 0.0 N N N

style V80 LER (* 0.08 sc) 1 0.0 N N N

style V100 LER (* 0.1 sc) 1 0.0 N N N

style V120 LER (* 0.12 sc) 1 0.0 N N N

style V140 LER (* 0.14 sc) 1 0.0 N N N

style V175 LER (* 0.175 sc) 1 0.0 N N N

style V240 LER (* 0.24 sc) 1 0.0 N N N

style V350 LER (* 0.35 sc) 1 0.0 N N N

regenauto on

style - issues the style command

S50   - names the style

LER   - specifies shape (shx) file

(* 0.05 sc) - system specific, multiplies the template size times the user

defined variable sc (plot scale for the final drawing)

1     - defines width factor

22.5  - defines obliquing angle for slant templates

N N N - answers NO to the remaining questions in the style command