CHAPTER 1: WHAT TINYTALK DOES AND WHY IT DOES IT

This chapter describes the facilities that Tinytalk gives you in your quest

to make your computer talk.  To understand how to use these facilities, you

need to know a little about how application programs use the screen

display, so we're going to talk about this and introduce some concepts that

we'll define in more depth later on.  If windows, video attributes and

lightbars don't make any sense to you yet, read on!

SECTION 1: HOW PROGRAMS ORGANIZE THEIR DISPLAYS

Modern application programs don't treat the screen like a scroll of paper

with new text coming in at the bottom; they consider it a "page" made up of

several sections.  This means that making a program talk isn't simply a

matter of "speaking anything that's written to the screen"; you also have

to know how the program uses the screen.  For example, when you're entering

or editing text in WordPerfect, the screen has two sections.  The first 24

lines show the text you're editing, and the bottom line shows a status

display.

We refer to these sections of the screen as "windows."  A window is an area

of the screen that a program uses differently from the rest of the screen.

These areas are always rectangular; they are defined by the row and column

coordinates of their top left and bottom right corners.  In the above

example, the text display is a window with a top left corner at line 1,

column 1 and a bottom right corner at line 24, column 80.  The status line

is a window from line 25, column 1 to line 25, column 80.

Programs often, but not always, visually mark the boundaries of their own

windows.  One way of doing this is to draw a box around the window using

line-drawing and corner "block graphics" characters.  Another way is to use

different video attributes for different windows.  Video attributes are

properties of displayed characters such as foreground color, background

color, brightness, underlining and blinking.  For example, a program might

display a window in a different color from the rest of the screen.

A program can do many different things with a window.  It can display

status information or instructions that seldom change (the second window in

WordPerfect is an example of this).  It can display a "lightbar menu,"

which is a list of options arranged either horizontally or vertically.  One

of the options is "highlighted," which means that it has different video

attributes from the rest of the options; for example, the foreground and

background colors might be reversed.  When you press cursor keys, the

highlight moves from one option to another; that's why it's called a

lightbar.

A window may display text that scrolls in and out of the area when you use

the cursor keys.  The main text window in WordPerfect is an example of this

kind of window.  Programs often have "pop-up windows" which are boxed-in

areas of the screen that contain messages, menus or prompts.  These are

often used together with lightbar menus.  For example, many programs have a

horizontal lightbar menu on the top line of the screen.  When you select an

option, a box with a vertical lightbar menu pops up underneath the

highlighted option.  Many programs will pop up "message boxes" when you

perform some action.  Pop-up windows can appear pretty much anywhere on the

screen, unlike fixed windows such as the ones we talked about in

WordPerfect.

Another common feature of full-screen programs is what we call "data entry

screens."  These are on-screen "forms" that consist of prompts followed by

highlighted areas.  You "fill in the form" by typing text into the

highlighted area, and move between fields (a field is a prompt/value area

combination) by using the cursor keys, tab key and ENTER key).

Most programs write characters directly to the screen instead of using the

rather slow and inflexible routines built into the computer's operating

system.  This means that a speech program can't just speak line-at-a-time

DOS output; it has to analyze the screen and read the changes in a way that

makes sense to the user.  Tinytalk provides a comprehensive set of

facilities for doing this.  Some of them work nearly automatically; with

others, you need to tell Tinytalk something about how the program is using

the screen.

Most commercial application programs won't fully talk "right out of the

box"; you'll have to do some configuring to make them talk the way you want

them to.  Most DOS-prompt commands and simple utility programs will talk

without any setup.

SECTION 2: APPLICATION MODE AND REVIEW MODE

When you're using Tinytalk, you're always in one of two modes.  Most of the

time you'll be in application mode, where you're running application

programs or giving commands at the DOS prompt.  In this mode, Tinytalk is

sitting in the background, watching the screen and your keystrokes and

sending screen output to your synthesizer.  Tinytalk always starts off in

application mode.

When you need to communicate with Tinytalk directly, for example to tell it

what parts of the screen you want it to read, you go into review mode (by

pressing a special "hot key").  In review mode, Tinytalk puts the program

you're running "on hold" and takes over the keyboard.  You can issue

commands to your synthesizer, move the cursor around to explore the screen

without interfering with the program you're running, and much more.  Once

you've done what you wanted to do, you can leave review mode and return to

your application.

SECTION 3: AN OVERVIEW OF TINYTALK'S FACILITIES

IMMEDIATE OUTPUT

All screen output sent through DOS or BIOS routines will be spoken as it

occurs.  You can turn this off completely, or you can set portions of the

screen to be silent.

USER-DEFINED PUNCTUATION

You can tell Tinytalk what, if any, punctuation characters to read.  You

can have different punctuation groups for reading versus typing, and for

review mode versus application mode.  You can tell Tinytalk how to

pronounce each punctuation character.

KEYBOARD ECHO

You can have your keystrokes spoken as words, as letters or not at all.

Tinytalk can announce any changes in "lock states" like caps lock.

AUTOMATIC CLEARING

Tinytalk will automatically clear any unfinished speech when you move the

cursor or when a window gets read.  You can choose whether or not regular

keystrokes should clear speech.

CURSOR TRACKING

When you move the cursor in a program, you can hear the text that it's

moving over.  You will hear characters that you delete with either the

backspace or delete keys.  You can specify how much text you want to hear.

REMAPPABLE HOT KEYS

You can read important parts of the screen from within an application

without going into review mode.  You can pick what keystrokes you want to

use as hot keys, and you can define your own hot keys to change Tinytalk's

settings without having to go into review mode.

REVIEW AND CONTROL MODE

Tinytalk has a comprehensive screen review mode for moving around the

screen without disturbing an application program.  In this mode, you can

also control synthesizer characteristic like pitch and speed, set up areas

of the screen to be monitored automatically and set various options for

Tinytalk itself.

AUTOMATIC WINDOW READING

You can have windows spoken automatically when they change.  You can tell

Tinytalk both when to speak them and how to speak them.

MODELESS REVIEW

Tinytalk provides an "independent cursor" that you can use to review the

screen while you're in an application program without having to go into

review mode.

CONTINUOUS DOCUMENT READING

When you're running a word processor or text editor, you can have Tinytalk

scroll through your entire document and read it to you.  If you're using a

synthesizer that supports indexing (see Chapter 3, Section 4), Tinytalk

will keep the speech from getting too far ahead of the cursor, so that you

can stop reading and then pick up where you left off.

TEXT HIGHLIGHTING

Tinytalk can highlight words or lines on the screen as it's reading them.

MULTIPLE CONFIGURATIONS WITH AUTOMATIC LOADING

Tinytalk can store up to 30 configurations (window definitions, mode

settings and voice characteristics) in memory.  You can switch from one

configuration to another with a few keystrokes, or you can have Tinytalk

automatically switch you to the correct configuration when you run a

program.  You can have separate configurations for individual screens

within a program, and Tinytalk can automatically switch between them based

on the presence of identifying text on the screen.

FORM FILLING

Many programs have "fill-in-the-form" data entry screens made up of prompts

and highlighted entry fields; you move between the fields with the cursor

keys.  Tinytalk will let you hear each prompt and field value as you move

to it, even when there are several fields on the same line.

KEY LABELLING

You can tell Tinytalk to say a word or phrase of your choice whenever you

hit a certain key.  You can have separate groups of key labels for each

application program you use.

VIDEO ATTRIBUTE ANNOUNCEMENT

If you want, Tinytalk can announce changes in video attributes when reading

text from the screen.  As you'll recall, programs often use attributes to

distinguish between sections of the screen.  Word processors also use them

to symbolize print characteristics like bold-facing and underlining.  You

can give names to attribute combinations; for example, you could have

yellow-on-blue text (which symbolizes italics in WordPerfect) announced as

"italic."

MISCELLANEOUS

Tinytalk provides several other features for making application programs

easier to work with; you'll learn more about these later on.  It can

automatically read column headings or titles as you move around a

spreadsheet or database browse screen; buzz at you if you accidentally hold

down the shift key and type a letter while you have caps lock on; ring a

bell when you enter text past a right margin you define or click the

speaker whenever the cursor passes over a video attribute that you select.

If an application displays a long line of punctuation characters, Tinytalk

will read only the first two if the rest are identical.  Tinytalk will

spell out any word that doesn't contain a vowel and can be told to spell

out "words" that have digits as well as letters (such as ham calls).  If

your synthesizer supports number processing, Tinytalk can use it when

saying "words" that contain only digits.

Tinytalk supports the 43 and 50-line screen displays that are available

with EGA and VGA video cards.