P C - T r a c k

V e r s i o n

2 . 1 4

-----------------------------

U S E R ' S

G U I D E

-----------------------------

S i m p l i f i e d

S a t e l l i t e

T r a c k i n g

Copyright (c)

1989-1991

Thomas C. Johnson

_______

____|__     |               (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

P C - T r a c k                   T a b l e  O f  C o n t e n t s

INTRODUCTION   . . . . . . . . . . . . . . . . . . . . . .   1

HISTORY  . . . . . . . . . . . . . . . . . . . . . . . .   1

GETTING STARTED  . . . . . . . . . . . . . . . . . . . .   3

RUNNING PC-TRACK  . . . . . . . . . . . . . . . . . .   3

USING THE ON-LINE HELP  . . . . . . . . . . . . . . .   3

SETUP  . . . . . . . . . . . . . . . . . . . . . . . . . .   5

SETUP OBSERVERS  . . . . . . . . . . . . . . . . . . . .   7

DESCRIPTION . . . . . . . . . . . . . . . . . . . .   7

LATITUDE  . . . . . . . . . . . . . . . . . . . . .   7

LONGITUDE . . . . . . . . . . . . . . . . . . . . .   7

MINIMUM ELEVATION . . . . . . . . . . . . . . . . .   8

TIME ZONE . . . . . . . . . . . . . . . . . . . . .   8

HEIGHT  . . . . . . . . . . . . . . . . . . . . . .   8

SETUP SATELLITES . . . . . . . . . . . . . . . . . . .    10

NAME  . . . . . . . . . . . . . . . . . . . . . .    10

CATALOG (SATELLITE) NUMBER  . . . . . . . . . . .    10

EPOCH YEAR  . . . . . . . . . . . . . . . . . . .    10

EPOCH DAY . . . . . . . . . . . . . . . . . . . .    10

DECAY RATE  . . . . . . . . . . . . . . . . . . .    10

INCLINATION . . . . . . . . . . . . . . . . . . .    11

RIGHT ASCENSION OF ASCENDING NODE . . . . . . . .    11

ECCENTRICITY  . . . . . . . . . . . . . . . . . .    11

ARGUMENT OF PERIGEE . . . . . . . . . . . . . . .    11

MEAN ANOMALY  . . . . . . . . . . . . . . . . . .    12

MEAN MOTION . . . . . . . . . . . . . . . . . . .    12

REVOLUTION NUMBER . . . . . . . . . . . . . . . .    12

FREQUENCY . . . . . . . . . . . . . . . . . . . .    12

FINISHING UP  . . . . . . . . . . . . . . . . . .    12

TRACK  . . . . . . . . . . . . . . . . . . . . . . . . .    14

GRAPHICS . . . . . . . . . . . . . . . . . . . . . . .    14

Getting Ready . . . . . . . . . . . . . . . . . . .    14

OBSERVATION POINT . . . . . . . . . . . . . . . .    15

MAP . . . . . . . . . . . . . . . . . . . . . . .    15

GLOBE VIEW  . . . . . . . . . . . . . . . . . . .    15

SATELLITES  . . . . . . . . . . . . . . . . . . .    15

Tracking  . . . . . . . . . . . . . . . . . . . . .    16

SPHERICAL OR LARGE MERCATOR PROJECTION SCREEN . .    17

SMALL MERCATOR PROJECTION SCREEN  . . . . . . . .    19

MAP/GLOBE . . . . . . . . . . . . . . . . . . . .    19

AZIMUTH/ELEVATION CHART . . . . . . . . . . . . .    20

OBSERVATION POINT BOX . . . . . . . . . . . . . .    20

STATUS BOX  . . . . . . . . . . . . . . . . . . .    20

SATELLITES BOX  . . . . . . . . . . . . . . . . .    21

DATE / TIME / INTERVAL BOX  . . . . . . . . . . .    22

P C - T r a c k                   T a b l e  O f  C o n t e n t s

TITLE BOX . . . . . . . . . . . . . . . . . . . .    23

HELP SCREEN . . . . . . . . . . . . . . . . . . .    23

FUNCTION KEYS AVAILABLE . . . . . . . . . . . . .    23

TEXT . . . . . . . . . . . . . . . . . . . . . . . . .    29

TEXT TRACKING SCREEN  . . . . . . . . . . . . . . .    30

FUNCTION KEYS AVAILABLE . . . . . . . . . . . . . .    30

CONFIGURATION  . . . . . . . . . . . . . . . . . . . . .    33

CONFIGURE DISPLAY    . . . . . . . . . . . . . . . . .    33

Selecting Text Color Category . . . . . . . . . . .    33

Select Background Category  . . . . . . . . . . . .    34

Change Text Color (F5)  . . . . . . . . . . . . . .    34

Change Background Color (F6)  . . . . . . . . . . .    34

CONFIGURE SYSTEM . . . . . . . . . . . . . . . . . . .    35

DEFAULT OBSERVATION POINT . . . . . . . . . . . . .    35

DEFAULT MAP . . . . . . . . . . . . . . . . . . . .    35

DEFAULT GLOBE VIEW  . . . . . . . . . . . . . . . .    35

DEFAULT SATELLITES  . . . . . . . . . . . . . . . .    36

DATA PATH . . . . . . . . . . . . . . . . . . . . .    37

MAP PATH  . . . . . . . . . . . . . . . . . . . . .    37

METRIC SYSTEM . . . . . . . . . . . . . . . . . . .    37

UTC TIME  . . . . . . . . . . . . . . . . . . . . .    38

FINISHING UP  . . . . . . . . . . . . . . . . . . .    38

CONFIGURE PRINTER  . . . . . . . . . . . . . . . . . .    39

Printer Codes . . . . . . . . . . . . . . . . . . .    39

APPENDIX A . . . . . . . . . . . . . . . . . . . . . . .    40

OBTAINING SATELLITE INFORMATION  . . . . . . . . . . .    40

APPENDIX B . . . . . . . . . . . . . . . . . . . . . . .    41

HOW TO USE THE ORBITAL DATA FROM NASA  . . . . . . . .    41

APPENDIX C . . . . . . . . . . . . . . . . . . . . . . .    42

GLOSSARY . . . . . . . . . . . . . . . . . . . . . . .    42

PC-TRACK 2.14                       Simplified Satellite Tracking

INTRODUCTION

HISTORY

PC-TRACK has its origin  in a satellite tracking program  I wrote

to track  the OSCAR  (Orbiting Satellite Carrying  Amateur Radio)

series of amateur radio satellites back in 1980. That program was

called  OSCAR  PATHFINDER and was the front cover feature article

in  the March 1982 edition  of "73 Magazine  For Radio Amateurs."

That  program was  written  for the  Apple  II Plus  computer  in

Applesoft BASIC. It was  a revolutionary program at that  time in

that it was the only program that tracked satellites in real time

(the Apple had no  clock) and displayed  the satellite path on  a

map on the Apple graphics screen. It was capable of only tracking

circular  orbiting satellites, but that was not a problem at that

time because all of the amateur radio satellites were in circular

orbits.

OSCAR PATHFINDER was so enthusiastically received that I made the

decision  to make some improvements  to the program  and offer it

commercially. This led  to a joint venture with a  good friend of

mine  (Thomas K. Ryan) and we called it Computer Applications and

with his marketing and businees management expertise sold quite a

number of copies  of Pathfinder II. We then  developed a  version

of   the  program  for  the  TRS-80  Model  III  computer  called

Pathfinder III. After a  year or so of selling these packages, we

decided (for various reasons) to pull  the plug on this and go on

to other things.

Later,  with  the  launch  of several  elliptical  orbit  amateur

satellites,  I tinkered with the program some more to include the

math  necessary  to  track  such  satellites.  This  resulted  in

Pathfinder X, which  was never published, but  I used it quite  a

bit  at home  for  my own  tracking  needs.  I had  also  started

rewriting  the program in UCSD Pascal because that offered a much

more readable and manageable structure for the program.

When I  got my IBM  PC/XT clone in 1985,  I had a  desire to port

Pathfinder X over to my new "supercomputer" since  it had an 8Mhz

clock and would be able to do faster computations and have better

graphics   capabilities.   Well,  after   many   years   of  many

distractions (other business ventures,  kids being born,  moving,

etc.) I  have finally put together my idea of a nifty package for

tracking satellites, called PC-TRACK.

PC-TRACK 1.1 was distributed late 1989. Version 2.0 contained new

User Guide                                                 Page 1

PC-TRACK 2.14                       Simplified Satellite Tracking

features   such  as  ability   to  track   up  to   8  satellites

simultaneously, multiple maps of different continents, and a text

track capability for non-EGA machines. Version 2.1 added 3D track

capabilities.

User Guide                                                 Page 2

PC-TRACK 2.14                       Simplified Satellite Tracking

GETTING STARTED

Refer  to the README file  included on the  distribution disk for

instructions on how to get PC-TRACK  installed and ready to run.

RUNNING PC-TRACK

To get PC-TRACK  running all you have  to enter at the DOS prompt

is PCT <RETURN>.

PC-TRACK   begins by drawing the background screen. There will be

a brief  pause at this  point while PC-TRACK   loads in  the help

screens,  data files,  and  graphics. This  takes  a few  seconds

(depending on the computer used).  You will see a message  at the

bottom of the hello  screen showing what data it is reading. Once

initialization is completed, you are presented with the main menu

screen.  This is a small shadowed window  on the left side of the

screen with the 4 main options listed.

The  four   options  are  SETUP,  TRACK,   CONFIGURE,  and  QUIT.

Selecting  one  of these  options is  done  simply by  moving the

cursor  (shown  by the selected name  being in a different  color

than the  rest of the  options listed) so that  it highlights the

module desired and pressing the RETURN key.

NOTE:  YOU MAY  ALSO SELECT  AN  OPTION  BY PRESSING  THE FIRST

LETTER  OF THE DESIRED  OPTION. FOR  EXAMPLE, IF  YOU WISHED TO

SELECT THE  TRACK OPTION, YOU WOULD  PRESS THE  'T' KEY (EITHER

UPPER OR LOWER CASE).

USING THE ON-LINE HELP

PC-TRACK   ASSISTANT, the on-line  help system for  PC-TRACK , is

available from anywhere in the program and can be accessed simply

by pressing the  F1 key.  The information presented  on the  help

screen will depend on where you are in the program.

For example, if you are  in the SETUP SATELLITES option with  the

cursor   on the  NAME field and  you press  the F1 key,  the help

screen  will pop up and tell you  how to enter information in the

NAME field.

Once  you  are in  the help  system, you  may browse  through the

information  provided by  pressing  the Page  Up,  Page Down,  Up

Arrow, or   Down Arrow. Page  Up does  the same thing  as the  Up

User Guide                                                 Page 3

PC-TRACK 2.14                       Simplified Satellite Tracking

Arrow  key and Page  Down does the  same thing as  the Down Arrow

key. You may get back to the original screen by pressing the HOME

key.

When going "down" in the system, you will be able to step through

every screen in the help  system. When going "up", you will  skip

up to the top screen  of the previous major topic in  the system.

This  allows you to move  quickly through the  screens to the one

you wish to see.

User Guide                                                 Page 4

PC-TRACK 2.14                       Simplified Satellite Tracking

SETUP

The  SETUP module is the built-in data manager for the satellites

and observation points to  be used in PC-TRACK.  With it you  can

enter, edit, and delete all information necessary to do satellite

tracking with PC-TRACK  quite easily.

To  select the SETUP module, move the cursor using the arrow keys

until  it highlights  SETUP and  press the  RETURN key.  (Or just

press  S.)  A second  menu window  will  appear with  the options

OBSERVERS and  SATELLITES. The OBSERVERS option  handles data for

the observation  points and SATELLITES  handles the data  for the

satellites. Select the desired option  by again moving the cursor

to highlight the one you want.

There are  various keys  that will  assist you  in the  entry and

editing  of observer or satellite data while in the SETUP module.

They are as follows:

F2 - ADD OBSERVER/SATELLITE

This  key   allows  you   to  create   a  new   entry  in   the

observer/satellite   file   where   information  about   a  new

observer/satellite  can be  put. When this key  is pressed, all

the  screen fields are  cleared so  you can  begin entering new

information.

F3 - REMOVE OBSERVER/SATELLITE

This key  does just the opposite  of the F2 key. It PERMANENTLY

removes  the  currently  visible  observer/satellite  from  the

file.  Since  this  action  is  PERMANENT  and  IRREVOCABLE,  a

warning window pops up to alert you to  the fact that this will

occur if you continue.  If you press RETURN in response to  the

warning window prompt, you  will remove that observer/satellite

from  the file. If you press ESC or any other key, you will not

remove the  observer/satellite and you  will go  back to  where

you were when you pressed the F3 key.

PAGE UP - PREVIOUS OBSERVER/SATELLITE

This key  will display the  previous observer/satellite in  the

file.  Observers/satellites   are  stored   in   the  file   in

alphabetical  order. Once the  observer/satellite you desire is

visible on the screen,  you may edit any of its fields. If  you

step past  the first observer/satellite  in the  file, the last

will be displayed.

PAGE DOWN - NEXT OBSERVER/SATELLITE

User Guide                                                 Page 5

PC-TRACK 2.14                       Simplified Satellite Tracking

This key  will display the next observer/satellite in the file.

If  you go past  the last  observer/satellite in  the file, the

first will be displayed.

User Guide                                                 Page 6

PC-TRACK 2.14                       Simplified Satellite Tracking

SETUP OBSERVERS

The OBSERVERS  option opens up the data entry window on the right

side  of the screen. This window shows  the six fields (pieces of

information)  held  for each  observation point  you have  in the

file.

The  six   pieces  of   information  necessary  to   describe  an

observation point are as follows:

DESCRIPTION

This field  is  your way  of  easily  identifying each  of  the

observation points you have in your file.  This is a text field

with room for up to 30 characters (anything you want).

For example,  you can put the  names of  the nearest associated

city with  the state, such as  Dayton, Ohio. Or  if you have  a

lot  of  observation points  entered,  you  might  devise  some

numbering  system  to systematic  identify each  location. This

could be something like OH-39N-84W, which  would be a  location

in  the  state of  Ohio, near  39  north latitude  and 84  west

longitude.  I'm sure  you can devise some  more sensible system

because  as  you  see  below  the  latitude and  longitude  are

already included.

LATITUDE

This  along  with   longitude  defines  the  location  of   the

observation point.

Latitude  is  entered  as  degrees  and  decimal  fractions  of

degrees. Latitude  will be entered as  a positive  number if it

is  in the northern  hemisphere and  negative (-)  for those in

the southern hemisphere.

For example  a position with  a northern hemisphere latitude of

34 degrees,  30 minutes,  and 15  seconds would  be entered  as

34.5042. That  same latitude in  the southern hemisphere  would

be entered as -34.5042.

This may be entered as a number between -90.0 to 90.0.

LONGITUDE

This along with latitude defines the  location on earth of  the

User Guide                                                 Page 7

PC-TRACK 2.14                       Simplified Satellite Tracking

observation point.

Longitude  is entered  like  latitude as  degrees  and  decimal

fractions of degrees.

Longitude  is  entered  as  positive  values  for  the  western

hemisphere (North  and  South  America,  Greenland,  etc.)  and

negative  for  the  eastern hemisphere  (Europe,  Asia, Russia,

etc). This  may  be entered  as  a  number between  -180.0  and

180.0.

MINIMUM ELEVATION

This  figure  describes  the  lowest  elevation  at  which  the

satellite  may  be  visible  from  the  observation  point.  An

observation  point with  a perfectly  clear horizon  around  it

would  have a minimum  elevation of  0.0 degrees.  If there are

mountains or  buildings around it  the minimum elevation  would

be higher.

This may be entered as a number between 0.0 and 90.0.

This  number is used  as a  filter for  displaying and printing

azimuth/elevation  data.   Only  satellite  look  angles   with

elevations above the minimum will be displayed or printed.

TIME ZONE

This describes the number  of hours from  UTC time (Coordinated

Universal Time)  your local  time zone  is. Time  zones in  the

western  hemisphere  are  entered  as  negative  while  eastern

hemisphere zones are entered as positive. For example,  Eastern

Standard  Time would  be entered  as  -5  hours from  UTC time.

Pacific Standard Time would be -8 hours.

Time Zone may be entered as whole numbers between -12 and 12.

NOTE: If you desire to track  using UTC time, make sure your

DOS clock is set to UTC time  (you can use the F10 key while

tracking) and use the CONFIG  SYSTEM option to tell PC-TRACK

To use UTC time.

HEIGHT

This describes the height your observation point is above  Mean

Sea Level (MSL).

User Guide                                                 Page 8

PC-TRACK 2.14                       Simplified Satellite Tracking

If  you are  not sure  what the  height is for  the observation

point, enter 0.0.

Height may be entered as any  value between -400.00 and  30,000

feet or between -122 and 9150 meters.

User Guide                                                 Page 9

PC-TRACK 2.14                       Simplified Satellite Tracking

SETUP SATELLITES

The SATELLITES  option also opens up the data entry window on the

right side of  the screen.  This window shows  the twelve  fields

used for each satellite you have in the file. This information is

available  from a  number  of sources.  See  Appendix A  for  the

cheapest  and  most reliable  source  for  this information.  The

twelve pieces  of information  necessary to describe  a satellite

are as follows:

NAME

This is a  11 character field  which you  can use  to name  the

satellite for easy identification.

CATALOG (SATELLITE) NUMBER

This is  a 5  digit number  assigned to  each satellite. It  is

used as the reference number in  the NASA source identified  in

Appendix A.

EPOCH YEAR

This is the  year of the specific time the rest of the data for

the  satellite is  effective. When  entering Epoch  year,  make

sure you include all  4 digits of the year, ie. 1989. Some data

sets such as those from NASA include only the last 2 digits  of

the year. PC-Track requires all 4.

EPOCH DAY

This is the day  and fraction of day  for the specific time the

data  is effective.  This  number defines  both the  julian day

(the whole  number  part  of the  value) and  the  time of  day

(fractional part of the value) of the data set.

The julian  day figure is  simply the  count of  the number  of

days that  particular date is from  the beginning  of the year.

(January 1 would have a julian day of 1. Feb 28 would be 59.)

This  number may  range from 1.0 to  366.999999999 (taking into

account leap years).

DECAY RATE

This is the rate of decay of the  orbital period (time it takes

User Guide                                                Page 10

PC-TRACK 2.14                       Simplified Satellite Tracking

to complete  one revolution)  due to  atmospheric friction  and

other  factors.  It is  a  real  number  measured  in terms  of

Revolutions per Day per Day (REV/DAY/DAY).

INCLINATION

This number describes the  angle of the  plane of the orbit  in

relationship to the earths equator.

Inclination  is 0  degrees for  a satellite  orbiting  directly

over the earths equator and orbiting  in the same direction  as

the earths  rotation.  It  is  180  degrees  for  an  satellite

revolving   directly  over   the  equator  counter   to  earths

rotation. An satellite  orbiting directly over the poles  would

have an inclination of 90 degrees.

Inclination is  given as a real  number of  degrees between 0.0

and 180.0 degrees.

RIGHT ASCENSION OF ASCENDING NODE

This  value describes  the  point the  satellites  crosses  the

equator   from  the   southern  hemisphere   to  the   northern

hemisphere (ascending node).  This measured eastward  along the

equator from the vernal equinox (right ascension).

It is given  and entered as a real  number of degrees from  0.0

to 360.0 degrees.

ECCENTRICITY

This is  a unitless  number which  describes the  shape of  the

orbit in  terms of how close  to a perfect  circle it is.  This

number is given in the range of 0.0 to less than 1.0.

An perfectly circular orbit would have an eccentricity of  0.0.

A number  greater than 0.0  would represent an elliptical orbit

with an  increasingly flattened shape  as the value  approaches

1.0.

ARGUMENT OF PERIGEE

This  value is the  number of  degrees from  the ascending node

the perigee point occurs. The perigee  point is the point where

the satellite  is the closest to  the earth  (assuming an orbit

which is elliptical to some degree).

User Guide                                                Page 11

PC-TRACK 2.14                       Simplified Satellite Tracking

This  number may  be entered  as a  real value between  0.0 and

360.0.

MEAN ANOMALY

This number  represents the angular  distance from the  perigee

point (closest point) to the  satellites mean position. This is

measured in degrees along  the orbital plane  in the  direction

of motion.

This  number is  entered like  the  argument  of perigee,  as a

value between 0.0 and 360.0.

MEAN MOTION

This is the number of  complete revolutions the satellite makes

in one day.

This number  may be  entered as  a value  greater than  0.0 and

less than 20.0.

REVOLUTION NUMBER

This represents  the number  of revolutions  the satellite  has

completed at the epoch time and date.

This number  is  entered as  an  integer  value between  1  and

99999.

FREQUENCY

This  represents  any   downlink  frequency  that  may  be   of

importance  to you.  This  frequency will  be displayed  on the

screens  and printed  on  the  printout  and will  be  adjusted

according to  the computed  doppler shift  associated with  the

relative range rate between the observer and the satellite.

This  number  is  entered  between  0.0  and  9999.999999   and

represents MHZ (megahertz).

FINISHING UP

When all of  the pieces of information  have been input for  an

satellite and you have no other  satellites to enter, press ESC

to get back to the main menu.

User Guide                                                Page 12

PC-TRACK 2.14                       Simplified Satellite Tracking

User Guide                                                Page 13

PC-TRACK 2.14                       Simplified Satellite Tracking

TRACK

The TRACK  module is  the  heart of  PC-TRACK.  It is  where  the

satellite  positions and  look  angles are  computed. Moving  the

cursor to  this option brings up  a choice of either  GRAPHICS or

TEXT. Select the type of tracking  you desire. If you do not have

an EGA video adapter, you must select the TEXT option.

GRAPHICS

The GRAPHICS TRACK option is the heart of PC-TRACK.  It tracks up

to eight satellites at  once on the  EGA graphics screen. It  can

display satellites using two types of graphic projections, either

spherical or Mercator.

Spherical  projections are  shown on  an image  of the  earth and

plotted in such a  way as to give a 3  dimensional effect.  There

are 4  different views of  the earth  for each of  the two  sizes

available.  The  view of the  globe is from  the equator, at  the

longitude chosen when selecting  the globe view (when setting  up

prior to tracking or with the F7 key). For example, selecting the

90 W  Lon globe view will show you  the earth from 0 deg latitude

(equator) and 90 deg west longitude. The view on the screen  will

always  be  from  that reference  point,  so  the  shapes of  the

satellite  orbits will look somewhat twisted due to the effect of

the rotation of the earth.

Mercator projections will show the positions of the satellites on

flat maps, with the  inherent distortion that provides.   It does

however, give  a somewhat  more accurate  picture of  the current

satellite  position, particularly  at higher  latitudes.   Due to

extreme distortions  at  latitudes  approaching  the  poles,  the

mercator projections only display positions to 80 degrees.

NOTE: If  either observer,  satellite, or graphics  data has

not  been loaded due to an improper path designation or some

other reason, then tracking  will not begin when you  select

this  option. An  error  window will  appear indicating  the

reason why  tracking cannot begin. Pressing  RETURN will put

you  back  at the  TRACK window.  If  this occurs,  you must

either check the paths  or enter new data (for  observers or

satellites).

Getting Ready

User Guide                                                Page 14

PC-TRACK 2.14                       Simplified Satellite Tracking

Selecting  this  option brings  up a  screen  with four  items to

select; the observation point, the map to use, the globe view  to

use, and the satellite(s) to track.

OBSERVATION POINT

The  default  observation point  selected  in  the CONFIG  SYSTEM

option  will appear. To change that observation point, use the up

and down arrow keys to select  the new one desired. Press  RETURN

to proceed  or ESC to go  back to the MAIN  MENU. The observation

points  displayed will  be  the  ones  in the  OBSERVER  database

defined using the SETUP OBSERVERS option.

MAP

Once  you select an observation point, the default map identified

in  the CONFIG SYSTEM option  is shown initially.  To select some

other map,  simply use  the up  and down arrow  keys to  show the

names  of the available maps.  When the one  you want is visible,

press  the RETURN key to proceed or the ESC key to go back to the

MAIN MENU.

GLOBE VIEW

Once you  select an map, the default globe view identified in the

CONFIG  SYSTEM option  will appear.  To select  some  other globe

view, simply use the up and down arrow keys to show the  names of

the available views. When the one you want  is visible, press the

RETURN key to proceed or the ESC key to go back to the MAIN MENU.

REMEMBER: You must have the MAPx.IMG and GLOBEx.IMG files in

the  map   path  specified  in  the   SETUP  SYSTEM  option.

Otherwise,  no  map  or   globe  will  be  displayed  during

tracking.

SATELLITES

PC-TRACK     is  capable   of   tracking  up   to  8   satellites

simultaneously. The selection  of the  ones to track  is done  as

follows:

There are 8 "slots" available to select satellites. Each slot may

have  any one of the  satellites defined in  the SETUP SATELLITES

option.  The cursor is initially positioned in slot number 1. The

default  satellites identified  in the  CONFIG SYSTEM  option are

shown. At this point you may:

User Guide                                                Page 15

PC-TRACK 2.14                       Simplified Satellite Tracking

Press the Page Up or  Page Down key. This will  move you to the

next slot  up or  down. If  you press  the page  down key,  the

cursor will move to the next slot  down, and the next satellite

down in the database will be showing. If  you press the page up

key,  the cursor will  move to  the next slot up  (will wrap to

the bottom  if you  are at  the top) and  display the  previous

satellite  in the database  (will also  wrap the  bottom of the

database if at the top).

Press the Up or Down arrow key. This will change the  satellite

displayed  to the previous  or next  satellite in the database.

The  up arrow will  select previous  satellite, the  down arrow

key will select the next.

Press  the Home  Key to  clear the  current slot  and  move the

cursor to the previous slot. This is handy to unselect a  slot.

You may  fill  any or  all slots  with satellites.  You do  not

necessarily have to fill them all.  You may have blanks  slots.

For  example,  you  could  set  it  up  so  there  are  only  2

satellites  to track, one in slot 2 and one  in slot 7. It's up

to you.

Press  RETURN  to complete  the  selection  process  and  begin

tracking.

Remember, the Page  Up and  Page Down keys  select the slot,  the

arrow keys select the satellite to go into the slot, the home key

clears  a slot, and the RETURN key finishes the selection process

and starts tracking.

Tracking

During  the tracking  process  information is  provided within  a

number  of separate  boxes on  the graphics  screen.  The general

locations  of these boxes on the track screen and the information

presented are described below. The  layout of the screen  depends

on  which projection  is selected  and for  mercator projections,

what size maps are selected.

User Guide                                                Page 16

PC-TRACK 2.14                       Simplified Satellite Tracking

SPHERICAL OR LARGE MERCATOR PROJECTION SCREEN

|===============================================================|

||========================================================||===||

||                                                        ||   ||

||                                                        ||   ||

||                                                        ||   ||

||                                                        ||   ||

||                                                        ||   ||

||                                                        || T ||

||                                                        ||   ||

||                                                        || I ||

||                                                        ||   ||

||                                                        || T ||

||                                                        ||   ||

||                                                        || L ||

||                                                        ||   ||

||              M A P   O R   G L O B E                   || E ||

||                                                        ||   ||

||                      B O X                             ||   ||

||                                                        ||   ||

||                                                        || B ||

||                                                        ||   ||

||                                                        || O ||

||                                                        ||   ||

||                                                        || X ||

||                                                        ||   ||

||                                                        ||   ||

||                                                        ||   ||

||                                                        ||   ||

||                                                        ||   ||

||                                                        ||   ||

||--------------------------------------------------------||---||

|                                             |----------------||

||--------------------------------------------|                ||

||        DATE TIME INTERVAL                  |  O B J E C T   ||

||              BOX                           |     B O X      ||

||-------------------------------------------------------------||

|---------------------------------------------------------------|

NOTE: IN THIS SCREEN, STATUS INFORMATION APPEARS ALONG THE BOTTOM

OF THE DATE TIME INTERVAL BOX.

User Guide                                                Page 17

PC-TRACK 2.14                       Simplified Satellite Tracking

User Guide                                                Page 18

PC-TRACK 2.14                       Simplified Satellite Tracking

SMALL MERCATOR PROJECTION SCREEN

|---------------------------------------------------------------|

||----------------------------------------||-------------------||

||                                        ||                   ||

||                                        ||                   ||

||                                        ||                   ||

||                                        ||      AZIMUTH/     ||

||                                        ||                   ||

||                                        ||     ELEVATION     ||

||                 M A P                  ||                   ||

||                                        ||       CHART       ||

||                                        ||                   ||

||                A R E A                 ||                   ||

||                                        ||                   ||

||                                        ||                   ||

||                                        ||-------------------||

||                                        ||-------------------||

||                                        ||                   ||

||                                        ||     OBSERVATION   ||

||                                        ||        POINT      ||

||                                        ||         BOX       ||

||                                        ||                   ||

||                                        ||                   ||

||----------------------------------------||-------------------||

||----------------------------------------------||-------------||

||                                              ||             ||

||                                              ||    STATUS   ||

||         S A T E L L I T E                    ||      BOX    ||

||                                              ||             ||

||            B O X                             ||-------------||

||                                              ||-------------||

||                                              ||             ||

||                                              ||   TITLE     ||

||                                              ||             ||

||                                              ||    BOX      ||

|------------------------------------------------|             ||

|------------------------------------------------|             ||

||       DATE TIME INTERVAL                     ||             ||

||             BOX                              ||             ||

||----------------------------------------------||-------------||

|---------------------------------------------------------------|

MAP/GLOBE

User Guide                                                Page 19

PC-TRACK 2.14                       Simplified Satellite Tracking

This is  the area where the maps of various portions of the world

are  displayed   for  use  in  plotting   the  current  satellite

positions.

AZIMUTH/ELEVATION CHART

The AZEL  chart shows  graphically the  look angles (azimuth  and

elevation) for each satellite. When a satellite is visible to the

observation point (defined as when the elevation  is greater than

the minimum elevation defined in the SETUP OBSERVERS option)  the

reference number  associated with  the satellite is  displayed on

the chart.  The number will be  placed on the chart  based on the

current  azimuth  (bearing)  and  elevation needed  to  point  an

antenna (or to look) at the satellite from the observation point.

The  distance  from  the  center  of  the  chart  represents  the

elevation.  The farther the point  is from the  center, the lower

the elevation.  Elevation is indicated by the numbers shown along

the  horizontal  line  on the  chart.  The  center  represents 90

degrees elevation or directly overhead.

The angular  displacement from  the vertical line  represents the

azimuth  from the observation point to the satellite. Azimuths is

numbered  clockwise around  the outside  of the  outermost circle

drawn on the chart. Zero degrees azimuth (straight up) represents

true north.

OBSERVATION POINT BOX

This box displays the name, latitude and longitude of the current

observation point.

STATUS BOX

This box shows  the status  of the various  modes available.  The

messages which will show up in this box are as follows...

REAL  TIME  TRACK   -  Indicates  PC-TRACK    is  tracking  the

satellite in real time. This is enabled by the F2 key.

FAST TRACK  - Indicates PC-TRACK   is computing and  displaying

satellite positions as  fast as possible ignoring the  internal

User Guide                                                Page 20

PC-TRACK 2.14                       Simplified Satellite Tracking

clock.  This is enabled by the F2 key.

PAUSING  -  Indicates PC-TRACK    has  temporarily  halted  the

tracking process after the F3 key  was pressed, and is  waiting

for the F3 key to be pressed again to resume tracking.

PRINTING  - Indicates  printing  is enabled  and  depending  on

which  ground track  mode  is selected,  will  print  satellite

position  and  look angles  for  the  satellite  selected.  The

satellite selected  will have its name displayed in a different

color than the rest of the  satellites listed in the SATELLITES

box. Printing is enabled by the F4 key.

G.T. (IR) - Indicates that  a + will  be placed on the map  and

the  AZEL  chart  for  each sub-satellite  point  in  which the

satellite  is  in range  of  the  observation point  when using

Mercator projections.   Will  mark satellite  position and  in-

range points when  using spherical projections. Enabled by  the

F6 key.

G.T.  (C) -  Indicates that a +  will be placed on  the map and

the  AZEL  chart for  each  sub-satellite  point  computed  for

Mercator  projections and  mark satellite  position and  ground

track   for  spherical   projections.   This  is   the  default

condition, and can be changed by the F6 key.

AUTOMAP - Indicates  PC-TRACK  will automatically select a  map

or  globe  which  will display  the position  of  the currently

selected  satellite  (selected  by  PGUP/PGDN  keys).  This  is

enabled by the F9 key.

SATELLITES BOX

The  satellite  box shows  all  the information  relating  to the

current position  of the satellites begin  tracked. The following

information is displayed for each satellite:

Reference  Number:  This is  the  number  associated  with  the

satellite, which  shows up  on the  map and  the azel chart  to

show its current position.

Name: This  is the  name given to  the satellite  in the  SETUP

SATELLITES option.

Latitude: Latitude of the current sub-satellite point.

User Guide                                                Page 21

PC-TRACK 2.14                       Simplified Satellite Tracking

Longitude: Longitude of the current sub-satellite point.

Altitude: Current altitude  of the satellite.  This will  be in

either kilometers or  statute miles depending on what you  have

configured the program to display in the CONFIG SYSTEM option.

Elevation: When  the satellite  is visible  to the  observation

point, the current elevation will be displayed here.

Azimuth:  When the  satellite  is visible  to  the  observation

point, the current azimuth will be displayed here.

Range:  This is  the  current  distance (again,  either  KM  or

Statute Miles)  from the  observation point  to the  satellite.

This  is displayed only  when the  satellite is  visible to the

observation point.

Frequency:  This  is   the  base  frequency  defined  for   the

satellite in the  SETUP SATELLITES option adjusted for  doppler

shift. This is displayed only when  the satellite is visible to

the observation point.

Phase:  This  is the current Mean Anomaly normalized to a range

of 0  to 255. This  is essentially  the portion of  the orbital

period  which  has elapsed  since  last  perigee  passage.  For

example, 0 will be at perigee and 128 will be at apogee.

Currently  Selected Satellite:  This  is shown  by  having  the

selected satellites name  displayed in a different color.  This

may be changed by using PGUP and PGDN keys.

DATE / TIME / INTERVAL BOX

PC-TRACK  is a time  based tracking package, which means  that it

first   obtains a  date and time  and then figures  out where the

satellite  is using  that date  and time.  This box  displays the

current  date and  time  used to  compute  the current  satellite

position.

The interval  is added  to the current  track time (shown  in the

DATE   TIME  INTERVAL BOX)  to generate  the next  date and  time

needed. This interval  is initially set  to 1  second. It may  be

modified as described below when in the Fast Track Mode. Interval

is fixed at 1 second and not displayed in Real Time Mode.

User Guide                                                Page 22

PC-TRACK 2.14                       Simplified Satellite Tracking

For  example,  if the  current time  shown  is 09:03:32,  and the

interval  is  00:00:05 (5  seconds) then  the  next time  used to

compute the satellite position will be 09:03:37.

TITLE BOX

This is where the advertisement goes.

HELP SCREEN

When the F1 help key is pressed a portion of  the track screen is

used  by  the PC-TRACK   ASSISTANT  to  display the  various help

screens. This is the very same help system used in other parts of

the program.  When you are done  with the help  system, press ESC

and the track  screen will be restored as it  was before help was

requested.

FUNCTION KEYS AVAILABLE

There  are several  function  keys available  to control  various

features of PC-TRACK  which affect the tracking process. They are

described as follows...

F1 - HELP

This is the  key which  brings up  the PC-TRACK   ASSISTANT  as

described earlier.  It functions  exactly as  it does  anywhere

else in the  program except the help  screens show in the  area

of the world map. Once you exit the  help system, the world map

is restored to its original condition before help began.

F2 - REAL TIME / FAST TRACK

This  key controls the tracking mode of PC-TRACK. When tracking

first  begins, it  is in the  REAL TIME TRACK  mode. This means

the  position   displayed  represents   the  actual   satellite

position according to the time set  in the computer (DOS time).

The displays will be updated at the  time interval shown in the

TIME INTERVAL BOX (fixed to 1 second).

Pressing the  F2 key changes this  to the FAST TRACK mode. This

mode ignores the clock in the  computer and simply computes and

displays the  satellite positions as fast  as it  can. The time

between each point  computed is determined by the time interval

shown  in  the  TIME  INTERVAL   box.  This  time  interval  is

changeable according to the procedures shown below.

The F2 key acts as a "toggle". This means  that if you press it

User Guide                                                Page 23

PC-TRACK 2.14                       Simplified Satellite Tracking

once, the mode  changes to FAST TRACK. When you press it again,

the  mode changes back  to REAL  TIME TRACK.  Pressing it again

changes the mode again to FAST TRACK, etc.

F3 - PAUSE TRACKING

This  key  may  be  pressed  at  any  time  during  tracking to

"freeze" the  display. This stops  all computations and  simply

waits for the F3  key to be  pressed again. Once it is  pressed

again, normal tracking resumes. When in  the PAUSE mode, all of

the other function keys perform normally.

F4 - ENABLE/DISABLE PRINTER

This  key enables output to  be sent to  the printer during the

tracking  process. The amount  of data  sent to  the printer is

determined by the  ground track mode  selected. If ground track

is  disabled,  or  in  the  IR  mode,  then  just  points  with

elevations  above the  minimum elevation  will be  printed.  If

ground  track  is  in  the  C  mode, then  all  points  will be

printed. The  currently selected satellite  will have its  data

printed.

F5 - REDRAW SCREEN

The  F5   key  simply  redraws  the   Map  or   Globe  and  the

Azimuth/Elevation chart to remove clutter from extended use  of

ground tracks.

F6 - GROUND TRACK

In  the  ground track  mode,  PC-TRACK    plots  the path  each

satellite has taken by marking each  point plotted. There are 3

possible modes of ground track...

- DISABLED

- IR

- CONT

GROUND TRACK DISABLED - No path will be plotted on the maps,

globes, or AZEL chart.  Just the satellite reference numbers

will be visible. Pressing the F6  key at this point will put

ground track into the IR mode.

GROUND  TRACK (In Range) - The STATUS BOX will display "G.T.

(IR)". With Mercator projections  leaves a ground track only

when the  satellite is in range.  With spherical projections

the ground  track is shown by  displaying each sub-satellite

point with a  black '+' on the globe. In  addition to this a

white point is placed in space showing the current satellite

User Guide                                                Page 24

PC-TRACK 2.14                       Simplified Satellite Tracking

position for  each  position computed,  whether in-range  or

not.

With mercator projections, a  '+' is placed on the  maps and

the AZEL chart at the current sub-satellite point.

Pressing the  F6 key at this point  will put ground track in

the CONT mode.

GROUND  TRACK (Continuous)  -  The STATUS  BOX will  display

"G.T.(C)".  This is  the condition  in effect  when tracking

begins.  This  leaves  a  ground  track  continuously.  With

spherical projections, a black '+' is placed on the globe at

each subsatellite point that  is in-range of the observation

point and  a  light  blue  '+' for  all  other  subsatellite

points. White points represent the actual satellite position

in space as described above.

With  mercator projections, A '+'  is placed on  the map and

the  AZEL  chart at  each  sub-satellite  location computed.

Pressing  the  F6 key  at this  point  will turn  off ground

track.

F7 - SELECT NEW MAP/GLOBE VIEW

This  allows  you  to  change  the  map  or  globe  view during

tracking. This  brings up the same  selection of  maps or globe

views which  were available when setting  up to  track. You may

select a  new map or  globe view simply by  pressing either the

up or  down arrow  keys until  the name  of the desired  map or

view is  shown. You  may then  press  either ESC  to keep  then

existing  map or RETURN  to begin  using the  new map  or globe

view  selected.  This key  is functional  only when  AUTOMAP is

disabled.

SPECIAL FEATURE:

When using spherical  projections, changing globe views may  be

accomplished  more  rapidly  by  pressing  ctrl     or  ctrl

(control right arrow or control left arrow).

NOTE:  Depending on  what map  or  globe  view you  select, the

observation point may not be visible on the new map.

CTRL F7 - SELECT PROJECTION

This  feature enables you  to switch  between the  two types of

projections  available  for  displaying the  satellite position

User Guide                                                Page 25

PC-TRACK 2.14                       Simplified Satellite Tracking

(Mercator and Spherical).

If you cannot locate a satellite on  the screen when using  the

Mercator projection world maps, check to the latitude, it  will

most  likely be  above 80  deg (north  or south)  which is  not

displayed in that projection.

ALT F7 - CHANGE SIZE

This  feature changes  the  size of  the  projection  currently

being viewed.   When using spherical projection, this will just

change the size of the globe.

When using  mercator projections, this  will change the  entire

screen  layout as  described earlier. With a  small screen size

selected,  you will  see  the satellite,  AZEL,  observer,  and

status boxes.

F8 - SELECT NEW OBSERVER

This  allows  you  to  change  the  observation  point   during

tracking.  This brings  up the  same selection  of  observation

points which were available when setting  up to track. You  may

select a  new observation point simply  by pressing either  the

up  or  down  arrow  keys  until   the  name  of  the   desired

observation point  is shown. You may  then press  either ESC to

keep existing observer or RETURN to select the new observer.

NOTE: If you are tracking with  local times, the position(s) of

the satellite(s)  may change,  since the  time zone offset  for

the  new observation point  may be  different than  that of the

old one.  This will result in  a different  effective UTC time,

which is used to predict the satellite(s) position(s).

F9 - AUTOMAP

This  enables  the automatic  map/globe view  selection feature

which checks  the current  position of  the selected  satellite

and makes sure it is visible on  the currently displayed map or

globe. If  not, it will check the available maps  or globes and

select  the  next one  which  will  show  the  position of  the

satellite.

This  key works like  a toggle,  ie. pressing  once enables it,

pressing  again disables  it.  Whatever map  or globe  view was

visible  at the time  AUTOMAP was  disabled will  remain on the

screen  until AUTOMAP  is again enabled  or a new  map or globe

view is selected using F7.

User Guide                                                Page 26

PC-TRACK 2.14                       Simplified Satellite Tracking

F10 - SET DOS CLOCK

This key  will take the currently  displayed date  and time and

set the DOS clock with them.

This  allows you  to easily  change the  real time  clock to  a

desired date  and time. The date  and time are changed by using

the arrow keys.

NOTE:  If you set  the DOS  clock with  this option,  the clock

will retain that new time even after quitting PC-TRACK.

ARROW KEYS - CHANGE DATE, TIME, AND INTERVAL

When in  the FAST TRACK  mode, you  have the ability  to change

the date, time and interval to whatever you wish.

To make changes, use the left and right  arrow keys to move the

highlighted cursor  to whatever parameter  you wish to  change.

For  example after  pressing F2  and  getting into  FAST  TRACK

mode, the  highlighted cursor will not be visible, but pressing

the left  arrow key will highlight  the SECONDS  portion of the

INTERVAL.  This  shows that  you  may  now  either increase  or

decrease that number. Once the parameter  you wish to change is

highlighted,  you may  do  so by  using the  up and  down arrow

keys.

The  +  or  -  sign  associated  with  the  interval  shows the

direction in time  the interval is changing the computed  time.

If it  is + (positive) then the computed time will be increase,

if it is - (negative) then the computed time will decrease.

If  the interval is  set to  00:00:00, then  you may experiment

with the date and time and  compute the satellite position  for

a  specific point  in time and  study it while  PC-TRACK  keeps

computing  all  data. This  is  different  than the  pause (F3)

feature which freezes computations.

NOTE: When the increment is 00:00:00, the frequency computed

and  displayed  for  visible  satellites  using  the   small

Mercator  projection   will  not  show  the   doppler  shift

incorporated in the displayed value.

PGUP/PGDN - CHANGE SELECTED SATELLITE

This   changes   the   currently   selected   satellite   shown

highlighted in the SATELLITES box.

User Guide                                                Page 27

PC-TRACK 2.14                       Simplified Satellite Tracking

The currently  selected satellite  is the  one which will  have

its data sent to  the printer and  will be used in the  AUTOMAP

mode to determine the currently visible map/globe view.

User Guide                                                Page 28

PC-TRACK 2.14                       Simplified Satellite Tracking

TEXT

The TEXT TRACK option is provided for computers which do not have

an EGA graphics display capability.

NOTE:  If either  observer  or satellite  data has  not been

loaded due  to an  improper path designation  or some  other

reason, then tracking  will not begin  when you select  this

option.  An  error window will appear  indicating the reason

why tracking cannot begin. Pressing RETURN will put you back

at the TRACK window.  If this occurs, you must  either check

the paths or enter new data (for observers or satellites).

Selecting  this option  brings up  a screen  with three  items to

select;   The   satellite,   Observation  point,   and   printing

instructions.

As described  earlier, your selections  are made by  simply using

either  the Up  or  Down  Key.  Once  the  desired  satellite  or

observation point is  visible, press  RETURN. To go  back to  the

MAIN MENU, press the ESC key.

The  initial  satellite  shown will  be  the  number one  default

satellite  (the top one) identified in  the CONFIG SYSTEM option.

The initial observation point will also be the default.

After  selecting which  satellite  to track  and the  observation

point, you will be asked to define what data is to be printed (if

printing  is enabled) during track.  You have a  choice of either

printing every point computed or only those which have elevations

greater than  the minimum elevation defined  for that observation

point. The  desired answer is again  provided by using the  Up or

Down keys and pressing RETURN.

Tracking will then begin.

User Guide                                                Page 29

PC-TRACK 2.14                       Simplified Satellite Tracking

TEXT TRACKING SCREEN

|---------------------------------------------------------------|

|                                                               |

|        CURRENT TRACKING DATA FOR OSCAR 10                     |

|     CURRENT OBSERVATION POINT: Dayton, Ohio                   |

|   39.45N Lat    84.15W Lon     Time Zone:  -5                 |

| MINIMUM ELEVATION:  2.00   HEIGHT ABOVE M.S.L.:  500 (Meters) |

| ------------------------------------------------------------- |

|  Date                Time (UTC)        Interval               |

|  June 13, 1990       07:43:14          + 00:10:00             |

|    ********** S A T E L L I T E   D A T A **********          |

| Revolution Number     Phase                                   |

| 2469                   31                                     |

| Latitude             Longitude                 Altitude (KM)  |

|   82.1 S                67.35 W                   2950        |

|                                                               |

|     ********** T R A C K I N G   D A T A ***********          |

|Azimuth              Elevation               Slant Range (KM)  |

|  155.55               11.50                      4923.4       |

|            Range Rate (KM/Sec)    Frequency (MHZ)             |

|            2.0                      145.808413                |

|        FAST TRACK          PAUSING            PRINTING        |

|                       PRESS F1 FOR HELP                       |

|                   PRESS ESC TO QUIT TRACKING                  |

|---------------------------------------------------------------|

This  screen displays essentially  the same information displayed

by the graphics screen  (except for only one satellite).  The top

of  the screen  displays  the  name  of  the  satellite  and  the

observation  point information.  This information  is simply  the

information entered using the SETUP OBSERVER option.

Revolution number  and Range  Rate are  provided on this  screen,

while they  are not in the graphics tracking mode. The range rate

is the change in  distance between the observation point  and the

satellite. If  it is positive, the distance  is increasing. Range

rate determines the doppler shift of the downlink frequency.

FUNCTION KEYS AVAILABLE

There  are several  function  keys available  to control  various

features of PC-TRACK  which affect the tracking process. They are

described as follows...

User Guide                                                Page 30

PC-TRACK 2.14                       Simplified Satellite Tracking

F1 - HELP

This  is the  key which  brings up  the PC-TRACK   ASSISTANT as

described earlier.

F2 - REAL TIME / FAST TRACK

This key controls  the tracking mode of PC-TRACK. When tracking

first  begins, it is  in the  REAL TIME TRACK  mode. This means

the  position   displayed  represents   the  actual   satellite

position according to the time set  in the computer (DOS time).

The displays will be updated at the time interval shown in  the

TIME INTERVAL (fixed to 1 second).

Pressing the F2 key changes  this to the FAST  TRACK mode. This

mode ignores the clock in the  computer and simply computes and

displays  satellite  positions  as  fast as  it  can.  The time

between each point computed is  determined by the time interval

shown  in  TIME INTERVAL.  This  time  interval  is  changeable

according to the procedures described below.

The F2 key acts as a "toggle". This means  that if you press it

once, the mode changes to FAST TRACK. When you press it  again,

the  mode changes back  to REAL  TIME TRACK.  Pressing it again

changes the mode again to FAST TRACK, etc.

F3 - PAUSE TRACKING

This  key  may  be  pressed  at  any  time  during  tracking to

"freeze" the  display. This stops  all computations and  simply

waits for the F3  key to be pressed  again. Once it  is pressed

again,  normal tracking resumes.  While in  the pause mode, you

may make changes to the time  increment (described below), turn

the printer  on or  off, and  select Real  Time  or Fast  Track

mode.

F4 - ENABLE PRINTER

This  key enables output to  be sent to  the printer during the

tracking  process. The amount  of data  sent to  the printer is

determined by  your response  to the  prompt during the  SELECT

process just  before tracking  began, either  all data  or just

points with elevations above the minimum elevation.

NOTE: If PC-TRACK  encounters a problem trying to print (out

of  paper,  off-line,    turned  off,  etc.)  printing  will

automatically be  disabled, and the PRINTING  message at the

lower portion  of the  screen will disappear.  Tracking will

continue.

User Guide                                                Page 31

PC-TRACK 2.14                       Simplified Satellite Tracking

The F4  key operates as a toggle just as the F2 and F3 keys. If

printing is enabled, just press F4 again to turn it off.

ARROW KEYS - CHANGE DATE, TIME, AND INTERVAL

When in  the FAST  TRACK mode, you  have the ability  to change

the date, time and interval to whatever you wish.

To make changes, use the left and right arrow keys to move  the

highlighted cursor  to whatever parameter  you wish to  change.

For  example after  pressing  F2  and getting  into FAST  TRACK

mode, the highlighted cursor will not  be visible, but pressing

the left  arrow key will highlight  the SECONDS  portion of the

INTERVAL.  This  shows that  you  may  now either  increase  or

decrease that number. Once the parameter  you wish to change is

highlighted,  you may  do so  by using  the up  and down  arrow

keys.

The  +  or  -  sign  associated  with  the  interval  shows the

direction in time the  interval is changing  the computed time.

If it is + (positive) then the computed time will be  increase,

if it is - (negative) then the computed time will decrease.

If  the interval is  set to  00:00:00, then  you may experiment

with the date and time and  compute the satellite position  for

a specific point  in time and  study it while  PC-TRACK   keeps

computing  all data.  This is  different  than the  pause  (F3)

feature which freezes computations.

NOTE: When the increment is 00:00:00, the frequency computed

and displayed for  visible points will not  show the doppler

shift incorporated in the displayed value.

User Guide                                                Page 32

PC-TRACK 2.14                       Simplified Satellite Tracking

CONFIGURATION

This module allows you to modify certain things about the program

which affect the  way the  program operates. The  things you  can

change are  text screen  colors, printer control  codes, distance

and time units, and the file paths.

CONFIGURE DISPLAY

This option allows you to easily change the colors you see on the

screen displays. This way you can get the screen displays to look

the way you want. You  are able to see the effects of  changes on

the screen  while you make them. You can make changes to both the

background and text colors with simple  key strokes. These colors

are  effective  throughout  PC-TRACK  ,  including  the  graphics

screen.  The only  colors which  are not  affected are  the maps,

globe  views, the AZEL chart  displays and the satellite position

marks on them.

NOTE:  While  graphics  mode  tracking  is  not  possible on  a

monochrome  video system,  all of  the  rest  of PC-TRACK   is,

including  the  text   base  track  screen.  Using  the   color

configuration  option  here  will  enable  the  colors  to   be

manipulated  until the  correct  gray shades  are  achieved  in

order to see the display adequately.

Here's how to do it...

Information is displayed  on the screen  in the form of  a matrix

(table) using  7 different categories for the text and background

colors...

- NORMAL

- HIGHLIGHTED

- DATA ENTRY

- TITLE

- SCREEN COLOR

- SHADOW COLOR (Background only)

Only  the combinations used in PC-TRACK  are shown on the matrix.

The  colors  for  both the  background  and  text  colors can  be

selected for each of these categories.

Selecting Text Color Category

User Guide                                                Page 33

PC-TRACK 2.14                       Simplified Satellite Tracking

Using the Left and Right arrow keys selects the category for text

color changes. The currently selected category is shown by a down

arrow pointing to it along the top of the matrix.

Select Background Category

Use the  Up  and  Down arrow  keys  to select  the  category  for

background  color changes.  The  currently  selected category  is

shown by a right arrow  pointing to it along the left side of the

matrix.

Change Text Color (F5)

Pressing this  key changes  the color of  the currently  selected

text category. There are 16 colors to choose from.

Change Background Color (F6)

Pressing this key changes the color of the currently selected

background  category. There are 2 sets of 8 identical colors. One

set causes the  text to  be non-blinking while  the other  causes

blinking text.

User Guide                                                Page 34

PC-TRACK 2.14                       Simplified Satellite Tracking

CONFIGURE SYSTEM

This option allows you  to define the default observation  point,

map, and satellites to  use in the TRACK option. This will enable

to you set up PC-TRACK  to track the satellites you want and then

not have  to  go through  the process  again unless  you wish  to

change  it. This reduces the  effort needed to  begin tracking to

just  a few keystrokes. It  also allows you  to define whether or

not to  use metric measurements, Coordinated  Universal Time, and

where PC-TRACK  looks on  your disk to find the files it needs to

operate with.

DEFAULT OBSERVATION POINT

The  default observation  point  is selected  first. The  initial

observation  point to  appear  is either  the  first one  in  the

observer  database  (if none  is  already selected),  or  the one

previously selected. To change that observation point, use the up

and down arrow keys  to select the new one  desired. Press RETURN

to proceed or ESC to finish  making changes in the CONFIG  SYSTEM

option.  The observation points displayed will be the ones in the

OBSERVER database defined using the SETUP OBSERVERS option.

NOTE: If there are no observers in the database (either none

entered yet or the  DATA PATH is incorrect), this  part will

be  skipped.  You  will not  be  able  to  select a  default

observer.

DEFAULT MAP

Once  you  select  an  observation  point,  the  default  map  is

identified. To  select a map,  simply use  the up and  down arrow

keys to  show the names of  the available maps. When  the one you

want is visible, press the  RETURN key to proceed or the  ESC key

to finish up with CONFIG SYSTEM.

NOTE: If there are no maps in the MAP PATH identified below,

this part will be skipped. You will not be able  to select a

default map.

DEFAULT GLOBE VIEW

Once you select a  map, the default globe view is  identified. To

select another globe view, simply use the up and  down arrow keys

to show the names of  the available views. When the one  you want

User Guide                                                Page 35

PC-TRACK 2.14                       Simplified Satellite Tracking

is  visible, press the  RETURN key to  proceed or the  ESC key to

finish up with CONFIG SYSTEM.

NOTE:  If there  are no  globes in  the MAP  PATH identified

below, this  part will be skipped.  You will not be  able to

select a default globe view.

DEFAULT SATELLITES

PC-TRACK   is  capable of  storing up  to 200  satellites in  its

database (using SETUP SATELLITES) and tracking up to 8 satellites

simultaneously. This option selects  up to 8 of the  possible 200

to be initially selected for GRAPHICS TRACK.

There are 8 "slots" available to select satellites. Each slot may

have  any one of the  satellites defined in  the SETUP SATELLITES

option. You are initially positioned in slot number 1.

NOTE:  If there are  no satellites  in the  database (either

none entered yet or  the DATA PATH is incorrect),  this part

will be skipped. You will not  be able to select any default

satellites.

At this point you may:

Press the Page Up or  Page Down key. This will  move you to the

next slot  up or  down. If  you press  the page  down key,  the

cursor will move to  the next slot down, and the next satellite

down in the database will be showing. If you press the page  up

key, the  cursor will move  to the next  slot up  (will wrap to

the bottom  if you  are at the  top) and  display the  previous

satellite  in the database  (will also  wrap the  bottom of the

database if at the top).

Press the Up or Down arrow key.  This will change the satellite

displayed to  the next  or previous satellite in  the database.

The up  arrow will  select previous  satellite, the  down arrow

key will select the next.

Press the  Home Key  to clear  the  current slot  and move  the

cursor to the previous slot. This is handy  to unselect a slot.

You  may fill  any or  all slots  with satellites.  You do  not

necessarily have to fill them all.  You may have blanks  slots.

For  example,  you  could  set  it  up  so  there  are  only  2

satellites  to track, one in slot 2 and one  in slot 7. It's up

User Guide                                                Page 36

PC-TRACK 2.14                       Simplified Satellite Tracking

to you.

Press RETURN  to complete the  selection of default  satellites

and move on to the PATH.

Press ESC to finish CONFIG SYSTEM.

Remember, the Page Up and Page  Down keys select the  slot, the

arrow  keys select the satellite to go into  the slot, the home

key clears  a slot,  and the  RETURN key  finishes the  default

satellite selection process.

DATA PATH

This  line tells PC-TRACK   what disk drive  and what directories

and subdirectory  on that disk to use as the storage area for the

data files it uses. This line of  text will be in the form of  an

MS-DOS path name. If  you have PC-TRACK  on your  C drive and you

have a special subdirectory  called "\TRACK\DATA", then you would

enter  "C:\TRACK\DATA\" on  this line. The  path defined  here is

where PC-TRACK   will look  for the OBJECTS.DAT  and OBSERVER.DAT

files.

NOTE: If  you enter a non-existent  path, an  error window will

appear and  will indicate that fact.  You must  press RETURN at

this point and try again. If you cannot remember  what the path

is, you  can just clear  the line out  by using the  BACKSPACE,

DELETE, OR CTRL X keys. This will specify the default path.

MAP PATH

This  line tells PC-TRACK   what disk drive  and what directories

and subdirectory  on that disk to use as the storage area for the

map and globe files  it uses. This line of  text will also be  in

the form of an MS-DOS path name. This is entered exactly the same

as described above.

See the note above on non-existent paths.

METRIC SYSTEM

This tells PC-TRACK  to use either metric (meters and kilometers)

or English  (feet and  statute miles) measurements  for distance.

Press the  INSert key to change  the answer or RETURN  to move to

another parameter, and ESC to finish the CONFIG SYSTEM option.

User Guide                                                Page 37

PC-TRACK 2.14                       Simplified Satellite Tracking

UTC TIME

This tells  PC-TRACK  to interpret the date and time displayed as

Coordinated Universal Time  (UTC) or Local Time.  Either 'UTC' or

'LOCAL'  will appear. When using  local time, PC-TRACK   uses the

time  zone defined for the selected  observation point to compute

UTC  time. This  time  zone is  defined  in the  SETUP  OBSERVERS

option.  If you select Local time display, PC-TRACK  will add the

time zone from the displayed time to get UTC time, since PC-TRACK

must use  UTC  time internally  to  compute satellite  data.  All

displays and printouts, however, will reflect the local time.

FINISHING UP

As described above, you may press  ESC any time to end the CONFIG

SYSTEM option. You  will be  asked if everything  is correct.  At

this point you may enter:

'Y' or RETURN to  save the changes you have made and go back to

the CONFIG menu.

'N' to go back and continue to make more changes.

ESC  to quit  without saving  the  changes and  go back  to the

CONFIG menu.

User Guide                                                Page 38

PC-TRACK 2.14                       Simplified Satellite Tracking

CONFIGURE PRINTER

This option allows you to  change the control codes sent  to your

printer to make it print the PC-TRACK  reports correctly.

Printer Codes

Printer codes are sequences of special control characters sent to

your printer. PC-TRACK  displays and accepts them from you as the

decimal  equivalents of  each  control character  separated by  a

slash (/). For example, the ESC character has a  decimal value of

27 so it is  shown on the screen as the  number "27". Notice each

print command  has a code sequence to turn it  on and one to turn

it  off. Some  codes  do not  need  to be  turned  off, so  those

portions are left blank.

Some  printers may  not  be capable  of  printing some  of  these

features.  In that  case,  leave the  CODE  TO TURN  ON  for that

feature blank.  You have up to 20  characters (including slashes)

to define each of the codes.

You may move the cursor around on the screen to  the desired code

sequence by using the arrow keys or just pressing RETURN.

User Guide                                                Page 39

PC-TRACK 2.14                       Simplified Satellite Tracking

APPENDIX A

OBTAINING SATELLITE INFORMATION

The  information  required  by PC-TRACK    can  be obtained  by

requesting  the  "NASA  PREDICTION  BULLETIN" or  the  "TWO  LINE

ORBITAL ELEMENTS" for each  of the satellites desired, referenced

by CATALOG NUMBER from:

PROJECT OPERATIONS BRANCH (CODE 513)

NASA/GODDARD SPACE FLIGHT CENTER

GREENBELT, MD. 20771

You  may  also  request a  package  of  prediction bulletins  for

certain categories of satellites such as weather or amateur radio

satellites rather than by individual CATALOG NUMBERS.

If you do not know the  CATALOG NUMBER of the satellites you wish

to  order  data  for,  you  can  request the  current  "SATELLITE

SITUATION  REPORT".  To the  best of  my  knowledge, there  is no

charge for  this document and it  can be obtained from  the above

address.

The  Satellite  Situation  Report  lists  almost  every satellite

currently  in   orbit.  Each   satellite  is  cataloged   by  the

International Designation  and by  Catalog number. It  also lists

the name,  launch date,  orbital period in  minutes, inclination,

apogee,  perigee,  and  transmitting  frequency.  By  using  this

document you can determine the catalog numbers of the  satellites

you wish to track with PC-TRACK.

User Guide                                                Page 40

PC-TRACK 2.14                       Simplified Satellite Tracking

APPENDIX B

HOW TO USE THE ORBITAL DATA FROM NASA

The  data required  by PC-TRACK   is  contained in  the Two  Line

Orbital  Elements. As described above, these  elements are a part

of the NASA  Prediction Bulletin  described in Appendix  A.   The

following is  an example of  the format of  the two line  orbital

elements received from NASA.

1 13923U          90153.41230725 0.00000215           11423-3 0  376

2 13923  98.7483 183.1938 0017844  74.4610 285.8536 14.22303248  9340

Here  are  the pieces  that PC-TRACK    requires, broken  out for

you...

LINE 1

EPOCH YEAR         PERIOD DECAY

||             RATE  |

||             ----------

1 13923U          90153.41230725 0.00000215           11423-3 0   376

|               ------------                      ---------------

|                     |                                  |

|               EPOCH DAY.FRACTION                       |

|                                                     NOT USED

----- CATALOG NUMBER

(ignore letter)

LINE 2

CATALOG

NUMBER

|     INCLINATION     ECCENTRICITY   MEAN ANOMALY

|       |                 |              |

|       |  RIGHT ASC.     |  ARGUMENT    |     MEAN MOTION

|       |  OF NODE        |  OF PERIGEE  |       |

----|  ------- -------- -------  ------- -------- -----------

2 13923  98.7483 183.1938 0017844  74.4610 285.8536 14.22603248  9340

---

|

REVOLUTION NUMBER --|

AT EPOCH (ignore last digit)

Notice that there  is no decimal point printed  for eccentricity.

The decimal point goes  in front of the number. For  example, the

number shown above for eccentricity would be entered as .0017844.

User Guide                                                Page 41

PC-TRACK 2.14                       Simplified Satellite Tracking

APPENDIX C

GLOSSARY

The  following  are some  terms  used in  the  satellite tracking

business and are described in layman's terms.

APOGEE

The  point  in  a  satellite's orbit  farthest  from  the Earth's

center.

ARGUMENT OF PERIGEE

The  angular  distance  measured  in  the  orbit  plane,  in  the

direction  of  motion  of  the  satellite,  from  the   point  of

intersection of the  orbit and equatorial planes, to  the perigee

point.  The perigee point  is the  point in  the orbit  where the

satellite is closest to the earth's surface.

ASCENDING NODE

Point at  which the satellite  crosses the equatorial  plane from

the  southern hemisphere  to the  northern hemisphere.  (See also

RIGHT ASCENSION OF THE ASCENDING NODE.)

AZIMUTH

The angle measured in  the plane of the  horizon from true  North

clockwise to the vertical plane through the satellite.

CATALOG NUMBER

A 5 digit number assigned to a cataloged orbiting satellite. This

number may be found in the NASA Satellite Situation Report and on

the NASA Prediction Bulletins.

COORDINATED UNIVERSAL TIME (UTC)

Also  known  as Greenwich  Mean Time  (GMT).  Local time  at zero

degrees     longitude at the Greenwich Observatory, England. Uses

24 hour clock, ie. 2:00 pm is 1400 hrs.

CULMINATION

The  point at which a  satellite reaches its  highest position or

elevation  in the sky relative to an observer. (Also known as the

Closest Point of Approach)

DECLINATION

The angular distance  from the equator to  the satellite measured

positive north and negative south.

User Guide                                                Page 42

PC-TRACK 2.14                       Simplified Satellite Tracking

DRAG

The  force  exerted on  a satellite  by  its passage  through the

atmosphere of the Earth, acting to slow the satellite down.

ECCENTRICITY

The  degree of flattening  of the orbit  or its  departure from a

circle.  The greater  the eccentricity,  the more  elliptical the

orbit is.

ELEMENT SET

See ORBITAL ELEMENTS.

EPHEMERIS

A tabulation  of a series of points which define the position and

motion of a satellite.

EPOCH

A specific time  and date which is used as  a point of reference;

the time  at  which  an element  set  for a  satellite  was  last

updated.

EQUATORIAL PLANE

An  imaginary plane running through  the center of  the earth and

the Earth's equator.

INCLINATION

The  angle between  the orbit  plane and  the Earth's  equatorial

plane,  measured counter-clockwise. 0  (zero) degrees inclination

would  describe a satellite orbiting in the same direction as the

Earth's  rotation  directly  above  the equator  (orbit  plane  =

equatorial  plane).   90  degrees  inclination   would  have  the

satellite orbiting directly over  both poles of the  earth (orbit

plane  displaced  90  degrees  from  the  equatorial  plane).  An

inclination  of 180  degrees  would have  the satellite  orbiting

again directly over the equator, but in the opposite direction of

the Earth's rotation.

INTERNATIONAL DESIGNATOR

An internationally agreed upon  naming convention for satellites.

Contains  the  last two  digits of  the  launch year,  the launch

number of the year  and the piece of the  launch, ie. A-indicates

payload, B-the rocket booster, or second payload, etc.

LATITUDE

Also  called  the  geodetic   latitude.  the  angle  between  the

perpendicular  to the Earth's surface (plane of the horizon) at a

User Guide                                                Page 43

PC-TRACK 2.14                       Simplified Satellite Tracking

location and the equatorial plane of the earth.

LONGITUDE

The angular  distance from the Greenwich  (zero degree) meridian,

along the  equator. This can is  measured either east or  west to

the  180th meridian (180  degrees) or 0 to  360 degrees west. For

example,  Ohio includes  85 degrees  west longitude,  while India

includes 85 degrees east longitude. But 85 degrees east longitude

could also be measured as 275 degrees west longitude.

MEAN ANOMALY

Angular distance measured in the orbit plane, in the direction of

motion, from the perigee point to the satellite's mean position.

MEAN MOTION

The number of complete  revolutions a satellite makes in  a given

unit  of time, usually measured in revolutions per day. (see also

DECAY)

NASA PREDICTION BULLETINS

Report published  by NASA  Goddard Space Flight  Center providing

latest orbital information on a particular satellite. This report

gives  information in 3 parts:  1) two line  orbital elements, 2)

Longitude  of the  South  to North  equatorial crossings,  and 3)

longitude  and  heights  of  the satellite  crossings  for  other

latitudes (besides the 0 degree latitude shown in part two).

OBJECT

A satellite. Something in space to be tracked.

ORBITAL ELEMENTS

Also called Classical Elements,  Satellite Elements, Element Set,

etc.  Includes the catalog Number; epoch  year, day, and fraction

of  day; period  decay  rate; argument  of perigee,  inclination,

eccentricity; right  ascension of  ascending node; mean  anomaly;

mean motion; revolution number at  epoch; and element set number.

This  data is contained in the TWO LINE ORBITAL ELEMENTS provided

by NASA in the NASA Prediction Bulletin.

PERIOD DECAY RATE

Also known as Decay. This is the tendency of a  satellite to lose

orbital velocity due to the influence of atmospheric drag and

gravitational forces.  A  decaying satellite  eventually  impacts

with the surface of the Earth or burns up in the atmosphere. This

parameter directly  affects the satellite's  MEAN MOTION.   It is

measured in  various  ways but  PC-TRACK  , the  NASA  Prediction

User Guide                                                Page 44

PC-TRACK 2.14                       Simplified Satellite Tracking

Bulletins and the Two Line  Orbital Elements use revolutions  per

day per day.

PERIGEE

The point  in the satellite's  orbit where  it is closest  to the

surface of the earth.

POSIGRADE ORBIT

Satellite motion which is  in the same direction as  the rotation

of the Earth.

RETROGRADE ORBIT

Satellite motion which  is opposite in direction  to the rotation

of the Earth.

REVOLUTION NUMBER AT EPOCH

The  number  of revolutions  or  ascending node  passages  that a

satellite  has completed at the  time (epoch) of  the element set

since  it was launched. The orbit number from launch to the first

ascending  node  is   designated  zero,  thereafter  the   number

increases by one at each ascending node.

RIGHT ASCENSION OF THE ASCENDING NODE

The angular distance from the vernal equinox measured eastward in

the  equatorial plane to the  point of intersection  of the orbit

plane where the satellite crosses the equatorial plane from south

to north.

SATELLITE SITUATION REPORT

A report  published by NASA  Goddard Space Flight  Center listing

all known  man-made Earth orbiting satellites.  This report lists

the Catalog  Number, International  Designator, Name,  Country of

origin,   launch  date,   orbital  period,   inclination,  beacon

frequency, and status (orbiting or decayed).

SUB-SATELLITE

Location of the point of intersection of the earth's surface by a

line between the center of the earth and the satellite (the point

on the earth directly beneath the satellite).

TWO LINE ORBITAL ELEMENTS

See ORBITAL ELEMENTS.

VERNAL EQUINOX

Also known as the first point of Aries, being the point where the

Sun crosses the Earth's equator going from south to north in  the

User Guide                                                Page 45

PC-TRACK 2.14                       Simplified Satellite Tracking

spring.  This point is fixed and represents the reference axis of

a coordinate system used in Astronomy and Astrodynamics.

User Guide                                                Page 46