WHATVGA version 1.50  17/jan/94

Copyright 1991,92,93,94   Finn Thgersen

All Rights Reserved

Command line switches:

whatvga [+acumos] [-ncr] [=1024] [/a]

where +chipset  forces WHATVGA to proceed as if this chipset was found.

The test for this chip set will be executed in order

to check for memory and revision.

No other chipsets will be tested.

-chipset  ensures that the tests for this chipset will NOT be

executed. Useful if a chipset is misidentified

or some earlier test gives problems.

=kbytes   Forces the number of Kbytes of video RAM on the

adapter to the specified value.

/a        Run automatic test and write results to a file.

The user responds with Y or N to each test and can

enter comments to the individual tests.

On startup you may be asked to select the video system

to test (say if you have a VESA driver loaded).

If the program seems to halt for more than a few seconds

try to press the enter key.

The recommanded way to use the program is to start it

with no command line parameters and play with it a bit.

See that the chipset, memory and DAC are ID'd properly

and that you can display at least some super VGA modes.

Then run the program with /a to do the full test.

For the /a test, the system should be as "clean" as

possible, as TSRs (including mouse and keyboard drivers)

and memory managers can sometimesaffect the results.

Try booting from a floppy.

Comments should only be entered when something is wrong

or needs clarification. Ie. there is no need to enter

"Fine" or "All ok" thats what the yes/no queries are for.

When something is wrong try to be as descriptive as

possible (remember, I can't see the screen).

Note that the files CHIPS.LST and WHATVGA.LST must be in the current

directory for WHATVGA to functiion.

If you have problems with the identification/startup:

- The screen goes blank or all white...

Try to run: WHATVGA /d

This will display the name of each chipset and waits for

you to press the Enter key BEFORE the chipset is tested.

When you have found the offending chipsets, try to run:

WHATVGA -Chips&Tech     (assuming Chips&Tech is the one)

Report the problem.

- The chipset is misidentified.

Try to run: WHATVGA -chipset

where chipset is one of the following:

Chips&Tech    Chips & Technologies

Paradise      Paradise/Western Digital

Video7        Video7

ET3000        Tseng Et3000

ET4000        Tseng ET4000, ET4000/W32

Trident       Trident 8800/8900/9000

Everex        Everex

ATI           ATI VGA Wonder/Graphics Ultra +/Pro

Genoa

Oak           Oak 37/67/77/87

Cirrus        Cirrus 5x0/6x0

Ahead         Ahead A&B

NCR           NCR 77c2x

Yamaha        Yamaha 6388

S3            S3 86c series

AL2101        Avance Logic ALG2101

MXIC          MXIC 860x0

Realtek       Realtek

PRIMUS        Primus P2000

Cirrus54      Cirrus CL-GD54xx

Cirrus64      Cirrus CL-GD64xx

Weitek        Weitek W5x86

COMPAQ        Compaq IVGA/AVGA/QVision

UMC           UMC 85c408

HMC           HMC HM86304

- Memory Size is identified wrong.

Run WHATVGA =2048    (or how many Kbytes you have)

Report the problem.

Whatvga has 6 options in the main menu:

1 Test standard VGA

Allows you to test the standard VGA video modes.

Draws a test pattern, including a color palette

and writes basic information as text on the display.

See 2) for the details.

2 Test extended modes

Allows you to test the extended video modes of the adapter.

Draws a test pattern, including a color palette

and writes basic information as text on the display.

On the upper and left edge every hundred pixels is marked.

when you press a key the testpattern is drawn again, this time

in the largest size the available video memory will support

("Virtual Screen"), allowing you to scroll around in the display

using the arrow keys. You should be able to move in the entire

pattern, but not to move so far right or down that the display wraps.

If you can move past the right edge (or can't reach it) then the

scroll function probably needs a factor 2 (divide or multiply)

in the SetVStart function.

When you press the Enter key a textscreen is shown with the

statistics for the mode, this screen also shown if the rutine

to analyse the mode from the register values (AnalyseMode) reaches

the same result. If not the values are shown as "Calculated".

By pressing "d" or "D" you can see a dump of all the VGA (and extended)

registers. The registers can be saved (appended) to the file REGISTER.VGA

by pressing "f" or "F".

3 Search for modes

Sequentially attempts to set modes 14h to 7Fh, each time testing

if the mode was succesfully set. If so the testpattern is drawn

and the mode is analysed.

5 Hardware Cursor test

This item attempts to place a 32x32 hardware Cursor in the

selected mode. The cursor can be moved around on the screen

with the arrow keys.

6 BitBLT test

This item first draws the testpattern, then draws the color palette

with BitBLT operations, then makes two blockcopies via BitBLT of

the two palettes, and finally draws a series of rectangles each smaller

than the last to test precision (you need to press a key for each box)

7 Line Draw test

This item attempts to draw lines from the center of the screen

in a "wheel" using different colors.

8 Read/Write bank test

This item copies the area with the color palette to the lower half

of the screen using seperate read/write banks.

0 Exit

Items 5-8 are only present if I believe the adapter supports the function,

though you can always try the test by pressing the key.

Special functions for test purposes (may hang your PC or worse):

B Register Bit test.

You are asked for the base register (such as 3C0h, 3C4h, 3CEh or

3d4h), starting index and starting bit number (0-7).

Then you select the mode for the test.

Now the test pattern is drawn in the desired mode, but using the

entire video memory (like the "virtual screen").

The selected bit will be changed for ~0.5 sec, then restored for

another 0.5 sec. This repeats until another bit is selected by

-, + or * or the test is stopped by the Esc key.

This can be used figure out the function of individual bits, the

Display Start extension bits being the obvious example (and the

intended target), but also clock bits etc.

!!NOTE!! this may hang your PC or expose your video card or monitor

to conditions beoynd their intended operational specifications.

While damage to your monitor or video card is extremely unlikely

(I haven't heard of a confirmed case since the old IBM MDA monitor),

it is theoretically possible and my insurence company refuses to

recognize any responsibility what so ever :-).

Hanging your PC is however a very real possibility.

Note the bit you were testing, reboot (power off may be required)

and restart from the NEXT bit.

D DAC 6/8 bit test

A test pattern is drawn in the selected 256color mode.

The palette is set up to use the full 256 of 16m palette (256

shades of each basic color). When the DAC is in 6bit mode there

are only 64 shades, thus the color bars will "break".

The keys 6 and 8 switches between 6bit and 8bit DAC mode

(if the 8bit DAC mode works :-) ).

Esc or Enter terminater the test.

Currently this only works for the AT&T,Brooktree and Sierra 24bit DACs.

The tests:

ID:      Is the Chip Set, DAC and video memory identified correctly?

Xres:    Are the modes (standard and extended) displayed correctly?

Display type, Switch settings and BIOS versions can cause

some modes to be unavailable or out of sync.

Scrl:    Can you display and scroll in the Virtual Screen ?

You should be able to move in the entire virtual screen,

but not to move so far right or down that the display wraps.

If you can move past the right edge (or can't reach it)

we probably miss a factor of two in the SetVStart function.

If the display wraps when the start crosses the middle (or 1/4)

of memory we have a problem with the high bits (the Y variable)

in the SetVStart function or with an enable bit.

Also part of video memory can be inaccessible in modes that

normally only use <half the memory causing the testpattern to

be overwritten.

Anal:    Are the modes (particularly the extended ones) analysed

correctly (Ie are there no "Calculated" fields) ?

Some modes, such as 2 and 4 color and mode 13h are

hard to ID on many adapters.

Cur:     Is the HardWare cursor shown correctly and can it be moved

around the display with the arrow keys ?

The cursor should look like a snipers sights with a dot in the

upper left corner.

Blit:    Is the second palette drawn correctly ? Are the palettes

copied correctly ? Are the boxes placed correctly

(each one pixels smaller in each direction) ?

Line:    Are the lines drawn from the middle to fill a rectangle ?

RWbank:  Is the palette (and text) copied correctly ?

Adapters tested:

ID: Xres: Scrl:  Anal:  Cur:  Blit:  Line:  RWbank:

Acumos:  See Cirrus 5401 and 5402

Ahead:

A

B             Ok  Ok*   Ok     Ok                         Ok

ATI:

18800         Ok  Ok    Ok     Ok                         N/A

18800-1

28800-2       Ok  Ok    Ok     Ok                         Ok

28800-4

28800-5

68800         Ok  Ok*   Ok     Ok                         Ok

Avance Logic:   .   .     .      .      .     .      .      .

AL2101        Ok  Ok    Ok     Ok     Ok*   Ok%    Ok%    Ok

Chips&Tech:

82c450

82c451

82c452        Ok  Ok*   Ok     Ok     Ok

82c453

82c455

82c456

82c457

F65520

F65530

Cirrus:         .   .     .      .      .     .      .      .

510/520       Ok                            N/A    N/A    N/A

610/620                                     N/A    N/A    N/A

5401(AVGA1)   No

5402(AVGA2)   Ok  Ok    Ok     Ok     Ok

5410

5420          Ok  Ok    Ok     Ok     Ok%^  N/A    N/A    N/A

5422          Ok  Ok    Ok     Ok     Ok^   N/A    N/A    N/A

5424          Ok  Ok    Ok     Ok     Ok^   N/A    N/A    N/A

5426          Ok  Ok    Ok     Ok     Ok^                 N/A

5428

543x

6205

6215

6225

6235

6410          Ok  Ok    Ok     Ok

6412

6420          Ok  Ok*   Ok     Ok                         No

6440

Compaq:

IVGS

AVGA

QVGA          Ok  Ok    Ok*    *      Ok    Ok*^

QVGA1280

Everex:

Genoa:          .   .     .      .      .     .      .      .

51/5200

53/5400

6100

62/6300

64/6600

7900

HMC:

HM86304       Ok  Ok*   Ok     Ok

IIT:

AGX-14

AGX-15

MXIC:

86000         Ok  Ok    Ok     Ok                         Ok

86010         Ok  Ok    Ok     Ok                         Ok

NCR:

77C20

77C21

77C22E        Ok  Ok    Ok     Ok     Ok#                 Ok

77C22E+

Oak:            .   .     .      .      .     .      .      .

OTI-37C       Ok

OTI-067       Ok  Ok    Ok     Ok                         Ok

OTI-077       Ok  Ok*   Ok     Ok                         Ok

OTI-087       Ok  Ok*   Ok     OK                         OK

Paradise/Western Digital:

PVGA1A        Ok  Ok

WD90C00       Ok  Ok    Ok     Ok

WD90C10

WD90C11

WD90C2x

WD90C30       Ok  Ok    Ok     Ok                         Ok

WD90C31       Ok  Ok    Ok     Ok     Ok*   Ok*           Ok

WD90c33       Ok  Ok    Ok*    Ok     Ok*                 Ok

Primus:

P2000         Ok  Ok    Ok*    Ok           Ok%           Ok

Realtek:

RT3106        Ok  Ok    Ok     Ok                         Ok

S3:             .   .     .      .      .     .      .      .

86c911                                                    N/A

86c911A/24    Ok  Ok    Ok     Ok     Ok    Ok     Ok     N/A

86c801                                                    N/A

86c805                                                    N/A

86c928                                                    N/A

Trident:

8800BR                                                    N/A

8800CS                                                    N/A

8900B         Ok  Ok    Ok*    Ok                         N/A

8900C         Ok  Ok    Ok*    OK                         N/A

9000                                                      N/A

8900CL        Ok  Ok    Ok     Ok

9000i         Ok  Ok    Ok     Ok

Tseng:          .   .     .      .      .     .      .      .

ET3000        Ok  Ok*   Ok     Ok                         Ok

ET4000        Ok  Ok    Ok     Ok                         Ok

ET4000W32     Ok  Ok    Ok     Ok     Ok%^                Ok

ET4000W32i

UMC:

UMC85c408     Ok  Ok    Ok     Ok                         Ok

Video7/Headland:

HT-208

HT-216

Weitek:

W5086

W5186         Ok* Ok    Ok*    Ok

W5286

P9000

Yamaha:

6388

* Note

# Not in 16 color modes

$ Not in 256 color modes

% Not in 32k/64k color modes

^ Not in 16m color modes

ATI Graphics Ultra Pro:

Mode 13h has problems above 64Kb.

Can't get the cursor to work yet

Avance Logic AL2101:

In interlaced modes the cursor is twice the normal size.

While copy via BitBLT works in HiColor modes, Fill does not.

LIne draw doesn't work in HiColor modes.

Chips&Tech 82c450:

There appears to be problems identifying this chipset

(black screen...).

Chips&Tech 82c452:

Mode 13h apparently can't cross the 64k line

Cirrus CL-GD54xx:

Mode 13h can't cross the 64k limit

Cursor OK except 16m modes (Hardware limitation ?)

Cirrus CL-GD6420.

Mode 13h has problems at the 128K and 256K borders.

Read/Write bank doesn't appear to work

Compaq Qvision 1024:

Cursor ok except 16m modes.

Scroll ok except 1024x768x16c

The BIOS hangs when mode 4Fh is set.

HMC HM86304:

Mode 13h can not cross 64k. 4 color modes don't work yet

MXIC 86000:

The detection of MX86000/86010 appears to work, but let me know if

you see otherwise.

Oak-077:

Mode 13h has problems beoynd 64k.

Oak-087:

The scroll rutine is only tested to 1MB.

Mode 13h has problems beoynd 64k.

Primus P2000:

Scroll has problems crossing the 1MB line in some modes.

Sympthom: An image from the 1st is overlayed on the image from the

2nd MB resulting in lots of flicker.

While copy via BitBLT works in HiColor modes, Fill does not.

Trident 8900B, 8900C:

In Mode 13h (and the 16color modes) the display can't cross the 512K

border, though once the display start is set>=512 it works.

Tseng ET3000:

Some 16color modes (640x480x16 and 800x600x16) has problems enabling

the upper 256KB.

WD90c31:

The hardware cursor is only XOR, and doesn't work properly in HiColor.

BitBlt sometimes skips blocks in 16c modes, and HiColor fill doesn't work

WD90c33:

The WD90c33 was only tested with 1mb, so the extra bits for accessing

and scrolling beoynd 1MB has not been tested.

The hardware cursor is only XOR.

Weitek:

The Weitek chip tested was the VGA part of a P9000 card with only 256K

VGA memory. The 5x86 will all be ID'd as 5186.

Generel Problems:

I can't tell the difference between the Sierra "Mark2" SC11482/3/4

(15 bits) and the "Mark3" SC11485/7/9 (15/16 bits) RAMDACs.

Search for modes doesn't understand 2 and 4 color modes.

In general 2 and 4 color modes are only supported partially.

Mode 0Fh and 11h (and other 2 color modes) has problems in some cases.

Mode 13h has a pixel double function. Most chips use Master Clock/2

(3C4h index 1 bit 3) for this, but some chips has their own system.

Some of the extended modes may only be implemented on

some otherwise similar boards, depending on make and BIOS rev.

Utilities:

DMPMEM:

DMPMEM dumps the ROMs to files named DMPxxxx.ROM (where xxxx is

the hex address of the ROM Ie. DMPC000.ROM for VGA).

Interrupt and Font addresses are saved to the file VECTORS.ROM

As Mouse drivers, national keyboard drivers, memory managers

etc can obscure the results the program should be run on a clean

system (empty CONFIG.SYS and AUTOEXEC.BAT say from a floppy).

TABLE:

Takes the register sets dumped to the file REGISTER.VGA and writes

them to REGISTER.TBL. If a register has the same value in all register

sets, the value is only shown the first time.

As the same file names are used every time you will have to delete

and rename the files as needed.

VESALIE:

Can change the version and amount of memory reported by your VESA driver.

Can also hide the VESA driver, so other applications don't see it.

Parameters:  /0 /1 /2  Makes the VESA driver report a specific VESA

VBE version (1.00, 1.01 or 1.02).

/on /off  Turns the VESA VBE on or off. By turning the

VBE off you can prevent applications from using

the VESA interface. Also if you try to load

another VESA driver, it may refuse to install

while the original driver is visible.

/Mxx      Forces the VESA driver to report the specified

amount (in Kbytes) of video memory.

/U        Unload the VESALIE program.

Note that this NOT a VESA driver, it just changes the behavior of

an already installed driver.

SHOWTEST:

Analyses the results in the .tst files generated when WHATVGA is

run with the /a option (WHVGAxxx.TST).

Parameters: SHOWTEST [tstfil] [outfil] [/r]

When run without parameters it lists the test files in the current

directory, showing the filename, chipset, memory, DAC and chipname.

When run with the name of a test file, the test results in this file

are written to the outfile. If no outfile is specified, the name of

the test file is used, with the extension changed to .txt

If the /r switch is used the register dumps for each mode is written

to the outfile (or to .reg).