SMARTDRIVE OVERHAUL

SMARTDRIVE is a disk cache program that works with

computers that have extended memory.  A disk cache uses

a portion of memory to store information that DOS reads

from disk.  DOS can read the information from the disk

cache much faster than it can read information from a

drive.  However, with DOS 6.0 Microsoft had their

SMARTDRIVE program default to both read caching and write

caching.  The problem with write caching is that if you

turned your computer off immediately after quitting a

program you could potentially lose data.

With DOS 6.2 SMARTDRIVE write caching is turned off by

default.  If you upgrade to DOS 6.2 from 6.0 and you had

SMARTDRIVE write cache your drives, then SMARTDRIVE will

leave write caching turned on.

You can force DOS to immediately write the cached data to

the hard drive, by typing at the system prompt:

smartdrv /c <ENTER>

With DOS 6.2 SMARTDRIVE will automatically flush a write

cached disk before returning you to the system prompt.

This means it gets the data from memory and writes it to

the hard drive before you are returned to the system

prompt.  If you are using DOS at the command line, you

should always exit your program and return to the DOS

prompt before you turn your machine off.

With DOS 6.2 they have added a new /X switch to turn off

write caching for all drives.

For most people, the SMARTDRIVE line will be in their

AUTOEXE.BAT file which is a text file that resides on the

root directory of the start up disk.  See 205 through 208

in Learning DOS for the Complete Novice, 2nd Edition for

examples and exact instructions on how to use EDIT to

selectively turn off write caching.  Those people with

exotic ("bus-mastering" hard disk controllers) such as

SCSI, or ESDI, may need to have SMARTDRIVE installed in

the CONFIG.SYS file.  The CONFIG.SYS file also resides in

the root directory of your start up disk and can be

viewed and modified with the DOS EDIT command.  See pages

209 through 211 in Learning DOS for the Complete Novice,

2nd Edition, for how to view and modify your CONFIG.SYS

file.

With DOS 6.2, you could add the /X switch to turn off

write caching like so:

SMARTDRV /X

You can also selectively disable write caching by

following the SMARTDRV command with the letters of your

hard drives.  Suppose you had hard drives C, D, E, and

you wanted to disable write caching on these drives.  Use

the DOS EDIT program to change the SMARTDRV line in the

AUTOEXEC.BAT file to read the following:

SMARTDRV C D E

The above line tells DOS to allow read caching while

disabling write caching for drives C, D, and E.

SmartDrive will not cache compressed drives (DoubleSpaced

drives), however, for increased performance, you can

cache the host drive.  If you are running DoubleSpace,

you can get information on all your drives by typing:

dblspace /i <ENTER>

If you are running DoubleSpace, the above command will

give you information about all your drives.  It will also

tell you which drive is your host drive.  Suppose you

have only one hard drive and are running DoubleSpace.

Suppose you typed the above command and found out that

your DoubleSpace host drive is called H.  If that is the

case, you can configure SMARTDRIVE to read cache (but not

write cache) the host drive by changing your SMARTDRIVE

line in your AUTOEXEC.BAT file to read:

SMARTDRV H

For more information on exactly how to change your

AUTOEXEC.BAT file get my book, Learning DOS for the

Complete Novice, 2nd Edition, and see pages 205 to 208.

SmartDrive will not cache network drives, or RAM drives.

TIP

If you are running Windows and you have your write

caching turned off, you will pay dearly.  Windows will

crawl.  You may find that it is better to have SMARTDRIVE

cache your hard drives and take the small risk of losing

data.  Windows can be very disk intensive.  If you cache

your hard drives, it really speeds things up.

With DOS 6.2, SMARTDRIVE will read cache CD-ROM drives.

This is a very nice feature because CD-ROM drives are

often too slow.  Caching should help performance.

If you want to see which drives SMARTDRV is caching, at

the system prompt type:

smartdrv <ENTER>

TIP

If after typing SMARTDRV <ENTER>, you notice that your

CD-ROM drive is not being cached there may be a perfectly

logical reason.  The line which loads the MSCDEX program

needs to be loaded BEFORE the SMARTDRV line.

To see how to modify an AUTOEXEC.BAT or CONFIG.SYS file

see Learning DOS for the Complete Novice, 2nd Edition.

For most people the line that loads the MSCDEX program

(for the CD-ROM) and SMARTDRV (disk cache) will be in the

AUTOEXEC.BAT file.  The AUTOEXEC.BAT file resides in the

root directory of the start up disk (usually drive C).

EXOTIC HARD DRIVES

If you are one of those people who have an exotic "bus

mastering" hard drive controller, you may need to have

SMARTDRIVE load in the CONFIG.SYS file.  For specific

instructions on how to modify a CONFIG.SYS file look in

Learning DOS for the Complete Novice, 2nd edition.  Try

using the following line in the CONFIG.SYS file:

DEVICE=C:\DOS\SMARTDRV.EXE /DOUBLE_BUFFER

The above SMARTDRIVE command needs to come before the

DEVICE command that loads EMM386.  Note that when in the

CONFIG.SYS file, SMARTDRV.EXE is used.  Remember, before

an AUTOEXEC.BAT or CONFIG.SYS start up file can take

effect, you must reboot your machine.  After you reboot

type:

smartdrv <ENTER>

With the above DEVICE=C:\DOS\SMARTDRV.EXE /DOUBLE_BUFFER

line in your CONFIG.SYS file the SMARTDRV <ENTER> command

will load SMARTDRIVE into upper memory.  After you do

that you can type the following:

smartdrv /s <ENTER>

Here, the /S stands for Status.  A table will appear on

your screen that looks something like:

DISK CACHING STATUS

drive    read cache    write cache    buffering

A:        yes            no            no

B:        yes            no            no

C:        yes            yes           no

S:        yes            no            no

Look in the column that says buffering.  If all the

entries say no then your hard drive does not need this

Double Buffering and you would be better off taking the

SMARTDRIVE line out of your CONFIG.SYS file and just

using a SMARTDRV line in your AUTOEXEC.BAT file.  This

Double Buffering business makes SMARTDRIVE operate

slower.  So, if your hard drive can support SMARTDRV in

the AUTOEXEC.BAT file, you will be better off.

If you want more information about the DOS 6.2 SMARTDRIVE

program type:

help smartdrv <ENTER>

Of course, you will have to have DOS 6.2 installed.  If

you need any help on using the excellent DOS HELP see

pages 192 - 194 in Learning DOS for the Complete Novice,

2nd Edition.

TIP

If you are using DOS at the command line, you should

always exit your program and return to the DOS prompt

before you turn your machine off.  Make sure the rest of

the family knows this, too.