* OptMouse.c

*

* Serial Port Optical Mouse Driver

*

* (c) Copyright 1989 J. Edward Hanway

*		      149 Scotch Pine Dr.

*		      Rochester, NY 14616

*

* This code may be freely redistributed for non-commercial purposes. There

* is NO WARRANTY on this program. The author assumes no responsibility for

* any damages resulting from use of this program. You know the drill.

*

Usage (from CLI only):

OptMouse [-m[iddle]] [device [unit]]

OptMouse [-k[ill]]

Examples:

OptMouse 		Starts mouse driver (in the background) on

serial.device, unit 0.

OptMouse foo.device 1	Starts mouse driver using foo.device, unit 1.

Unit is optional, but if you want a unit other

than zero, you must specify the device, even

if it's just serial.device.

OptMouse -middle	Starts mouse driver. Translates middle button

OptMouse -moo		events into shift-left button events.

OptMouse -m foo.dev 1	You get the idea.

OptMouse -kill		Stops mouse driver. (Frees serial port.)

OptMouse -kwack

Background:

The M3 (or M2) is a serial, optical mouse made by Mouse Systems

Corp. of Santa Clara, CA.  They are very nice, true optical mice (as

in no-moving parts, not the "opto-mechanical" market-speak like

the Amiga mouse) and are (were? still?) used by Sun Microsystems

for their workstations.  According to the "Optical Mouse Technical

Reference Manual" the mice come with a variety of connectors, including

ones which have DB25 serial connectors and are powered by a separate

power pack, but	the one I got had an RJ11C (modular telephone)

connector, which I wired to get power from the Amiga 2000 serial port

as follows:

DB25S					    RJ11C

to Amiga				    to mouse

3+------+

Pin 3 (Data) ----------|      |2

1|      |------------- Pin 4 (Signal)

Pin 10 (-12V)----------| 1488 |

14|      |7

+----|      |----------------------------+

|    +------+				   |

|					   |

|   1+------+3			   |

Pin 9 (+12V) -----+----| 7805 |----+-------- Pin 3 (+5V)   |

|    +---+--+    |			   |

.01uF ===      2|	  === .01uF		   |

|	   |	   |			   |

Pin 7 (GND)  -----+--------+-------+---+---- Pin 2 (GND)   |

|		   |

+-------------------+

I managed to cram this circuit inside a DB25-to-modular adapter.

Caveats:

As far as I can tell, the events that this program generates are

identical to genuine mouse events, but Digi-Paint (version 1)

doesn't like me, and hangs when I use the optical mouse to try to

load a picture. I know of no other incompatibilies with "normal"

programs which read the mouse via Intuition. Games which take over

the machine and/or go straight to the hardware will obviously not

work.

Gurus, which require you to click the left mouse button to continue,

apparently go straight to the hardware (what else can they do?) and,

thus, don't work.

I have not tested this with any add-on serial port (I can't afford

them). Even if I could, half of a $300 board for a mouse when you

can buy the Boing optical mouse for $100 ain't very cost-effective,

so I suppose this will remain just a hack unless (until?) the price

of add-on serial ports comes down severely.

Apparently these mice can be had for cheap at hamfests, etc. because

they're not strictly compatible with a PC, I guess. If you do find

one, make sure you get the reflective mouse pad with it!

Disclaimer:

Mouse Systems has nothing to do with this and has never heard of me.