SOUND_ED.DOC version .98 ⌐ 1990,by ADCsystems

Howard Dortch or Mike Coriell c/o

ADCsystems

1628 Grandview Av.

Portsmouth,OH. 45662

(614)354-1289

The following documentation is an explanation of the

functions of the SOUND_ED editor/sampler software. The software

should run on any standard Amiga computer, including an A3000

with 2.0 software. The program should NOT be run in overscan mode

available with the release of 2.0. Any screen width larger than

640 pixels wide may have problems operating properly. See the bug

list at the end of this document

REQUESTERS ---------------------------------------------------

PLEASE WAIT

When the Please Wait requester is active on the screen,

it is recommended that no other operations are attempted or a

program lock-up may occur. This is a minor bug that will soon be

corrected.

LOAD - SAVE

This requester allows loading and saving files for the

editor. Select the file you wish to load by clicking the mouse

on the file and select LOAD. Or you may double click on the file

and it will load. There are string gadgets for the path and file

names should you require paths other than the ones shown.

L - R

This requester will appear when loading a file that has no

CHAN chunk type in the header. Select L to load the file in the

upper or LEFT edit window. Select R to load the file in the lower

or RIGHT edit window.

LOW MEMORY

This requester will appear when your computer encounters

a low memory condition. During the course of editing the memory

may become too segmented to do what you want. Select the OK

gadget,save your files and try another method.

SAVE FILES FIRST?

Some operations will overwrite a file in an edit window.

This requester will appear to allow you to save your work before

you continue.

MENUS --------------------------------------------------------

EDIT

NEW - LEFT

This selection will erase the contents of the left buffer

and reset the buffer size to 10,000 bytes. It will redraw the

graphic window to a zero line. If you have a file loaded in the

left buffer a requester will appear asking if you want to save

the file first. If you select NO the operation will continue. If

you select YES the operation will quit, allowing you to save the

file before continuing.

NEW - RIGHT

This selection will erase the contents of the right buffer

and reset the buffer to 10,000 bytes. It will redraw the graphic

in the right window to show a zero line. If you have a file

loaded in the right buffer a requester will appear asking if you

want to first save the contents of the buffer.

NEW - BOTH

This selection will erase the contents of both the left and

right buffers,and reset the buffer sizes to 10,000 bytes each. If

there is a file loaded a requester will appear first for the left

side then for the right side. If you select YES for the left

requester the operation will be aborted allowing you to save your

work.

QUIT

This selection will exit the program. You can also select

the right "A"miga key with "Q" and get the same result.

TASK

ICON

This selection will close the editor window and install a

title bar in the screens title bar. This will allow you to

perform other operations that require the use of a CLI  or

workbench. To reopen the SOUND_ED editor window select the title

with the LEFT mouse button then press the RIGHT mouse button and

the window will re-open. It is possible to run a second version

of the SOUND_ED program, allowing for a separate editing

session. However keep in mind this will use more memory depending

on the file sizes you are editing. You can also iconify the

program using the right "A"miga and the "I" keys

FILE

IFF

This selection is the default file type mode. This item

should have the check mark selected when you first run the

program. When attempting to load a file if it is an IFF type file

it will load with no problem. If you do not change the selection

you can save the file as an IFF type file. If the file does not

load,the words "BAD FORMAT TRY RAW" will appear in the filename

box in the upper left corner of the window. Use the right mouse

button to activate the menus and select FILE - RAW. The check

mark will then be to the left of the word RAW. Now you can

attempt the load the file as raw data. If the file you loaded

sounds like distortion try the AUX mode. This mode converts DC

signals found on MAC and Atari computers to AC files for the

Amiga.

RAW

This selection will load a file as RAW data. It will load

any file regardless of type. You can then use the editor to cut

out unwanted portions. Some programs want RAW data to play. You

can load a file as an IFF and save it as RAW data when needed.

The RAW data does not save rate and volume information. The

default setting for a raw file is 8,000 samples per second,full

volume. Raw files cannot be saved in stereo mode.

AUX

This selection is used to load MAC and Atari 8 bit sound

files. You can load MAC files and save them as Amiga IFF or RAW

files. Likewise you can load RAW of IFF files and save them as

RAW MAC or Atari 8 bit sound files.

DIGITIZER

SOUND_ED

This selection sets the record mode to use the SOUND_ED

digitizing hardware.

PS_3.0

This selection is used for Perfect Sound 3.0 digitizing

hardware.

GENERIC

This selection is used for most generic types of digitizers

including PS_2.0.

MAIN SCREEN GADGETS (starting upper left)---------------------

LEFT:

This box is for the left side file name. It may also contain

status reports about a file you are attempting to load.

LOAD

This is a gadget that brings up a file requester. The file

selections can be double clicked using the left mouse button or

select the LOAD gadget in the requester window to load a file.

After a file has been selected, if it has left - right data

stored in the file, it will load in the proper edit window. If

there is no "CHAN" data stored with the file a requester will

appear allowing you to select the side you wish to load the file.

This requester will find the mouse so you don't have to chase

around the screen to find it.

RANG

This is the range "in/out" gadget. Use the mouse to select a

range in the edit window by moving the pointer to the start of an

area you wish to zoom into. Then hold down the left button and

drag the mouse to the right over the range you want to see. The

area you have dragged the mouse over will change to grey to show

you the area you are ranging into. Let up on the mouse button and

select RANG. You will then see a ZOOMED portion of the file.

Selecting the RANG gadget without selecting a range and you will

ZOOM OUT by a factor of 2 to allow you to see a larger range.

Other edit functions will work on this RANGED section. The SIZ

box will display the number of bytes per pixel when you first

press the mouse button. The SIZ will increase as you move the

mouse to let you know how much you are ranging into. See

TIM,BEG,END also.

CUT

This gadget will cut a section from the file. You must first

have a range selected in the edit window. The area colored grey

will be cut from the file. The CUT section will be held in a CUT

buffer ready for you to PASTE in either edit window. Warning if

you cut another section the first one will be lost so be careful!

COPY

This gadget will copy a range from one window to the other.

You must first select a range in the opposite window to use the

COPY gadget. The COPY gadget does not use a buffer.

MONO - STER

This gadget toggles between mono and stereo MODE for record

and playback. If you use the STER (stereo) mode remember that the

file lengths must be the same size in order to work properly.

Also keep in mind that when you save a file this MODE will be

saved with the data. If you have STER selected both left and

right buffers will be saved.

PLAY

This gadget will play the file on the side you selected. If

you should have the STER mode selected both buffers will play

VOL

This is a slider gadget that allows you to change the volume

of the sound being played. The number to the right of the slider

is a number from 1 to 64 indicating the level of sound.

MAIN SCREEN GADGETS (second row)------------------------------

TIM

This box displays the time duration of the file. If you have

a range selected, it will show the time of the range. The format

for the time is MM:SS.MICROS

SIZ

This box is the size of the file or range in bytes. If youclick the mouse left button in the edit window and release, thesiz will display how many bytes are represented by one pixel onthe screen.

SAVE

This gadget will bring up a file requester for saving a

file. If you have a range selected only the range will be saved.

The file type will also be used. If you have loaded a file using

IFF and select RAW in the menus the file will be converted to

RAW. Check all your settings prior to a save.

OUT

This gadget selects RANGE out. If you have ranged into a

section of a file use this gadget to see the entire file again.

PAST

Use this gadget to paste a section you have previously CUT.

To paste a section move the mouse to the position of the file you

wish to paste to then press the left mouse button to set the

paste point. Then select the PAST gadget to perform the

operation.

EFF

This gadget selects the effects window (see the section

effects). The gadget is a toggle select. When selected the gadget

will highlight grey to close the effects window select the gadget

again.

REC

Use this gadget to select the recorder window.(see the

section RECORD for more information)

LOOP

This gadget will set loop on for playback. Loop mode is

active when the gadget is hilited grey. To turn looping off

select the gadget again and it will return to blue.

RATE

This gadget sets the playback rate in samples per second. It

also has a string gadget attached to set the playback rate to a

specific value.

MAIN SCREEN GADGETS (bottom row)------------------------------

BEG

This box indicates the BEGining time. If you select a range,

the point where you set the start of the range will be indicated

in the box as the beginning time for the range. If you do not

have a range selected this value should be 00:00.000000. This

time value is relative to the beginning of the file. For instance

if you have a file that has a total time of 2 seconds, and you

set the start of a range by clicking the mouse down half way

through the file the beginning time should be about 1 second.

<<<<

This gadget allows you to scroll through a ranged section of

the file. It will slowly move the graphic to allow you to find an

area of interest. This only works when you have a range selected.

MAIN SLIDER

When you first run the program, there is a long black bar in

the center of the screen. This is a slider gadget used when a

range has been selected. The bar will shrink to show the

percentage of file that has been ranged into and the relative

position of the range. To move to a different position in the

file grab the slider with the mouse and move it. The size of the

range wont change, just the position.

>>>>

This gadget allows you to scroll towards the end of a ranged

section of the file.

END

This box indicates the END of a range relative to the

beginning of the file. If no range has been selected, the display

will show the total size of the file (same as TIM). The format

for the ending time display is MM:SS.MICROS.

EFFECTS WINDOW GADGETS (upper left)---------------------------

RAMP BEG  - RAMP END  - SETL - SETR

This set of gadgets selects the RAMP function. If you want

to start a file at full volume and decrease the volume toward the

end, you would move the BEG slider to the right (100%) and the

END slider to the left (0%), then select SETL. The file will be

redrawn with the amplitude (volume) tall at the start and short

at the end. When played the sound volume will taper off to

nothing toward the end of the file. Move the BEG slider to the

left and the END slider to the right and the sound will get

louder towards the end. If you have a RANGe selected the RAMP

will work only on the range.

REV

This gadget will REVERSE the file, or display it in reverse.

The end of the file will be the beginning. To change it back

select REV again.

CLONE L>R - R>L

This set of gadgets allow you to copy or CLONE the entire

file from right to left or left to right. Use this to keep a

temporary copy of the file you are working with in case you make a

mistake, CLONE it back and try again.

MIX L>R - R>L

This set of gadgets allows you to mix two files together,

creating one file with the sounds of both. The file buffer you

are mixing to will be lost so save your work!

LEFT VOL -C- - +B+

This set of gadgets is used to BOOST or CUT the amplitude of

a file. After a mix the overall volume is lower than either of

the two files you mixed. Select the +B+ BOOST gadget and the file

will increase in amplitude. If the file is not as loud as you

want select the +B+ BOOST gadget as many times as necessary to

get the desired result. CAUTION, the +B+ BOOST operation can

cause the file to be too loud,and cause distortion, so be

careful!

LEFT DELAY - SETL

This set of gadgets will introduce delay into the file. Move

the slider and play the file. When you get the amount of delay

you want select the SETL gadget and the file will have that

amount of delay permanently installed.

HARD FILTER

This gadget turns the audio filter on or off as desired. If

you turn the filter off you will be able to hear higher frequency

sounds. When this filter is selected the power light on the front

of the computer will get dim or bright depending on the state of

the filter selection.

FILE FILTER  LOW MID HIGH

This set of gadgets will filter frequencies from the file.

Save your work first. When this filter system is used the file

data will be altered!

RATE CONVERTER

This set of gadgets allows you to change the rate of the

file. If you have a file sampled at 20,000 samples per second and

you want it to change to 12,000 samples per second, move the

slider or use the string gadget to select the new rate, then

select the SETL gadget and the rate will be converted. Selecting

a slower rate will take less memory for the file. CAUTION,

sampling rates below 10,000 samples per second will sound noisy

so be careful when performing this operation.

TRANSPOSE

This set of gadgets alters the rate up and down 1/4 step

1/2 step or a full octave. Alter the rate and play the sound. If

you want to use the new rate select the SETL,SETR gadgets below

the rate converter and the file will be adjusted to the new rate.

RECORD WINDOW ------------------------------------------------

The record window opens over a section of the edit window. You

can't edit a file with the RECORD window open, nor can you

ICONify the program with the record window open. To return to the

EDIT function select the EDIT gadget.

MEM

This gadget shows the amount of memory allocated for

recording. You may select a larger amount of memory to be

allocated by changing the string gadget or by using the slider

below. CAUTION if you allocate all the available memory for the

left side the right side wont be able to allocate much more than

is showing. if you are after stereo, balance the memory between

the two files.

REC

Select this gadget when you are ready to record. Remember to

select the type of hardware in the menus (DIGITIZER) before you

start.

MONO - STER

This is a toggle select gadget that sets the record MODE. If

you have the STER option set the RATE on the LEFT file will set

the rate for recording for both sides.

PLAY

This gadget allows you to PLAY the file before returning to

the edit session. You may choose to resample if you don't get what

you want in the first try.

VOX

Set the VOX level by moving the slider gadget. VOX is the

level at which the recorder will turn on. If you have the vox set

high the recorder function will wait till it gets a loud enough

sound to start recording. Its like a sound activated switch

After the recording has started the VOX has no effect. VOX is not

supported on the PS_3.0 hardware.

MON

This is the MONITOR gadget. You can use it listen to the

digitizer prior to recording.

EDIT

This gadget gets you back to the editor.

SYNC

This gadget allows simultaneous tracking of the opposite

file. In other words if you record on the left side you can hear

the right side file played along with the recording, so you can

synchronize the two files for stereo. You must have MONO mode

selected to use this feature. Then select STEReo mode to hear the

result. SYNC is not supported for the PS_3.0 hardware.

RAT

This gadget sets the rate in samples per second for the

recording. There is a string gadget attached so you can set an

rate.

KNOWN BUG LIST

1> Filenames with spaces should not be used.

2> You cant cut odd number of bytes from the file.

3> Vox and Sync have not been implemented in this release.

4> Scan bar during play is not implemented yet.

5> When the PLEASE WAIT window is active dont click on the

EFF gadget you may open another window that wont close.

6> Playback on stereo must have the same file size or the

shortest file may wrap around.

7> Answer all requests before doing anything else.

8> The MEMory selection in the record window will not

balance in STEREO mode,you will have to do it manually

by using the string gadget.

9> The program wants to be in the root directory of a disk

or a system requester will pop up. Select CANCEL and go

on with the session.