SCSIMounter 2.0

===============

Removable media partition mounter for Kickstart 2.0 or higher.

Refer to 'SCSIMounter.guide' for an AmigaGuide version of this

document.

WANTED: Someone who translates this document to German!

------------------------------------------------------------------------

Versions

--------

2.02 - Terrible bug fixed which caused SCSIMounter to fail on 68000

machines.

2.00 - SCSIMounter is a commodity now

bi-lingual user interface

1.30 - MASK tooltype added

1.20 - Minor changes to the CLI interface. New tool type 'NOUNMOUNT'

1.10 - ReadArgs() CLI Interface added

1.04 - Now works with beta kickstarts (V 36)

Keyboard equivalents for everything (including list view)

Layout bug fixed

No funtional changes.

1.03 - Made more tolerant :-)

1.02 - Fix for gvpscsi.device (GVP Series II) added.

------------------------------------------------------------------------

Disclaimer

----------

SCSIMounter may be freely distributed for non-commercial purposes only.

The entire risk as to the quality and performance of this program is

with you.

The author assumes no responsibility or liability whatsoever with

respect to your use or inability to use of this software.

------------------------------------------------------------------------

Purpose

-------

Most owners of removable media SCSI drives are stuck with the problem

that their software doesn't handle disk changes correctly.

For example, the current Commodore implementation doesn't mount the

partitions on a removable media SCSI drive (e.g. SyQuest or Ricoh)

when there was no cartridge in the drive at boot time.

Even worse, after another cartridge with a different Rigid Disk Block

(i.e. a different partition table) has been inserted, the system

gets totally confused and reports read/write errors when trying

to access the device which can lead to loss of important data on

the cartridge.

Until now, the only possibility to work around this problem was to

reboot the system every time a new cartridge has been inserted.

SCSIMounter allows you to mount and unmount partitions without rebooting

the machine. It sports a nice 2.0-style point-and-click user interface

that makes handling removable media a piece of cake.

------------------------------------------------------------------------

Secondary use

-------------

SCSIMounter can also be used to mount partitions that have intentionally

not been mounted (e.g. the wb_1.3: partition on an A3000 when running

under 2.0). You don't have to create a mountlist for it!

------------------------------------------------------------------------

Requirements

------------

� Any Amiga with a SCSI host adaptor that supports the Commodore

RDB (Rigid Disk Block) and 'SCSI Direct' standards.

� Kickstart 2.0 or higher (version 37.175 preferred)

The program has been tested on an A3000 with Commodore's scsi.device

and on an A2500 with a Microbotics HardFrame (HardFrame.device) but

I see no reason why it shouldn't work on any other host adaptor.

The program may be of no use for owners of certain GVP controllers

which support removable media correctly (Though not all GVP software

might do so).

------------------------------------------------------------------------

Before you begin

----------------

First, determine the name of the device driver of your hard disk

controller card.

Examples of popular controllers and their devices:

manufacturer/controller           device

--------------------------------------------------

Commodore A590/A2091/A3000        scsi.device

Old GVP                           scsidev.device

GVP Series II                     gvpscsi.device

Supra SupraDrive                  harddisk.device

Advanced Storage Systems Nexus    nexus.device

Microbotics HardFrame             HardFrame.device

ICD AdSCSI                        icddisk.device

IVS Trumpcard Professional        ivs_scsi.device

BSC ALF2/3                        ALF.device

BSC Oktagon 2008                  Oktagon.device

--------------------------------------------------

Click once one the SCSIMounter icon and select 'Information' from the

'Icons' menu.

Add or change the following tool type:

DEVICE=Name_of_your_scsi_device_here

for example if you use a HardFrame controller write:

DEVICE=HardFrame.device

Don't add path names here, only the name of the device is required.

Take care to capitalize the device name correctly (exec device names

are case sensitive!)

NOTE

SCSIMounter will not work with the Commodore A2090(A) controller card

because this product does not support Commodore's own 'SCSI direct' and

'RDB' standards properly.

------------------------------------------------------------------------

Start SCSIMounter

-----------------

SCSIMounter can be used either from Workbench or from the CLI.

* Workbench use

Double click on the SCSIMounter icon. After a short time, a window

with a listview and three buttons will appear (assuming CX_POPUP

is set to `yes'). If the window doesn't appear several things could

have gone wrong:

� Do you use Kickstart 2.0 or higher?

� Did you specify and spell the name of your device driver correctly?

� Does your host adaptor support RDB?

� Is there any memory left on your system?

� Do you have commodities.library (supplied with Workbench 2.04)

in libs:?

Requesters will try to tell you the cause if SCSIMounter fails to run.

* CLI use

Type 'SCSIMounter gui device Name_of_your_scsi_device'

example:

1> SCSIMounter gui device HardFrame.device

If the 'gui' argument is not given, SCSIMounter will be started

in command line mode as described later in this document.

------------------------------------------------------------------------

Panels

------

1) Main panel

The main panels shows all devices connected to the SCSI port with

information about their capacity and current status. The currently

selected unit is higlighted.

UNIT - This is the unit number of a SCSI device.

Up to seven devices can be connected to the SCSI bus

DRIVE TYPE - Manufacturer and product specification

REV. - Revision of Hardware, Firmware and ROM of device

CAP. - Device capacity in Megabytes or Kilobytes

STATUS - Current Status of device. If the device is busy or

not accessable due to the lack of media (if it's a

removable media drive) the status will be set to

'not ready'

There are three 'action buttons' present:

HIDE - Put SCSIMounter to sleep. To wake it up again, use the

hotkey as specified in CX_POPKEY or the Commodities Exchange

program on you system partition. Closing the window

using its closegadget has the same effect.

UPDATE - Re-scan SCSI bus and update display

MOUNT - Mount partitions on selected drive

For every action buttons there's also a menu item with identical

function. All buttons have keyboard equivalents (marked with an

underscore in the label text). The listview can be scrolled using

the arrow up and arrow down keys on your keyboard rather than using

the mouse.

There's one important menu item that has no button equivalent:

QUIT - Quits SCSIMounter and removes it from the list of commodity

programs

2) 'Partition Selector' panel

This panel shows all available partitions and allows you to

individually mount them.

CHECKGADGETS - Determine if the particular partition should

be mounted or not.

checked: mount corresponding partition

not checked: don't mount specified partition

Other partition Information on the partition selector -

Number of Partition == keyboard equivalent

Device name of partitions

Size of partitions

Dostype (FileSystem) of partition

Examples of dostypes:

DOS0 - Old filesystem partition

DOS1 - FastFileSystem partition

UNI1 - Unix boot partition

RESV - Reserved partition

CANCEL - Go back to main panel, don't mount anything

MOUNT - Unmount all currently mounted partitions on the selected

device and mount the new partitions (if any).

UNMOUNT ALL - If this gadget is checked, SCSIMounter will try

to unmount all partitions currently mounted on the

selected unit. If there are locks on this device

there will be a 'BUSY' icon on your Workbench screen.

WARNING:

This option can be dangerous if used with non-removable

devices because it will unmount all partitions

including your boot partition (SYS:) forcing you to

reboot your computer. Use it with care!

------------------------------------------------------------------------

Operation

---------

SCSIMounter is pretty self-explanatory. Nevertheless, here are some

basic operation procedures.

* Mount a new cartridge

� Click on the listview entry of the desired device. (Make sure its

status is 'ready')

� Click on the 'Mount' button

The 'select partitions' panel will ask you which partitions to mount

� Chosse the partitions you would like to mount

� Make sure that the 'Unmount all' gadget is checked

� Click on 'Mount' button on the partition selector panel

SCSIMounter will automatically unmount all existing partitions

on the device and mount the new partitions.

* Add a SCSI drive that was not present at boot time

� Click on the 'Update' button

The new drive's name and other information should appear in the

listview

� Click on the corresponding entry

� Click on the 'Mount' button

and so on...

* Mount a partition that has the automount flag not set

� Click on the listview entry of the desired device. (Make sure its

status is 'ready')

� Click on the 'Mount' button

� On the partition selector panel, make sure the 'unmount all' check

gadget is *not* checked

� Select 'Unselect All' from the 'Edit' menu or press 'Rt. Amiga-U'

� Click on the check gadget of the partition(s) to mount

� Click on the 'Mount' gadget or press 'Rt. Amiga-M'

WARNING !!!

Don't unmount your boot partition (where you system files live) or

you'll soon be forced to reboot you machine...

------------------------------------------------------------------------

Keyboard Equivalents

--------------------

* Requesters

gadget         keystroke activation     menu shortcut

-------------------------------------------------------

OK             O                        Rt. Amiga-O

Cancel         C                        Rt. Amiga-C

* Main window & Project menu

gadget         keystroke activation     menu shortcut

-------------------------------------------------------

About          none                     Rt. Amiga-A

Mount          M                        Rt. Amiga-M

Update         U                        Rt. Amiga-U

Quit           Q                        Rt. Amiga-Q

Listview up    Arrowkey up              none

Listview dowm  Arrowkey down            none

* Partition Selector

gadget         keystroke activation     menu shortcut

-------------------------------------------------------

Mount          M                        Rt. Amiga-M

Cancel         C                        Rt. Amiga-C

Select All     none                     Rt. Amiga-A

Unselect All   none                     Rt. Amiga-U

Partitions     0-9                      none

------------------------------------------------------------------------

Tool Types

----------

The following tool types are supported:

DEVICE    - See above

NOUNMOUNT - If this tool type is set, the 'Unmount All' gadget is not

checked by default and other partitions on the same unit are

not removed. This option is useful if you don't use

SCSIMounter with removable media. Dont set this tool type

for removable media!

MASK      - Default DMA Mask. If not specified, the mask in the RDB will

be used. Don't specify this tool type unless you know what

you're doing.

CX_POPUP  - If you want the SCSIMounter window to pop up the first time

you double-click on the SCSIMounter disk icon, set this

to yes:

CX_POPUP=yes

If you want SCSIMounter to run in the background the first

time it is being started e.g. if you're running it from the

WBStartup drawer, set this to no:

CX_POPUP=no

CX_POPKEY - Key combination for the commodity `hotkey'. Default:

CX_POPKEY=alt shift f10

CX_PRIORITY Commodity priority. Default value:

CX_PRIORITY=0

LANGUAGE  - Determines the language to use for the user interface

Possible values:

LANGUAGE=German

LANGUAGE=English

This tool type overrides global language settings. Remove

it if you want to use the system's default language...

PUBSCREEN - Name of the public screen on which SCSIMounter should open.

------------------------------------------------------------------------

Command Line Use

----------------

For the advanced user, SCSIMounter can be used in command line mode

by specifying all required arguments on the command line. In CLI mode,

it is not possible to individually select partitions. All partitions on

the particular unit will be mounted.

Note that SCSIMounter will not attempt to open commodities.library

and add itself to the list of commodities in CLI mode except when

the `GUI' argument (see below for more info) is given.

SCSIMounter supports 2.0-style ReadArgs() CLI options. The command

template looks like that:

D=DEVICE/K,U=UNIT/N,N=NOUNMOUNT/S,GUI/S,QUIET/S

DEVICE    - Specifies the device used for you hard drive controller

e.g. scsi.device.

If this argument is left out, DEVICE defaults to

'scsi.device'.

UNIT      - Specifies the SCSI unit number of the drive to be mounted.

e.g. 6

This argument is not required if the GUI option is used.

NOUNMOUNT - If this switch is set, SCSIMounter will not unmount old

partitions. It basically works the same as the 'Unmount All'

check gadget in the 'partition selector' panel. If you're

mounting removable cartridges, use this option with care!

GUI       - If this keyword is used, SCSIMounter will open its

'unit selector' panel as if it was started from Workbench.

QUIET     - Don't output information on selected unit and mounted

partition but output fatal error messages. Ideal for use

in shell scripts

Examples:

1> scsimounter device HardFrame.device unit 6 quiet

(mounts all partitions on SCSI unit 6 connected to a HardFrame

controller and is quiet about device and partitions)

2> scsimounter d u 3

Unit Drive Type                  Rev.    Cap.  Status

3    SyQuest   SQ555             FGJ     43 M  ready

Unmounting device: sq0

Mounting device  : sq0

(mounts all partitions on SCSI unit 3 connected to a Commodore

A590/A2091/A3091 controller and unmounts old partitions on the

same unit. Outputs full device and partition information)

------------------------------------------------------------------------

Known Problems

--------------

� The program only removes the dos entries for the volumes and devices.

It doesn't remove the filesystem process and associated memory

allocations. Apparently the current FS doesn't support the ACTION_DIE

packet. I'm still looking for a legal way to solve this problem.

------------------------------------------------------------------------

Credits

-------

Author: Martin A. Blatter

Pfaff�cherstr. 59

CH-8913 Ottenbach

Switzerland

Work address: Relog AG

Attn: Martin A. Blatter

Bruggerweg 2

CH-8037 Zurich

Switzerland

(Relog AG develops high quality software for the

Amiga computer)

E-Mail: blatter@amiga.physik.unizh.ch

cbmvax!cbmehq!cbmswi!zethos!blatter

Bug reports or suggestions are welcome. If you find this program useful,

feel free to send a donation to the above address.

This program uses Relog's ITools(tm), the object-oriented user interface

system (c) 1991 by Christian A. Weber and Relog AG.

Special thanks to  � Christian A. Weber

� Peter Simeon for the SCSI inquiry code

� Michael-Wolfgang Hohmann for the AmigaGuide

version of the manual.

� Andreas F. Bobak for the new icon

------------------------------------------------------------------------

Trademarks

----------

All trademarked product and company names used herein are used for

identification purposes only. No infringement of trademarks is intended.

Trademarks remain property of the trademark holder.