Degrader V1.21  1991-92 Chris Hames.  All rights reserved.

-----------------------------------------------------------

You upgrade your system and what happens?  The idiots who wrote your

favorite program didn't follow the guidelines and it no longer works.  This

is where degrader comes in, it will try to degrade your system so that the

old brain dead program will work on your machine.  You can also just use it

to boot off floppy drives other than DF0:, force your machine to PAL or

NTSC, turn the audio filter on or off and other things.

***************************************************************************

When you run the program from workbench you are presented with

various option buttons which you can toggle on(down & highlighted)/off:-

50Hz			Changes the screen refresh rate to 50Hz (PAL).

You can use this to view the extra lines a PAL

machine has that a NTSC machine normally doesn't.

Requires ECS Agnus and a appropriate monitor.

60Hz			Changes the screen refresh rate to 60Hz (NTSC).

Requires ECS Agnus and a appropriate monitor.

50HzSystem		Modifies GfxBase to give values that will make

some programs think this machine is PAL.

Useful in Kickstart 1.2/1.3.

60HzSystem		Modifies GfxBase to give values that will make

some programs think this machine is NTSC.

Useful in Kickstart 1.2/1.3.

AudFilterOff		Most Amiga's have a audio filter which can be

switched off and the power light usually

dims as well.

AudFilterOn		Most Amiga's have a audio filter which can be

switched on and the power light usually

brightens as well.

NoFastMem		What this does depend on whether you selected

for the machine to reset.

If reset, it will stop any memory besides chip

memory from being added to the system during

reset.

If you don't select a reset option it will patch

the system so no Fast memory will be allocated

by programs.

HalfMegChip		Sets the amount of chip memory your system has to

512K.  Some programs don't like it if you have

1 or 2 meg of chip memory so use this option

to trick the program into thinking you only have

512K of chip memory.

If you don't select a reset option it resorts

to some nasty memory list manipulation.

OneMegChip		Sets the amount of chip memory your system has to

1024K.  Some programs don't like it if you have

2 meg of chip memory so use this option

to trick the program into thinking you only have

1024K of chip memory.

If you don't select a reset option it resorts

to some nasty memory list manipulation.

NoMemAt			Allows you to stop a certain memory block from

being added to the system memory list.  If you

don't select a rest option it just gives that

memory block a priority of -128 so it is not

allocated until you run out of other memory.

NOTE: you must turn the Button "ON" as well as

enter value.

AddMem			Adds memory to the memory list.  Useful if you

have a non-autoconfig memory board and want

your memory added at reset time so it is used

a lot more instead of your chip memory.

NOTE: you must turn the Button "ON" as well as

enter values.

NoCache			Turns off your Cache & Burst modes of 68020 or

higher processors.

Privilege		Changes move SR,<ea> to move CCR,<ea> and

changes move CCR,<ea> to move SR,<ea>.

MOVE SR,<ea> is privileged on processors 68010+.

If a program gives a 00000008 GURU consistantly

this might help.  Very few programs read the

status register directly, even fewer get it wrong

as well.

TimeWaste		Slows down your machine based on the value you

give 65535 to 1.  The smaller the number the

slower your machine gets.

Won't work with many programs as they take over or

turn off the interrupt I use.

NOTE: you must turn the Button "ON" as well as

enter value.

BlockDevice		Stops programs from opening a specified device.

DF0: is -> DF0:

DF1: is -> DF1:

DF2: is -> DF2:

DF3: is -> DF3:		Use these to swap drives around.  Set a drive to

DF4: to make it unuseable.  Hence it won't take

up precous chip memory.

Only works with a reset operation.

After you have selected the options you want you have a choice of

three buttons at the bottom of the screen.  To do it now only, to survive

one reset only or to survive resets until the left mouse button is held

down at reset.

If  you  do a reset selection your machine will reboot and at reset

time  your screen will go dark blue->light blue->dark orange->light orange.

Note  the  reset  features of this program is Kickstart specific but should

work with at least:-

Kickstart 1.2 ROM.

Kickstart 1.3 ROM and A3000 Kickstart 1.3 file.

Kickstart 2.04 ROM and A3000 Kickstart 2.04 file and zkicked 2.04.

***************************************************************************

If you start degrader from a Shell/CLI you can all the keywords shown in

the window display as described above, except the DF0: stuff.

And there a few more:-

Floppies		This is the Shell/CLI interface to changing

floppies.  Give it 4 numbers with values 0-4

SurviveReset		Reset then do all the goodies.

SurviveResets		Reset then do all the goodies.  And still be

there to do it next reset.  Keep it up until

the user press the left mouse button at reset.

NoEditOp		Don't bring up the window to edit options.

***************************************************************************

Examples of using Degrader from a Shell/CLI:-

"Degrader AUDFILTEROFF"

Bring up option window with audio filter selected.

"Degrader AUDFILTERON NOEDITOP"

Turn the audio filter on.  The option window is not used.

"Degrader 60HZ 60HZSYSTEM"

Bring up option window with 60HZ and 60HZSYSTEM selected.

"Degrader ADDMEM $100000 5 10 $400000 SURVIVERESETS NOEDITOP"

Add 1 meg of memory at $400000 at each reset from now on.

"Degrader TIMEWASTE 50 SURVIVERESET NOEDITOP"

Reset with hopefully a lot slower machine.

"Degrader FLOPPIES 0444 SURVIVERESETS NOEDITOP"

Reset and only have DF0: available.

***************************************************************************

The most common way to get a brain dead game to work is to try this

"Degrader NOFASTMEM NOCACHE PRIVILEGE FLOPPIES 0444 SURVIVERESET NOEDITOP"

***************************************************************************

Please note this program uses the ColdCapture, KickTagPtr and

sometimes patches LVOAddMemList, LVOAllocMem, LVOAvailMem, LVOOpenDevice so

good virus detectors, like mine :-), will give you warnings.

This program may be copied as long as it is accompanied by this doc file

both unaltered & no money is charged (except for a reasonable copying fee

such as what Fred Fish charges).  All use of this program is at your own

risk & I assume no responsibility or liability for any problems that occur.

If you want to contact me to give me a porsche or similar,

Chris Hames

Internet:	bytey@werple.apana.org.au

bytey@phoenix.apana.org.au

Fido Net: 	3:633/353

If the above addresses don't work, get the latest version of one of my

other programs such as DirWork or PC-Task which are updated more

frequently.