PC-Dial(tm)

A Communications Program

For the IBM Personal Computer

(C)Copyright 1984 by Jim Button

Version 1.4

Jim Button

ButtonWare

P.O. Box 5786

Bellevue, WA 98006

Source ID : CL2925

Compuserve: 71435,2012

(206)746-4296

-

TABLE OF CONTENTS

GENERAL INFORMATION:.................................... 1

GETTING STARTED......................................... 2

USING  PC-Dial's AUTOMATIC CAPABILITIES................. 3

USING THE FUNCTION KEYS................................. 4

COMMANDS................................................ 9

THE "PROFILE" FILE (PC-DIAL.PRO)........................12

STOPPING THE PROGRAM....................................12

COMMAND KEY SUMMARY  (A "quick-reference" sheet)........13

ERROR MESSAGES..........................................14

SOME FREQUENTLY ASKED QUESTIONS:........................14

DISCLAIMER..............................................15

PERMISSION TO COPY:.....................................15

THE USER-SUPPORTED CONCEPT..............................15

-

PC-Dial(tm) USERS GUIDE               Page 1.

GENERAL INFORMATION:

PC-Dial is a general purpose asynchronous communications program.

Although it employs a simple and straightforward approach to

communications, its capabilities surpass those of some of the more

expensive communications programs on the market today.

With PC-Dial you'll be able to:

-   Communicate at speeds up to 9600 baud (limited by the speed of

your serial port and modem.)

-   Exchange (send or receive) text files with other computers.

While sending text files, you may dynamically send selected

records from the file and skip past other records not to be

sent.

-   Exchange (send or receive) non-text (exe or com) files with

other computers which support the "X-Modem" protocol.

-   Use modems which don't support Auto-dial.

-   Use modems which do support Auto-dial. This includes not only

the Hayes SmartModem, but also a variety of other modems which

use other commands to AutoDial a phone number.

-   Have PC-Dial automatically redial a phone number over and over

again, until connection is made.

-   Capture incoming data in a disk file.

-   Setup your own "filters" to remove or change undesirable

characters in the incoming data.

-   Dynamically slow down the speed at which you transmit, so as

not to overrun the computer at the other end.

-   Dynamically alter your communications parameters, such as

Parity, Speed, and number of data bits.

-   Build automatic logon sequences of any length, to enable you to

log on to a variety of remote computers without having to

remember phone numbers, passwords and access commands.

-   Change the colors on your display (if you have a color display)

to suit your personal taste.

-   Move back and forth between different directories on your hard

disk (if you have DOS 2.0) for retrieving and sending data.

-   Communicate properly with computers which use "XON/XOFF"

characters.

In short, PC-Dial is easy to use, easy to understand, powerful, and

small in size.

( 1 )

-

PC-Dial(tm) USERS GUIDE               Page 2.

PC-Dial requires a 64K or larger IBM PC (with PC-DOS 1.0 or 1.1), a

96K or larger IBM PC (with PC-DOS 2.0 or later), one disk drive,

and a serial I/O port with a Modem.  You must also have a video

monitor capable of displaying 80 characters per line.

GETTING STARTED.

1.  Put your PC-Dial diskette into the "A:" drive and type "A:DOC".

This will produce a printed set of documentation for you.

2.  Read the documentation.

3.  Copy the "PC-DIAL.COM" program onto any disk which has been

formatted with PC-DOS. This will become your communications

disk.

4.  File your PC-Dial distribution diskette in a safe place. You

may need it someday for "backup" purposes.

5.  Turn on your modem and make sure that you are properly

connected to the phone line and to your PC.

6.  To run PC-Dial, assign the "logged on" drive to the disk that

contains the PC-Dial program, like this:

B:

Then type the program name, like this:

PC-DIAL

7.  You will be asked to choose one of five pre-chosen

communications protocols. You can probably choose number two,

as this is by far the most common one in use. To see what these

protocols are, you will have to turn to your IBM BASIC manual

(the one which came with your PC).  Turn to section 4 and find

the documentation for the "OPEN 'COM..." statement.

8.  Place a phone call to the computer with which you wish to

communicate. When you hear the high pitched squeel from the

other end, you can switch your modem to "data" mode and you

should be on the air.  If you have an autodial modem, you

should read carefully the section on using autodial modems.

9.  When you are through communicating, type in a "(Ctrl)\"  (while

holding down the Ctrl key, type the backwards slash) and this

will cause PC-Dial to return control back to DOS.

10.  Switch your modem back to "talk" mode.

( 2 )

-

PC-Dial(tm) USERS GUIDE               Page 3.

USING  PC-Dial's AUTOMATIC CAPABILITIES

For normal operations, it is indeed possible to use PC-Dial knowing

only that whatever you enter through the keyboard will be sent out

over the phone line.  However, to really make use of the program,

you need to understand just a few of the "neat" features.

Although PC-Dial can do the normal transfer of files using either

X-Modem protocol or ASCII transfer with XON/XOFF standards, it has

the unique and powerful ability to send one record each time the

"send record" key is depressed.  This, coupled with the ability to

preview each record before sending it, and to skip forward in the

transmit file to selectively transmit records, makes the program

flexible enough to adjust to almost any requirement.

The record that is sent can be data intended for the remote

computer. It might also be a command to be sent to your

"intelligent" modem.  Or, it can be a command which PC-Dial will

intercept and act upon. For example, I've created the following

file which allows me to call up a local computer and log onto it:

\COM1:0300,N,8,1

AT DT 575-3011

BUTTON

JIM

BELLEVUE, WA

JGB

I have built one of these little files for each computer that I

call regularly.  One of the files is named "SOURCE.LOG", another is

named "COMPUSER.LOG", and another is named "DOWJONES.LOG".

When I first start PC-Dial and am ready to communicate, I push F9

("send record") and PC-Dial asks me "FILE NAME TO SEND:".  I enter

the name of my "automatic logon" file for the remote computer with

which I wish to communicate.  I then press F9 again.  PC-Dial reads

the first record (\COM1:0300,N,8,1) and sees that it's a command to

automatically set the communications protocol for that computer. It

does it.  I press (F9) again. The program reads the next record (AT

DT 575-3011) and since it's not a command, it sends it out over the

line.  As it turns out, my Hayes SmartModem sees this and realizes

that it's a command to it to dial the phone number.  It dials the

number.

When I hear the beep from the remote computer, I know that the

connection is completed. Also, the remote computer detects my

presence and asks me for my last name.  I press F9 again.  PC-Dial

reads the next record (BUTTON) and sends it out over the line.  The

remote computer asks me for my first name. I press F9 again and the

PC reads the next record (JIM) and sends it. The remote computer

asks me what city I'm calling from. I press F9 and PC-Dial sends

the record (BELLEVUE, WA).  The remote computer asks me for the

password, I press F9, and PC-Dial sends the last record (JGB).

Notice that I don't have to remember to enter anything special in

( 3 )

-

PC-Dial(tm) USERS GUIDE               Page 4.

order to automatically log on to the remote computer.  All I need

to do is press F9 each time that input is needed from me.  Since I

built the logon file with the proper commands, everything else

happens automatically.

You should also know that I could freely intermix data from the

keyboard with pressing of F9. Thus, if something went wrong in the

logon sequence, such as the remote computer saying "We are taking a

survey today, please answer a question...", I could type some input

from the keyboard which would be sent over the line. I could later

go back to using F9 to complete my logon sequence, or I could just

continue to do it manually from the keyboard.

In order to create one of these "logon" files, you'll need an

editor program, such as Edlin, Wordstar, or Personal Editor.  Then,

you will need to keep track of all the keys you press while you're

calling and logging on to the remote computer. When you have done

this once, you can use your editor program to create a "logon" file

for that computer and you're then ready to have it all happen

automatically, as described above.

This is an extremely simple concept, but it gives PC-Dial

tremendous power and flexibility.  It's so easy to use, easy to

implement, and pleasant to work with that I'm surprised it's not

available in all other communications programs.

Another use for this "send record" capability is in the sending of

electronic mail.  You can build your letter using your editor

program, and save it in a file called "LETTER". Then, call up your

information utility and when you get to the spot where they prompt

you for the first line of your letter, just push F9 and send the

first line of your "LETTER" file. Each time they ask you for the

next line, push F9.  The remote computer thinks that you certainly

are a fast and accurate typist!  It would get confused if you sent

it the whole file at once, (as some other communications programs

force you to do), because it was programmed to prompt you before

the sending of each line. No problem when you're using PC-Dial.

After you have sent the last line of your "LETTER" file, you just

continue communications from the keyboard.

USING THE FUNCTION KEYS.

While PC-Dial is running you will see a command key summary line

printed at the bottom of your screen. The command keys may be

pushed at any time the program is running and will take effect

immediately. (Exceptions to this are while you are in "send file"

mode (F7), or are uploading/downloading files using the "X-Modem"

protocol). During the X-modem up/download process, the only command

keys active will be F2(stop receive) and F10(stop send). If either

of these keys is pressed, the file transfer will be discontinued.)

(F1) - Receive file.

Pressing (F1) tells PC-Dial that you would like to receive a file.

( 4 )

-

PC-Dial(tm) USERS GUIDE               Page 5.

You will be asked for the File name to receive into.  Enter any

valid "filespec". Example: "B:INMAIL.TXT".  (Omit the quote marks

and the trailing period.)  From then on, every character coming in

on the communications line will automatically be saved on the disk

file which you have named.  If the file didn't exist previously, it

will be created. If the file did exist previously, all incoming

data will be added at the end of the previous data.

This facility is excellent for keeping a "log" of your entire

communications session. After you are finished, you can go back in

with your editor program and clean up the disk file containing the

record of your session (removing the parts which are unimportant).

Then, just print the whole thing on your printer and save it for

future reference.

If you wish to receive the file using "X-Modem" protocol (which

automatically does error checking and error recovery), wait until

the remote computer says it is ready to send the file, then press

(F1) and enter the filespec followed by the characters =X.

Example: "B:DOWNLOAD.COM=X".  Caution: When you are using X-Modem

for file transfer, you are required to use 8-data bits in your

communications protocol.  Read the section on (F4) changing

communications parameters.  With X-Modem transfer, a new file will

always be created. If one existed previously by the same name, it

will first be deleted.

(F2) - Stop Receive of file.

Pressing (F2) will "close" the disk file and stop storing the

incoming characters.  Later on, you could always press (F1) and

reopen the same file to save more incoming data following the

original data.  If you are receiving a file using "X-Modem, stay

away from (F2) unless you have to abort the download.  The file

will automatically be closed when the sending computer tells you

that the last data in the X-Modem transmission has been sent.

(F3) - Echo ON/OFF.

Normally, the computer which you are communicating with will echo

your data back to you, which causes it to be printed on your

screen.  However, sometimes the remote computer won't do this for

you. In this case, you will not be able to see your keystrokes as

you enter them.  When this happens, pressing (F3) will turn "echo"

on, and PC-Dial will automatically display your data as you enter

it. Pressing (F3) again will turn "echo" off again. Each time you

press the key, "echo" will switch to the opposite state.  If you

ever see "double" characters on your screen, it is because "echo"

is on and the remote computer also is echoing your data back to

you. Just press (F3) to turn "echo" off at this end, and everything

should look normal again.

(F4) - Change Communications Parameters.  Sometimes you will want

to change one or more of your communications parameters.  You may

need to slow down your speed from 1200 baud to 300 baud. Or you may

need to change the number of "data bits" from 7 to 8. Pressing (F4)

( 5 )

-

PC-Dial(tm) USERS GUIDE               Page 6.

will allow you to make these changes from the keyboard.  You will

be given six choices. Choice number 1 is to return to

communications. Use this if you pressed (F4) accidentally and don't

wish to make any changes. The next two choices are some standard

protocols for 300 baud transmission speed. The next two choices are

some standard protocols for 1200 baud transmission speed. The last

choice, called "SPECIFY" will let you type in an entire

communications specification, assuming that none of the above

choices is adequate.

For almost all cases, one of the four predefined communications

protocols will be adequate. Select number 6 "SPECIFY" only if you

have to.  You then will be shown a "skeleton" of the standard entry

format and asked to completely overstrike it with your new

specification.  Your textbook on accomplishing this is the IBM

BASIC manual which came with your PC.  You will have to turn to

section 4 in the manual and locate the pages which describe the

"OPEN COM..." statement.  If you can understand this material, you

will have no trouble entering whatever communications protocol is

required.

(F5) - Set the "DELAY" counter.

When you are sending files or records to the remote computer, you

will occasionally come across a computer which can't receive data

as fast as you can send it.  This is normally handled automatically

by the remote computer sending you an "x-off" character, in which

case PC-Dial will stop sending until the remote computer sends an

"x-on" character. In those cases where the remote computer doesn't

know about x-on and x-off and also can't receive as fast as you can

send, they will lose some of your data.  This will be seen by you

as missing parts of your message or file as they are echoed to your

screen.

You can slow down your sending speed to give the remote computer

time to catch its breath between characters by using the (F5) key.

This allows you to set the "delay counter", which simply tells

PC-Dial to count to some number after each character that it sends.

This, of course, slows the output down.  You will be shown the

current value of the delay counter, which is 0 at startup. You will

have to enter some new number, which will become the value of the

delay counter. Some experimentation will be required on your part

to discover a number just large enough to slow things down to the

right amount.  Values of a few hundred will slow things down a

little bit (this computer counts fast!)  and values in the multiple

hundreds will slow things down much more.

(F6) - Change Directory.

If you are using DOS 2.0 or a later version, with perhaps a "hard

disk", it is nice while running a program to be able to change to a

different directory without having to stop and restart the program.

PC-Dial lets you change directories whenever you press (F6). You

will be asked which directory you would like to change to. Just

give the name of the new directory. You'll be told whether or not

( 6 )

-

PC-Dial(tm) USERS GUIDE               Page 7.

the change was successful.  This works just like the DOS "CHDIR"

command.  It's very convenient, when you're sending or receiving

files, to be able to change to the appropriate directory right in

the middle of your communications session.

(F7) - Send File.

When you wish to send a file to the remote computer, you can press

(F7) to send the entire file without stopping. You will be asked to

enter the name of the file to be transmitted.  Enter a valid file

specification.  Example: "B:SAMPLE.DOC". (Omit the quote marks and

the final period).  After receiving the file name, PC-Dial will

start transmitting the file, so be sure not to press (F7) until the

remote computer is ready to receive your data. Once transmission

begins, data will be sent continuously until either the entire file

has been sent, or you have pressed (F9) or (F10).

If you will be sending your file using "X-Modem" protocol, wait

until the remote computer has signaled that it is ready to receive

your file.  Then press (F7) and enter the file name followed by the

characters "=X".  Example: "A:DEMO.EXE=X".

(F8) - Turn the PEEK window off or on.

Normally, you will want the bottom line of the screen to show your

command summary menu. However, when you are transmitting a file in

"one record at a time" mode (F9) it is handy to see which record

you are about to transmit, before you press F9 to send it. That

way, if you want to override the record and enter from the keyboard

instead, or just turn off record transmission, you can do so.

Pressing (F8) turns on the peek window. The bottom line of your

screen will now display the record that's "in the batter's box"

(the one that's destined to be transmitted, if you choose to do

so).  This is very handy for your automatic logon sequences.

When the peek window is on you have several options available with

respect to the record being shown in the window.

First, you can ignore the record being shown and provide input

through the keyboard.

Second, you can transmit the record being displayed as if you had

entered it from the keyboard, by pressing (F9).

Third, you can skip past the record in the window to the next

record in the file, by pressing the "cursor down" arrow (the "2"

key on the numeric keypad at the right side of your keyboard).

Fourth, you can skip over multiple records in the file searching

for a certain character sequence. To do this, press the "cursor

right" key (the "6" key on the numeric keypad). You will be shown a

">" prompt. Enter a character string that you would like to search

for in the transmit file, followed by the "return" key. All records

in the transmit file will be bypassed until the character string

you entered is discovered starting in position 1 of a record in the

( 7 )

-

PC-Dial(tm) USERS GUIDE               Page 8.

file.

You can precede the search string with question marks, in which

each question mark will stand for one character to be skipped over

before the comparison is made. For example:

???TH

This example will locate records which start out "SMITH" or

"ARITH", etc.  (The "TH" must start in at the fourth position in

the record).

If you precede the search string with a "greater than" sign,

records will be found which contain the search string at any

location.

>TH

This will locate records which start out "THANK" or "PARENTHESES",

etc.

The peek window will remain on until you press (F8) again, or until

a \PEEK OFF command is read from your transmission file.  If you

are running with the peek window on but can't remember which

function key you need to push, just push (F8) to get your menu

back. View it, then push (F8) again to continue with the peek

window.

(F9) - Send Record.

If you wish to send your file one record at a time, with you

controlling when each record will be sent, use (F9). You will be

asked to enter the name of the file to be transmitted. Enter a

valid file name.  PC-Dial will then wait for you to press (F9)

again before transmitting the first record in your file.  Each time

you press (F9), one more record will be sent out.  In this manner,

you can control the sending of records, usually sending one each

time you observe that the remote computer is ready to receive it.

However; if you transmit your file using X-modem protocol, it will

all be sent without pausing. The "one record at a time" only

applies to upload of ASCII files.

When the record in the window is one of the predefined commands,

PC-Dial will not pause after sending it. Commands will continue to

be sent automatically until data appears in the window which is not

a command.

Rather than transmitting the record in the window, you may choose

to skip past it. This can be done by pressing the "cursor down"

arrow. If you do this, PC-Dial will bypass the record in the peek

window and retrieve the next record from the transmit file.  Or,

you can press the "cursor right" arrow. If you do this, you will be

prompted for a character string to be searched for in the transmit

file. The prompt is ">". Enter the character string followed by the

"return" key. PC-Dial will then bypass all records in the transmit

file until it finds the character string start in column 1 of a

record in your transmit file.

( 8 )

-

PC-Dial(tm) USERS GUIDE               Page 9.

Incidentally, there is a nice way to view an ASCII dataset just to

see what's in it. You can press (F9) to open it, then use the

cursor-down key to view each record in the peek window, without

sending anything over the phone line. Press (F10) when you're

through looking at it.

There is also a "retransmit record" capability provided. When you

are in "send record" mode, you can retransmit the record which was

most recently transmitted. To do this, press (Alt)R. Each time you

press these two keys together, the record will be retransmitted.

This feature makes it easy to redial a phone number if you get a

busy signal. Just wait a few minutes and press (Alt)R to redial the

number.

Your file will be closed for you when you have sent the last

record, or when you press (F10). Note that you can press (F7) at

any point in time and transmission mode will switch from "one

record at a time" to "the whole remainder of the file".

You may have commands for PC-Dial imbedded in your transmit file.

These begin with a backwards slash "\" and have a command encoded

in CAPITAL LETTERS. (See the section on commands).  When PC-Dial

detects one of these commands, it will act on it and will not send

the command out over the phone line as data.

(F10) - Stop Sending the File.

If you wish to stop sending the file before you reach the end of

it, you can press (F10).  PC-Dial will close the file and

discontinue the sending of file data.  If you have been in "send

file" mode (F7), as opposed to "send record" mode (F9), it may take

a while for DOS to empty all of its communications buffers.  Don't

panic. Eventually (after a few more lines of output), the buffers

will all be empty and you will see the data stop going out over the

phone line.

COMMANDS.

Certain commands can be imbedded in your disk files to be acted

upon by PC-Dial whenever it comes across one while transmitting

file data. This is typically done at the start of a file which will

be used to help you automatically log on to a remote computer.  You

may wish to create a series of different files, one for each

computer you will wish to call up and log onto. The commands at the

front of this file can instruct PC-Dial to change its

communications parameters to match those of the remote computer, or

to change the characters which it will "filter" out of the incoming

data, etc.

Here are the commands which can be used.  Important: all commands

must be entered in UPPER CASE only. If they are not, they will be

treated as data and sent out over the phone line.

\CHDIR - Change from the current DOS directory to a new one.

( 9 )

-

PC-Dial(tm) USERS GUIDE              Page 10.

Example: \CHDIR \GAMES   (change to the "games" directory)

\CHDIR \       (change to the "root" directory)

This command works exactly like the DOS "CHDIR" command. Of course,

to use it you must be using DOS version 2.0 or a later version.

\COLOR - Change the colors on the display screen.

Example: \COLOR 14,1

This command will change the colors to be used on your display

screen.  The first number is the color of the foreground (the

characters), and the second number is the color of the background

and border. These color numbers can be found in your IBM BASIC

manual, section 4, under the topic: "COLOR statement".  The

foreground color must be a number from 0 to 15 inclusive, and the

background color must be a number from 0 through 7. In the example

above, the foreground color will be yellow, and the background

color will be blue.

\COM - Change the communications parameters.

Example: \COM1:0300,S,7,1

This command will allow you to change the communications port, the

transmission speed, the type of parity, the number of data bits, or

the number of stop bits to be used during communications. Your

textbook on how to code this is chapter 4 in the IBM BASIC manual

which came with your PC.  You will need to read the pages

describing "OPEN COM..." to learn this command. When PC-Dial

receives this command, it closes its communications port and then

immediately reopens it using the new specifications.

\DELAY - Change the "delay" counter.

Example: \DELAY 500

This command will change the delay counter for you, to slow down

your outgoing transmissions if needed. Read the description on

function key (F5) to understand this.

\FILTER - Establish filters for the incoming data.

Example: \FILTER 12,13,10,0

"Filters" are instructions to PC-Dial to either remove or replace

certain characters as they arrive from the remote computer. The

filter values are entered in pairs. The first value in the pair is

the ASCII value (base 10) of the incoming character to be acted

upon.  The second value in the pair is the ASCII value of the

character to be substituted whenever the first value is found.  In

the example above, if a "Form Feed" (12) is received, it will be

replaced with a "Carriage Return" (13). If the second value of the

pair is a zero, nothing will be substituted for the character.

Instead, it will be entirely removed from the incoming data stream.

In the example above, if a "Line Feed" (10) is found, it will be

( 10 )

-

PC-Dial(tm) USERS GUIDE              Page 11.

removed with nothing substituted for it. These ASCII values can be

found in "Appendix G." at the back of your IBM BASIC manual.

You may establish a maximum of 20 filters. Each time the \FILTER

command is read, the old filters are discarded and the new ones are

established.

\PEEK - Turn the "PEEK" window ON or OFF.

Example:  \PEEK ON

\PEEK OFF

The peek window is the bottom line of your screen. It is normally

turned off, so that the bottom line shows a brief menu of all the

available function keys.

When you are transmitting a file in "semiautomatic" mode (one

record each time you press (F9)), it is very handy to be able to

see which record you are ABOUT to transmit IF you press (F9). That

way, you can decide in advance whether or not to go ahead with the

transmission.  With the peek window on, you will be able to preview

the record that is awaiting transmission. If you decide not to send

it, or to override it with manual entry from the keyboard, you can

do so.

\RECYCLE - Branch back to top of "transmit" file.

Example: \RECYCLE

When the \RECYCLE command is encountered in the transmit file, the

current transmit file is closed, then re-opened at the front of the

file. \RECYCLE cannot be "scanned" past by using the "cursor down"

or "cursor right" keys, as can the other commands.  It is usually

placed at the end of an automatic logon file, so that the entire

sequence can be redone if desired.  If you scan for a record that

doesn't exist beyond your current position, the scan will resume at

the top of the file if you have a \RECYCLE command at the end. In

this way, you will be able to scan for records that are previous to

your current position.  "What (you ask) will happen if the record

I'm scanning for doesn't exist at all?  Will the program loop

forever scanning through the file?" No, after the second scan is

completed the program will stop scanning.

\REPEAT - Repeatedly try to dial a phone number.

Example: \REPEATATDT746-4296>CONNECT

The data after the "\REPEAT" and before the ">" is sent to the

modem.  Income from the line is then scanned repeatedly until the

data after the ">" is located anywhere in the stream.  If the data

is discovered, connection is assumed to be established and the

operator is notified via an alarm.  If the data is not discovered

after about 1 minute, the command is repeated.  This continues

until the operator stops the sequence.

( 11 )

-

PC-Dial(tm) USERS GUIDE              Page 12.

\REM - Reminder to the operator.

Example: \REM You are about to log on to "The Source".

When PC-Dial receives the \REM command, it prints the data portion

on your screen.

\TOFILE - Branch to a different transmit file.

Example: \TOFILE SOURCE.LOG

When the \TOFILE command is encountered, the current transmit file

is closed, and the indicated file is opened as the transmit file.

Unlike the \RECYCLE command, the \TOFILE command can be skipped

over using the "cursor down" or "cursor right" keys.  Thus, you

could have a series of \TOFILE commands in your file, and select

the appropriate one by using the cursor keys and then pressing (F9)

to cause transfer to the selected file to occur.

THE "PROFILE" FILE (PC-DIAL.PRO).

Whenever PC-Dial is started, it looks on the default disk drive for

a file called "PC-DIAL.PRO".  If it finds it, it executes all of

the commands it finds in the file.  I have provided a "default"

profile file for you to show you how it's done, and to get you

started in case you don't care to provide your own.  However, if

you wish to change the file to suit your own operating conditions,

you should do so.  You can use any of the commands found in the

above section.  This would be a good place, for example, to place

your "\COLOR" command to initialize your screen colors.

STOPPING THE PROGRAM.

When you are finished communicating, you can end your session by

pressing (Ctrl)\. To do this, press down on the (Ctrl) key on the

left side of your keyboard. While holding this key down, also press

the "\" (backwards slash) key.  This will cause PC-Dial to close

all disk files and return control to the PC-DOS operating system.

( 12 )

-

PC-Dial(tm) USERS GUIDE              Page 13.

COMMAND KEY SUMMARY  (A "quick-reference" sheet)

F1 - RECEIVE FILE.  Opens a disk file. Saves all incoming data.

The open is for "Append". (see note 1)

F2 - END RECEIVE.   Closes the "receive" file.

F3 - ECHO.          Toggles "ECHO" (to screen) "on" or "off".

F4 - PARMS.         Allows changing the communications parameters.

F5 - DELAY.         Allows slowdown of outbound characters.

F6 - CHDIR.         Changes the current directory (DOS 2.0)

F7 - SEND FILE.     Sends an entire file.

(see note 1). (see note 2).

F8 - PEEK.          Toggles "Peek" to ON/OFF. When on, shows

the record that will be sent next.

F9 - SEND RECORD.   Same as F7, but only one record is sent each

time F9 is pressed.  You may switch back and

forth between F7 and F9.

(see note 1). (see note 2).

F10- STOP SEND.     Closes the "send" file.

(Alt)R              Retransmit a record.

(Alt)B              Send a "BREAK" character

*Note 1: Here are some example file names:

?A         Displays directory on "A:" drive

?C:*.LOG   Displays a directory of all *.LOG files

on "C:" drive

AB.XYZ     File is "AB.XYZ" in ASCII mode

AB.XYZ=X   File is "AB.XYZ" using "X-Modem" protocol

*Note 2: The following "commands", if found in your "SEND" file,

will be acted upon (and not transmitted).

(Use all CAPITAL letters).

\CHDIR     Changes the current directory on disk. Code

as: \CHDIR newname

\COLOR fg,bg

fg and bg are the foreground and background

color numbers from the BASIC manual.

\COM       Resets your communications parameters. Code

as: \COM1.0300,S,7,1  (format used by BASIC)

\DELAY     Sets "delay" counter for file transmissions.

\FILTER    Sets "FILTERS" for the incoming data.

Code as: \FILTER from,to,from,to....

Values are in decimal, ASCII character set.

Example:  \FILTER 10,0,12,13

\PEEK      Turns "Peek" on or off. Peek lets you see

your records before you transmit them.

Code as:  \PEEK ON   or   \PEEK OFF

\RECYCLE   Branches to top of transmit file. Cannot be

scanned past.

\REPEAT    Repeat dials a phone number until completion.

Example:  \REPEATATDT7464296>CONNECT

\REM       A comment.  All data after "REM" will be

printed on screen.

\TOFILE    Transfers to a new transmit file. Example:

\TOFILE B:SOURCE.LOG

( 13 )

-

PC-Dial(tm) USERS GUIDE              Page 14.

ERROR MESSAGES.

To keep the size of the program as small as possible, PC-Dial uses

the standard error messages as discussed in your IBM BASIC manual.

The program will try to recover from all errors, and will always

issue the line number in the program where the error occurred (of

no value to you, but of immense value to me) and the error number.

For the more common errors, it will also print out a brief

description of the error.

You can look up these error numbers in your IBM BASIC manual.  They

are found in "Appendix A" at the back of the book. With each number

is a complete description of the error. After reading this

description, the error will usually be apparent.

SOME FREQUENTLY ASKED QUESTIONS:

1.  "Will it help if I buy more RAM memory for my PC?".  ANSWER:

No, PC-Dial fits very nicely into 64K and would not benefit

from any additional memory.

2.  "In what language was PC-Dial written?".  ANSWER: It was

written in BASIC, then compiled with the BASIC Compiler.  Some

portions were done in Assembler language.

3.  "Can I obtain the source code for the program?".  ANSWER: No,

it's considered to be proprietary.

4.  "What is your update policy?".  ANSWER: All registered users

will be notified by mail of each new update.  However, I don't

foresee a lot of updates to this program.

5.  "Do you have any other programs that you're distributing?".

ANSWER: Yes, PC-File III(tm), a database manager program.

6.  "Can you send me a FREEWARE(tm) catalog?".  ANSWER: No,

ButtonWare and PC-Dial(tm) are not associated with

Freeware(tm).

7.  "Is there a version of PC-Dial for other computers?".  ANSWER:

No.

8.  "Do you pay any taxes on money received for PC-Dial?".  ANSWER:

Yes, I fully report all income to the IRS.  In addition, state

and local taxes totaling 7.9% are paid on all money received

from within the state of Washington.

( 14 )

-

PC-Dial(tm) USERS GUIDE              Page 15.

DISCLAIMER.

In no event will the Author be liable to you for any damages,

including any lost profits, lost savings or other incidental or

consequential damages arising out of the use of or inability to use

this program, even if the Author has been advised of the

possibility of such damages, or for any claim by any other party.

PERMISSION TO COPY:

Clubs and other non-profit organizations are granted permission by

the author to freely copy this program and documentation and share

it with their members, so long as:

1.  No price is charged for the software or documentation.

However, a distribution cost may be charged for the cost of the

diskette, so long as it is not more than $6 total.

2.  Club members are informed of the user-supported concept and

encouraged to support it with their donations.

3.  The program or documentation are not modified in any way and

are always distributed together.

THE USER-SUPPORTED CONCEPT.

User-Supported Software

If after evaluating this program you find it to be useful,

I trust you to send a $25 payment.

Jim Button

P.O. Box 5786

Bellevue, WA  98006

You are encouraged to copy and share this program with others.

User-supported software is an experiment in distributing computer

programs, based on these beliefs:

1.  That the value and utility of software is best assessed by the

user on his/her own system.

2.  That the creation of personal computer software can and should

be supported by the computing community.

3.  That copying of programs should be encouraged, rather than

restricted.

Anyone may request an evaluation copy of a user-supported program

by sending a blank, formatted disk to the author of the program.

An addressed, postage-paid return mailer must accompany the disk

( 15 )

-

PC-Dial(tm) USERS GUIDE              Page 16.

(no exceptions, please). A copy of the program, with documentation

on the disk, will be sent by return mail.  A second way to obtain a

copy of the program for evaluation is from a friend or computer

club.  After you have had a chance to use and evaluate the program

in your own environment, you are trusted to either forward a

payment to the author, or to discontinue use of the program.

Free distribution of software and voluntary payment for its use

eliminates costs for advertising and copy protection schemes.

Users obtain quality software at reduced cost. They can try it out

before buying, and do so at their own pace and in the comfort of

their own home or office.  The best programs will survive, based

purely on their quality and usefulness.

Please join the experiment.

If you believe in these ideals, your contribution is solicited to

help make them work.

Many of you have written requesting an invoice so that you could

initiate payment through your company accounting system.  The

following invoice is provided for those of you in this category, or

for those who wish an invoice for your tax records. Perhaps it will

serve the purpose.

( 16 )

-

--------------------

INVOICE

--------------------

Purchased from:

JIM BUTTON

ButtonWare        Wash. State Tax # C 600 508 801

P.O. Box 5786

Bellevue, WA  98006

DATE:     /     /                           Invoice No. 840101

--------------------------------------------------------------

PRICE     PRICE

PRODUCT                    QTY       EACH      EXTENDED

-------------------------  ---       -----     --------

"PC-Dial"

version 1.4

program and

documentation             1         $25         $25

--------------------------------------

PLEASE PAY THIS AMOUNT    $25    TOTAL

--------------------------------------------------------------

Please make check payable to :JIM BUTTON

You may retain this Invoice for your tax records.

Payments sent from within Washington State:  Please add 7.9%

for state and local tax, total is $26.98

( 17 )

######