--------------------------------------------------------------------------AX25.DOC         AX.25 Amateur Packet-Radio Link-Layer Protocol                    Version 2.0  October 19842.  AX.25 Link-Layer Protocol Specification2.1  Scope and Field of Operation        In  order  to  provide  a  mechanism  for  the   reliable transport  of  data  between  two  signaling  terminals,  it   is necessary  to define a protocol that can accept and deliver  data over a variety of types of communications links.  The AX.25 Link-Layer  Protocol is designed to provide this service,  independent of any other level that may or may not exist.        This protocol conforms to ISO Recommendations 3309,  4335 (including DAD 1&2) and 6256 high-level data link control  (HDLC) and  uses  some terminology found in these  documents.   It  also conforms with ANSI X3.66, which describes ADCCP, balanced mode.        This  protocol  follows,  in principle,  the  CCITT  X.25 Recommendation,  with the exception of an extended address  field and  the addition of the Unnumbered Information (UI)  frame.   It also follows the principles of CCITT Recommendation Q.921  (LAPD) in the use of multiple links, distinguished by the address field, on a single shared channel.        As  defined,  this  protocol will work  equally  well  in either half- or full-duplex Amateur Radio environments.        This protocol has been designed to work equally well  for direct  connections between two individual  amateur  packet-radio stations or an individual station and a multiport controller.        This  protocol allows for the establishment of more  than one  link-layer  connection  per  device, if  the  device  is  so capable.        This  protocol  does not  prohibit  self-connections.   A self-connection  is considered to be when a device establishes  a link  to  itself using its own address for both  the  source  and destination of the frame.        Most  link-layer  protocols assume that one  primary  (or master)  device  (generally  called  a  DCE,  or  data   circuit-terminating equipment) is connected to one or more secondary  (or slave)  device(s)  (usually  called a DTE,  or  data  terminating equipment).   This type of unbalanced operation is not  practical in a shared-RF Amateur Radio environment.  Instead, AX.25 assumes that  both  ends  of  the link are of  the  same  class,  thereby eliminating  the two different classes of devices.  The term  DXE is  used in this protocol specification to describe the  balanced type of device found in amateur packet radio.2.2  Frame Structure        Link  layer packet radio transmissions are sent in  small blocks of data, called frames.  Each frame is made up of  several smaller groups, called fields.  Fig.1 shows the three basic types of  frames.  Note that the first bit to be transmitted is on  the left side.        First        Bit Sent          Flag      Address     Control     FCS      Flag        01111110  112/560 Bits  8 Bits    16 Bits  01111110                Fig. 1A -- U and S frame construction        First        Bit Sent Flag      Address      Control    PID    Info.       FCS      Flag 01111110  112/560 Bits   8 Bits   8 Bits  N*8 Bits   16 Bits  01111110                Fig. 1B -- Information frame construction        Each field is made up of an integral number of octets (or bytes), and serves a specific function as outlined below.2.2.1  Flag Field        The flag field is one octet long.  Since the flag is used to  delimit  frames, it occurs at both the beginning and  end  of each  frame.  Two frames may share one flag, which  would  denote the  end of the first frame, and the start of the next frame.   A flag consists of a zero followed by six ones followed by  another zero,  or  01111110 (7E hex).  As a result of bit  stuffing  (see 2.2.6,  below),  this sequence is not allowed to  occur  anywhere else inside a complete frame.2.2.2  Address Field        The address field is used to identify both the source  of the  frame and its destination.  In addition, the  address  field contains  the  command/response information  and  facilities  for level 2 repeater operation.          The encoding of the address field is described in 2.2.13.2.2.3  Control Field        The  control field is used to identify the type of  frame being  passed  and  control several attributes  of  the  level  2 connection.   It  is  one octet in length, and  its  encoding  is discussed in 2.3.2.1, below.2.2.4  PID Field        The  Protocol  Identifier  (PID) field  shall  appear  in information  frames (I and UI) only.  It identifies what kind  of layer 3 protocol, if any, is in use.        The PID itself is not included as part of the octet count of the information field.  The encoding of the PID is as follows:  M      L  S      S   B      B  yy01yyyy AX.25 layer 3 implemented.  yy10yyyy AX.25 layer 3 implemented.  11001100 Internet Protocol datagram layer 3 implemented.  11001101 Address resolution protocol layer 3 implemented.  11110000 No layer 3 implemented.  11111111 Escape character.  Next octet contains more Level 3           protocol information.        Where:        A y indicates all combinations used.        Note:        All  forms  of  yy11yyyy and yy00yyyy  other  than  those listed  above  are  reserved  at this time  for  future  level  3 protocols.   The  assignment of these formats is  up  to  amateur agreement.   It  is  recommended that the  creators  of  level  3 protocols   contact  the  ARRL  Ad  Hoc  Committee   on   Digital Communications for suggested encodings.2.2.5  Information Field        The  information field is used to convey user  data  from one  end of the link to the other.  I fields are allowed in  only three  types of frames:  the I frame, the UI frame, and the  FRMR frame.   The  I  field can be up to 256 octets  long,  and  shall contain  an integral number of octets.  These  constraints  apply prior to the insertion of zero bits as specified in 2.2.6, below.  Any  information  in the I field shall be passed along  the  link transparently,  except  for the zero-bit  insertion  (see  2.2.6) necessary  to prevent flags from accidentally appearing in the  I field.2.2.6  Bit Stuffing        In  order to assure that the flag bit sequence  mentioned above  doesn't appear accidentally anywhere else in a frame,  the sending  station  shall monitor the bit sequence for a  group  of five  or more contiguous one bits.  Any time five contiguous  one bits  are sent the sending station shall insert a zero bit  after the  first  one  bit.   During frame  reception,  any  time  five contiguous  one  bits  are  received,  a  zero  bit   immediately following five one bits shall be discarded.2.2.7  Frame-Check Sequence        The  frame-check sequence (FCS) is a  sixteen-bit  number calculated  by  both the sender and receiver of a frame.   It  is used  to  insure that the frame was not corrupted by  the  medium used to get the frame from the sender to the receiver.  It  shall be calculated in accordance with ISO 3309 (HDLC) Recommendations.2.2.8  Order of Bit Transmission        With  the  exception of the FCS field, all fields  of  an AX.25 frame shall be sent with each octet's least-significant bit first.  The FCS shall be sent most-significant bit first.2.2.9  Invalid Frames        Any frame consisting of less than 136 bits (including the opening  and closing flags), not bounded by opening  and  closing flags, or not octet aligned (an integral number of octets), shall be  considered  an  invalid frame by the link  layer.   See  also 2.4.4.4, below.2.2.10  Frame Abort        If a frame must be prematurely aborted, at least  fifteen contiguous ones shall be sent with no bit stuffing added.2.2.11  Interframe Time Fill        Whenever   it  is  necessary  for  a  DXE  to  keep   its transmitter  on  while  not actually  sending  frames,  the  time between frames should be filled with contiguous flags.2.2.12  Link Channel States        Not applicable.2.2.13  Address-Field Encoding        The  address  field of all frames shall be  encoded  with both the destination and source amateur call signs for the frame.  Except  for the Secondary Station Identifier (SSID), the  address field  should  be made up of upper-case alpha and  numeric  ASCII characters only.  If level 2 amateur "repeaters" are to be  used, their call signs shall also be in the address field.        The  HDLC address field is extended beyond one  octet  by assigning  the  least-significant  bit of each  octet  to  be  an "extension bit".  The extension bit of each octet is set to zero, to indicate the next octet contains more address information,  or one,  to  indicate  this is the last octet of  the  HDLC  address field.   To make room for this extension bit, the  amateur  Radio call sign information is shifted one bit left.2.2.13.1  Nonrepeater Address-Field Encoding        If  level  2 repeaters are not being  used,  the  address field is encoded as shown in Fig. 2.  The destination address  is the call sign and SSID of the amateur radio station to which  the frame is addressed, while the source address contains the amateur call  sign  and SSID of the station that sent the  frame.   These call signs are the call signs of the two ends of a level 2  AX.25 link only.        First        Octet Sent                     Address Field of Frame        Destination Address         Source Address        A1 A2 A3 A4 A5 A6 A7    A8 A9 A10 A11 A12 A13 A14        Fig. 2 -- Nonrepeater Address-Field Encoding        A1  through A14 are the fourteen octets that make up  the two  address  subfields of the address

field.   The  destination subaddress is seven octets long (A1 thru A7), and is sent  first.  This  address sequence provides the receivers of frames  time  to check  the  destination address subfield to see if the  frame  is addressed to them while the rest of the frame is being  received.  The  source  address subfield is then sent in octets  A8  through A14.   Both  of these subfields are encoded in the  same  manner, except  that  the last octet of the address field  has  the  HDLC address extension bit set.        There  is  an octet at the end of each  address  subfield that contains the Secondary Station Identifier (SSID).  The  SSID subfield  allows an Amateur Radio operator to have more than  one packet-radio station operating under the same call sign.  This is useful when an amateur wants to put up a repeater in addition  to a regular station, for example.  The C bits (see 2.4.1.2,  below) and H bit (see 2.2.13.2, below) are also contained in this octet, along with two bits which are reserved for future use.        Fig.  3A shows a typical AX.25 frame in  the  nonrepeater mode of operation.        Octet   ASCII   Bin.Data  Hex Data        Flag            01111110     7E         A1       K     10010110     96         A2       8     01110000     70         A3       M     10011010     9A         A4       M     10011010     9A         A5       O     10011110     9E         A6     space   01000000     40         A7     SSID    11100000     E0         A8       W     10101110     AE         A9       B     10000100     84         A10      4     01100100     68         A11      J     10010100     94         A12      F     10001100     8C         A13      I     10010010     92         A14    SSID    01100001     61       Control    I     00111110     3E         PID    none    11110000     F0         FCS    part 1  XXXXXXXX     HH         FCS    part 2  XXXXXXXX     HH        Flag            01111110     7E        Bit position    76543210      Fig. 3A -- Nonrepeater AX.25 frame        The frame shown is an I frame, not going through a  level 2 repeater, from WB4JFI (SSID=0) to K8MMO (SSID=0), with no level 3  protocol.   The P/F bit is set; the  receive  sequence  number (N(R)) is 1; the send sequence number (N(S)) is 7.2.2.13.1.1  Destination Subfield Encoding        Fig.  3 shows how an amateur call sign is placed  in  the destination address subfield, occupying octets A1 thru A7.             Octet   ASCII   Bin.Data   Hex Data              A1       W     10101110      AE              A2       B     10000100      84              A3       4     01101000      68              A4       J     10010100      94              A5       F     10001100      8C              A6       I     10010010      92              A7     SSID    CRRSSID0           Bit Position-->   76543210          Fig. 3 -- Destination Field Encoding        Where:        1.  The top octet (A1) is the first octet sent, with  bit             0  of each octet being the first bit sent, and bit  7             being the last bit sent.        2.  The  first (low-order or bit 0) bit of each octet  is             the HDLC address extension bit, which is set to  zero             on all but the last octet in the address field, where             it is set to one.       3.   The  bits marked "r" are reserved bits.  They may  be             used in an agreed-upon manner in individual networks.              When not implemented, they should be set to one.        4.  The  bit marked "C" is used as  the  command/response             bit of an AX.25 frame, as outlined in 2.4.1.2 below.        5.  The  characters of the call sign should  be  standard             seven-bit  ASCII  (upper  case only)  placed  in  the             leftmost seven bits of the octet to make room for the             address  extension  bit.  If the call  sign  contains             fewer  than six characters, it should be padded  with             ASCII spaces between the last call sign character and             the SSID octet.        6.  The  0000  SSID is reserved for  the  first  personal             AX.25 station. This establishes one standard SSID for             "normal" stations to use for the first station.2.2.13.2  Level 2 Repeater-Address Encoding        If  a  frame  is to go through  level  2  amateur  packet repeater(s), there is an additional address subfield appended  to the end of the address field.  This additional subfield  contains the call sign(s) of the repeater(s) to be used.  This allows more than one repeater to share the same RF channel.  If this subfield exists,  the  last octet of the source subfield has  its  address extension  bit  set to zero, indicating that  more  address-field data  follows.  The repeater-address subfield is encoded  in  the same  manner  as the destination and  source  address  subfields, except for the most-significant bit in the last octet, called the "H" bit.  The H bit is used to indicate whether a frame has  been repeated or not.        In  order  to provide some method of  indicating  when  a frame has been repeated, the H bit is set to zero on frames going to  a repeater.  The repeater will set the H bit to one when  the frame  is retransmitted.  Stations should monitor the H bit,  and discard  any frames going to the repeater (uplink frames),  while operating  through  a repeater.  Fig. 4 shows how  the  repeater- address subfield is encoded.  Fig. 4A is an example of a complete frame after being repeated.                Octet   ASCII   Bin.Data  Hex Data                 A15      W     10101110     AE                 A16      B     10000100     84                 A17      4     01101000     68                 A18      J     10010100     94                 A19      F     10001100     8C                 A20      I     10010010     92                 A21    SSID    HRRSSID1              Bit Order  -->    76543210              Fig. 4 -- Repeater-address encoding   Where:   1.  The  top octet is the first octet sent, with bit  0  being        sent first and bit 7 sent last of each octet.   2.  As  with  the  source and  destination  address  subfields        discussed  above, bit 0 of each octet is the HDLC  address        extension  bit, which is set to zero on all but  the  last        address octet, where it is set to one.   3.  The  "R"  bits are reserved in the same manner as  in  the        source and destination subfields.   4.  The  "H" bit is the has-been-repeated bit.  It is  set  to        zero  whenever a frame has not been repeated, and  set  to        one by the repeater when the frame has been repeated.        Octet    ASCII   Bin.Data  Hex Data        Flag             01111110     7E         A1        K     10010110     96         A2        8     01110000     70         A3        M     10011010     9A         A4        M     10011010     9A         A5        O     10011110     9E         A6      space   01000000     40         A7      SSID    11100000     E0         A8        W     10101110     AE         A9        B     10000100     84         A10       4     01101000     68         A11       J     10010100     94         A12       F     10001100     8C         A13       I     10010010     92         A14     SSID    01100000     60         A15       W     10101110     AE         A16       B     10000100     84         A17       4     01101000     68         A18       J     10010100     94         A19       F     10001100     8C         A20       I     10010010     92         A21     SSID    11100011     E3       Control     I     00111110     3F         PID     none    11110000     F0         FCS     part 1  XXXXXXXX     HH         FCS     part 2  XXXXXXXX     HH        Flag             01111110     7E        Bit position     76543210      Fig. 4A -- AX.25 frame in repeater mode        The  above frame is the same as Fig. 3A, except  for  the addition of a repeater-address subfield (WB4JFI, SSID=1).  The  H bit is set, indicating this is from the output of the repeater.2.2.13.3 Multiple Repeater Operation        The  link-layer AX.25 protocol allows  operation  through more  than  one  repeater, creating  a  primitive  frame  routing mechanism.   Up to eight repeaters may be used by  extending  the repeater-address subfield.  When there is more than one  repeater address,  the repeater address immediately following  the  source address  subfield  will be considered the address  of  the  first repeater  of  a multiple-repeater chain.  As a  frame  progresses through  a chain of repeaters, each successive repeater will  set the  H bit (has-been-repeated bit) in its SSID octet,  indicating that  the  frame has been successfully repeated through  it.   No other  changes  to the frame are made (except for  the  necessary recalculation of the FCS).  The destination station can determine the  route  the frame took to each it by  examining  the  address field.        The  number of repeater addresses is variable.   All  but the last repeater address will have the address extension bits of all  octets  set to zero, as will all but the  last  octet  (SSID octet) of the last repeater address.  The last octet of the  last repeater address will have the address extension bit set to  one, indicating the end of the address field.        It should be noted that various timers (see 2.4.7, below) may  have  to be adjusted to accommodate  the  additional  delays encountered  when a frame must pass through  a  multiple-repeater chain,  and  the return acknowledgement must travel  through  the same path before reaching the source device.        It  is anticipated that multiple-repeater operation is  a temporary

method of interconnecting stations over large distances until  such  time that a layer 3 protocol is in use.   Once  this layer 3 protocol becomes operational, repeater chaining should be phased out.2.3  Elements of Procedure2.3.1       The  elements of procedure are defined in terms  of  actions that occur on receipt of frames.2.3.2  Control-Field Formats and State Variables2.3.2.1  Control-Field Formats        The control field is responsible for identifying the type of  frame  being sent, and is also used to  convey  commands  and responses  from  one  end of the link to the other  in  order  to maintain proper link control.        The  control  fields  used in AX.25 use  the  CCITT  X.25 control fields for balanced operation (LAPB), with an  additional control field taken from ADCCP to allow connectionless and round-table operation.        There are three general types of AX.25 frames.  They  are the Information frame (I frame), the Supervisory frame (S frame), and  the  Unnumbered  frame (U frame).  Fig. 5  shows  the  basic format  of  the  control field associated  with  these  types  of frames.              Control-Field      Control-Field Bits                  Type         7  6  5  4  3  2  1  0                  I Frame          N(R)   P    N(S)   0                S Frame          N(R)  P/F S  S  0  1                U Frame        M  M  M P/F M  M  1  1                  Fig. 5 -- Control-field formats        Where:        1.  Bit 0 is the first bit sent and bit 7 is the last bit             sent of the control field.        2.  N(S) is the send sequence number (bit 1 is the LSB).        3.  N(R)  is  the receive sequence number (bit 5  is  the             LSB).        4.  The  "S" bits are the supervisory function bits,  and             their encoding is discussed in 2.3.4.2.        5.  The  "M" bits are the unnumbered frame modifier  bits             and their encoding is discussed in 2.3.4.3.        6.  The  P/F bit is the Poll/Final bit.  Its function  is             described in 2.3.3.  The distinction between  command             and response, and therefore the distinction between P             bit and F bit, is made by addressing rules  discussed             in 2.4.1.2.2.3.2.1.1  Information-Transfer Format        All I frames have bit 0 of the control field set to zero.  N(S)  is  the sender's send sequence number  (the  send  sequence number  of  this frame).  N(R) is the sender's  receive  sequence number (the sequence number of the next expected received frame).  These numbers are described in 2.3.2.4.  In addition, the P/F bit is to be used as described in 2.4.2.  2.3.2.1.2  Supervisory Format        Supervisory  frames  are denoted by having bit 0  of  the control  field set to one, and bit 1 of the control field set  to zero.   S  frames  provide  supervisory  link  control  such   as acknowledging or requesting retransmission of I frames, and link-level window control.  Since S frames do not have an  information field,  the  sender's send variable and  the  receiver's  receive variable are not incremented for S frames.  In addition, the  P/F bit is used as described in 2.4.2.2.3.2.1.3  Unnumbered Format        Unnumbered frames are distinguished by having both bits 0 and 1 of the control field set to one.  U frames are  responsible for  maintaining additional control over the link beyond what  is accomplished  with  S  frames.  They  are  also  responsible  for establishing  and  terminating link connections.  U  frames  also allow  for the transmission and reception of information  outside of   the  normal  flow  control.   Some  U  frames  may   contain information and PID fields.  The P/F bit is used as described  in 2.4.2.2.3.2.2  Control-Field Parameters2.3.2.3  Sequence Numbers        Every  AX.25  I  frame shall be  assigned,  modulo  8,  a sequential  number  from  0 to 7.  This will allow  up  to  seven outstanding I frames per level 2 connection at a time.2.3.2.4  Frame Variables and Sequence Numbers2.3.2.4.1  Send State Variable V(S)        The send state variable is a variable that is internal to the  DXE  and  is never sent.  It contains  the  next  sequential number  to  be assigned to the next transmitted  I  frame.   This variable is updated upon the transmission of each I frame.2.3.2.4.2  Send Sequence Number N(S)        The send sequence number is found in the control field of all  I  frames.  It contains the sequence number of the  I  frame being sent.  Just prior to the transmission of the I frame,  N(S) is updated to equal the send state variable.  2.3.2.4.3 Receive State Variable V(R)        The receive state variable is a variable that is internal to the DXE.  It contains the sequence number of the next expected received I frame.  This variable is updated upon the reception of an  error-free  I  frame whose send sequence  number  equals  the present received state variable value.2.3.2.4.4  Received Sequence Number N(R)        The  received sequence number is in both I and S  frames.  Prior  to  sending an I or S frame, this variable is  updated  to equal  that  of  the received  state  variable,  thus  implicitly acknowledging  the  proper  reception of all  frames  up  to  and including N(R)-1.2.3.3  Functions of Poll/Final (P/F) Bit        The  P/F bit is used in all types of frames.  It is  used in  a  command  (poll) mode to request an immediate  reply  to  a frame.   The  reply  to this poll is  indicated  by  setting  the response  (final)  bit  in  the  appropriate  frame.   Only   one outstanding  poll condition per direction is allowed at  a  time.  The procedure for P/F bit operation is described in 2.4.2.2.3.4  Control Field Coding for Commands and Responses        The following commands and responses, indicated by  their control field encoding, are to be use by the DXE:2.3.4.1  Information Command Frame Control Field        The  function  of  the  information  (I)  command  is  to transfer   across  a  data  link  sequentially  numbered   frames containing an information field.        The  information-frame control field is encoded as  shown in  Fig. 6.  These frames are sequentially numbered by  the  N(S) subfield to maintain control of their passage over the link-layer connection.                          Control Field Bits                           7 6 5 4 3 2 1 0                            N(R) P  N(S) 0                    Fig. 6 -- I frame control field2.3.4.2  Supervisory Frame Control Field        The supervisory frame control fields are encoded as shown in Fig. 7.        Control Field Bits       7   6   5   4   3   2   1   0        Receive Ready     RR        N(R)    P/F  0   0   0   1        Receive Not Ready RNR       N(R)    P/F  0   1   0   1        Reject            REJ       N(R)    P/F  1   0   0   1                Fig. 7 -- S frame control fields                          The Frame identifiers:C or SABM          Layer 2 Connect RequestD or DISC          Layer 2 Disconnect RequestI                  Information FrameRR                 Receive Ready. System Ready To ReceiveRNR or NR          Receive Not Ready. TNC Buffer FullRJ or REJ          Reject Frame. Out of Sequence or DuplicateFRMR               Frame Reject. Fatal ErrorUI                 Unnumbered Information Frame. "Unproto"DM                 Disconnect Mode. System Busy or Disconnected.2.3.4.2.1  Receive Ready (RR) Command and Response        Receive Ready is used to do the following:   1.  to  indicate  that the sender of the RR is  now  able  to        receive more I frames.   2.  to  acknowledge  properly  received I frames  up  to,  and        including N(R)-1, and   3.  to clear a previously set busy condition created by an RNR        command having been sent.        The status of the DXE at the other end of the link can be requested  by  sending a RR command frame with the P-bit  set  to one.2.3.4.2.2  Receive Not Ready (RNR) Command and Response        Receive Not Ready is used to indicate to the sender of  I frames  that  the receiving DXE is temporarily  busy  and  cannot accept any more I frames.  Frames up to N(R)-1 are  acknowledged.  Any I frames numbered N(R) and higher that might have been caught between states and not acknowledged when the RNR command was sent are not acknowledged.        The RNR condition can be cleared by the sending of a  UA, RR, REJ, or SABM frame.        The status of the DXE at the other end of the link can be requested  by sending a RNR command frame with the P bit  set  to one.2.3.4.2.3  Reject (REJ) Command and Response        The  reject frame is used to request retransmission of  I frames  starting  with N(R).  Any frames that were  sent  with  a sequence number of N(R)-1 or less are acknowledged.  Additional I frames may be appended to the retransmission of the N(R) frame if there are any.        Only  one  reject  frame condition  is  allowed  in  each direction  at  a time.  The reject condition is  cleared  by  the proper  reception of I frames up to the I frame that  caused  the reject condition to be initiated.        The status of the DXE at the other end of the link can be requested  by sending a REJ command frame with the P bit  set  to one.2.3.4.3  Unnumbered Frame Control Fields        Unnumbered  frame control fields are either  commands  or responses.        Fig.  8 shows the layout of U frames  implemented  within this protocol.        Control Field                Type     Control Field Bits                                            7  6  5  4  3  2  1  0Set Asynchronous Balanced Mode-SABM  Cmd    0  0  1  P  1  1  1  1Disconnect-DISC                      Cmd

0  1  0  P  0  0  1  1Disconnected Mode-DM                 Res    0  0  0  F  1  1  1  1Unnumbered Acknowledge-UA            Res    0  1  1  F  0  0  1  1Frame Reject-FRMR                    Res    1  0  0  F  0  1  1  1Unnumbered Information-UI           Either  0  0  0 P/F 0  0  1  1                Fig. 8 -- U frame control fields2.3.4.3.1  Set Asynchronous Balanced Mode (SABM) Command        The  SABM  command  is  used  to  place  2  DXEs  in  the asynchronous balanced mode.  This is a balanced mode of operation known as LAPB where both devices are treated as equals.        Information fields are not allowed in SABM commands.  Any outstanding  I frames left when the SABM command is  issued  will remain unacknowledged.        The  DXE  confirms  reception and acceptance  of  a  SABM command   by  sending  a  UA  response  frame  at  the   earliest opportunity.   If  the  DXE is not capable of  accepting  a  SABM command, it should respond with a DM frame if possible.2.3.4.3.2  Disconnect (DISC) Command        The  DISC  command is used to terminate  a  link  session between  two  stations.  No information field is permitted  in  a DISC command frame.        Prior  to  acting on the DISC frame,  the  receiving  DXE confirms acceptance of the DISC by issuing a UA response frame at its  earliest opportunity.  The DXE sending the DISC  enters  the disconnected state when it receives the UA response.        Any  unacknowledged  I frames left when this  command  is acted upon will remain unacknowledged.2.3.4.3.3  Frame Reject (FRMR) Response2.3.4.3.3.1       The FRMR response frame is sent to report that the  receiver of  a frame cannot successfully process that frame and  that  the error condition is not correctable by sending the offending frame again.  Typically this condition will appear when a frame without an  FCS  error  has  been received  with  one  of  the  following conditions: 1.  The  reception of an invalid or not implemented  command  or      response frame. 2.  The reception of an I frame whose information field  exceeds      the agreed-upon length.  (See 2.4.7.3, below.) 3.  The  reception  of an improper N(R).  This  usually  happens      when the N(R) frame has already been sent and  acknowledged,      or when N(R) is out of sequence with what was expected. 4.  The reception of a frame with an information field where one      is  not  allowed, or the reception of a U or S  frame  whose      length is incorrect.  Bits W and Y described in  2.3.4.3.3.2      should both be set to one to indicate this condition.      5.  The reception of a supervisory frame with the F bit set      to  one,  except  during a  timer  recovery  condition  (see      2.4.4.9), or except as a reply to a command frame sent  with      the  P  bit  set to one. Bit W  (described  in  2.3.4.3.3.2)      should be set to one. 6.  The reception of an unexpected UA or DM response frame.  Bit      W should be set to one. 7.  The reception of a frame with an invalid N(S).  Bit W should be     set to one.        An  invalid N(R) is defined as one which points to  an  I frame  that previously has been transmitted and acknowledged,  or an  I  frame which has not been transmitted and is not  the  next sequential I frame pending transmission.          An  invalid N(S) is defined as an N(S) that is  equal  to the  last transmitted N(R)+k and is equal to the  received  state variable  V(R),  where  k is the maximum  number  of  outstanding information frames as defined in 2.4.7.4 below.        An  invalid  or not implemented command  or  response  is defined  as a frame with a control field that is unknown  to  the receiver of this frame.2.3.4.3.3.2       When a FRMR frame is sent, an information field is added  to the  frame that contains additional information indicating  where the  problem  occurred.  This information field is  three  octets long and is shown in Fig. 9.                     Information Field Bits 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0  0  0  0  0  Z  Y  X  W   V(R)    C   V(S)  0  Rejected Frame                                    R            Control Field             Fig. 9 -- FRMR frame information field  Where:  1.  The rejected frame control field carries the control  field       of  the frame that caused the reject condition.  It  is  in       bits 0-7 of the information field.  2.  V(S)  is  the  current send state variable  of  the  device       reporting the rejection (bit 9 is the low bit).  3.  The  CR bit is set to zero to indicate the  rejected  frame       was a command, or one if it was a response.  4.  V(R)  is the current receive state variable of  the  device       reporting rejection (bit 13 is the low bit).  5.  If W is set to 1, the control field received was invalid or       not implemented.  6.  If  X  is  set  to 1, the  frame  that  caused  the  reject       condition  was considered invalid because it was a U  or  S       frame  that had an information field that is  not  allowed.        Bit W must be set to 1 in addition to the X bit.  7.  If  Y is set to 1, the control field received and  returned       in   bits   exceeded  the  maximum   allowed   under   this       recommendation in 2.4.7.3, below.  8.  If  A is set to 1, the control field received and  returned       in bits 1 to 8 contained an invalid N(R).  9.  Bits 8, and 20 to 23 are set to 0.2.3.4.3.4  Unnumbered Acknowledge (UA) Response        The  UA  response  frame  is  sent  to  acknowledge   the reception  and  acceptance of a SABM or DISC  command  frame.   A received command is not actually processed until the UA  response frame  is  sent.  Information fields are not permitted  in  a  UA frame.2.3.4.3.5  Disconnected Mode (DM) Response        The  disconnected  mode response is sent whenever  a  DXE receives  a  frame  other  than a SABM or UI  frame  while  in  a disconnected  mode.   It  is  also sent to  request  a  set  mode command,  or  to indicate it cannot accept a  connection  at  the moment.  The DM response does not have an information field.        Whenever a SABM frame is a received, and it is determined that  a  connection is not possible, a DM frame  shall  be  sent.  This  will  indicate  that the called  station  cannot  accept  a connection at that time.        While a DXE is in the disconnected mode, it will  respond to  any command other than a SABM or UI frame with a DM  response with the P/F bit set to 1.2.3.4.3.6  Unnumbered Information (UI) Frame        The   Unnumbered  Information  frame  contains  PID   and information fields and is used to pass information along the link outside the normal information controls.  This allows information fields  to go back and forth on the link bypassing flow  control.  Since   these  frames  are  not  acknowledgeable,  if  one   gets obliterated, there is no way to recover it.  A received UI  frame with  the  P bit set shall cause a response  to  be  transmitted.  This response shall be a DM frame when in the disconnected  state or  a  RR  (or  RNR, if appropriate)  frame  in  the  information transfer state.2.3.5  Link Error Reporting and Recovery        There are several link-layer errors that are  recoverable without  terminating the connection.  These error situations  may occur  as  a  result  of  malfunctions  within  the  DXE,  or  if transmission errors occur.2.3.5.1  DXE Busy Condition        When  a  DXE  becomes temporarily  unable  to  receive  I frames,  such  as when receive buffers are full, it will  send  a Receive Not Ready (RNR) frame.  This informs the other DXE   that this  DXE  cannot handle any more I frames at the  moment.   This condition is usually cleared by the sending of a UA, RR, REJ,  or SABM command frame.2.3.5.2  Send Sequence Number Error        If the send sequence number, N(S), of an otherwise error-free  received frame does not match the receive  state  variable, V(R),  a  send sequence error has occurred, and  the  information field will be discarded.  The receiver will not acknowledge  this frame, or any other I frames, until N(S) matches V(R).          The  control  field of the erroneous I frame(s)  will  be accepted so that link supervisory functions such as checking  the P/F  bit can still be performed.  Because of this  updating,  the retransmitted I frame may have an updated P bit and N(R).2.3.5.3  Reject (REJ) Recovery        REJ  is  used  to request a retransmission  of  I  frames following  the  detection  of a N(S) sequence  error.   Only  one outstanding  "sent  REJ" condition is allowed at  a  time.   This condition  is  cleared  when  the  requested  I  frame  has  been received.        A DXE receiving the REJ command will clear the  condition by  resending all outstanding I frames (up to the  window  size), starting with the one indicated in N(R) of the REJ frame.2.3.5.4  Time-out Error Recovery2.3.5.4.1  T1 Timer Recovery        If  a DXE, due to a transmission error, does not  receive (or  receives and discards) a single I frame or the last I  frame in  a sequence of I frames, it will not detect  a  send-sequence-number  error,  and therefore will not transmit a REJ.   The  DXE which transmitted the unacknowledged I frame(s) shall,  following the  completion of time-out period T1, take appropriate  recovery action  to determine when I frame retransmission should begin  as described  in 2.4.4.9, below.  This condition is cleared  by  the reception of an acknowledgement for the sent frame(s), or by  the link being reset.  See 2.4.6.2.3.5.4.2  Timer T3 Recovery        Timer  T3 is used to assure the link is still  functional during  periods of low information

transfer.  Whenever T1 is  not running  (no  outstanding I frames), T3 is used  to  periodically poll  the  other DXE of a link.  When T3 times out, a RR  or  RNR frame  is transmitted as a command and with the P bit  set.   The waiting  acknowledgement  procedure  (2.4.4.9,  below)  is   then executed.2.3.5.5  Invalid Frame or FCS Error        If  an invalid frame is received, or a frame is  received with  an FCS error, that frame will be discarded with  no  action taken.2.3.5.6  Frame Rejection Condition        A  frame  rejection condition occurs  when  an  otherwise error-free  frame  has been received with one of  the  conditions listed in 2.3.4.3.3 above.        Once  a  rejection  error occurs, no more  I  frames  are accepted  (except for the examination of the P/F bit)  until  the error is resolved.  The error condition is reported to the  other DXE by sending a FRMR response frame.  See 2.4.5.2.4  Description of AX.25 Procedures        The  following  describes the procedures used  to  setup, use, and disconnect a balanced link between two DXE stations.2.4.1  Address Field Operation2.4.1.1  Address Information        All   transmitted  frames  shall  have   address   fields conforming  to  2.2.13, above.  All frames shall  have  both  the destination device and the source device addresses in the address field,  with the destination address coming first.   This  allows many links to share the same RF channel.  The destination address is  always  the address of the station(s) to receive  the  frame, while the source address contains the address of the device  that sent the frame.        The destination address can be a group name or club  call sign if the point-to-multipoint operation is allowed.   Operation with  destination addresses other than actual amateur call  signs is a subject for further study.2.4.1.2  Command/Response Procedure        AX.25  Version 2.0 has implemented  the  command/response information   in  the  address  field.   In  order  to   maintain compatibility    with   previous   versions   of    AX.25,    the command/response information is conveyed using two bits.        An  upward-compatible AX.25 DXE can determine whether  it is  communicating  with  a DXE using an  older  version  of  this protocol by testing the command/response bit information  located in  bit 7 of the SSID octets of both the destination  and  source address subfields.  If both C bits are set to zero, the device is using  the  older protocol.  The newer version  of  the  protocol always has one of these two bits set to one and the other set  to zero, depending on whether the frame is a command or a response.        The  command/response  information is  encoded  into  the address field as shown in Fig. 10.   Frame Type          Dest. SSID C-Bit    Source SSID C-Bit  Previous versions             0                    0  Command (V.2.0)               1                    0  Response (V.2.0)              0                    1  Previous versions             1                    1              Fig. 10 -- Command/Response encoding        Since  all  frames  are  considered  either  commands  or responses, a device shall always have one of the bits set to one, and the other bit set to zero.        The  use  of the command/response  information  in  AX.25 allows  S frames to be either commands or responses.   This  aids maintenance   of  proper  control  over  the  link   during   the information transfer state.2.4.2  P/F Bit Procedures        The next response frame returned by the DXE to a SABM  or DISC command with the P bit set to 1 will be a UA or DM  response with the F bit set to 1.          The next response frame returned to an I frame with the P bit  set  to 1, received during the information  transfer  state, will be a RR, RNR, or REJ response with the F bit set to 1.        The next response frame returned to a supervisory command frame  with the P bit set to 1, received during  the  information transfer state, will be a RR, RNR, or REJ response frame with the F bit set to 1.        The  next  response frame returned to a S  or  I  command frame  with  the  P bit set to 1, received  in  the  disconnected state, will be a DM response frame with the F bit set to 1.        The  P  bit  is used in  conjunction  with  the  time-out recovery condition discussed in 2.3.5.4, above.        When not used, the P/F bit is set to zero.2.4.3  Procedures For Link Set-Up and Disconnection2.4.3.1  LAPB Link Connection Establishment        When  one DXE wishes to connect to another DXE,  it  will send  a SABM command frame to that device and start  timer  (T1).  If  the other DXE is there and able to connect, it  will  respond with  a UA response frame, and reset both of its  internal  state variables  (V(S)  and V(R)).  The reception of  the  UA  response frame  at  the  other  end will  cause  the  DXE  requesting  the connection  to  cancel the T1 timer and set  its  internal  state variables to 0.        If the other DXE doesn't respond before T1 times out, the device requesting the connection will re-send the SABM frame, and start  T1  running  again.  The DXE should  continue  to  try  to establish  a  connection  until it has  tried  unsuccessfully  N2 times.  N2 is defined in 2.4.7.2, below.        If,  upon  reception of a SABM command, the  DXE  decides that  it  cannot enter the indicated state, it should send  a  DM frame.        When  receiving a DM response, the DXE sending  the  SABM should  cancel  its  T1 timer, and  not  enter  the  information-transfer state.          The  DXE sending a SABM command will ignore  and  discard any  frames except SABM, DISC, UA, and DM frames from  the  other DXE.        Frames  other  than UA and DM in response to  a  received SABM  will  be  sent  only after the link is set  up  and  if  no outstanding SABM exists.2.4.3.2  Information-Transfer Phase        After establishing a link connection, the DXE will  enter the  information-transfer  state.  In this state,  the  DXE  will accept  and  transmit I and S frames according to  the  procedure outlined in 2.4.4, below.        When  receiving a SABM command while in the  information-transfer  state,  the  DXE will follow  the  resetting  procedure outlined in 2.4.6 below.2.4.3.3  Link Disconnection2.4.3.3.1       While  in  the information-transfer state,  either  DXE  may indicate a request to disconnect the link by transmitting a  DISC command frame and starting timer T1 (see 2.4.7).2.4.3.3.2       A DXE, upon receiving a valid DISC command, shall send a  UA response  frame  and enter the disconnected state.  A  DXE,  upon receiving  a  UA  or DM response to a sent  DISC  command,  shall cancel timer T1, and enter the disconnected state.2.4.3.3.3       If  a UA or DM response is not correctly received before  T1 times out, the DISC frame should be sent again and T1  restarted.  If  this happens N2 times, the DXE should enter the  disconnected state.2.4.3.4  Disconnected State2.4.3.4.1       A  DXE  in  the disconnected state  shall  monitor  received commands  and react upon the reception of a SABM as described  in 2.4.3.1 above and will transmit a DM frame in response to a  DISC command.2.4.3.4.2       In the disconnected state, a DXE may initiate a link  set-up as outlined in connection establishment above (2.4.3.1).  It  may also  respond  to  the  reception  of  a  SABM  and  establish  a connection, or it may ignore the SABM and send a DM instead.2.4.3.4.3       Any  DXE receiving a command frame other than a SABM  or  UI frame  with the P bit set to one should respond with a  DM  frame with  the  F  bit  set to one.  The  offending  frame  should  be ignored.2.4.3.4.4       When  the DXE enters the disconnected state after  an  error condition  or if an internal error has resulted in the DXE  being in  the  disconnected  state, the DXE  should  indicate  this  by sending  a  DM response rather than a DISC frame and  follow  the link  disconnection procedure outlined in 2.4.3.3.3, above.   The DXE  may then try to re-establish the link using the link  set-up procedure outlined in 2.4.3.1, above.2.4.3.5  Collision Recovery2.4.3.5.1  Collisions in a Half-Duplex Environment        Collisions  of  frames in a half-duplex  environment  are taken  care  of  by  the  retry  nature  of  the  T1  timer   and retransmission count variable.  No other special action needs  to be taken.2.4.3.5.2  Collisions of Unnumbered Commands        If sent and received SABM or DISC command frames are  the same,  both  DXEs  should  send a UA  response  at  the  earliest opportunity, and both devices should enter the indicated state.        If sent and received SABM or DISC commands are different, both  DXEs should enter the disconnected state and transmit a  DM frame at the earliest opportunity.2.4.3.5.3  Collision of a DM with a SABM or DISC        When  an  unsolicited  DM  response  frame  is  sent,   a collision  between it and a SABM or DISC may occur.  In order  to prevent  this  DM from being misinterpreted, all  unsolicited  DM frames  should  be transmitted with the F bit set to  zero.   All SABM  and DISC frames should be sent with the P bit set  to  one.  This will prevent any confusion when a DM frame is received.2.4.3.6  Connectionless Operation        In  Amateur  Radio,  there  is  an  additional  type   of operation  that is not feasible using level 2 connections.   This operation  is  the  round table, where several  amateurs  may  be engaged  in one conversation.  This type of operation  cannot  be accommodated by AX.25 link-layer connections.        The   way   round-table  activity

is   implemented   is technically  outside  the AX.25 connection, but still  using  the AX.25 frame structure.        AX.25 uses a special frame for this operation, called the Unnumbered  Information (UI) frame.  When this type of  operation is  used,  the  destination  address  should  have  a  code  word installed  in  it to prevent the users of that  particular  round table from seeing all frames going through the shared RF  medium.  An example of this is if a group of amateurs are in a round-table discussion  about  packet  radio, they could put  PACKET  in  the destination  address,  so  they would receive  frames  only  from others in the same discussion.  An added advantage of the use  of AX.25  in this manner is that the source of each frame is in  the source  address  subfield,  so  software  could  be  written   to automatically display who is making what comments.        Since  this  mode  is connectionless, there  will  be  no requests for retransmissions of bad frames.  Collisions will also occur, with the potential of losing the frames that collided.2.4.4  Procedures for Information Transfer        Once  a  connection  has been  established,  as  outlined above, both devices are able to accept I, S, and U frames.2.4.4.1  Sending I Frames        Whenever  a DXE has an I frame to transmit, it will  send the  I frame with N(S) of the control field equal to its  current send  state  variable V(S).  Once the I frame is sent,  the  send state  variable  is  incremented  by one.  If  timer  T1  is  not running, it should be started.  If timer T1 is running, it should be restarted.        The DXE should not transmit any more I frames if its send state variable equals the last received N(R) from the other  side of  the link plus seven.  If it were to send more I  frames,  the flow control window would be exceed, and errors could result.        If  a  DXE is in a busy condition, it may  still  send  I frames as long as the other device is not also busy.        If  a  DXE is in the frame-rejection mode, it  will  stop sending I frames.2.4.4.2  Receiving I Frames2.4.4.2.1       If  a DXE receives a valid I frame (one with a  correct  FCS and  whose  send sequence number equals  the  receiver's  receive state variable) and is not in the busy condition, it will  accept the  received I frame, increment its receive state variable,  and act in one of the following manners:         1.  If it has an I frame to send, that I frame may be sent with the     transmitted N(R) equal to its receive state variable V(R) (thus     acknowledging the received frame).  Alternately, the device may     send a RR frame with N(R) equal to V(R), and then send the I      frame. 2.  If there are no outstanding I frames, the receiving device will     send a RR frame with N(R) equal to V(R).  The receiving DXE may     wait a small period of time before sending the RR frame to be sure     additional I frames are not being transmitted.2.4.4.2.2           If  the  DXE  is  in a busy condition,  it  may  ignore  any received  I  frames without reporting this condition  other  than repeating the indication of the busy condition.        If  a busy condition exists, the DXE receiving  the  busy condition   indication  should  poll  the  sender  of  the   busy indication periodically until the busy condition disappears.          A  DXE may poll the busy DXE periodically with RR or  RNR frames with the P bit set to one.        The  reception  of  I  frames  that  contain  zero-length information  fields  shall be reported to the next level  but  no information field will be transferred.2.4.4.3  Reception of Out of Sequence Frames        When  an I frame is received with a correct FCS, but  its send sequence number, N(S), does not match the current receiver's receive  state  variable, the frame should be discarded.   A  REJ frame  shall be sent with a receive sequence number equal to  one higher (modulo 8) than the last correctly received I frame if  an uncleared  N(S) sequence error condition has not been  previously established.   The  received state variable and poll bit  of  the discarded  frame should be checked and acted upon, if  necessary, before discarding the frame.  2.4.4.4  Reception of Incorrect Frames        When  a  DXE receives a frame with an incorrect  FCS,  an invalid  frame, or a frame with an improper address,  that  frame shall be discarded.2.4.4.5  Receiving Acknowledgement        Whenever an I or S frame is correctly received, even in a busy condition, the N(R) of the received frame should be  checked to  see if it includes an acknowledgement of outstanding  sent  I frames.   The T1 timer should be cancelled if the received  frame actually  acknowledges previously unacknowledged frames.  If  the T1  timer is cancelled and there are still some frames that  have been sent that are not acknowledged, T1 should be started  again.  If  the T1 timer runs out before an acknowledgement is  received, the  device  should proceed to the  retransmission  procedure  in 2.4.4.9.2.4.4.6  Receiving Reject        Upon receiving a REJ frame, the transmitting DXE will set its  send  state variable to the same value as  the  REJ  frame's received sequence number in the control field.  The DXE will then retransmit  any  I  frame(s) outstanding at  the  next  available opportunity conforming to the following: 1.  If the DXE is not transmitting at the time, and the  channel      is  open,  the  device  may commence  to  retransmit  the  I      frame(s) immediately. 2.  If   the   DXE  is  operating  on  a   full-duplex   channel      transmitting  a UI or S frame when it receives a REJ  frame,      it may finish sending the UI or S frame and then  retransmit      the I frame(s). 3.  If   the   DXE  is  operating  in  a   full-duplex   channel      transmitting  another I frame when it receives a REJ  frame,      it  may  abort  the  I  frame  it  was  sending  and   start      retransmission of the requested I frames immediately. 4.  The DXE may send just the one I frame outstanding, or it may      send  more than the one indicated if more I frames  followed      the  first  one not acknowledged, provided the total  to  be      sent does not exceed the flow-control window (7 frames).        If the DXE receives a REJ frame with the poll bit set, it should  respond with either a RR or RNR frame with the final  bit set before retransmitting the outstanding I frame(s).2.4.4.7  Receiving a RNR Frame        Whenever  a  DXE  receives a RNR  frame,  it  shall  stop transmission  of  I  frames until the  busy  condition  has  been cleared.   If timer T1 runs out after the RNR was  received,  the waiting  acknowledgement  procedure  listed  in  2.4.4.9,  below, should  be  performed.  The poll bit may be used  in  conjunction with  S  frames  to test for a change in  the  condition  of  the busied-out DXE.2.4.4.8  Sending a Busy Indication        Whenever a DXE enters a busy condition, it will  indicate this  by sending a RNR response at the next  opportunity.   While the  DXE is in the busy condition, it may receive and  process  S frames,  and if a received S frame has the P bit set to one,  the DXE should send a RNR frame with the F bit set to one at the next possible  opportunity.   To  clear the busy  condition,  the  DXE should  send either a RR or REJ frame with the received  sequence number equal to the current receive state variable, depending  on whether the last received I frame was properly received or not.2.4.4.9  Waiting Acknowledgement        If timer T1 runs out waiting the acknowledgement from the other DXE for an I frame transmitted, the DXE will restart  timer T1  and transmit an appropriate supervisory command frame (RR  or RNR)  with  the  P  bit set.  If the  DXE  receives  correctly  a supervisory  response frame with the F bit set and with  an  N(R) within  the  range from the last N(R) received to the  last  N(S) sent  plus  one, the DXE will restart timer T1 and will  set  its send  state  variable  V(S) to the received N(R).   It  may  then resume   with   I  frame  transmission  or   retransmission,   as appropriate.  If, on the other hand, the DXE receives correctly a supervisory response frame with the F bit not set, or an I  frame or  supervisory command frame, and with an N(R) within the  range from  the last N(R) received to the last N(S) sent plus one,  the DXE will not restart timer T1, but will use the received N(R)  as an  indication of acknowledgement of transmitted I frames  up  to and including I frame numbered N(R)-1.        If timer T1 runs out before a supervisory response  frame with  the  F  bit set is received, the  DXE  will  retransmit  an appropriate supervisory command frame (RR or RNR) with the P  bit set.  After N2 attempts to get a supervisory response frame  with the  F bit set from the other DXE, the DXE will initiate  a  link resetting procedure as described in 2.4.6, below.2.4.5  Frame Rejection Conditions        A  DXE  shall initiate the frame-reset procedure  when  a frame  is received with the correct FCS and address field  during the information-transfer state with one or more of the conditions in 2.3.4.3.3, above.        Under these conditions, the DXE will ask the other DXE to reset  the  link by transmitting a FRMR response as  outlined  in 2.4.6.3, below.        After sending the FRMR frame, the sending DXE will  enter the  frame reject condition.  This condition is cleared when  the DXE that sent the FRMR frame receives a SABM or DISC command,  or a DM response frame.  Any other command received while the DXE is in the frame reject state will cause another FRMR to be sent  out with the

same information field as originally sent.        In  the  frame rejection condition, additional  I  frames will not be transmitted, and received I frames and S frames  will be discarded by the DXE.        The DXE that sent the FRMR frame shall start the T1 timer when  the  FRMR is sent.  If no SABM or DISC  frame  is  received before the timer runs out, the FRMR frame shall be retransmitted, and   the  T1  timer  restarted  as  described  in  the   waiting acknowledgement section (2.4.4.9) above.  If the FRMR is sent  N2 times without success, the link shall be reset.2.4.6  Resetting Procedure2.4.6.1       The   resetting  procedure  is  used  to   initialize   both directions  of  data  flow  after  a  nonrecoverable  error   has occurred.   This resetting procedure is used in the  information-transfer state of an AX.25 link only.2.4.6.2       A DXE shall initiate a reset procedure whenever it  receives an unexpected UA response frame or an unsolicited response  frame with  the  F bit set to one.  A DXE may also initiate  the  reset procedure  upon receipt of a FRMR frame.  Alternatively, the  DXE may  respond to a FRMR by terminating the connection with a  DISC frame.2.4.6.3       A  DXE  shall  reset the link by sending a  SABM  frame  and starting  timer  T1.  Upon receiving a SABM frame  from  the  DXE previously connected to, the receiver of a SABM frame should send a  UA  frame back at the earliest opportunity, set its  send  and receive  state  variables,  V(S) and V(R), to zero  and  stop  T1 unless it has sent a SABM or DISC itself.  If the UA is correctly received by the initial DXE, it resets its send and receive state variables, V(S) and V(R), and stops timer T1.  Any busy condition that previously existed will also be cleared.        If  a  DM response is received, the DXE  will  enter  the disconnected  state  and  stop timer T1.  If timer  T1  runs  out before  a UA or DM response frame is received, the SABM  will  be retransmitted  and timer T1 restarted.  If timer T1 runs  out  N2 times,  the  DXE  will  enter the  disconnected  state,  and  any previously existing link conditions will be cleared.     Other  commands  or  responses received by  the  DXE  before completion of the reset procedure will be discarded.2.4.6.4       One  DXE  may request that the other DXE reset the  link  by sending  a  DM response frame.  After the DM frame is  sent,  the sending DXE will then enter the disconnected state.2.4.7  List of System Defined Parameters2.4.7.1  Timers        To   maintain  the  integrity  of  the  AX.25   level   2 connection, use of these timers is recommended.2.4.7.1.1  Acknowledgement Timer T1        The  first timer, T1, is used to make sure a DXE  doesn't wait  forever  for a response to a frame it  sends.   This  timer cannot be expressed in absolute time, since the time required  to send frames varies greatly with the signaling rate used at  level 1.   T1  should take at least twice the amount of time  it  would take  to send maximum length frame to the other DXE, and get  the proper response frame back from the other DXE.  This would  allow time for the other DXE to do some processing before responding.        If  level  2 repeaters are to be used, the  value  of  T1 should be adjusted according to the number of repeaters the frame is being transferred through.2.4.7.1.2  Response Delay Timer T2        The  second timer, T2, may be implemented by the  DXE  to specify  a maximum amount of delay to be introduced  between  the time an I frame is received, and the time the resulting  response frame is sent.  This delay may be introduced to allow a receiving DXE to wait a short period of time to determine if there is  more than  one frame being sent to it.  If more frames  are  received, the  DXE can acknowledge them at once (up to seven), rather  than acknowledge  each individual frame.  The use of timer T2  is  not mandatory,  but  is recommended to  improve  channel  efficiency.  Note  that, on full-duplex channels, acknowledgements should  not be delayed beyond k/2 frames to achieve maximum throughput.   The k parameter is defined in 2.4.7.4, below.2.4.7.1.3  Inactive Link Timer T3        The third timer, T3, is used whenever T1 isn't running to maintain  link integrity.  It is recommended that whenever  there are  no  outstanding  unacknowledged I  frames  or  P-bit  frames (during  the information-transfer state), a RR or RNR frame  with the  P  bit set to one be sent every T3 time units to  query  the status  of the other DXE.  The period of T3 is  locally  defined, and  depends greatly on level 1 operation.  T3 should be  greater than T1, and may be very large on channels of high integrity.2.4.7.2  Maximum Number of Retries (N2)        The maximum number of retries is used in conjunction with the T1 timer.2.4.7.3  Maximum Number of Octets in an I Field (N1)        The maximum number of octets allowed in the I field  will be 256.  There shall also be an integral number of octets.2.4.7.4  Maximum Number of I Frames Outstanding (k)        The  maximum number of outstanding I frames at a time  is seven.                              END.