-------------------------------------------------------------------------

SCSI Preferences for the Amiga 3000

-------------------------------------------------------------------------

by Martin A. Blatter (blatter@ifi.unizh.ch)

This small program allows you to change the bits in the Amiga 3000's

battery backed up memory that control some parameters of the embedded

SCSI host adaptor.

IT REQUIRES AMIGADOS 2.0 OR HIGHER AND AN A3000! It won't run on an

A2000!

SCSI Prefs is Freely Distributable Software. The entire risk as to the

quality and performance of this programs is with you.

Put SCSI Prefs in your prefs directory. It offers the same look and

feel as the standard 2.0 Preferences editors, so you'll imediately

find yourself familiar with its user interface.

--------------------------------------- Long SCSI Timeout (Seacrate Mode)

This checkbox determines how long the host adaptor waits for connected

drives. It's especially useful for owners of drives that take a long

time to spin up. Drives manufactured by Seagate are candidates for this

setting. With this option enabled the controller will wait 2 seconds

instead of 128 ms.

(Some drives by Imprimis and Maxtor also need long timeouts)

---------------------------------------------------- Synchronous Transfer

When this option is enabled, the host adaptor will initiate synchronous

transfers (hence the name of this option :-) with hard drivers capable

of doing them.

(Hard drives made by Imprimis, Conner Peripherals and Quantum Fujitsu

usually support synchronous transfers)

--------------------------------------------------- Support Multiple LUNs

Each SCSI ID can support several Logical Unit Numbers. I can't think of

a use for more than one LUN right now, but that situation can change...

Leave this option disabled for now, beacause it apparently can cause

troubles with hard drives that respond on more than one LUN address.

---------------------------------------------------- Host Adaptor SCSI ID

The Amiga (resp. its SCSI controller) is usually set to SCSI address 7.

If you want to use a SCSI device with your Amiga that can't be jumpered to

another address than 7 , you'll have to type in another number here. Be

sure not to use a SCSI ID that is already assigned to device attached

to your Amiga. (The internal Quantum hard drive is set to SCSI ID 6).

The value of SCSI ID's can range from 0 to 7.

Take care! If you experiment with SCSI ID's, always keep a copy of this

program on a disk. Your hard drive won't boot if both, the Amiga and your

hard drive have the same SCSI ID...

-------------------------------------------------------------------- Note

All changes made with SCSI Preferences won't have an effect until you

reset your Amiga.

-------------------------------------------------------------------------

Please send suggestions, comments and bug reports to:

Martin A. Blatter, Relog AG

Internet: blatter@amiga.physik.unizh.ch, blatter@ifi.unizh.ch or

cbmehq!cbmswi!zethos!blatter@cbmvax.commodore.com

Snail Mail: Pfaffaecherstr. 59, CH-8913 Ottenbach/ZH, Switzerland