A68k - a freely	distributable assembler	for the	Amiga

by Charlie Gibbs

with special thanks to

Brian R. Anderson and Jeff Lydiatt

(Version 2.42 -	January	10, 1989)

Note:  This program is Freely-Distributable, as opposed to	Public

Domain.	 Permission is given to	freely distribute this program provided	no

fee is charged,	and this documentation file is included	with the program.

This assembler is based on	Brian R. Anderson's 68000 cross-

assembler published in Dr. Dobb's Journal, April through June 1986.

I have converted it to produce AmigaDOS-format object modules, and

have made many enhancements, such as macros and	INCLUDE	files.

My	first step was to convert the original Modula-2	code into C.

I did this for two reasons.  First, I had access to a C	compiler, but

not a Modula-2 compiler.  Second, I like C better anyway.

The executable code generator code	(GetObjectCode and MergeModes)

is essentially the same	as in the original article, aside from its

translation into C.  I have almost completely rewritten	the remainder

of the code, however, in order to remove restrictions, add enhancements,

and adapt it to	the AmigaDOS environment.  Since the only reference book

available to me	was the	AmigaDOS Developer's Manual (Bantam, February

1986), the assembler and the remainder of this document	work in	terms

of that	book.

RESTRICTIONS

Let's get these out of the way first.  There are a few things that I

have not yet implemented, and some outright bugs that would take too long

to correct for this version.

o	The verification file (-v) option is not supported.  Diagnostic

messages always	appear on the console.	They also appear in the

listing	file, however (see extensions below).  You can produce

an error file by redirecting console output to a file -	the

line number counter and	final summary are displayed on stderr

so you can still see what's happening.

o	The file names in the INCLUDE directory	list (-i) must be separated

by commas.  The	list may not be	enclosed in quotes.

o	Labels assigned	by EQUR	and REG	directives are case-sensitive.

o	The following directives are not supported, and	will be	flagged	as

invalid	op-codes:

OFFSET

NOPAGE

LLEN

PLEN

NOOBJ

FAIL

FORMAT

NOFORMAT

MASK2

I feel that NOPAGE, LLEN, and PLEN should not be defined within	a

source module.	It doesn't make sense to me to have to change your

program	just because you want to print your listings on	different

paper.	The command-line option	"-p" (see below) can be used as a

replacement for	PLEN; setting it to a high value (like 32767) is a

good substitute	for NOPAGE.  The effect	of LLEN	can be obtained

by running the listing file through an appropriate filter.

EXTENSIONS

Now for the good stuff:

o	Labels can be any length that will fit onto one	source line

(currently 127 bytes maximum).	Since labels are stored	on the

heap, the number of labels that	can be processed is limited only

by available memory.

o	Since section data and user macro definitions are stored in the

symbol table (see above), they too are limited only by available

memory.	 (Actually, there is a hard-coded limit	of 32767 sections,

but I doubt anyone will	run into that one.)

o	The only values	a label	cannot take are	the register names - the

assembler can distinguish between the same name	used as	a label,

instruction name or directive, macro name, or section name.

o	Section	and user macro names appear in the symbol table	dump, and

will also be cross-referenced.	Their names can	be the same as any

label (see above); the assembler can sort them out.

o	INCLUDEs and macro calls can be	nested indefinitely, limited only

by available memory.  The message "Secondary heap overflow -

assembly terminated" will be displayed if memory is exhausted.

You can	increase the size of this heap using the -w parameter

(see below).  Recursive	macros are supported; recursive	INCLUDEs

will, of course, result	in a loop that will be broken only when

the heap overflows.

o	The EVEN directive forces alignment on a word (2-byte) boundary.

It does	the same thing as CNOP 0,2.

(This one is left over from the	original code.)

o	Branch (Bcc) instructions to a previously-defined label	will be

automatically converted	to short form if possible.  This feature is

not available for forward branches, since in pass 1 the	assembler

doesn't yet know how far the branch must go.  You can, however,

ask A68k to tell you which instructions	can be coded as	short

branches by using the -f command-line switch (see below).

o	Backward references to labels within the current CODE section

will be	converted to PC	relative addressing with displacement

if this	mode is	legal for the instruction.

o	If a MOVEM instruction only specifies one register, it is converted

to the corresponding MOVE instruction.	Instructions such as

MOVEM D0-D0,label will not be converted, however.

o	ADD, SUB, and MOVE instructions	will be	converted to ADDQ, SUBQ,

and MOVEQ respectively if possible.  Instructions coded	explicitly

as (for	example) ADDA or ADDI will not be converted.

o	ADD, CMP, SUB, and MOVE	to an address register are converted to

ADDA, CMPA, SUBA, and MOVEA respectively, unless (for ADD, SUB,

or MOVE) they have already been	converted to quick form.

o	ADD, AND, CMP, EOR, OR,	and SUB	of an immediate	value are converted

to ADDI, ANDI, CMPI, EORI, ORI,	and SUBI respectively (unless the

address	register or quick conversion above has already been done).

o	If both	operands of a CMP instruction are postincrement	mode, the

instruction is converted to CMPM.

o	Operands of the	form 0(An) will	be treated as (An) except for

the MOVEP instruction, which always requires a displacement.

o	The SECTION directive allows a third parameter.	 This can be

specified as either CHIP or FAST (upper- or lower-case).  If this

parameter is present, the hunk will be written with the	MEMF_CHIP

or MEMF_FAST bit set.  This allows you to produce "pre-ATOMized"

object modules.

o	The synonyms DATA and BSS are accepted for SECTION directives

starting data or BSS hunks.  The CHIP and FAST options mentioned

above can also be used,	e.g. BSS name,CHIP.

o	The following synonyms have been implemented for compatibility

with the Aztec assembler:

CSEG is	treated	the same as CODE or SECTION name,CODE

DSEG is	treated	the same as DATA or SECTION name,DATA

PUBLIC is treated as either XDEF or XREF, depending on

whether or not the symbol in question has been

defined in the current source module.

A single PUBLIC directive can name a mixture

internally-	and externally-defined symbols.

o	The ability to produce Motorola	S-records is retained from the

original code.	The -s option causes the assembler to produce

S-format instead of AmigaDOS format.  Relocatable code cannot be

produced in this format.

o	Error messages consist of three	parts.

The	position of the	offending line is given	as a line number

within the current module.  If the line	is within a macro expan-

sion or	INCLUDE	file, the position of the macro	call or	INCLUDE

statement in the outer module is given as well.	 This process

is repeated until the outermost	source module is reached.

Next, the offending	source line itself is listed.

Finally, the errors	for that line are displayed.  A	flag

(^) is placed under the	column where the error was detected.

o	Named local labels are supported.  These work the same as the

local labels supported by the Metacomco	assembler (nnn$) but

can be formed in the same manner as normal labels, except that

the first character must be a backslash	(\).

o	The following synonyms have been implemented for compatibility

with the Assempro assembler:

ENDIF is treated the same as ENDC

= is treated the same as EQU

| is treated the same as ! (logical OR)

o	Quotation marks	(") can be used as string delimiters

as well	as apostrophes (').  Any given string must begin

and end	with the same delimiter.  This allows such statements

as the following:

MOVEQ	'"',D0

DC.B	"This is Charlie's assembler."

Note that you can still	define an apostrophe within a string

delimited by apostrophes if you	double it, e.g.

MOVEQ	'''',D0

DC.B	'This is Charlie''s assembler.'

o	If any errors are found	in the assembly, the object code file

will be	scratched, unless you specified	the -k (keep) flag

on the command line.

o	The symbols .A68K, .a68k, .a68K, and .A68k are automatically

defined	as SET symbols having absolute values of 1.

This enables a source program to determine whether it is

being assembled	by this	assembler, and is effectively

insensitive as to whether or not it is checked in upper	case.

o	A zeroth positional macro parameter (\0) is supported.	It

is replaced by the length of the macro call (B,	W, or L,

defaulting to W).  For instance, given the macro:

moov	MACRO

move.\0	\1,\2

ENDM

the macro call

moov.l	d0,d1

would be expanded as

move.l	d0,d1

o	If an INCLUDE file doesn't generate any code and no listing

file is	required (including suppression	of the listing using

NOLIST), it won't be read again in pass 2.  The statement

numbers	will be	bumped to keep in proper alignment.  This

can really speed up assemblies that INCLUDE lots of EQUates.

o	The ORG	directive is supported.	 It works like RORG, except

that it	takes the actual address to be jumped to, rather

than an	offset from the	start of the current section.

The given address must be in the current section.

As far as A68k is concerned, the only real difference

between	ORG and	RORG is	that the ORG value must	be

relocatable, while the RORG value must be absolute.

THE SMALL CODE / SMALL DATA MODEL

Version 2.4 implements a rudimentary small	code/data model.

It consists of converting any data reference to	one of the following

three addressing modes:

address	register indirect with displacement (using A4)

(for references	to the DATA or BSS section)

program	counter	indirect with displacement

(for references	to the CODE section)

absolute word

(for absolute and 16-bit relocatable values)

These conversions do not take place unless a NEAR directive is

encountered.  Any operands on the NEAR directive are ignored.

Conversion is done for all operands until a FAR	directive is

encountered.  NEAR and FAR directives can occur	any number of

times, enabling	conversion to be turned	on and off at will.

Backward references which cannot be converted (e.g. external

labels declared	as XREF) will remain as	absolute long addressing.

All forward references are assumed to be convertible, since during

pass 1 A68k has	no way of telling whether conversion is	possible.

If conversion turns out	to be impossible, invalid object code will

be generated - an error	message	("Invalid forward reference") will

indicate when this occurs.

Although the small	code/data model	can greatly reduce the

size of	assembled programs, several restrictions apply:

o	Small code and small data models are active simultaneously.

You can't have one without the other, since during pass 1

A68k doesn't know whether forward references are to CODE

or to DATA/BSS.

o	Programs can consist of	a maximum of two sections,

one CODE, the other DATA or BSS.  If you try to	define

a third	section, the message "Too many SECTIONs" will

be displayed.  The NEAR	directive is active only within

the CODE section.

o	While the NEAR directive is active, external labels (XREF)

must be	declared before	they are used, CODE section references

must be	with 32K of the	current	position (i.e. expressible as

PC-relative), and DATA/BSS section references must be in the

first 64K of the DATA/BSS section (i.e.	expressible as

address	register indirect with displacement).  Any instructions

which do not satisfy these requirements	cannot be detected in

pass 1,	so A68k	has no choice but to display an	error message

in pass	2 ("Invalid forward reference") which in this case

indicates that invalid code has	been generated.	 To properly

assemble such instructions, you	can temporarily	disable

conversion with	a FAR directive, then resume afterwards

with another NEAR directive.

o	Conversion cannot be done for references between modules.

All external references	must be	left as	absolute long.

o	A68k assumes that register A4 points to	the start of the

DATA/BSS section plus 32768 bytes.  A4 must be preloaded

with this value	before executing any code converted by the

NEAR directive.	 One way to do this is to code the instruction

that loads the register	prior to the NEAR directive.  Another

way is to use a	MOVE.L with immediate mode, which is never

converted.  Here are examples of the two methods:

LEA	data+32768,a4		NEAR

NEAR				MOVE.L	#data+32768,a4

<remainder of code>		    <remainder of code>

BSS				BSS

data:				data:

<data areas>		    <data areas>

END				END

I'll be the first to admit that this is a very crude and ugly

implementation.	 I hope	to improve it in future	versions.

HOW TO USE A68k

The command-line syntax to	run the	assembler is as	follows:

a68k <source file name>

[<object file name>]

[<listing file name>]

[-d]

[-e<equate file	name>]

[-f]

[-h<header file	name>]

[-i<INCLUDE directory list>]

[-k]

[-l<listing file name>]

[-o<object file>]

[-p<page depth>]

[-q[<quiet interval>]]

[-s]

[-t]

[-w[<hash table	size>][,<secondary heap	size>]]

[-x<listing file name>]

[-y]

[-z[<debug start line>][,<debug	end line>]]

These options can be given in any order, and the source	file name can

appear before all switches, after them,	or anywhere in the middle.

Option values, if any, must immediately	follow the keyword with

no intervening spaces.

If	the -o keyword is omitted, the object file will	be given a default

name.  It is created by	replacing all characters after the last	period in

the source file	name by	"o".  For example, if the source file name is

"myprog.asm", the object file name defaults to "myprog.o".  A source name

of "my.new.prog.asm" produces a default object file name of "my.new.prog.o".

If the source file name	does not contain a period, ".o" is appended to it

to produce the default object file name.

The default value for the listing file name is arrived at in the same

way as the object file name, except that ".lst" is appended instead of ".o".

If you don't specify this parameter, no listing file will be produced.

If you specify -x (see below), -l (with	the default name) is assumed,

although you can still use this	parameter if you wish.

The default value for the equate file name	is arrived at in the same

way as the object file name, except that ".equ" is appended instead of ".o".

The INCLUDE directory list	is a list of directory names separated by

commas.	 No embedded blanks are	allowed.  For example, the specification

-imylib,df1:another.lib

will cause INCLUDE files to be searched	for first in the current directory,

then in	"mylib", then in "df1:another.lib".

The -d keyword causes symbol table	entries	(hunk_symbol) to be written

to the object module for the use of symbolic debuggers.

The -f keyword causes any forward branches	(Bcc, BRA, BSR)	that

could be converted to short form to be flagged.	 A68k can't convert them

automatically because it doesn't know in pass 1 how far the branch will

be.  This option tells you which instructions could be manually	converted.

The -k keyword causes the object file to be kept if any errors were

found.	Otherwise, it will be scratched	if any errors occurred.

The -l keyword causes a listing file to be	produced.  If you want

the listing file to include a symbol table dump	and cross-reference,

use the	-x keyword instead (see	below).

The -p keyword causes the page depth to be	set to the specified value.

If omitted, a default of 60 lines (-p60) is assumed.

The -q keyword changes the	interval at which A68k displays	the

current	line number (the default is every 10 lines, i.e. -q10).	 If

you specify -q0	or -q without a	value, no line numbers will be displayed.

This will speed	up assemblies slightly by reducing console I/O.	 If -q

is specified as	a negative number (e.g.	-q-10),	line numbers will still

be displayed at	the specified interval,	but will be given as positions

within the current module (source, macro, or INCLUDE) rather than

as a total statement count - the module	name will also be displayed.

The -s keyword, if	specified, causes the object file to be	written	in

Motorola S-record format.  If omitted, AmigaDOS	format will be produced.

The default name for an	S-record file has ".s" appended to the source name,

rather than ".o"; this can still be overridden with the -o keyword, though.

The -t keyword allows tabs	in the source file to be passed	through

to the listing file, rather than being expanded.  In addition, tabs will

be generated in	the listing file to skip from the object code to the

source statement, etc.	This can greatly reduce	the size of the	listing

file, as well as making	it quicker to produce.	Do not use this	option

if you will be displaying or listing the list file on a	device which

does not assume	a tab stop at every 8th	position.

The -w keyword specifies the size of the fixed memory areas that

are allocated.	The first parameter gives the number of	entries	that

the hash table will contain (defaulting	to 2047).  This	should be enough

for all	but the	very largest programs.	The assembly will not fail if

this value is too small, but may slow down as a	result of A68k having

to search many long hash chains.  I've heard that you should really

specify	a prime	number for this	parameter, but I haven't gone into

hashing	theory enough to know whether it's actually necessary.

The second	parameter of the -w keyword specifies the size of the

secondary heap (defaulting to 1024 bytes, which	should be enough

unless you use very deeply nested macros and/or	INCLUDE	files with long

path names).

You can specify either or both parameters.	 For example:

-w4093		secondary heap size remains at 1024 bytes

-w,2000		hash table size	remains	at 2047	entries

-w4093,2000	increases the size of both areas

If	you're really tight for memory, and are assembling small modules,

you can	use this keyword to shrink these areas below their default sizes.

At the end of an assembly, a message will be displayed giving the sizes

actually used, in the form of the -w command you would have to enter to

allocate that much space.  This	is primarily useful to see how much

secondary heap space was used.

NOTE: All other table storage (e.g. the actual symbol table) is

allocated as required (currently in 8K chunks).

The -x keyword works the same as -l, except that a	symbol table

dump, including	cross-reference	information, will be added to the end

of the listing file.

The -y keyword causes hashing statistics to be displayed.	First

the number of symbols in the table is given, followed by a breakdown

of hash	chains by length.  Chains with length zero denote unused hash

table entries.	Ideally	(i.e. if there were no collisions) there should

be as many chains with length 1	as there are symbols, and there	should

be no chains of	length 2 or greater.  I	added this option to help me

tune my	hashing	algorithm, but you can also use	it to see whether you

should allocate	a larger hash table (using the first parameter of the

-w option, see above).

The -z keyword is provided	for debugging purposes.	 You can cause

the assembler to list a	range of source	lines, complete	with line number

and current location counter value, during both	passes.	 For example:

-z		lists all source lines

-z100,200	lists lines 100	through	200

-z100		lists all lines	starting at 100

-z,100		lists the first	100 lines

If	you wish to override the default object	and (optionally) listing

file names, you	can omit the -o	and -l keywords.  The assembler	interprets

the first three	parameters without leading hyphens as the source, object,

and listing file names respectively.  Anything over three file names is	an

error, as is attempting	to respecify a file name with the -o or	-l keywords.

TECHNICAL INFORMATION

The actual	symbol table entries (pointed to by the	hash table,

colliding entries are linked together) are stored in 8K	chunks which

are allocated as required.  The	first entry of each chunk is reserved

as a link to the next chunk (or	NULL in	the last chunk)	- this makes

it easy	to find	all the	chunks to free them when we're finished.  All

symbol table entries are stored	in pass	1.  During pass	2, cross-reference

table entries are built	in the same group of chunks, immediately following

the last symbol	table entry.  Additional chunks	will continue to be

linked in if necessary.

Symbol names and macro text are stored in another series of linked

chunks.	 These chunks consist of a link	pointer	followed by strings

(terminated by nulls) laid end to end.	Symbols	are independent	entries,

linked from the	corresponding symbol table entry.  Macros are stored as

consecutive strings, one per line - the	end of the macro is indicated by

an ENDM	statement.  If a macro spans two chunks, the last line in the

original chunk is followed by a	newline	character to indicate that the

macro is continued in the next chunk.

Relocation	information is built during pass 2 in yet another series

of linked chunks.  If more than	one chunk is needed to hold one	section's

relocation information,	all additional chunks are released at the end of

the section.

The secondary heap	is built from both ends, and it	grows and shrinks

according to how many macros and INCLUDE files are currently open.  At

all times there	will be	at least one entry on the heap,	for the	original

source code file.  The expression parser also uses the secondary heap to

store its working stacks - this	space is freed as soon as an expression

has been evaluated.

The bottom	of the heap holds the names of the source code file and

any macro or INCLUDE files that	are currently open.  The full path is

given.	A null string is stored	for user macros.  Macro	arguments are

stored by additional strings, one for each argument in the macro call line.

All strings are	stored in minimum space, similar to the	labels and user

macro text on the primary heap.	 File names are	pointed	to by the fixed

table entries (see below) - macro arguments are	accessed by stepping past

the macro name to the desired argument,	unless NARG would be exceeded.

The fixed portion of the heap is built down from the top.	Each entry

occupies 16 bytes.  Enough information is stored to return to the proper

position in the	outer file once	the current macro or INCLUDE file has been

completely processed.

The diagram below illustrates the layout of the secondary heap.

Heap2 +	maxheap2 ----------->  ___________________________

|				  |

|	  Input	file table	  |

struct InFCtl *InF ---------> |___________________________|

|				  |

|	  Parser operator stack	  |

struct OpStack *Ops --------> |___________________________|

|				  |

|	  (unused space)	  |

struct TermStack *Term -----> |___________________________|

|				  |

|	  Parser term stack	  |

char *NextFNS --------------> |___________________________|

|				  |

|	  Input	file name stack	  |

char *Heap2 ----------------> |___________________________|

The "high-water mark" for NextFNS is stored in char *High2,

and the	"low-water mark" (to stretch a metaphor) for InF is stored

in struct InFCtl *LowInF.  These figures are used only to determine

the maximum heap usage.

AND FINALLY...

Please send me any	bug reports, flames, etc.  I can be reached

on Mind	Link (604/533-2312), at	any Panorama (PAcific NORthwest	AMiga

Association) meeting, or via Jeff Lydiatt or Larry Phillips.

(I don't have the time or money to live on Usenet or CompuServe, etc.)

Charlie	Gibbs

2121 Rindall Avenue

Port Coquitlam,	B.C.  V3C 1T9