Degrader V1.30  1991-93 Chris Hames.  All rights reserved. (17700 bytes)

-------------------------------------------------------------------------

You  upgrade  your  system  and  what  happens?   The idiots who wrote your

favorite program didn't follow the guidelines and it no longer works.  This

is  where degrader comes in, it will try to degrade your system so that the

old brain dead program will work on your machine.  You can also just use it

to  boot  off  floppy  drives other than DF0:, force your machine to PAL or

NTSC,  turn  the audio filter on or off and other things.  Please note that

many  of Degrader's functions are hacks in that they may not work on future

versions  of  the  Amiga  hardware  or software.  The Early Startup Control

provided  in  Kickstart  3.0 (just hold down both mouse buttons at reset to

use) will help getting old software to work as well.  Use its options where

possible as they are more likely to work.

***************************************************************************

If you don't know what all the technical junk is about but just want to get

a program to work:-

The most common way to get a brain dead program to work is to try this

"Degrader NOFASTMEM NOCACHE PRIVILEGED FLOPPIES 0444 SURVIVERESET NOEDITOP"

Or if you are running Kickstart 3.0 you should try something like

"Degrader NOFASTMEM PRIVILEGED SURVIVERESET NOEDITOP" and then use the Early

Startup Control to disable all disk devices except DF0:, disable CPU caches

and select the ECS or Original chipset.

If  you  what maximum speed on a machine with a data cache (68030+) and AGA

(A1200/A4000) when the program works fine without Degrader but is NONDOS so

you can't use SetPatch or CPU to turn on the extra features

"Degrader ENLIGHTEN CACHESON SURVIVERESET NOEDITOP"

***************************************************************************

When  you  run  the  program  from workbench you are presented with various

option buttons which you can toggle on(down & highlighted)/off:-

50Hz

Changes  the  screen  refresh rate to 50Hz (PAL).  You can use this to view

the  extra  lines  a  PAL machine has that a NTSC machine normally doesn't.

Good  for  NONDOS  disk  games and demos that are designed for PAL machines

only  Requires  ECS  Agnus  or  Alice  and a appropriate monitor.  Note has

varied  effects with a AGA chipset depending on the screen mode.  Kickstart

3.0 Early Startup Control has a similar function.

60Hz

Changes  the  screen  refresh  rate  to 60Hz (NTSC).  Requires ECS Agnus or

Alice  and  a  appropriate  monitor.   Note  has  varied effects with a AGA

chipset  depending on the screen mode.  Kickstart 3.0 Early Startup Control

has a similar function.

50HzSystem

Modifies  GfxBase  to  give  values that will make some programs think this

machine  is  PAL.   Useful in Kickstart 1.2/1.3.  Doesn't seem to work well

with  Kickstart  2.0.   Kickstart  3.0  Early Startup Control has a similar

function.

60HzSystem

Modifies  GfxBase  to  give  values that will make some programs think this

machine  is  NTSC.  Useful in Kickstart 1.2/1.3.  Doesn't seem to work well

with  Kickstart  2.0.   Kickstart  3.0  Early Startup Control has a similar

function.

AudFilterOff

Most  Amiga's  have  a audio filter which can be switched off and the power

light usually dims as well.

AudFilterOn

Most  Amiga's  have  a  audio filter which can be switched on and the power

light usually brightens as well.

NoFastMem

What this does depend on whether you selected for the machine to reset.  If

reset,  it will stop any memory besides chip memory from being added to the

system during reset.  If Kickstart 2.0 or later is being used you can still

add  memory yourself.  So if you wanted to save your Fast ram for a certain

purpose  you  could  use  this  option  then when your machine boots up use

ADDMEM  to  add  the memory and you will have the full amount free.  If you

don't select a reset option it will patch the system so no Fast memory will

be allocated by programs.

HalfMegChip

Sets  the  amount  of  chip  memory your system has to 512K.  Some programs

don't  like  it if you have 1 or 2 meg of chip memory so use this option to

trick  the program into thinking you only have 512K of chip memory.  If you

don't  select  a  reset  option  it  resorts  to  some  nasty  memory  list

manipulation.

OneMegChip

Sets  the  amount  of  chip memory your system has to 1024K.  Some programs

don't  like it if you have 2 meg of chip memory so use this option to trick

the program into thinking you only have 1024K of chip memory.  If you don't

select a reset option it resorts to some nasty memory list manipulation.

NoMemAt

Allows  you  to  stop a certain memory block from being added to the system

memory  list.  If you don't select a reset option it just gives that memory

block  a priority of -128 so it is not allocated until you run out of other

memory.  NOTE:  you must turn the Button "ON" as well as enter value.

AddMem

Adds memory to the memory list.  Useful if you have a non-autoconfig memory

board  and  want  your  memory added at reset time so it is used a lot more

instead  of your chip memory.  NOTE:  you must turn the Button "ON" as well

as enter values.

NoCache

Turns  off  your  Cache  & Burst modes of 68020 or higher processors.  Some

games  do  what  is  called  self  modifying code and often end up with the

processor  executing  what  is  already  in  the cache instead of what they

intended.  Kickstart 3.0 Early Startup Control has a similar function.

Privileged

Changes  move  SR,<ea>  to  move  CCR,<ea>.   MOVE SR,<ea> is privileged on

processors  68010+.   If  a program gives a 00000008 GURU consistently this

might  help.   Very  few  programs  read the status register directly, even

fewer get it wrong as well.

TimeWaste

Slows  down  your  machine  based  on  the  value you give 65535 to 1.  The

smaller  the  number  the  slower  your machine gets.  Won't work with many

programs  as  they  take  over or turn off the interrupt I use.  NOTE:  you

must turn the Button "ON" as well as enter value.

BlockDevice

Stops programs from opening a specified device.

DF0: is -> DF0:

DF1: is -> DF1:

DF2: is -> DF2:

DF3: is -> DF3:

Use  these  to  swap  drives  around.   Set  a  drive  to  DF4:  to make it

unuseable.  Hence it won't take up precious chip memory.  Only works with a

reset operation.

CachesOn

Currently  the  system  doesn't turn on the data cache, until SetPatch time

(provided  you  have  one  of  course  (68030+)).   This  will  turn on the

instruction  cache  and  burst  as well as the data cache and could get you

extra  speed  if  it  is  not  possible  to run SetPatch before the program

normally.

Enlighten

This  is only relevant if you have the AGA chipset in your machine (such as

A1200's  and  A4000's).   The  extra bandwidth and features aren't normally

turned  on  until SetPatch is executed.  If a NONDOSDISK program is written

correctly  enough  for  it to run with the extra bandwidth and new features

then  it  could gain a lot of speed in graphics operations with them turned

on.  It only works with Kickstart 3.0.

PromoteAll

This  only  works  with  Kickstart 2.0 or later.  It allows you to make all

programs  that  call  OpenScreen  use  a specific monitor.  You must have a

capable  monitor  and have it also in your devs/monitors drawer or similar.

NOTE:   you must turn the Button "ON" as well as enter value.  Here are the

values of some monitor types and uses:-

$11000 Make all opened screens 60Hz.

$91000 Make all opened screens use the NTSC DBL scan modes.

$21000 Make all opened screens 50Hz.

$a1000 Make all opened screens use the PAL DBL scan modes.

$00000 Remove the monitor part from open screens.

You  can  also  do  some  complex things like force most screens to SUPER72

800*300 mode by giving a value of $81020.

NoView

This is usefull for AGA machines it will create a blank display and get the

system very close to a ECS state.  Does not work with reset options.

***************************************************************************

After  you  have  selected  the  options you want you have a choice of four

buttons  at  the bottom of the screen.  To do it now only, to do it now and

then  load  and  run  a bootblock, to survive one reset only, or to survive

resets  until the left mouse button is held down and the right mouse button

is not down at reset.

If you do a reset selection your machine will reboot and at reset time your

screen will go dark blue->light blue->dark orange->light orange.  Note that

some options require Kickstart specific code but should work with at

least:-

Kickstart 1.2 ROM.

Kickstart 1.3 ROM and A3000 Kickstart 1.3 file.

Kickstart 2.04 ROM and A3000 Kickstart 2.04 file

Kickstart 2.05 ROM (37.300)

Kickstart 3.00 ROM (39.109)

The  screen  should go green at reset time if Degrader needs to be familiar

with the Kickstart to do one of the selected options, but is not.

***************************************************************************

If  you  start degrader from a Shell/CLI you can use all the keywords shown

in  the  window  display  as  described above, except the DF0:  stuff.  And

there a few more:-

Floppies

This  is  the  Shell/CLI interface to changing floppies.  Give it 4 numbers

with values 0-4.

RunBootBlock

Start the NDOS disk in DF0:.

SurviveReset

Reset then do all the goodies.

SurviveResets

Reset  then  do  all  the goodies.  And still be there to do it next reset.

Keep it up until the user press the left mouse button at reset.

NoEditOp

Don't bring up the window to edit options.

***************************************************************************

Examples of using Degrader from a Shell/CLI:-

"Degrader AUDFILTEROFF"

Bring up option window with audio filter selected.

"Degrader AUDFILTERON NOEDITOP"

Turn the audio filter on.  The option window is not used.

"Degrader 60HZ 60HZSYSTEM"

Bring up option window with 60HZ and 60HZSYSTEM selected.

"Degrader ADDMEM $100000 5 10 $400000 SURVIVERESETS NOEDITOP"

Add 1 meg of memory at $400000 at each reset from now on.

"Degrader TIMEWASTE 50 SURVIVERESET NOEDITOP"

Reset with hopefully a lot slower machine.

"Degrader FLOPPIES 0444 SURVIVERESETS NOEDITOP"

Reset and only have DF0:  available.

***************************************************************************

Thanks to the people who have helped with testing etc, especially:-

Pekka Heino

Andrew Morison

Michael Chamberlain

***************************************************************************

Common reasons why programs fail on one machine and work on others:-

1.   Use processor to do critical timing loops and hence on different speed

machines  fail  to  delay the correct amount.  This is often the reason why

custom  disk  reading  routines fail.  Turning off your fast memory, caches

and  anything  else  that  makes your machine faster than the slowest Amiga

might help.

2.   Use  self-modifying  code.  The processor can end up running different

instructions  to  the  the  ones  just loaded or modified because the cache

still holds the old values.  You can turn off your caches.

3.   Load  into memory at a specific location.  One machine might have more

system  hardware  or a new OS could allocate more memory before the machine

boots  up  and  the  program  then  overwrites  memory  already being used.

Turning  off  or disconnecting any extra hardware, and disabling all drives

except for DF0:  often helps with this problem.

4.   Use  the hardware directly and don't write zeros to reserved/undefined

bits.   The  next  machine comes along and those bits now do something that

the program didn't intend.  If you have a AGA machine use the early startup

menu  to  select the ECS or original chipset.  Using the NoView option just

before executing the program will often do the trick as well.

***************************************************************************

Please  note  this  program  uses the ColdCapture, KickTagPtr and sometimes

patches  LVOAddMemList,  LVOAllocMem,  LVOAvailMem,  LVOOpenDevice  so good

virus detectors, like mine :-), will give you warnings.

This  program  may  be copied as long as it is accompanied by this doc file

both  unaltered  & no money is charged (except for a reasonable copying fee

such  as  what  Fred Fish charges).  All use of this program is at your own

risk & I assume no responsibility or liability for any problems that occur.

If you want to contact me to give me a porsche or similar,

Chris Hames

Internet:	bytey@werple.apana.org.au

bytey@phoenix.apana.org.au

FidoNet: 	3:633/353

If  the  above  addresses  don't  work, get the latest version of one of my

other   programs  such  as  DirWork  or  PC-Task.