Computer Viruses: An Introduction

A very simple definition of computer viruses is:

"A program that modifies other programs by placing a copy of

itself inside them."

This definition is somewhat simplified, and does not cover all virus

types, but is sufficient to show the major difference between viruses and

so-called "Trojan" programs, which is that the virus replicates, but the

Trojan does not.  (The definition does not cover the so-called

"companion"-type viruses, however).

A Trojan is a program that pretends to do something useful (or at least

interesting), but when it is run, it may have some harmful effect, like

scrambling your FAT (File Allocation Table) or formatting the hard disk.

Viruses and Trojans may contain a "time-bomb", intended to destroy

programs or data on a specific date or when some condition has been

fulfilled.

A time bomb is often designed to be harmful, maybe doing something like

formatting the hard disk.  Sometimes it is relatively harmless, perhaps

slowing the computer down every Friday or making a ball bounce around the

screen.  However, there is really no such thing as a harmless virus.  Even if

a virus has been intended to cause no damage, it may do so in certain cases,

often due to the incompetence of the virus writer or unexpected hardware

or software revisions.

A virus may be modified, either by the original author or someone else, so

that a more harmful version of it appears.  It is also possible that the

modification produces a less harmful virus, but that has only rarely

happened.

The damage caused by a virus may consist of the deletion of data or

programs, maybe even reformatting of the hard disk, but more subtle damage

is also possible.  Some viruses may modify data or introduce typing errors

into text.  Other viruses may have no intentional effects other than just

replicating.

Two different groups of viruses occur on PCs, boot sector viruses (BSV)

and program viruses, although a few viruses belong to both groups.

A BSV infects the boot sector on a diskette.  Normally the boot sector

contains code to load the operating system files.  The BSV replaces the

original boot sector with itself and stores the original boot sector

somewhere else on the diskette or simply replaces it totally.  When a

computer is then later booted from this diskette, the virus takes control

and hides in RAM.  It will then load and execute the original boot sector,

and from then on everything will be as usual.  Except, of course, that

every diskette inserted in the computer will be infected with the virus,

unless it is write-protected.

A BSV will usually hide at the top of memory, reducing the amount of

memory that the DOS sees.  For example, a computer with 640K might appear

to have only 639K.

Many BSVs are also able to infect hard disks, where the process is similar

to that described above, although they may infect the master boot

record instead of the DOS boot record.

Program viruses, the second type of computer viruses, infect executable

programs, usually .COM and .EXE files, but sometimes also overlay files.

An infected program will contain a copy of the virus, usually at the end,

but in some cases at the beginning of the original program.

When an infected program is run, the virus may stay resident in memory

and infect every program run.  Viruses using this method to spread the

infection are called "Resident Viruses".

Other viruses may search for a new file to infect, when an infected

program is executed.  The virus then transfers control to the original

program.  Viruses using this method to spread the infection are

called "Direct Action Viruses".  It is possible for a virus to use both

methods of infection.

Most viruses try to recognize existing infections, so they do not infect

what has already been infected.  This makes it possible to inoculate

against specific viruses, by making the "victim" appear to be infected.

However, this method is useless as a general defense, as it is not

possible to inoculate the same program against multiple viruses.

In general, viruses are rather unusual programs, rather simple, but written

just like any other program.  It does not take a genius to write one - any

average assembly language programmer can easily do it.  Fortunately, few of

them do.

Now - to correct some common misconceptions, here are a few bits of

information about what viruses cannot do.

A virus cannot spread from one type of computer to another.

For example, a virus designed to infect Macintosh computers cannot

infect PCs or vice versa.

A virus cannot appear all by itself, it has to be written, just

like any other program.

Not all viruses are harmful; some may only cause minor damage as

a side effect.

A virus cannot infect a computer unless it is booted from an

infected diskette or an infected program is run on it.  Reading

data from an infected diskette cannot cause an infection.

A write-protected diskette cannot become infected.

There is no way a virus can attach itself to data files, so

viruses cannot be distributed with them.  However, a BSV can

be distributed on data diskettes.

The F-PROT package will provide protection against viruses, but there are

other methods that also should be used.  Before I list them, I want to

warn you against three methods that are of very limited use.

One anti-virus measure consists of making every executable file read-only,

by issuing commands like

ATTRIB +R *.EXE

This is actually not a bad idea, but it will not provide much protection

against viruses.  Most program viruses will remove this protection before

they infect files, and restore it afterwards.  Making files read-only will

of course have no effect on BSVs.  The main purpose of this method is actually

to protect the user from his own mistakes, because this makes it harder to

delete programs by mistake.  However, some viruses are stopped by this

method, "Lehigh" and "South African" in particular.

Another method is to hide the COMMAND.COM file, by giving the following

sequence of instructions:

MKDIR C:\HIDDEN

COPY COMMAND.COM C:\HIDDEN

DEL COMMAND.COM

add SHELL=C:\HIDDEN\COMMAND.COM /P to CONFIG.SYS

add SET COMSPEC=C:\HIDDEN\COMMAND.COM to AUTOEXEC.BAT

This method is quite useless, to say the least.  Few viruses infect

COMMAND.COM, and some of them are able to do it, even if it has been

hidden, using this method.

A third useless method is to change the name of COMMAND.COM and patch other

programs so they use the new name.  Somebody who had only heard of the "Lehigh"

virus got this "bright" idea.  Apparently he thought that all other viruses

operated like it, so he wrote and distributed a program to do this

automatically.  He thought it was a general cure for the virus problem, but

he was wrong.

On the other hand, there are a number of ways to provide useful protection.

Rule #1 is: MAKE BACKUPS!!!  Keep good backups (more than one) of

everything you do not want to lose.  This will not only protect you

from serious damage caused by viruses, but is also necessary in the

case of a serious hardware failure.

Never boot a computer with a hard disk from a diskette because that

is the only way the hard disk could become infected with a BSV.

(well, strictly speaking, it can happen if you run a "dropper"

program too, but that happens extremely rarely).

Should you, by accident, have left a non-bootable diskette in

drive A: when you turn the computer on, the message

Not a system disk.

may appear.  If the diskette was infected with a virus, it will now

be active, but may not have infected the hard disk yet. if this

happens, turn the computer off, or press the reset button.  It is

important to note that pressing Ctrl-Alt-Del will not be sufficient,

as a few viruses can survive that.

If the computer has no hard disk, but is booted from a diskette,

you should always use the same diskette, and keep it write-protected.

Keep all diskettes write-protected unless you need to write to them.

When you obtain new software on a diskette, write-protect the

diskette before you make a backup copy of it.  If it is not possible

to make a backup of the diskette, because of some idiotic

copy-protection, I do not recommend using the software.

Be really careful regarding your sources of software.  In general,

shrink-wrapped commercial software should be "clean", but there

have been a few documented cases of infected commercial software.

Public-Domain, Freeware and Shareware packages do not have to be

any more dangerous - it all depends on the source.  If you obtain

software from a BBS, check what precautions the SysOp takes against

viruses.  If he does not screen the software made available for

downloading, you should find another source.

Check all new software for infection before you run it for the

first time.  It is even advisable to use a couple of scanners from

different manufacturers, as no single scanner is able to detect

all viruses.

Obtain Shareware, Freeware and Public-Domain software from the

original author, if at all possible.

Look out for any "unusual" behavior on your computer, like:

Does it take longer than usually to load programs ?

Do unusual error messages appear ?

Does the memory size seem to have decreased ?

Do the disk lights stay on longer than they used to ?

Do files just disappear ?

Anything like this might indicate a virus infection.

If your computer is infected with a virus - DON'T PANIC!  Sometimes a

badly thought out attempt to remove a virus will do much more damage

than the virus could have done.  If you are not sure what to do, leave

your computer turned off until you find someone to remove the virus for

you.

Finally, remember that some viruses may interfere with the disinfection

operation if they are active in memory at that time, so before attempting

to disinfect you MUST boot the computer from a CLEAN system diskette.

It is also a good idea to boot from a clean system diskette before

scanning for viruses, as several "stealth" viruses are very difficult do

detect if they are active in memory during virus scanning.