|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- |
| ITU-T RECOMMENDATION SUMMARY |  |  |  |
| Rec. No. : I.311 |  | Title : B-ISDN General network aspects |  |
| Study Group : XVIII - ISDN |  |  |  |
| Version : New |  | Date of adoption : 1991 | Notes : |
| Recommendation I.311 describes networking techniques, signalling principles, traffic control and resources management for B-ISDN. Networking techniques: B-ISDN networking techniques include network layering aspects and applications of the virtual path and virtual channel. Signalling principles: In B-ISDN, the use of the asynchronous transfer mode (ATM) allows for a multiplicity of service types and characteristics and for the locial separation of signalling from user information streams. A user may have multiple signalling entities connected to the network call management via separate ATM virtual channel connections. Recommendation I.311 identifies the signalling capabilities needed in B-ISDN and the requirements for establishing signalling communication paths. Traffic control and ressource management: The B-ISDN, which is based on the ATM technique, is designed to transport a wide variety of traffic classes satisfying the required performance in terms of user and network requirements. To cope with these requirements, an ATM-based network will provide the following traffic control capabilities: connection admission control, usage parameter control, priority control, congestion control. |  |  |  |
| To order the complete text of this Recommendation, please use the Order Form for ITU-T Recommendations. An electronic version of this form is available on ITUDOC (Winword 2.0: UPI=ITU-5265; ASCII: UPI=ITU-2488). |  |  |  |