|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- |
| ITU-T RECOMMENDATION SUMMARY |  |  |  |
| Rec. No. : Q.81.3 |  | Title : Stage 2 description for number identification supplementary services |  |
| Study Group : XI - Switching and Signalling |  |  |  |
| Version : Revised |  | Date of adoption : 1991 | Notes : |
| CALLING LINE IDENTIFICATION PRESENTATION (CLIP) AND CALLING LINE IDENTIFICATION RESTRICTION (CLIR) Recommendation Q.81.3 defines the stage 2 of the integrated services digital network (ISDN) as provided by the telecommunications operators for the Calling Line Identification Presentation (CLIP) and Calling Line Identification Restriction (CLIR) supplementary services. Stage 2 identifies the functional capabilities and the information flows needed to support the service description. The stage 2 description also identifies user operations not directly associated with a call (see Recommendation I.130). Recommendation Q.81.3 is defined according to the methodology specified in Recommendation Q.65. Recommendation Q.81.3 does not formally describe the relationship between these supplementary services and the basic call but, where possible, this information is included for guidance. In addition, Recommendation Q.81.3 does not specify the requirements where the service is provided to the user via a private ISDN. Recommendation Q.81.3 does not specify the requirements for the allocation of defined functional entities within a private ISDN; it does however define which functional entities may be allocated to a private ISDN. Recommendation Q.81.3 does not specify the additional requirements where the service is provided to the user via a telecommunications network that is not an ISDN. The CLIP supplementary service provides the called party with the possibility to receive identification of the calling party. The CLIR supplementary service enables the calling party to prevent presentation of its ISDN number to the called party. The CLIP and CLIR supplementary services are applicable to all telecommunication services. Recommendation Q.81.3 is applicable to the stage 3 Recommendations for the ISDN CLIP and CLIR supplementary services, as defined in Recommendation I.130. |  |  |  |
| To order the complete text of this Recommendation, please use the Order Form for ITU-T Recommendations. An electronic version of this form is available on ITUDOC (Winword 2.0: UPI=ITU-5265; ASCII: UPI=ITU-2488) |  |  |  |