Albert Swarthout

3456 E. Green St.

Pasadena, CA 91107

INSTRUCTIONS FOR

CONSTRUCTION AND USE

OF COMPUTER/FAX

INTERFACE CABLE

Construction:

(See fig. below)

- Cutting red* lead, connect one side to resister.

- Connect other end of resister to black lead of 9V battery connector.

- Connect red lead of battery connector to other red phone cable lead.

Operation:

- Connect cable between phone line jack of fax machine and phone line jack of computer fax/modem.

- Insert document to be scanned into fax machine.

- Initiate manual fax receive on computer (refer to fax software documentation).

- Pick up fax machine handset and listen for high pitch tone from computer.

- When heard, initiate fax transmission from machine (usually by pressing “Start” or “Send”).

* In the event that the modular phone cable does not use colored leads use middle two conductors.

NOTE:	Either end of cable may be connected to either device. Also, polarity of battery leads is irrelevant 	but, to clarify the diagram, a given polarity was arbitrarily selected.