GateSim(TM) - A Gate Level Simulator

Version 1.0 Demo

To run the automated demo, insert the demo disk in drive a, type a:<Enter>,

and type gatesim lights<Enter>.  You may temporarily halt a simulation by

pressing the space bar.  Press it again to continue.  At the end of each

horizontal simulation display, you may press any key to continue.

This demo version has all the features of the production version except the

following:

1.  There is a limit of 100 nodes in the netlist file.

2.  The result file facility has been removed.

Please copy this diskette freely.

On your GateSim demonstration disk you should find the following files:

!readme.doc      This file.

gatesim.com      The main simulator executable file.

gatesim.000      These two simulator overlay files need to be in the same

gatesim.001         directory as the .com file.

gatesim.doc      An on-line condensed manual for GateSim.

lights.sch       An OrCAD schematic file of an example circuit.  This design

can be viewed on screen if you have a copy of OrCAD 3.0.

A printout of the circuit is included with this demo disk.

lights.wir       OrCAD EDIF file produced from the .sch file using OrCAD's

netlist utility with the /p option.

lights.net       Net list file produced by NetTran from the OrCAD EDIF file

mapping to National Semiconductor gate array cells and

timing information.  This netlist file is read into the

simulator during the demo.  The netlist translator,

NetTran, and vendor mapping libraries such as for National

Semiconductor gate arrays are available separately from

Tanner Research, Inc.

lights.vec       The input vector file that specifies the signals going into

the example circuit.

lights.sim       This file contains the simulator commands to run the demo.

When you start the simulator by typing "gatesim lights"

simulator commands are taken from this file instead of

from the keyboard.

lights.cmp       The production output test vectors.  Fault simulation

compares this file with the output from a simulation of

the circuit with a fault in it.  This vector set also is

used by the vendor's tester hardware.

lights.cm2       A much smaller test vector set that still gives 100% fault

coverage.

The Example Circuit

--- ------- -------

Our example circuit implements a traffic light controller.  It accepts a clock

time base and produces signals for the green, yellow, and red traffic signals

in both the east-west direction and the north-south direction.  In addition,

the walk and don't walk signals are generated for both directions.  The don't

walk signal flashes first and then turns on solid.

Eight D flip-flops form a binary ripple counter that divides down the clock.

Various logic gates decode the states of the counter to generate the required

traffic control signals.  The ripple counter outputs do not all change at once

resulting in hazards or glitches that the simulator will help uncover.

The Automatic Demo

--- --------- ----

The automated demo begins by turning on the monitoring of the simulation on

the screen.  (The simulation runs faster if the monitoring is supressed.)

Next the netlist is loaded from a file.  The netlist file may be hand

generated or created automatically by NetTran (available separately) from an

OrCAD schematic.  The NEtfile command specifies the file (lights.net) to be

read in.

The PAttfile statement gives the name of the vector file (lights.vec) to be

read in during the simulation.  The vector file contains the specification for

the input signals to be applied to the circuit during the simulation.  Within

the pattern or vector file, waveforms may be generated by specifying the

signal value for each time using a .pattern statement.  The .clk statement

produces repetitive waveforms.  The file lights.vec has examples of both of

these methods for generating input signals.

Back in the .sim file, the VIewvector command makes the input vectors appear

in the simulator display, mixed with the simulator output.  Input vectors

show up with a ">" character in the first column.

The .hex statement defines a set of up to four signals that can be viewed

during the simulation as a hex digit.  Later when the name "TOP" appears

in a .TAB statement, it refers to the four signals collectively as a hex

digit.

The simulator itself has no fixed time base.  Simulation times are in terms of

integers.  Vendor libraries, however, are created with a specific choice of

time unit.  The National Semiconductor gate array library, whose timing is

used in this netlist, was created with numbers representing 1/10ths of

nanoseconds.  Thus a simulator time of 50000 represents 5usec.  The DP command

sets how the time units are displayed during a simulation.  In this case, we

want to display times in nanoseconds, so we specify dividing simulator units

by 10 and showing 1 digit after the decimal point.

The nodes and hex digits to be displayed during the simulation are listed in

order in a .TAble statement.  The semicolons serve to space out the resulting

simulation signals across the screen.

We begin a simulation for 5usec.  The node names are listed vertically at the

top of the screen and the signal values scroll vertically underneath.  Since

we did not specify a display time step in the sim command, a line of display

is shown on the screen whenever any of the monitored signals changes.  This is

the most detailed viewing option that will allow you to easily trace activity

through your circuit.  You will see any glitches on the monitored nodes.

Notice however that the times listed along the left hand side are not evenly

spaced.  There is a time corresponding to each monitored transition but there

is no time listed during periods of inactivity.  We will see an alternative

display style shortly.

As the simulation progesses, you can see that there is varying amounts of

activity in the circuit depending on how far down the ripple chain a clock

signal propogates.  Each time a signal changes from the previous line of

display, it is shown in a different color to highlight the change.

After the 5usec simulation completes, we decide to monitor an additional

four signals as a hex digit.  We add this item to the existing display list

with the '+' option to the .TAble statement.  We continue with the simulation

for another 5usec with another SImulate statement.  SImulate + continues the

simulation for a length of time equal to the last simulation time.

Next we change the display from vertical scrolling to horizontal.  Horizontal

mode does not allow as many signals to be displayed on the screen at once, but

more time values of the signals may be seen.  The simulation continues.

The waveform statement changes to displaying signals as waveforms instead of

binary digits.  Hex digits continue to be displayed using characters.  In

this simulation you can clearly see a glitch on the east-west green signal

about the time that yellow is turning off and red is turning on.  With this

display mode, it is hard to tell just how wide the glitch is relative to

the other signals.

We continue the simulation with a sampled display option by specifying a time

step in the sim statement.  Note that this change does not affect the way the

simulator simulates your circuit, only the way you view the results.  During

this simulation, we see from the clock waveform that the time is displayed

evenly but we see that the green signal glitch has disappeared.  This is

because we are sampling the simulator output at exactly the clock rate so

the glitch resulting from a clock transition has gone away by the time the

waveform is next sampled for display.

We decrease the display time step and simulate again.  Now we can see

just how big the glitch is relative to a clock time.

If Vdd and Gnd are used as logic signals in a circuit, the fault simulator

will note that these nodes are not testable - by definition, those nodes are

stuck at 1 or stuck at 0.  We use the FOrcenode command to set these two nodes

to already tested (T) so that they won't be reported as untestable.

We begin a fault simulation using the file lights.cmp.  This file or a

derivative of it will be the one sent to the vendor to run on their tester.

The goal of the designer is to choose the vectors so that the shortest set

of vectors will catch as many possible fabrication faults as possible.

The file lights.cmp is similar to a simulator output file but has several

important differences.  First, tester hardware has no way of telling if a

signal is unknown (X) so their must be no Xs in the .cmp file.  Instead there

may be a character (.) that instructs the tester not to compare the signal

on that cycle.  Testers sample the chip outputs according to the tester clock

so the vectors in the comparison file must be on clock boundaries.  Long

comparison files can also be optimized to make use of the repeat capability

of some tester hardware.

The fault simulation proceeds by forcing a node to 0 and then simulating the

circuit.  If the simulation output differs from the comparison file, that

node is marked as 0-testable.  The same node is then forced to 1 and another

simulation is begun.  If the output differs from the comparison file then

this node is 1-testable.  If the node is both 0-testable and 1-testable than

it is testable.  Then another node is forced to 0 and the process repeats,

until all nodes have been checked for testability.  The simulator reports the

total number of nodes checked and the number that are testable.

The NDelay command reports the delay for all possible paths between the two

specified nodes.  This static timing analysis tool allows you to find critical

paths in your circuit.  The -c option tells how many clock-to-Q pathways

through D flip flops to follow before abandoning a search path.  In this case

we set it to a large number because we want to find all paths.

For the example circuit there are six different paths between the node

TEST_POINT and the node DONT_NS, the longest of which has a rise time delay of

11.9ns and a fall time delay of 11.8ns.  Each of the paths is listed.  You can

trace them along on the schematic.  The node names that have three underscore

characters in them are nodes that were created when mapping the cells on the

schematic to their implementation.  For example, the OR gate, U18, is

implemented by a NOR gate followed by an inverter.  The node in between these

two gates was assigned the name U18___000.  Unlabeled nodes that do appear on

the schematic have names like U18_P3 which refer to the node connected to pin

3 of U18.

A Self-Guided Tour of GateSim

- ---- ------ ---- -- -------

At this point you may wish to copy the entire contents of the demo diskette to

a directory on your hard disk or RAM disk, especially if you continue beyond

the automated demo and modify some files.  You may also wish to print out the

files !readme.doc, lights.sim, lights.vec, and gatesim.doc for reference as

you proceed.

Here are some suggestions for exercising GateSim by executing commands not

included in the automated demo.

1.  Edit the file lights.sim that contains the simulator commands for the

demo.  Change one or two things (e.g. remove all the DLay commands) save the

file and type again:

>gatesim lights<Enter>

Please make a copy of lights.sim before modifying it so you or others may

later run the original automated demo file.

2.  Start up GateSim without a command file by typing

>gatesim<Enter>

Now you can type commands to the simulator and have them executed immediately.

The command line allows for TopDOS or EMACS-like commands such as ^F, ^B, ^A,

and ^E for forward, backward, beginning of line and end of line.  There is

also ^D for forward delete and ^K for delete to end of line.  Command history

may be displayed with ^R and re-executed with ^P and ^N commands.  The home,

end, and arrow keys on the numeric keypad also move the cursor or traverse the

history of commands.  The Tab key performs completion on file names.  Type ?

to get a help screen.

3.  To start with, type in the opening commands of lights.sim to load in the

netlist and test vector file.  Use the TAB command to select the nodes of

interest for viewing during a simulation.

4.  Start a simulation with the SIM command.  Note how you can pause the

simulation with the space bar or interrupt it by hitting any other key.

5.  If you have an EGA card, try the 43 line mode (EGA ON) or try vertical

waveform output (HO OFF) and (WA ON).

6.  Try to track down all the hazards and glitches in the design.

7.  Shorten the fault simulation or change the comparison file so they don't

catch all faults.  For example "FS 0 100000 -c lights" leaves one node

untested.  Take a look at the file lights.cm2.  This vector set has been

reduced from the file lights.cmp to make a shorter set but still have complete

fault coverage.  This condensing is often necessary for large chips with test

vector sets that otherwise would exceed the maximum number of vectors that

hardware testers can hold.

8.  Try the -t option of the NDelay command.  This option shows you the path

under consideration as the simulator traverses the tree of possible paths.

The maximum delay for rise time and fall time is displayed as it progresses.

To order your full functional version of GateSim, call (818) 795-1696 or write

to us at the address below.

GateSim is a trademark of Tanner Research, Inc.  Other brand and product names

are trademarks or registered trademarks of their respective holders.

Copyright (c) 1988 Tanner Research, Inc.

Tanner Research, Inc.

128 West Del Mar Boulevard

Pasadena, CA  91105

(818) 795-1696