GateSim(TM) - A Gate Level Simulator

Condensed Manual  Version 1.0

1.  OVERVIEW:

The Tanner Research, Inc. simulator GateSim is an event-driven four-state

(0,1,Z,X) digital logic simulator with the capability of running a 80286/80386

processor in virtual protected mode to simulate extremely large designs.  The

inner simulation loop is written in assembly language for extremely high

simulation speed.  Circuit primitives include boolean gates, tri-state

drivers, and/or/invert gates, D flip-flops, latches, pullups, repeatable

waveform signals, and RAM/ROM models.  Each primitive supports gate-specific

timing delays which may vary from instance to instance.  Times are handled as

32-bit integers, allowing extremely long simulations.  In addition to its

simulation capabilities, GateSim contains extensive features to support design

verification, including flexible output formatting, automatic comparison of

simulation output against an "expected result" file, critical path

determination, and fault simulation.

2.  COMMAND INPUT:

GateSim uses a simple command line interface, patterned after the SILOS

simulator, but including many TopDOS-like features to facilitate command

input.  Commands to the simulator are distinguished by their first two letters

(not case sensitive).  Comments are allowed in all files and from the

keyboard, and consist of a $ followed by any number of characters up to an end

of line.  Continuation lines are allowed by placing a '+' as the first

character on the succeeding line.  Commands may also be input from a file,

either by placing the file name on the DOS command line when the simulator is

invoked (e.g., gatesim mychip.sim) or by entering a command (e.g., "INput

mychip.sim") within the simulator.  Command files may be nested up to four

deep.  The default extension for command files is SIM.

3.  KEYBOARD EDITING:

The keyboard input lines can be edited as in TopDOS, with Epsilon-like

keystrokes, plus history (^R) and recall (^P and ^N).  Arrow keys on the

numeric keypad are alternatives to move the cursor.  There are also keyboard

macros available (F1 through F10), which can be defined by a command of the

form:

@Fn=<text>|<text>...

The | character is interpreted as a carriage return and can also be used from

the command line directly to input multiple commands on one line.  You can

also invoke a keyboard macro from the command line (or an input file) by a

line of the form @Fn).  The macros can be displayed by typing ^V.  Filename

completion is also supported, using the <Tab> key.

At any time during the execution of most commands (e.g., SIM, DISPLAY, etc.),

you can press space bar or ^S to halt the display.  A message <Pause> will be

displayed, and pressing any other key will allow the command to continue.  If

you press ^C or a displayable character (e.g., 'A'..'Z'), the message

<Halt> (Y/N/Q/P)?  will be displayed.  Typing ^C or 'Y' to this prompt causes

the command execution to terminate, but execution of any further commands from

an input file will then continue.  Typing a 'Q 'will terminate this command

and return control to the keyboard, ignoring all input files.  Typing a 'P'

will terminate execution of the current command and return command to the

keyboard, but the pointer within the current command file will be retained so

that commands execution can resume via the 'RF' command (for Resume File).

Any other key (e.g, 'N') will allow execution to continue.

4.  INSTALLATION:

To run the simulator, you must copy the files GATESIM.COM, GATESIM.000, and

GATESIM.001 from their home to a directory on your path.  The latter files

contain the simulator overlay procedures.  Due to the overhead of loading the

overlay files during command input, it is strongly recommended that these

files reside on a RAM disk.

5.  GENERAL COMMAND SYNTAX:

Each command has its own syntax, but the general form is

COmmand [parameters]* [options]*

Note that only the first two letters of the command name are signficant; the

others are optional and are shown in lower case to help clarify the meaning of

the command.

The options for a command typically take the form '-X [parameter]' or

'+X [parameter]', where X is the option letter (not case sensitive) and

the parameter may or may not be required, depending on the option.

When file names are included in the command parameters, they typically must

be prefaced by '<' for input or '>' for output.  These characters may not

be required if the file name can be otherwise unambigously parsed, but in

general it is safe to use the '<' and '>' characters.  In addition, the output

can be appended to the output file by using '>>' in front of the file name.

6.  SIM COMMANDS:

ADdnet [<]filename

Read in more parts and add them to the network (default extension NET)

ANybad [ON|OFF|?]

ANYBAD is a boolean variable maintained by the simulator to allow

accumulation of verify results for use in 'IF' commands.  The parameters

have the following meaning:

ON  -   ANYBAD:=TRUE

OFF -   ANYBAD:=FALSE

?   -   ANYBAD:=ANYBAD OR BADVERIFY

COmments [on|off|?]

Enable/disable the display of comments in the simulation results.  If

the '?' is used, the current state of the comments will be displyed.

DIsplay [#n] [t0 [TO] t1 [STEP dt]] [>[>]dispfile] [-H] [-N]

Display RESULTFILE with given time parameters, optionally to a file of

default extension OUT.  If no file is given, the display is to the screen.

The -H option turns off the hiliting of signal changes on the screen.

The -N forces lines repeated due to a non-zero step to have all signals

forced to 'X'.

If the parameter #n is used, then the output is reformatted according

to TAB #n (see the .TAB command).

DLay d

Wait for d milleseconds.  Useful to slow down the command stream from a

file to the point that you can read it during demos.

DOs

Push to DOS.  Return to GateSim by typing the DOS command 'EXIT'

DP pos show shift

This command controls formatting decimal points in the times output by

the simulator.  The meaning of the three parameters is as follows:

Pos  - Divide the (integer) time by 10^Pos

Show - Display only this many digits to the right of the '.'

Shift- 0 ==> lose trailing digit  (digit  0)

1 ==> lose leading digit   (digit 10)

If Pos=0 (the default), no formatting is performed.  Typically, unless

extremely large times are needed, it is safe to set Shift to 1.

Here are some examples, using a time of 17234:

Pos = 0                            Output = '      17234'

Pos = 1, Show = 1, Shift = 1       Output = '     1723.4'

Pos = 1, Show = 1, Shift = 0       Output = '      1723.'

Pos = 4, Show = 3, Shift = 0       Output = '      1.723'

Pos = 4, Show = 3, Shift = 1       Output = '     1.723 '

Pos = 4, Show = 4, Shift = 1       Output = '     1.7234'

EGa43 [on|off]

Enable or disable the EGA 43 line mode display.  Default is off

ERrorfile >[>]filename

Open the file to make a copy of all error/warning messages.  If appending

is specified but the file does not exist, it will first be created.

EXit

Quit the simulator

FOrcenode [nodename|nibblename|nodespec] [[~]option]

Force nodes, including wildcards (? and *).  The options are:

0     - force to 0

1     - force to 1

X     - force to X

.     - force to remain at current value

Z     - unforce

U     - force undetected stuck at 0 and 1

U0    - force undetected stuck at 0

U1    - force undetected stuck at 1

G     - force detected stuck at 0 and 1

G0    - force detected stuck at 0

G1    - force detected stuck at 1

M     - mark the node

A tilde (~) causes nodes to be forced to the inverse of the given state.

FSim   [t0 [TO] t1 [STEP dt]] -C cmpfile [>[>]nodeStatFile] [-R] [-W n] [+W n]

Do fault simulation for all undetected faults in the network, comparing

against the cmpfile (default extension CMP).   The comparison file must

look like a simulator output file, lines of the form:

<time> <signals> <ret>

where a '.' in the signals is a wild card for comparison.

The -R option means to use random node ordering.  Outputting the results

can be done to a file (default extension FLT), suitable for reading back

in as FORCE statements.

The -W/+W options specify the time window during which the simulation

results must match the compare file.  The default window is [-0,0], so

a comparison is done only at the comparison file time.

GAtes [gatetype|Unnn[.suffix]] [-D] [-S delta] [+S delta] [>[>]filename]

List all gates of a given type or with a given U number/suffix.

For example,

GA AND*     gives all AND or AND-OR-INVERT gates

GA U132     gives all gates with a U number of 132

The -D option gives detailed information about all pins on each gate,

as well as delay information and a dump of memory allocated to the

gate record (largely for debugging purposes).

The -S/+S options are used to adjust the setup times of all matching

D flip-flops and latches by the delta time given (-S ==> decrease setup

time, +S ==> increase).  If these options are used, no display is given,

and the only allowable gate type is '*', but a U number may be specified.

HELP or  ?

Give a help screen

.HEX [busname=node3,node2,node1,node0]*

Define a hex nibble for output or input.  It is allowable to have

a node name of ;, which means that the bit of the nibble is ignored.

HOriz [on|off|?]

Enable/Disable the horizontal display of simulation results.  The

default is OFF, so that results are displayed vertically, allowing

up to 80 columns of output on the screen.  If '?' is the parameter,

the current state of display will be shown

IF [NOT] condition THEN command[|command]*

Conditions supported:

Badverify - set by all verify/compare operations

Anybad    - set via ANybad commands

INput [<]filename

Take commands from a file (may be nested up to four deep)

LIstnode [nodename|nibblename|nodespec] [-Q] [[~]state]* [>[>]filename]

Display the status of matching nodes, including wildcards (? and *).

All states given must match for a node to be displayed, and tilde

means the inverse of a state (e.g., ~0 ==> nodes not forced to 0).

The nodes are output in alphabetical order, unless the -Q option is

used, in which case only the totals are given.  The states are:

0     - nodes forced to 0

1     - nodes forced to 1

X     - nodes forced to X

.     - nodes forced to anything

B     - nodes forced to binary values

U     - nodes with undetected stuck at fault(s)

U0    - nodes with undetected stuck at 0 fault

U1    - nodes with undetected stuck at 1 fault

G     - nodes with no undetected stuck at fault(s)

G0    - nodes with detected stuck at 0 fault

G1    - nodes with detected stuck at 1 fault

*     - all matching nodes (default)

>n    - all nodes with greater than n transitions

<n    - all nodes with less than n transitions

=n    - all nodes with exactly n transitions

M     - all marked nodes

Examples:

LI *    U    -- list all nodes with undetected faults

LI RES* B    -- list all nodes on the RES bus forced to binary state

MEmory [>memFile] [<memFile]

Save/restore the state of the simulator.  This can generate very large

files and is not very robust--the simulator versions must be identical

AND the simulator must be located at exactly the same place in memory

to reload a file (default extension .MEM).  This command is illegal in

virtual protected mode.

MOn [on|off|?]

Enable/disable display of the results during simulation. If ? is used,

the current monitor state is shown.

NDelay sourceNodes destNodes [minDel] [maxDel] [>[>]filename] [options]

Output all paths from source to destination nodes which have

delay >= minDelay (default=0) and <=maxDel (default=32K-1).

Options:

-Q          : just endpoints of the path

-R          : reverse order

-S          : include setup time in PD

-L n        : only go through n Latches in any one path

-C n        : don't go through more than n DFF CLK pins/Latch G pins

-X nodeSpec : exclude all matching nodes from search

-T          : display as a tree

-M          : exclude marked nodes from path

The node specs have the form   [~][name | #n]   where #n ==> TAB #n.

Note that the node name can have a wild card.  The ~ in a nodespec

implies negation.

NEtfile file

Read in a network file (default extension NET).  See the appendix on

netlist format for a detailed description of the format for each gate.

The network is in a text file and is part oriented.  The netlist format

is very much like SILOS.  Each element has the general form:

node1 .PARTNAME [time]* [node]*

Example:

OUT .NAND 10 12 IN1 IN2 IN3

Each time a comment of the form:  '$ Unn=...' is encountered at the

beginning of the line, the value nnn is saved as an integer for the

instance number, and that will be output along with the gate name whenever

GateSim displays information about that gate (see GAte and NOde commands).

The gate suffix is set to 0 when such a comment is encountered and

is incremented each time a new gate is encountered.  The gate suffix

can be modified by a comment of the form #sss.  The gate name is

output in the form PARTNAME [Unnn.sss].

Memory devices have a slightly different format:

RAMname .SRAM [taccess [thiZ]] CSnode WEnode A0 A1 .. An / D0 .. Dm

ROMname .ROM  [taccess [thiZ]] CSnode        A0 A1 .. An / D0 .. Dm

The name is used for referring to the part in the memory commands.

It is also the name of the Intel HEX file loaded when the part is

intialized for simulation.

PAttfile filename

Use the file as the vector file (default extension VEC).  The vectors

consist of .CLK statements, followed by a .PATT statement, with or without

a fixed delta time.  See the netlist format for a description of the

.CLK statements.   The PATT statement has the format:

.PATT [delta [startTime]] [signalName]*

[ [time|.time] [signalValues] <ret> ]*

If no delta is given, the time must be included on each input line.

However, a '.' means to repeat the last vector input interval, and

a '.n' means to skip n time units from the previous vector.

A statement of the form

IF node =  state THEN SKIP [+|-] n

or   IF node <> state THEN SKIP [+|-] n

or   IF node =  state THEN HALT

or   IF node <> state THEN HALT

or   IF node =  state THEN ECHO string

or   IF node <> state THEN ECHO string

does just what you think.  A skip with n=0 means NOP, while n=-1

implies to repeat the IF, etc.

Also, if the first character of the vector (after the optional time)

is a K, you will be prompted for keyboard vector input, with the

remainder of the line made available for editing.

A signal value of '.' means no change from the previous state

PDelay sourceNodes destNodes [minDel] [maxDel] [>[>]filename] [options]

Identical to the NDelay command except that the only source nodes

considered are the matching nodes which are also outputs of D flip-flops

or .CLK gates, and destination nodes must be the D pins of D flip-flops

or latches.

PUllup [node]*

Force certain nodes to have pullups on them, so that if the node

state goes to Z, it will instead be pulled to 1.

QUit

Quit the simulator

RAndomdelay n

Set the random delay on inputs from the vector file to the range 0..n.

The default is 0.

REsult resultfile

Output simulation results to the resultfile (default extension OUT).

The contents of this file can be used to DIsplay, STore, or VErify

the simulation results.

RF [command [|command]* ]

Resume input from a file (having typed 'P' to halt) after executing

the command(s) specified.  This command is most useful during a simulation

initiated within a command file, since you can Pause the simulation,

run any number of diagnostic commands (including pushing to DOS), then

continue the simulation by typing "RF SIM .".

SCroll [on|off]

Enable scrolling the header display during verify/simulate/display.

Default is OFF, so the header stays on the screen

SFights [on|off|?]

Enable/disable fight reporting (default=ON).  If ? is used, the current

state of showing fights will be displayed.

SImulate  [t0 [TO] t1 [STEP dt]] [-C cmpfile [-W n] [+W n]] [-H] [-G] [-Y]

Simulate over the given time range.  If t0 is less than the previous

ending simulation time, you will be asked if you want to restart, unless

you override by giving the -Y option.

You can also compare against a file of default extension CMP (no output

will be generated), and the simulation will stop (and the Badverify

flag set) if any discrepancies are found.  A '.' in the cmpfile will

always result in a match for the given node.  If the -G option is used,

the simulation will continue even after Badverify is set.  The +W/-W

options give a comparison time window about the times for the lines

in cmpfile.  The default window is [-0,+0].

You can also do "SIM +" to continue a simulation with the same time range

(e.g., sim 0 1000 followed by "SIM +" is the same as sim 1000 2000).

Similarly, a "SIM ." will complete a halted simulation.

The -H option turns off the hiliting of signal changes (for MOnitor ON).

STore [#n] [t0 [TO] t1 [STEP dt]] [>[>]dispfile] [-N] [-I] [-D]

Same as DIsplay, but if no file is given, the result is stored to the

file DISPLAY.OUT.  STore options have the following meaning:

-N  ==> output in NCR format (time on right, suppress repeated

lines due to step by changing signals to 'X')

-I  ==> used in conjunction with -N to keep the NCR format

without suppressing repeated lines

If the parameter #n is used, then the output is reformatted according

to TAB #n (see the .TAB command).

.TAb [ [+] [#n] [node [[state]] |nibble|;]* | ? ]

Select the nodes to be output during simulation or display.  The #n

selects which table is used (TAB 1 to TAB 10); no such parameter implies

TAB 0, which is the default used for simulation/display.  The transitions

on nodes/nibbles in TAB 0 will be saved to a file for later display or

verification.  A semicolon leaves a space in the display.  A '+' means

append the list to the current table.  If a state (0,1,Z,X) is placed

after a node name in brackets, that node will be displayed in a hilited

color whenever it is in that state.

Entering a parameter of ? will force a list of all TAB statements.

VErify [t0 [TO] t1 [STEP dt]] [<verffile] [>[>]diffile] [[+|-]O offset]

Verify RESULTFILE with given time parameters against verffile (default

extension VRF).  Optionally, store (or append) the results to diffile

(default extension DIF).  Sets the badverify flag if a discrepancy

is detected.  If no file is given, the discrepancies are displayed

using various colors to distinguish them.  The +O/-O offset is used

to do a comparison with a time offset.  +O means to add the time

to the current simulation results before comparing, while -O means

to add the time to VERFFILE before comparing.

VIewvector [on|off|?]

Enable/disable the display of vectors as comments.  If a ? is used, the

current state of vector display is shown.  The default state is ON.

WAveform [on|off|?]

Enable/disable the display of signals as a waveform (using character

graphics) instead of characters (0,1,Z,X).   This feature is only

supported on EGA displays.  If a ? is used, the current state of waveforms

is displayed.  The default state is OFF.

WIndow n

Set the number of lines used for comments when performing a horizontal

display or simulation.  The default is 4.  As more lines are used for

comments, obviously fewer lines are available for signal display.

7.  MEMORY COMMANDS:

DUmp memName [startaddr [endaddr]] [>[>]file]

Dump the RAM/ROM of the given name (with optional address range) to a file

or the screen if no file is specified.

VRam memName [startaddr [endaddr]] <file [>[>]difffile]

Verify the contents of memory against a hex file and display the

results to the screen or to a file.  Sets the badVerify flag if

any differences are encountered

LOadmem memName [startaddr [endaddr]] <file

Load the contents of RAM/ROM from an INTEL HEX file.  When ROMs are

read from the netlist file, a file of the name memName.HEX is opened

and the ROM is initialized.

POkemem memName startaddr [value]*

Modify memory with the values

FIllmem memName startaddr endaddr value

Fill a range of memory with a value

APPENDIX A.    NETLIST FORMAT

Comments are marked by a '$' character up to the end of the line.  A

'+' on the beginning of a line means that the line is a continuation

of the previous line.  Each element has its own syntax and should be

placed on its own line(s).  Each input node can optionally be

preceded by a '-' sign to show that the input is inverted before the

gate function is computed.  All delays are parsed as 16 bits integers

and thus must be in range 0..32767.

Inverter:

OUT	.INV  [trise] [tfall] IN

Boolean gates: ( up to 9 inputs)

OUT	.AND  [trise] [tfall] IN1 IN2 ...

OUT	.NAND [trise] [tfall] IN1 IN2 ...

OUT	.OR   [trise] [tfall] IN1 IN2 ...

OUT	.NOR  [trise] [tfall] IN1 IN2 ...

OUT	.XOR  [trise] [tfall] IN1 IN2 ...

OUT	.XNOR [trise] [tfall] IN1 IN2 ...

Tri-State Inverter: (OE is active high)

OUT	.TINV [trise] [tfall] [tZH] [tZL] [tHZ] [tLZ]  OE  IN

D Flip-Flop:

Q	.DFF [tRiseQ]  [tFallQ]  [tsetQ]  [tclrQ]

+            [tRiseQB] [tfallQB] [tsetQB] [tclrQB]

+            [tsetup] [thold] [tMinPulseWidth]

+            QBAR SETBAR CLRBAR D CLK

Latch: (transparent while G is high)

Q	.LATCH [tRiseD] [tFallD] [tRiseG] [tFallG] [tSet] [tClr]

+            [tsetup] [thold] [tMinPulseWidth]  SETBAR CLRBAR D G

And-Or-Invert: (max of 24 terms, 8 and gates)

OUT	.AOI [tRise] [tFall] A0 A1 ... / B0 B1 .. / C0 C1 ...

Or-And-Invert: (max of 24 terms, 8 and gates)

OUT	.OAI [tRise] [tFall] A0 A1 ... / B0 B1 .. / C0 C1 ...

CMOS Transfer Gate:

OUT	.CXFR [tRise] [tFall] EN ENBAR IN

RAM: (maximum of 14 address pins, 8 data pins )

NAME	.SRAM [tAccess] [tHiZ] [tsetupD]

+             CSBAR WEBAR A0 A1 .. An / D0 .. Dn

ROM: (maximum of 14 address pins, 8 data pins )

NAME	.ROM  [tAccess] [tHiZ]

+             CSBAR       A0 A1 .. An / D0 .. Dn

PULLUP:

NODE    .PULLUP

CLK: (repeatable waveform)

NODE    .CLK time state [time state]* [.REP time]

GateSim is a trademark of Tanner Research, Inc.  Other brand and product names

are trademarks or registered trademarks of their respective holders.

Copyright (c) 1988 Tanner Research, Inc.

Tanner Research, Inc.

128 West Del Mar Boulevard

Pasadena, CA  91105

(818) 795-1696