The printer ports on the IBM DB25 connector are as follows

LPT1        LPT2       Pin         Signal              Bit#

632d        888d       2-9         Data 0-7            0-7

633d        889d       15          Error (input)       3

633d        889d       13          Select (input)      4

633d        889d       12          Paper End (input)   5

633d        889d       10          Acknowledge (input) 6

633d        889d       11          Busy (input)        7

634d        890d       1           Strobe              0

634d        890d       14          AutoFeed (out)      1

634d        890d       16          Initialize (out)    2

634d        890d       17          Select In (out)     3

The author has successfully interfaced the MFJ-1224 RTTY/CW interface to the

printer port as follows:

Port 890d  bit 3 set up for key line output

Port 889d  bit 7 set up for receiver input