SuperMorse CW Program Documentation

------------------------------------------------

Copyright 1990 by Power Mountain Systems all rights reserved

P.O. Box 161, Cora, Wyoming 82925

(307) 367-4766

THIS PROGRAM IS WRITTEN FOR THE IBM PC, XT, OR AT COMPUTERS (OR

EQUIVALENT) WITH AT LEAST ONE SERIAL PORT.

This program is SHAREWARE and as such is owned by Power Mountain

Systems.  If you use the program you are required to register it.  Registration

is only $25.00.  If we have upgraded the program you'll receive the latest

version upon registration.

If you have purchased the PMX-200 (or an equivalent product) from Power

Mountain Systems then you are already registered, and have the latest version.

The PMX-200 is a device to interface between your computer and your

short wave receiver.  It allows you to decode very weak CW & RTTY signals.  You

can read more about this project in the April/May 1990 issues of Radio-

Electronics.  Current pricing is available if you write to the above address.

This program may work well on other CW systems but we obviously cannot

know how your equipments' RS-232 port may be wired.  We also cannot guarantee

that the interface you're using is adequate.  Noise immunity is critical for

good CW performance.

CW output to the interface from the computer is sent via pin #20.  CW

data from the interface to the computer is sent via pin #8.  Pin 1 & 7 is wired

to ground.  Voltage levels are RS-232 standard.

OPERATION

=========

The first time you run this program you'll need to run the speed test

option in the first menu.  After that you can just go right into the program

(option 'A') and get started.

The program starts at 25 words per minute and in transmit mode.  When a

key is hit a window will pop up and ask you what port you're using.  After that

you're ready to transmit or receive CW.

Keyboard commands are as follows:

----------------------------------------------------------------

[ = go to receive mode

] = set new speed

{ = set new Com port

} = end session and go to DOS

+ = increment speed by 1 WPM

- = decrement speed by 1 WPM

For large changes in speed you can use ']' and for small changes use

'+' and '-'.  After changing speed use '[' to get back into receive mode.

To start transmission all you do is start typing.  You'll see that your

cursor moves to the transmit window.  You'll hear a beep when your keyboard

buffer is full.  With a little practice you'll get used to how far you can get

ahead.

As each letter is sent it is displayed on the screen.

Good Luck, and have fun!  Please pass this program and this

documentation around in unaltered condition.

--  Larry Ashworth --

Power Mountain Systems

04/10/90