QSTMUF13.BASQSTMUF13.BAS is a frequency propagation forecast program to obtain the MUF(maximum usable frequency) based on the Ottawa 10.7 cm Solar Flux Numberor the Sunspot Number for a particular date over a particular path.  It alsocalculates the initial Great Circle bearing and the distance between thepoints in kilometers, statute miles and nautical miles.QSTMUF13 is an updated and modified version of the QST magazine MUF programwith some color added, additional error trapping, a default home base, adefault target added.  It also gets the Solar Flux number from the computerenvironment, and reads the computer date for a default forecast date ifdesired.  Of course, you can manually enter a Solar Flux number or SunspotNumber, too.  It uses the BASIC LCOPY command to optionally print thepropagation forecast to the printer, will optionally save it in a file, andwill display it on the screen.A number of default reference coordinates are presented on the screen toassist you as a memory jog.  These can be modified by you to show anyreference points you wish.  See lines 310-390 of the program.  These can bechanged in any word processor which saves in true ASCII format, or in BASICwith the EDIT LINE nnn command, saving afterwards.Likewise you are offered a default Transmitter location description in Line440 and its Latitude and Longitude in lines 470 and 480.  These too can bechanged in your word processor or in BASIC to your own location.A default target for a path you frequently use is described in line 630 andits latitude and longitude given in lines 660 and 670.  These too you canchange to suit yourself.Note that all south latitudes and east longitudes are specified with a minussign preceding the numbers.  Latutudes and longitudes are entered in degrees and decimal parts of a degree for simplicity.The program will find the system date and use it if you wish.  Naturally yourcomputer must be set to the correct date when the program is run for this feature to work.The program normally looks in the DOS ENVIRONMENT for the value attached tothe variable SOLARFLUX, and will use it if you wish.  This is because I usea program which contacts NOAA and obtains the 10.7 cm Solar Flux number andsets it in the environment ready for use by several propagation forecastprograms.  (If interested in this program contact me at CIS 74066,635 and Iwill get it to you.)  You can manually set the Solar Flux or Sunspot Numberin the program, and you can set the SOLARFLUX variable by enteringSOLARFLUX=nnn at the DOS prompt, where nnn is the Solar Flux numbered obtainedfrom any reliable source.  (Try WWV at H+18.)  And if you don't know theSolar Flux number it offers a default in lines 1020-1060 which you can alsochange to any desired figure in your word processor or in BASIC.When you choose to copy the forecast to a text file you are asked if thedefault filename should be used, and you can change it as you wishin your word processor or BASIC in lines 3220 and 3230. QSTMUF13.DAT isa typical printout which can be TYPEd to your screen as a sample.Except for the use of LCOPY to print the forecast to the printer, theprogram will compile with no errors in QUICKBASIC 4.5 should you wish to doso, but the resulting file is about twice as big when it becomes an .EXE file.I do not claim to have written the basic forecast routines.  These are fromQST Magazine and the National Bureau of Standards (see credits in the REMstatements at the end).  I have only enhanced the already fine work of theprevious programmers.If you have any questions or difficulties contact me on CompuServe at74066,635.                 -Paul Heim