G8BPQ Host Mode

From Version 4.00, a new application interface to the switch will be provided.

This will replace the existing interfaces (TNC2, PK Host and KISS). New

software should if possible use the new interface - interface routines

supporting the old modes, and WA8DED Host mode, will be provided as TSRs

to support existing application software.

The interface is normally called using INT 7Fh. From version 4.03 the interrupt

number may be changed - The value is specified in BPQCFG.TXT as INTERRUPT=nnn,

where nnn is in decimal. The function required is in AH, and the stream in AL.

The system supports 64 streams, or sessions, numbered from 1 - 64.

;

;

;	COMMANDS SUPPORTED ARE:

;

;	Note that in all cases where a string is returned, ES:DI must point to

;	a buffer supplied by the user, and the switch will copy the data to

;	this buffer. The code does NOT return pointers to its own data.

;	AH = 1	Set application mask to value in DL.

;		Set application flags to value in CL.

;		  If top bit of CL is set, monitored frames will be available.

;		  Use function 11 to receive them.

;

;	AH = 2	Send frame in ES:SI (length CX)

;

;

;	AH = 3	Receive frame into buffer at ES:DI, length of frame returned

;		in CX.  BX returns the number of outstanding frames still to

;		be received (ie. after this one) or zero if no more frames

;		(ie. this is last one).

;

;

;

;	AH = 4	Get stream status.  Returns:

;

;		CX = 0 if stream disconnected or CX = 1 if stream connected

;		DX = 0 if no change of state, or DX = 1 if

;		       the connected/disconnected state has changed.

;

;	AH = 5	ACK stream status change. Must be called to allow another

;		status change to be reported.

;

;

;	AH = 6	Session control.

;

;		CX = 1 connect to node

;		CX = 2 disconnect

;		CX = 3 return user to node

;

;

;	AH = 7	Get buffer counts for stream.  Returns:

;

;		BX = number of frames queued for receive

;		CX = number of un-acked frames to be sent

;		DX = number of buffers left in node

;

;

;	AH = 8	Port control/information.  Called with a stream number

;		in AL returns:

;

;		ES:DI = CALLSIGN (10 bytes space padded)

;

;		The following are not supported by the current version,

;		but I expect to add them soon.

;

;		AX = Radio port on which channel is connected (if level 2)

;		BX = L2 paclen for the session

;		CX = L2 maxframe for the radio port (if level 2 session)

;		DX = L4 window size (if L4 circuit, or zero)

;

;

;	AH = 10	RAW (KISS) transmit frame.  Data pointed to by ES:SI, of

;		length CX, is transmitted as a HDLC frame on the radio

;		port (not stream) in AL.

;

;

;	AH = 11 Get Trace (RAW Data) Frame into ES:DI, Length to CX. Data

;		is as received off air (ie with calls in AX.25 format, etc)

;

;

;	The following additions have been proposed. Similar functions may

;	be provided, but not necessarily in this format.

;

;

;	AH = 0	Get node/switch version number and description.  On return

;		AH = major version number and AL = minor version number,

;		and user's buffer pointed to by ES:SI is set to the text

;		string normally output by the USERS command. CX is set to the

;		length of the text string.

;

;

;	AH = 9	Fetch node/application callsign & alias or application names

;

;		AL = application

;		BL = 0 to get Callsign, 1 to get Application Name

;

;		number:

;

;		0 = node

;		1 = BBS

;		2 = HOST

;		3 = SYSOP etc. etc.

;

;		Returns string with alias & callsign or application name in

;		user's buffer pointed to by ES:SI length CX.  For example:

;

;		"WORCS:G8TIC"  or "TICPMS:G8TIC-10".

;

;

The following sample of code shows how to find the Interrupt Number to use

to call the switch, and check that the switch is loaded.

NOTBPQERROR	DB	'Switch not found$'

VERSERROR	DB	'Version not compatible with Node Software$'

G8BPQ		DB	'G8BPQ'

BADCONFIGMSG	DB	'Configuration file read error',0DH,0AH,'$'

CONFIGFILENAME	DB	'BPQCFG.BIN',0

NOCONFIGMSG	DB	'Configuration file BPQCFG.BIN not found',0DH,0AH,'$'

;

;	BPQCFG FIRST 128 BYTES

;

CONFIGTABLE	LABEL	BYTE

;

;	CONFIGURATION DATA STRUCTURE

;

;	DEFINES LAYOUT OF CONFIG RECORD PRODUCED BY CONFIGURATION PROG

;

;	LAYOUT MUST MATCH THAT IN CONFIG.C SOURCE

;

C_NODECALL	DB	10 DUP (0)	 ; OFFSET = 0

C_NODEALIAS	DB	10 DUP (0)	 ; OFFSET = 10

C_BBSCALL	DB	10 DUP (0)	 ; OFFSET = 20

C_BBSALIAS	DB	10 DUP (0)	 ; OFFSET = 30

;

C_OBSINIT	DW	0		 ; OFFSET = 40

C_OBSMIN	DW	0		 ; OFFSET = 42

C_NODESINTERVAL	DW	0		 ; OFFSET = 44

C_L3TIMETOLIVE	DW	0		 ; OFFSET = 46

C_L4RETRIES	DW	0		 ; OFFSET = 48

C_L4TIMEOUT	DW	0		 ; OFFSET = 50

C_BUFFERS	DW	0		 ; OFFSET = 52

C_PACLEN	DW	0		 ; OFFSET = 54

C_TRANSDELAY	DW	0		 ; OFFSET = 56

C_T3		DW	0		 ; OFFSET = 58

DW	0		 ; OFFSET = 60

DW	0		 ; OFFSET = 62

C_IDLETIME	DW	0		 ; OFFSET = 64

C_EMSFLAG	DB	0		 ; OFFSET = 66

DB	0

C_BBS		DB	0		 ; OFFSET = 68

C_NODE		DB	0		 ; OFFSET = 69

C_HOSTINTERRUPT	DB	0		 ; OFFSET = 70

C_DESQVIEW	DB	0		 ; OFFSET = 71

C_MAXLINKS	DW	0		 ; OFFSET = 72

C_MAXDESTS	DW	0

C_MAXNEIGHBOURS	DW	0

C_MAXCIRCUITS	DW	0		; 78

C_TNCPORTLISTO	DB	16 DUP (0)	 ; OFFSET = 80

C_IDINTERVAL	DW	0		; 96

C_XXXXXXXX	DW	0		; 98    ; SPARE (WAS DIGIFLAG)

C_MINQUAL	DW	0		; 100

C_HIDENODES	DB	0		; 102

C_L4DELAY	DW	0		; 103

C_L4WINDOW	DW	0		; 105

C_BTINTERVAL	DW	0		; 106

X_UNPROTO	DB	9 DUP (0)	; 108	; NOW SPARE

C_BBSQUAL	DW	0		; 117

DB	(CONFIGTABLE+128-$) DUP (0)

CONFIGHANDLE	DW	0

NODE:

INT	7FH

RET

INIT:

;

;	GET NODE INTERRUPT NUMBER FROM CONFIG FILE

;

MOV	DX,OFFSET CONFIGFILENAME

MOV	AH,3DH

MOV	AL,0			; READ ONLY

INT	21H			; OPEN IT

JC	NOCONFIGFILE

MOV	CONFIGHANDLE,AX

MOV	BX,AX

MOV	DX,OFFSET CONFIGTABLE

MOV	CX,128

MOV	AH,3FH

INT	21H			; READ

CMP	AX,CX

JNE	SHORTCONFIG

JMP SHORT PROCESSCONFIG

NOCONFIGFILE:

MOV	DX,OFFSET NOCONFIGMSG

JMP SHORT CONFIGERR

SHORTCONFIG:

MOV	DX,OFFSET BADCONFIGMSG

CONFIGERR:

MOV	AH,9

INT	21H

MOV	AX,4C01H

INT	21H			; EXIT

PROCESSCONFIG:

MOV	BX,CONFIGHANDLE

MOV	AH,3EH

INT	21H			; CLOSE IT

MOV	AL,C_HOSTINTERRUPT	; INTERRUPT

MOV	BYTE PTR NODE+1,AL	; PATCH NODE CALL INSTRUCTION

;

;	GET NODE VERSION

;

PUSH	DS

MOV	AH,0

ADD	AX,AX			; VECTORS ARE 4 BYTES LONG

ADD	AX,AX

MOV	SI,AX

XOR	AX,AX

MOV	DS,AX

LDS	SI,DS:[SI]		; GET POINTER TO HOST SUPPORT CODE

SUB	SI,7

MOV	DI,OFFSET G8BPQ

MOV	CX,5

REP CMPSB			; MAKE SURE SWITCH IS LOADED

JE	SWITCHOK

POP	DS

MOV	DX,OFFSET NOTBPQERROR

MOV	AH,9

INT	21H

JMP SHORT EXIT

SWITCHOK:

CMP	BYTE PTR DS:[SI],MAJORVERSION

JNE	BADVERSION

CMP	BYTE PTR DS:1[SI],MINORVERSION

JE	VERSIONOK

BADVERSION:

POP	DS

MOV	DX,OFFSET VERSERROR

MOV	AH,9

INT	21H

EXIT:

MOV	AX,4C00H

INT	21H

BADPORTVAL:

MOV	AH,9

MOV	DX,OFFSET ERRORMSG

INT	21H

RET

VERSIONOK:

POP	DS

;

..........

The release disk also contains a demonstration terminal program, TERM4,

written by G8IMB (in PASCAL) to demonstate the use of the new interface from

a high level language. Note, however, that it was written for version 4.02,

and therefore only works with the Host Mode interrupt configred at 7Fh

John Wiseman, G8BPQ @ GB7BAD

20/2/91

#########