//=========================================\\

**************||                                           ||*****************

**  **  **  **||  Packet GOLD 1.2 Test Drive Cmd Reference ||** ** ** ** ** **

**************||                                           ||*****************

\\=========================================//

Quick Start:

1.  Be sure you know which serial port is attached to your TNC.

2.  Serial Cable must include hardware handshaking lines (standard

cable, or be sure to wire pins 2,3,4,5,6,7,20).

3.  Turn on the TNC.

4.  If this is the FIRST time you've run the Test Drive version:

Please turn down volume on the radio until program starts.

The program will attempt to "talk" to the TNC without

resetting it. Extra received frames heard during this startup

phase may delay startup.

Note: This is not required after the first time running the

program, and may not be necessary if the channel is fairly quiet.

5.  Type     PKTDEMO [Enter]     to startup the program.

The program will not RESET the tnc unless it has no other way

to establish communication.  Therefore, it MAY take some time

to initially startup. One the TBAUD rate (Computer to TNC rate)

is set, the program turns on HOST mode and commences to load

the parameters found in STARTUP.TNC.  You will see the parameters

being set as the TNC is configured.

If the startup FAILS, try turning off the TNC, and turning it

back on again. Then rerun the program.  You will see the LEDs

change state if the configuration is proceeding.

6.  Set the radio volume and squelch (if appropriate) according to

the instructions in your AEA manual.  The objective is to have

the DCD (Data Carrier Detect) LED (Light Emitting Diode) "track"

the presence of a decodeable signal present at the receiver.

For FM, VHF operation, setup your radio volume with the squelch

OPEN, until the DCD LED is lit (if your TNC has true carrier detect,

you will run with open squelch at all times).  Then turn the

squelch up until the DCD light is extinguished.

The Threshold control on the PK232 is used to adjust the TNC

to follow the presence of signal. Turn it up until the DCD light

is ON, then turn it down until the DCD goes off with no signal

present.

BAUD RATES: To change TBAUD (the baud rate between the TNC and

the computer) use [Alt-S] to enter the setup area, then choose

SETUP from the menu, then COMMUNICATIONS.  Pick your new baud

rate.  It will NOT take effect until you shut down the program

and rerun it.  Note: Changing baud rates RESETS the tnc, but

once a baud rate is selected, it is hardly ever changed. Use

4800, 9600 or if you have a new PK88, 19200. Slower rates are

discouraged.

Once the program is running

Connecting:

1.  Type the callsign of the "target" station on the command entry line.

Don't type "C CALLSIGN" just type the callsign by itself.

2.  Push the connect key, [F7]

3.  When connected, enter the remote station operator's name using

the [Alt-N] key. Now, each time you connect to that station, the

operator's name will appear.

4.  After you have tried a few connects, use the Setup [Alt-S] menu

to add a "quick connect" name and path.  Enter a few of the

names and paths of stations you might call routinely.  Then

leave this submenu, return to the main program, and depress

the connect key [F7], highlight a "quick connect" and hit

the [Enter] key.

Advanced NET/ROM connect:

1.  Type NODE_ALIAS | NEXT_NODE_ALIAS |  etc. etc. | DESTINATION_CALLSIGN

2.  Hit the connect key [F7].  Program will establish a connect to each

NET/ROM type node and continue until final DESTination callsign is

reached.

Note: Some nodes (KAnodes for example) aren't really NET/ROM

type nodes. They will not allow connecting to an ALIAS-ssid as will

NET/ROM.  A true NET/ROM or clone will permit multiple connects to

the node ALIAS using SSID's.  For example, the BGBEAR node (a real

NET/ROM node) will allow you to connect to:

BGBEAR-1, BGBEAR-2, .... BGBEAR-15.

With Packet-GOLD, the SSID is appended automatically, depending on

which "stream" is used to establish the connect. This means you can

setup QUICK CONNECTS to several stations, all using the same starting

NODE ALIAS, and have multiple connects going, all using the same

NET/ROM node.  Think about this feature for a while, it's very

useful.

Packet-GOLD allows you to setup a large list of Quick-Connects,

storing the NET/ROM path, digipeaters, etc.  When you want to connect,

just hit [F7] without ANY text on the command/text entry line.

The program will display a "pick list" of Quick-Connect stations.

Emergency Communications: Using Tactical Callsigns

The Quick-Connects list allows you to "name" the station and path.

For emergency work, set the "name" to the tactical callsign. This

means you use an FCC licensed callsign for the actual communication,

but refer to the Tactical Callsign in your Quick-Connects list.

Disconnecting:

1.  Use [F4] key to display the appropriate session screen.

2.  Hit [Alt-F7] key combination. Use the Normal Disconnect unless

you have a reason to abruptly disconnect with a higher priority

disconnect.

Multiple Connects:

1.  Type in another callsign, and hit [F7], the connect key.

2.  Use the NEXT session key [F4] to switch between connects.

Monitoring while connected:

1.  This is automatic.

2.  You can ZOOM in/out on a connect screen with Alt-Z toggle.

3.  Go to the Network View screen using [F2] key if you wish

to scroll back or see a larger Network View.

4.  Try the Text Search [F9] key. Enter a portion of the text

you might want to find.  Entering "c,p" or "[c,p]" will cause

a search for connect requests. Or enter "CQ" (note, case is

not important) to find CQ.  Entering "[UI]" is sometimes

interesting, as it will seek out all unproto info frames, such

as CQ or ID frames.  To continue a search, finding additional

occurrances of the target string, use Search Again [Alt-F9].

Sending CQ:

1.  Go to the Network View screen.

2.  Go into Unproto mode, using ALT-U

3.  Type your CQ message and hit [Enter] key.

4.  You may wish to alter your UNPROTO path/id. To do this, type

UNPROTO CQ [F10]  or  UNPROTO CQ VIA W1AW [F10].

5.  Note: When you leave Network View screen with Next [F4] or

Previous Sessions [F3] key, you are taken out of Unproto Mode.

To reenter Unproto CQ mode, use [Alt-U]. This feature guards

against text being sent out in error.

Viewing a parameter:

1.  Type the parameter or abbreviated version of parameter on

the command line.

2.  Hit the CMD key, [F10] to read the parameter value from the TNC.

Changing a parameter:

1.  Type the parameter (or abbreviation) and a value on the

command line.

2.  Hit the CMD key, [F10] to store the value in the TNC.

Changing a parameter group:

1.  Type DISPLAY (or just DISP) on the command line.

2.  Hit the CMD key, [F10].

3.  Point at the parameter group you wish to view/change.

4.  Make your changes.

5.  Return to the program with [Alt-X] or [ESC] key.

Changing Program Setup Options:

1.  Use [Alt-S] key (setup) to enter pulldown menu area.

Note: [ESC] or multiple [ESC] keystrokes will return you to the

program.

2.  Explore the menu system.

You can change a number of things in the TEST DRIVE version,

but they will not "stick" or be saved when time runs out.

3.  If you have a Color display, try changing the monitor settings

shown under the Setup/Monitor. You can switch to 43/50 lines

if you have an EGA or VGA monitor.  The color settings can be

changed as well, or switched back to the defaults.

Under the Other parameters option, you will find a number of

other interesting things to change.

There's much more in Packet-GOLD that we can explain in a brief file.

When you purchase the program, a 75-page manual is included, along with

other information. Additionally, you get telephone support for your

questions.

Here is a brief listing of some of the command areas in Packet-GOLD.

Some of the functions may not be available in the "test drive" version.

Subject Area                            Command Format          Comment

--------------------------------------  ----------------------  --------------

Change Mode (PK232 only, no effect on PK88):

Enter Mode name (e.g. baudot)   BAUDOT [F10]            Changes Mode

Use .TNC file                   [F10]                   Pick List

Key Down/Up (Non Packet)        [ESC]                   When text sent

Parameters and Program Options:

View Parameter                  Parm [F10]              Shows Param

Set  Parameter                  Parm Setting [F10]

View/Set Groups of Params       DISPLAY [F10]

Example                   DISPLAY M [F10]         Monitor Parms

Load Parameter File (.TNC)      [F10]                   From Pick List

Setup Options                   [Alt-S]                 Menu

Note: One or more [ESC] key

puts away menu system.

CQ, Connecting and Disconnecting:

CQ : NetView Screen Only        [Alt-U]                 Unproto Mode

Connect & Quick Connect         [F7]                    Pick List

Next Session (if multiple)      [F4]                    Smart Queue

Setup Quick Connect             [Alt-S]                 From Menu

Connected station Name          [Alt-N]                 Fill in

Change Displayed Callsign       [Alt-C]                 Fill in

Station Information Edit        [Alt-E]                 "QSL" card

Disconnect                      [Alt-F7]                Pick List

Previous Session Text           [F3]                    Switch Window

Jump to session                 [Ctrl-Fn]               On stream "n"

Monitoring:

Net View Full screen            [F2]                    Switch Window

Zoom in/out on Connect Scrn     [Alt-Z]                 Zoom

Mheard                          [Alt-F2]                Shows List

Binary Filter On/Off            [Alt-B]                 Toggle

Checking Connect List           [Ctrl]                  Hold down key

Display Monitor Params:         DISPLAY M [F10]

Conferencing:

Conference Mode Window          [Alt-F1]                Need 2 or more

Conference Info                 [Alt-V]                 sessions to do

Previous Sessions Log           [F3]                    a conference.

Network View Screen             [F2]

Disconnect                      [Alt-F7]                After removing

from Confer

Maildrop:

Maildrop                        [Alt-F8]                See AEA Docs

Leave Maildrop                  [B]                     Bye

Files and Text:

Send Brag Tape                  [Alt-F6]                .BRG files

Start/Check File Transfer       [F6]

Find Text                       [F9]                    Fill in blank

Find Next Text Occurrance       [Alt-F9]

Copy & Paste                    [F5]                    Sub menu

Edit Clipboard, File            [F5]                    Sub menu

Memory Functions:

Wipe Window (Erase Screen)      [Alt-W]                 Current view

Check Memory Usage              [Alt-M]                 All views

Printing/Saving to Disk:

Turn On/Off Save Session        [Alt-Y]                 Popup window

Turn On/Off Print/Capt          [Ctrl-PtrSc]            To LPT/FILE

Entry Line Editing:

Erase Text Entry Line           [Alt-X]                 Delete line

Retrieve erased line            [Alt-R]                 Restore

Insert Date                     [Alt-D]

Insert Time                     [Alt-T]

Insert Alt-n text               [Alt-0..9]

Show ASCII special char         [Alt-A]                 Popup Table

Use Alt-S, Setup, Alt-n msgs to edit these              Menu

Calibrate, Reset, from DIrect Terminal:

Enter Direct terminal           [Alt-F3]                For tests

Leave Direct Terminal           [Alt-F3]

While in Direct Terminal (after Alt-F3):

Use RESET to clear TNC then hit * until you get a "cmd:"

prompt. Then Leave Direct Terminal. Not normally required.

Calibrate functions (when in Direct Terminal Mode ONLY):

Entering the Calibrate mode     CALIBRATE [Enter]

Key Down/Up (Toggle)            [K]

Change to Mark/Space tone       [Space]

Send Dual Tone                  [D]

Quit Calibrate Mode             [Q]

Note: Useful to check levels, tones, etc.

See AEA docs for more information.

Execute a DOS command:

ExecDOS [cmdname params]        [F10]

To Format a Disk                Execdos Format A:/S [F10]

Leaving/Quitting Packet-GOLD

Quit to DOS (terminate)         [Alt-F10]

Shell to DOS (temporary)        [Alt-F4]

Return from DOS Shell           Type EXIT

For ordering information, read ORDER.DOC.