SCANNER.DOC

The following is a guide to the hobby of monitoring the

Public Service bands, and some general tips for beginners to

the hobby.

This guide is broken down into 3 parts:

(1) EQUIPMENT

(2) SCANNING TIPS

(3) SECTION 605, ECPA OF 1986, RESPONSIBILTY

EQUIPMENT

---------------------------------------------------------

SCANNER TYPES: tunable, crystal, programable

Tunable: Many types of tunable recievers are available to the

scanner user. most of these have poor image rejection and

were low-end units or mutiband type radios. These were

subject to intermod and are only good for a few very strong

signals and local coverage. Poor choice.

Crystal: Crystal-controlled scanners have been around since

the 60's. These radios very from 1 channel to as many as 16

and are very good performers. They allow a set, 1 frequency-

1 channel capability, and have good image rejection and

filtering. These radios allow the first time user to sample

frequencies at relativly low cost per channel, about 5

dollars per crystal.

Crystal controlled recievers are also good for professional

users as the cost is low and most Public Service users only

need to monitor a few set interim channels.

Programable: These are by far the best for the serious

scanner hobbiest. Programable recievers have good S/N

figures, good image rejection, and for the most part very

little intermodulation. They also allow greater flexability

by allowing the user to change channels without spending

money for crystals. Most all of the programable radios have

LED or LCD readouts that give frequency, channel number, and

other user information. All have memory backup to retain the

contents of what has been programed. Programable recievers

are by far the best cost per dollar units.

These radios use to cost 300 to 700 dollars a few years ago

but with computer technology, low IC cost , they now can be

had for a little over 100.00. The main compromise in price

these days for a programable scanner, is channels, frequency

coverage, and special features.

A minimum setup would be 20 channels, and the 118-136 Mhz

aircraft band. 150.00 for this setup.

* Scanner Tips *

Now that the radio is sitting there, how  do you know what

frequencies to enter ? Most manuals that come with scanners

have a spectrum breakdown to help new users find a ballpark

area of interest.

Frequency Spectrum Breakdown   30Mhz  to  512Mhz

30 to 50 low-band VHF

military,police,fire,business, new cordless phones 46-49mhz

mostly older systems in rural areas,mobile phone(not used in

larger areas.paging systems used extensivly.Subject to long

range skip conditions and solar activity.

50 to 54Mhz Amatuer 6 meter band

54 to 72mhz TV Ch. 2-6

72 to 88Mhz low power industrial

88 to 108Mhz FM Broadcast Band

108 to 136 (AM) Aircraft band,Tower frequencies start at

118.000

136 to 144Mhz land mobile (Military)

144 to 148Mhz Amatuer 2 meter band

148 to 150.600 Mhz military

150.6 to 152Mhz Business

152 to 153Mhz RCC mobile phones and taxi cabs

153 to 154Mhz Public Transportation

154 to 155Mhz fire depts.

155 to 156Mhz State police

156 to 157Mhz Marine Band,ship to coast,ship to ship,Coast

Guard,

Bridges,toll Booths,Locks,inter-coastal waterways.

157 to 158Mhz Marine operations,Coast

Guard.156.800(Distress,Calling)

158 to 160Mhz State Police

160 to 162Mhz Marine phone,various users

162 to 174Mhz Govt. including all agencies

174 to 220Mhz TV Ch.7-14

220 to 225Mhz Amatuer radio

225 to 400Mhz UHF Aircraft (Military),Satelittes,Experimental

Work

400 to 406Mhz Wx Satelittes

406 to 420Mhz Govt land mobile 430 to 440Mhz Fixed

440 to 450Mhz Amatuer 70cm Band

450 to 453Mhz Business, Power co.'s

453 to 454Mhz Local Police Depts.

454 to 455Mhz RCC Mobile Phone

455 to 456Mhz TV News Teams,Rebroadcasts

456 to 458Mhz Public Transport,Business,etc.

458 to 460Mhz Local Police Depts.

460 to 460.5Mhz Local Police Depts.

460.5 to 461Mhz Local Fire Depts.

462 to 463Mhz Business,GMR Service,REACT Teams

463 to 464Mhz Fire Dept. (Med Ch's)

464 to 470Mhz Public Safety,Industrial,Land Transportation

470 to 512Mhz Broadcasting,Shared With Local Public Safety

and other users.

As you can see there is a lot of ground to cover or

monitor.

Antennas: Most of the antennas that come with these

radios are sufficient for local area coverage up to 75 mile

radius from the listening post. If you can put up and

external antenna, your range can be up to 200 miles or more

depending on the antenna type.

External antennas:

Omni:  omnidirectional, 360 degree radius

signal pickup.

Yagi:  directional, high gain

beam:  high gain, highly directional

discone: high gain, wide bandwith, omnidirectional

Your choice of external antennas depends on what

services you are looking to hear most. Beam antennas are good

for long range, single frequency coverage.

Omni's pull in many signals, where discone and yagi's

tend to have higher gain and wider bandwiths. A good

compromise to all these antennas is the Discone, with wide

coverage and good signal puling power.

Once you have your radio and antenna set up, keep in mind

these tips: keep your squelch control at a low theshhold

level, you will hear more. Weak stations can sometimes be

improved by re-orienting your antenna. Use NOAA Wx. stations

for your area, when aligning your antenna or testing. These

stations are continuous broadcast and can gauge

reciever/antenna performance.

Strong carriers can either be real signals or harmonics

caused by other equipment, computers, printers, clocks, TV's,

etc. Reorienting the scanner antenna sometimes will keep

these signal from locking up the scanner. If the area you are

in is really bad, you can buy filters that lock out these

adjacent signals. Some phantom signals are caused by the

radio itself. These are called "intermod" and "birdies",

Intermod is caused by two internally generated signals

causing a third order harmonic that locks up the scanner.

"Birdies are caused by clock and timing signals in

microprocessor controlled radio's, most of these occurr at

only a few frequencies over the entire range of the radio and

can not be locked out. Most makers of scanners have installed

filters to keep this from happening.

SEARCH SCANNING:

Searching for new frequencies with a programable scanner

is accomplished by entering "upper and lower" frequency

limits. A word of note, only enter a 1 megahertz search area.

The reason for this is, the smaller the search area the more

likelyhood of finding new frequencies .I.E  155-156 mhz.

Another tip is to put the delay function on during

search mode. This allows you to note the frequency and also

allows for a reply. When monitoring a transmission, take

notes as to ID codes and or FCC call letters. Most public

service users are required to ID their station at least once

every 5 minutes.

OTHER TIPS:

Inclement weather, storms, major events etc. have a tendency

to increase radio traffic. During storms, monitor local road

crews, rescue services, and other public service bands. The

local officials generally are the people " out in the thick

of it", and you can gleen much info on area road conditions

or detours etc. from this monitoring.

MOBILE INSTALLATIONS:

Check first your local laws covering radios capable of

monitoring police frequencies in a vehicle, before installing

a scanner in your car. The city of Philadephia at one time

had a law making it illegal and a 1st offense punishable by

a 500.00 dollar fine and confiscation of the equipment.

Most radios today have a 12 volt DC negative ground

option or jack for modile use. Once a suitable location and

power requirements are met, all that is left is mounting the

unit and running the antenna. These instructions are

generally included in the owners manual.

**** SECTION 605, ECPA, AND RESPONSIBILITY ****

As a hobbiest, it is important to observe a few rules

pertaining to the scanner hobby. First use of information

overheard via a scanner for personal gain, criminal purposes,

or interference  with public safety, is not only illegal but

strongly enforced. Chasing ambulances, fire trucks etc. will

cause you to quickly loose your licence and may get you

arrested. Other responsibilty falls into one important

category: Restraint. If you happen to stumble across a set of

frequencies used by local or federal law enforcement

officials, keep in mind that disclosure of any information

you may hear could interfere with a drug stake-out, hostage

situation, or other life threatening senario.

There are situations where news media people, in their

zeal to cover a story have caused the deaths of hostages

after showing up on a scene after overhearing the operation

on their scanners. Police, Fire and Goverment officials have

jobs to do, and don't need interference from the public. Keep

what you hear to yourself.

The Electronic Communications Privacy Act of 1986

This bill which is currently pass the senate and in the house

of representitives, is a bill designed by the cellular lobby

to restrict what you can and cannot listen to. In it's

present form it makes it illegal to "willfully" monitor any

two-way communication primarily seen as closed to the public.

This includes mobile phone, remote broadcast pickup, and

certain other services, this law is seen by the FCC and the

Justice Dept. as being unenforcable, but as with any bad law

it will no doubt have to be tested in court before it is

reformed or rewritten.

Footnote: Scanning can be and enjoyable hobby and give the

listener a better insight into where his/her tax dollars are

being spent. Public servants are there for helping the people

and enforcing the laws. The United States is the only country

in the world where monitoring is allowed, It is a privilage

that few other countries show their citizens. Be responsible

and enjoy!################################################################