MAPPER2 .. A DX MAP DRAWING and Propagation Program

for IBM PC Compatibles with graphics capability.

Version 2.0

by

Dennis P. Murray

11165 McDonald Street

Culver City, Ca. 90230

CompuServ 72770,2765

20 October, 1987

This program, written for BASIC compilers such as MICROSOFT QUICK

BASIC or BORLAND TURBO BASIC, displays a map of the world centered on

a user-selectable QTH, and does propagation predictions based upon a

modified MINIMUF method. It shows the regions of sunlight and darkness

around the world, and updates these regions automatically at user-

specified time intervals. It can also accept arbitrary times and dates

at the user option. The program accepts DXCC prefixes or

latitude/longitude values for the DX location, draws the path from the

home QTH to that location, and displays the range, azimuth and wave

launch elevation angle from the home QTH. The user can select either

short or long paths; the sunspot number; antenna height, gain, and

polarization for both home QTH and DX station; transmit power; and the

minimum wave launch angle to be used for propagation prediction. The

propagation prediction gives the MUF (maximum usable frequency), LUF

(lowest usable frequency) based upon E-layer screening, and the

minimum number of F-layer hops for the  minimum launch angle.

Predictions of signal strength are made for Ham Band frequencies which

include D-layer absorption, reflection losses, and transmit and

receive antenna losses due to ground plane reflection at low elevation

angles.

This program is  written for IBM PC compatible computers with an

EGA Graphics capability and color display. The source code provided

could be modified by the user to use other graphics displays supported

by your BASIC compiler. The program runs quite quickly on an AT clone

with a hard disk and 640k of RAM, but it might be a 'dog' on a floppy-

only machine because it must read some long files for the map data.

The author does not intend to support this software, but is

simply releasing it into the public domain for non-commercial use by

HAMS who know what they are doing with their computer. The BASIC

source code is included for that reason.

The author encourages the distribution of this program to anyone

who might find it useful, or onto bulletin boards which deal with HAM

radio or shortwave listening. Please make sure to include ALL the

required files in any uploading however. These files are:

1- MAPPER2.BAS      The Program Source Code

2- MAPPER2.EXE      the Q-BASIC v3.0 executable code

3- MAPPER.ATL       An Atlas of DXCC countries and their

latitudes, longitudes, and DX prefix.

This is an ASCII file ,sorted by prefix,

which can be added onto by the user with

a word processor or text editor.

4- MAPPER.DEF       A short file used to store some default

values used by the program.

5- MAPPER.SCR       A large file which is used to store and

retrieve the screen data for a world map

centered on the user's location. This

file is rewritten whenever the user

changes the 'Home' location. ( It is

interesting to view the world as seen

from DX locations.) This data is read

into a large array in memory to permit

rapid redrawing of the map as the sun

moves. The file is written into the

integer array NSTORE by BLOAD and BSAVE

commands. Because the program uses a lot

of memory to speed things up, this array

is DYNAMIC and does not generally reside

in the normal BASIC data segment. The

version of the program supplied for

Q-BASIC uses the CALL PTR86 subroutine

supplied by MICROSOFT, and this module

must be specified during the linking

process. In TURBO-BASIC the VARSEG

command can be used to find the array

segment used in lines 3120 and 12140.

6- MAPPER2.DOC      This instruction file.

7- WORLDMAP.DAT     A large ASCII file containing the

lat/lon of the world's borders. This is

used to recompute the map whenever the

map center point is redefined.

INSTRUCTIONS:

To run this program simply enter 'MAPPER2'. After an initial

message explaining the program is displayed, and you press any key to

proceed, you will be presented with a 7-choice initial menu which

allows you to change the default values of:

1- Sunspot number

2- Home QTH latitude and longitude

3- The time interval at which the daylight/darkness areas are

redrawn.

4- Home antenna height, gain, and polarization.

5- DX station antenna height, gain, and polarization.

6- Transmitter power output.

7- Minimum elevation angle for propagation predictions.

Pressing the appropriate number key will put up prompts to allow

changes to these variables, and will return you to this menu after a

change is entered. Pressing <ENTER> at the menu prompt will store

these new values in the MAPPER.DEF file and proceed on to the map

drawing.

The map drawing will first put up a blank box, then calculate the

position of the sun at the time and date currently held by the DOS,

draw the solar terminator ( it looks like a satellite ground trace ),

and color the sun-lit portions of the world in red. It will then go on

to draw the country boundaries, color the oceans blue, and color the

USA green. The land areas which are in darkness will show up in black.

The program may miss coloring part of the oceans blue because of the

position of the solar terminator blocking the PAINT commands used in

the subroutine at line 8000 ... . I got tired of adding ocean points

to this routine to cover all the contingencies.

After the map is drawn a Menu bar will appear on the bottom line of

the screen, with 9 choices selected by the function keys:

f1-   Select DX Prefix.

You will be prompted to enter a DX prefix ( eg. ZS for

S. Africa ). The program will then search the DX atlas

for an occurrence of this prefix. It will display all

of the entries ( ie ZS1,ZS2,... ) that contain this

string and ask you to select one. If the number of

candidate entries is large you can simply <ENTER> to

continue the search.

When a selection is made the program will fetch

the location of the prefix, make a propagation

prediction for the currently selected date and time,

draw the path from your QTH to that location, and

display sundry useful information at the top of the

screen. On the left of the screen the signal strength

predictions will be shown for all Ham bands for which

the frequency is more than 80% of the lowest usable

frequency (LUF), less than 120% of the maximum usable

frequency (MUF), and whose predicted signal strength

is higher than -20dB relative to 0.5 microvolts into a

50 ohm receiver input.

For each band which meets these criteria I display the

frequency, the absorption losses (Labs), the ground

reflection losses (Lref), the transmit and receive

antenna losses due to ground reflection ( Ltx, Lrx ),

and signal strength ( Prcv ). The losses are given in

dB. The signal strength is given in dB relative to 0.5

microvolts into 50 ohms. This is about an S-1 signal

level for most Ham receivers, and S-9 usually

corresponds to 50 microvolts at the antenna terminals.

Thus a signal strength of 40 dB would correspond to a

S-9 prediction.

The user should be cautioned that the signal strength

predictions are very uncertain. I only calculate paths

via the F2 layer, and there a great number of other

propagation modes possible which may be better or

worse than the pure F2 mode. Don't blame the author if

this program says that you can't possibly work

Australia when you are hearing VK's with S-9 signals.

These are only crude indications of the likelihood of

a path being open.

f2-Specify Country Name.

Same as option 1 except that you are prompted for the

name of a country ( or a subset of that name such as

'AUS', which will find both Austria and Australia ).

The search process is insensitive to the case of your

entry.

f3-Specify Lat/Lon.

Same as the above option except you will be prompted

to enter the latitude and longitude of the desired DX

location.

f4-Select Date/Time.

Prompts for a new date and time to be used to

calculate solar terminator and make propagation

predictions. <ENTER> responses to either of these

prompts will fetch the current DOS value of date or

time. This option will also turn off the automatic

updating of the terminator and map redraw.

f5-Use Real Time.

This option will reset the above option to the real

(DOS) date and time and enable the auto update

feature.

f6-Select Short Path.

This toggles the path from the long path option back

to short path.

f7-Select Long Path.

This option toggles long path calculations.

f8-Change Parameters.

This returns you to the 7 choice initial menu to

change the various default parameters such as sunspot

number, antenna heights, power, etc.

f9-Quit.

Exits the program and returns you to DOS.

The program is entirely menu-driven so you don't have to remember

any special commands to use it.

Good luck!!

Some Technical Information:

The propagation is based upon the MINIMUF method, but with some

modifications of the author.

1-Rather than use the standard MINIMUF method of only looking at

the first and last ionospheric bounce points ( the two control

point method ), I calculate the propagation at every hop and

select the worst-case condition out of all of them. This results

in generally much more pessimistic MUF and LUF estimates than

MINIMUF.

2- I have included the E-layer screening frequency and lowest-

usable frequency ( LUF ) estimation technique given in chapter 4

of 'Amateur Radio Software ', by John Morris GM4ANB , RSGB

Books, Lambda House, Cranborne Road, Potters Bar, Hertfordshire

EN6 3JW. The subroutine starting at line 45000 contains this

calculation.

3-I have used what apparently is the HAM convention for longitudes

being positive-West for the input and output in this program, in

the MIMIMUF subroutine, and in the DX Atlas file MAPPER.ATL.

However, in the internals of the program I immediately reverse

the sign of the longitude. Being an engineer by training I think

in terms of right-handed coordinate systems! Therefore those who

want to modify the program should be wary of the sign of the

longitude being used.

4-Note that I have changed the names of the variables in the

MINIMUF subroutine from the standard values to names which are

descriptive of what the variable actually is.

5-I have augmented the propagation predictions with a signal

strength prediction. For each ionospheric control (bounce) point

an estimate of the D-layer absorption is made according to the

method in chapter 5 of Davies, "Ionospheric Radio Propagation",

NBS monograph 80, 1965. The total D-region absorption is summed

up for all control points. The subroutine at line 46000 contains

this calculation.

6-In addition, a calculation of the loss due to ground reflection

is made for each ground reflection point along the path. At each

of these points the map on the screen is checked to see if that

point is colored blue. If so, it is assumed that the bounce

point is at sea, and the reflection coefficient for sea water is

calculated ( dielectric constant of 80, conductivity of 4 mho/m

). If the reflection point is over land then a dielectric

constant of 4 and conductivity of 0.01 mho/m are used. The

Fresnel reflection coefficients for both horizontal an vertical

polarization are power-averaged to account for the random

polarization of the wave due to Faraday rotation at HF

frequencies. I have also modified the reflection coefficients to

account for surface roughness. A RMS surface roughness of 4m is

used on the sea, and 15m for land points. The subroutine at line

35000 makes the reflection calculations.

7-To account for the fact that most Hams have fairly low antenna

heights, thus suffer large losses at low elevation angles, I

explicitly calculate the multipath cancellation losses for each

antenna in the subroutine at line 36000. This is the standard

Fresnel reflection calculation without surface roughness

included. Most urban locations will be worse than this

calculation suggests because buildings in the foreground will

actually shadow the low angle line-of-sight and modify the

reflection area. If you know you have poor line-of-sight you

should modify the minimum launch angle setting at the initial

menu appropriately.