RATS Open Systems Environment (ROSE)

The Radio Amateur Telecommunications Society

206 North Vivyen Street

Bergenfield, New Jersey  07621

United States of America

01-201-387-8896

Table of Contents                                 Table of Contents

Forward - RATS Open System Environment                     i

Introduction to the ROSE X.25 Packet Switch                1

System Overview and Features                               1

What ROSE Provides                                         2

Disconnecting from a Station                               3

Information Bulletin                                       3

Local Digipeating                                          3

Networking with ROSE                                       4

Local switching                                            4

Multi-switch networking                                    4

Entry and Exit Digipeaters                                 5

Monitoring Transmissions                                   5

How to determine where a connection originated             6

ROSE X.25 Packet Switch Messages                           7

Tips and Tricks                                            7

Appendix A - Disconnect and Reset Codes Used               8

Appendix B - CCITT Data Country Codes                     10

RATS Membership Application                               15

Forward - RATS Open System Environment          Forward - RATS Open System Environment

The  Radio  Amateur  Telecommunications  Society  (RATS)  is

dedicated to  the improvement  of communications  systems in  the

Amateur Radio  Service.    This  objective  has  been  guided  by

individuals who are willing to develop software, operate, and use

systems which  push the  current state  of the  art.   Our packet

switch, the  ROSE X.25  Packet Switch, and communications server,

ROSErver/PRMBS, have from their inception been ambitious projects

providing  increased  functionality  to  the  users  and  network

management.     These   systems   were   developed   to   support

communications using  conventional packet  radio equipment.   Any

AX.25 TNC  user can  access  a  network  of  ROSE  switches,  and

likewise any  W0RLI-compatible packet  bulletin board  system can

exchange mail with ROSErver/PRMBS.

The objective  was not  another home-grown  packet switch or

BBS, but  to  add  features  needed  by  the  users  and  network

management while  also  facilitating  interoperability  with  (or

through) other  networks.   The vehicle for this interoperability

was the  then emerging  Open  Systems  Interconnection  Reference

Model  (OSI-RM)  developed  jointly  by  ISO  (the  International

Organization   for    Standardization)   and   the   CCITT   (the

International Consultive  Committee on  Telephone and Telegraph).

Adherence to  the model has provided a modular framework in which

protocols could  be tested,  used, and  replaced as new solutions

(software and hardware) became available.

We chose  to base  our  systems  on  OSI  because  it  is  a

blueprint  for   communications   not   bound   to   the   design

methodologies or  marketing objectives  of private companies like

IBM or DEC, or of governmental agencies such as the U.S.  Dept of

Defense. Instead,  these protocols have been developed and agreed

upon by  both user  groups and telecommunications carriers.  This

blueprint defines  the various aspects of communications in terms

of a  seven-level stack.   For  example, the  switch provides all

required network services needed to interconnect remote users.

These include:

Connection establishment - creating a data path through a network

Data transfer - transfer of data between users will be free of:

Most bit-errors - the kind that turn an "A" into a "B";

Sequencing errors - the kind that change the order of the data;

Undetected packet loss - dropped or missing packets;

Undetected packet duplication - unrequested retransmission;

Connection clearing - the orderly termination of communications.

i

The OSI reference model is the blueprint that was applied to

facilitate the  evolution of  the ROSErver/PRMBS Message Handling

System.   This system  began its development as a packet bulletin

board system  (or PBBS),  but has  outgrown this  label by adding

interoperability support  for CCITT X.400 Message Handling System

and DoD  Internet RFC822  message headers,  providing for  remote

file and  database requests, and remote execution of applications

for a  user.   The  system  is  progressing  toward  support  for

Directory  Services,   CCITT  X.500  and  Management  Information

Services, ISO 9596.

The progress  of OSI-based development has been fraught with

difficulties, including  apathy, "Why Change?"; limited resources

of developers;  the collection  of dialogues that became known as

the "protocol  wars".  Many of these problems occurred because we

recognized the  impact of  OSI very  early and as such were faced

with no  base of  software or  expertise from  which to build and

many of  the required  standards were  not yet  defined, or  were

defined poorly.  These difficulties have been overcome, since the

momentum of  the interest  in OSI protocols to support multimedia

electronic mail  (X.400), directory  services (X.500),  and other

applications in  the marketplace  today has  helped to  fuel  our

efforts now  that a  larger community  exists for the exchange of

ideas and problem resolutions.

In any  communications environment there are always real and

artificial  boundaries  where  special  handling  is  needed  for

communications to  occur.    In  amateur  radio  we  have  local,

regional and  area nets  for traffic  handling, while  these  are

geographically based  boundaries they  are  artificial,  since  a

moderately equipped HF station can easily cross those boundaries.

In the  commercial land-line  based communications  systems these

boundaries also  occur, and  in fact  are encouraged  in order to

facilitate management  of the  equipment involved such as modems,

telephone lines, microwave stations, etc.  The term Domain is one

term that  is used to describe a large collection of systems that

interoperate in a cooperative manner.

A  domain   name  or  identifier  is  assigned  to  specific

collection of communications systems to identify the political or

management group responsible for proper operation of the systems.

In order  to keep the size of the list of Domain Identifiers to a

minimum the  identifiers are  based upon  a tree-like  structure,

"njit-eies.mailnet.edu" is  an example  of a  system  name  where

"edu"  is   the  domain   name  for   the  educational/university

communications networks  and "mailnet"  is a  domain  within  the

"edu" domain,  or a  sub-domain.   There can  be many  levels  of

domains.   The management  group responsible  for the  top  level

domain can add sub-domains as needed without having to notify the

groups managing  the other  top level  or global  domains.   This

allows flexibility  that is  especially important  to dynamic and

fast growing  networks, such  as networks  found in  the  Amateur

Radio Service.

ii

In order  to fully  integrate the  worldwide  Amateur  Radio

Service into  the global  OSI community we needed a unique domain

identifier for  OSI-based Amateur Radio systems.  This identifier

had to  account for  national identity  in order  to provide  the

basis for  recognition by  the regulatory  bodies in each nation.

This objective  had one  serious logical caveat:  we did not want

to request  a piece  of the  global domain  name space  from each

country with an Amateur Radio activity.  To do so would have been

a nightmare  of paperwork  and expense.   What  was needed  was a

global Domain  Identifier for the Amateur Radio Service.  ISO and

CCITT recognized  needs of  certain activities  and organizations

such as  Amateur Radio  when they  devised the global domain name

scheme. Under  ISO is a place for "Identified Organizations" (ISO

6523). Since  the Amateur Radio Service is recognized as a global

service by  the International Telecommunications Union (ITU), the

International Consultive  Committee  for  Radio  (CCIR)  and  the

International Amateur  Radio Union (IARU),  we approached ISO for

a  global   domain  assignment.     After   some  discussion  the

International Code Designator (ICD) identifying the Amateur Radio

Service was  issued.   With  the  Amateur  Radio  ICD,  OSI-based

Amateur Radio  systems will  be known  by, and  accepted by, non-

Amateur systems operating throughout the world.

RATS will  continue the  development of user applications to

support and  expand the needs of the Amateur Radio community.  We

will continue  to work  with  other  individuals  and  groups  to

cooperatively develop new and innovative applications and support

systems.

iii

RATS Open Systems Environment (ROSE)

ROSE X.25 Packet Switch

User Manual

The Radio Amateur Telecommunications Society

206 North Vivyen Street

Bergenfield, New Jersey  07621

United States of America

01-201-387-8896

Introduction to the ROSE X.25 Packet Switch          Introduction to the ROSE X.25 Packet Switch

The ROSE  X.25 Packet  Switch is a connection-oriented, Open

Systems Interconnection packet switch which conforms to the CCITT

Recommendation X.25  and provides  the user  with a  functionally

rich network  interface.   The user  interface to  the ROSE  X.25

Packet Switch  has been  designed with  the average user in mind.

Current users  who are familiar with networking using digipeaters

(C CALLSIGN VIA DIGI, DIGI) will find that we have continued this

basic concept in the ROSE X.25 Switch user interface.

The network will accept data from you and will notify you if

there is  a possibility  that data has been lost.  The network is

100% reliable unless you are otherwise notified.

Users no  longer need  to know each step through the network

to get  to the  desired destination.  The network will handle all

routing of  connections as defined by the routing tables that the

network manager has set up.

The only two things you need to know to make calls using the

ROSE X.25  Packet Switch are the call sign of the switch local to

you, and  the network address (Area Code/Exchange in USA), of the

point you  want to exit the network.  It's like knowing where the

telephone  is   in  your  house  and  knowing  the  phone  number

(Telephone Network Address) of the person you are calling.

Future  applications  will  provide  directory  information,

similar to  555-1212, and  other  applications  that  the  system

manager   may   choose   to   upload,   such   as   Clusters   or

Conferencing/Round-Tables.

System Overview and Features          System Overview and Features

Written in  the C  language by  Thomas A. Moulton, W2VY, the

ROSE X.25  Packet Switch  is based  on  the  popular  AX.25  Link

protocol and  the CCITT  X.25 Packet  protocol.  The use of the C

language  allows   rapid  "porting"  of  the  software  to  other

hardware.

ROSE X.25 Packet Switch offers the following features:

*  Support for AX.25 Level 2 Users - Any standard TNC.               *  Support for AX.25 Level 2 Users

*  Support for X.25 Level 3 Users - BBS can directly               *  Support for X.25 Level 3 Users

interface with network; allows more efficient support

for multiple simultaneous users on one BBS.

*  Enhanced Digipeater Support - Higher throughput due to               *  Enhanced Digipeater Support

fewer retries through one switch.

*  Faster Response Time - Switch is will retransmit               *  Faster Response Time

information as needed using Hop-by-Hop Acknowledgements

providing higher throughput.

1

*  Online Information - Information/Help bulletin.               *  Online Information

*  FCC and foreign Government (PTT) acceptable AX.25 Level 2               *  FCC and foreign Government (PTT) acceptable AX.25 Level 2

SOURCE and DESTINATION Identification - the call signs                    SOURCE and DESTINATION Identification

of both the station of origination and termination

appear at each end of the connection.

*  Station Identification integrity maintained - Call signs               *  Station Identification integrity maintained

traverse the network without ANY changes.

*  Proper Transmitter Licensee Identification - Switch               *  Proper Transmitter Licensee Identification

always identifies its transmissions with its own call

sign, not the call sign of ANY user.

*  Simple Addressing - Only need to know the address of the               *  Simple Addressing

desired exit point of the network, not all the

intermediate steps, true Implicit Addressing.

*  State of the art addressing - Addressing is based on the               *  State of the art addressing

universally accepted telecommunications numbering plan.

*  Network Determined Routing - Network manager determines               *  Network Determined Routing

best path, eliminating need for broadcasting of routing

information to other switches.

*  Dynamic Route Selection - Network will automatically               *  Dynamic Route Selection

attempt alternative paths to remote switches.

*  Predetermined Network Paths - Network manager tells each               *  Predetermined Network Paths

switch which paths to use, will not attempt impossible

links because another switch was heard during a band

opening.

*  Easily Extendible Networking Plan - no need to re-learn               *  Easily Extendible Networking Plan

how to connect to another station because of a new

switch being added.

*  Support for Emergency Operations - A switch can be added               *  Support for Emergency Operations

to the network to provide service from the afflicted

area without modifications to the existing network.

*  Security System for Remote Control - authentication of               *  Security System for Remote Control

user who requests to view or modify configuration.

What ROSE Provides          What ROSE Provides

The  ROSE   X.25  Packet  Switch  allows  users  to  connect

transparently through  the Amateur  Packet Network  to  a  remote

station without the worry of setting up connections on a step-by-

step basis.   All a user needs to do is connect through the local

switch specifying the remote station's network address.

2

Before  ROSE,  you  had  to  know  the  call  sign  of  each

digipeater/node in the path to reach the station.

To connect  to a  station with a ROSE X.25 Packet Switch you

only need  to know  the call  sign of  your local  switch and the

network address  (telephone area code and exchange) of the switch

local to the station.

For example, to connect to W2VY from anywhere in the USA:

C W2VY via (call sign of Your local switch), 201478

It's as easy as dialing a telephone.

Disconnecting from a Station          Disconnecting from a Station          ____________________________

Before we  get into  the various  ways you can issue connect

requests through  a ROSE  X.25 Network, it is always good to know

how to  get out!  You just disconnect like you normally would, if

you are using a BBS send the "BYE" command, if you are talking to

another person  hit Control-C  and type "D" at the "cmd:" prompt.

Don't worry  about doing  something wrong,  it won't  bother  the

switch, if  you find  something that  does then I've got a bug to

fix!  Please tell me!

Information Bulletin          Information Bulletin          ____________________

The switch contains a configurable information message which

can be used for network information, meeting notices and any text

that is  desired by the network manager.  To get this message you

just need  to connect  to the  switch and  enter return.  If your

local switch  is N2DSY-3  you would just type: "C N2DSY-3".  When

you get  the ***  CONNECTED message  hit return.   You  will then

receive a  line indicating  the version  (release  date)  of  the

switch and  the text that was loaded by the manager.  The version

is important  to know when reporting any bugs.  When all the text

has been sent, the switch will automatically disconnect.

Local Digipeating          Local Digipeating          _________________

This mode  of operation  is straightforward  and provides  a

familiar mode  of operation to continue WITHIN the local network.

The ROSE  X.25 Packet  Switch will only digipeat frames with just

ONE call  sign (its  own) in  the "via" field of the AX.25 frame.

The digipeat  call sign  is usually  the call  sign of the switch

with a  "-2" suffix,  and the  switch call  sign is generally the

station call  sign with a "-3" suffix.  In any case the suffix of

the digipeater will be one less than the suffix of the switch.

For the N2DSY-3 switch the following will work:

"C N2FWI V N2DSY-2"

3

But the following will be ignored:

"C N2FWI V N2DSY-2,KD6TH-5

Because of the extra digipeater field, you may need to use

local switching and an exit digipeater, see below.

Networking with ROSE          Networking with ROSE          ____________________

There is only one new concept for users to learn in order to

use the  advanced networking capabilities of the ROSE X.25 Packet

Switch. Each  switch has  a unique,  local, six-digit  "address."

This address  is the  telephone area code and the exchange of the

location the switch is serving.  This address is used to indicate

to the  network where the station you want to communicate with is

located.

Local switching          Local switching          _______________

Instead of  digipeating you  may want  to use  the  advanced

functionality of  the  switch  to  reduce  channel  overhead  and

increase the  overall throughput.   If  you want  to  connect  to

another station  that uses  the  same  switch  (N2DSY-3,  address

201744 in this case) that you use, you can do this as shown;

For example: "C N2FWI Via N2DSY-3,201744"

(where you want to connect to N2FWI)

In the  preceding example you specified N2DSY-3 because that

was where  you entered  the ROSE  X.25 Network, and you specified

201744 because  that was  where you wanted to leave the ROSE X.25

Network.   In this  case both  the entry and exit points were the

same location, like a digipeater.

This initially  looks like  a two hop digipeater connection,

but in  reality the ROSE X.25 Packet Switch gets into the picture

and makes  the connection more reliable.  The switch will receive

the connect  request from  you, establish  a connection with you,

and then  attempt to  establish a  connection with  N2FWI.   This

arrangement provides  less congestion  within the network because

the acknowledgements are only between adjacent stations.

Multi-switch networking          Multi-switch networking          _______________________

In order  to connect  to another  station at a remote switch

one must  know the  address for  that switch.   If KB7UV uses the

switch "718956"  a user would type: "C KB7UV V N2DSY-3,718956" to

connect to  him.  In later versions the network will have support

for directory  services enabling  you to use the actual telephone

number of the Amateur as the address instead of being required to

know their local switch address.

4

Entry and Exit Digipeaters          Entry and Exit Digipeaters          __________________________

If the  nearest ROSE X.25 Packet Switch is not local to you,

you may  need to  include  an  extra  digipeater  address.    For

instance if  K2MFF-2 is  the digipeater  you need  to access  the

nearest switch  (N2DSY-3), then  you could  use a connect command

such as;

C WA6KJD Via K2MFF-2,N2DSY-3,619372

A digipeater may also be needed at the exit point of the network:

C WA6KJD Via N2DSY-3,619372,WA6KJD-2

As well as Both:

C WA6KJD Via K2MFF-2,N2DSY-3,619372,WA6KJD-2

Monitoring Transmissions          Monitoring Transmissions

Let us  first look  at  what  happens  when  you  set  up  a

connection.  For the purpose of example we will look at a network

consisting of two switches.

N2DSY-3 = = = = = X.25 = = = = = KA2VLP-3

201744                             609426

Little Falls                   Hightstown

W2VY                                          KB1BD

When I want to connect to Bob, KB1BD, I type:

C KB1BD Via N2DSY-3,609426

Which will cause my TNC to transmit:

W2VY>KB1BD,N2DSY-3,609426 <C>

Where the <C> means that it is a Connection request.

At this point N2DSY-3 will accept the connect request by sending:

KB1BD>W2VY,609426,N2DSY-3* <UA>

Where the <UA> is an "Unnumbered Acknowledgement" confirming and

accepting the Connection request.

Note that N2DSY-3 is marked (*) as the transmitting station.

5

At the  other end  the connect request will exit the network with

KA2VLP-3 sending:

W2VY>KB1BD,201744,KA2VLP-3* <C>

Note that KA2VLP-3 is marked (*) as the transmitting station.

Assuming Bob's station is on the air and not busy, he will accept

the connect request by sending:

KB1BD>W2VY,KA2VLP-3,201744 <UA>

And his TNC will print to the terminal:

*** CONNECTED TO W2VY VIA KA2VLP-3,201744

This indicates the correct path to W2VY for Bob.

How to determine where a connection originated          How to determine where a connection originated          ______________________________________________

When monitoring  a channel you will see the switch call sign

preceded or  followed by  a six-digit  number.   This is the area

code and exchange of a switch within a ROSE X.25 Network.

Example:

If you were to see a monitored frame such as:

WA6KJD>W2VY,619372,N2DSY-3* <I>:Carol says Hi!

Where  the   <I>   indicated   the   transmission   contains

Information.

This would indicate that the N2DSY-3 switch was transmitting

information to  W2VY on  behalf of  WA6KJD who  is at Switch

address 619372.

And if you saw:

W2VY>WA6KJD,N2DSY-3,619372 <I>:Hi Dad, back on HF yet?

Would be a frame sent by W2VY going to WA6KJD@619372.

Connections with DX Stations look about the same but have an

extra numeric field:

VE7APU>W2VY,3020,615423,N2DSY-3* <I>: Hi Tom.

Is a  frame from  Canada (Data Country Code 3020), Area Code

615-423.  For a complete list of DCC's see Appendix B.

6

ROSE X.25 Packet Switch Messages          ROSE X.25 Packet Switch Messages

"*** Disconnect *** nnnn"          "*** Disconnect *** nnnn"

This message is sent when your connection to the other station is

cleared.  The four-digit number (nnnn) describes the reason for

disconnection.  For your convenience the following table is a

list of the codes that are normally seen.  The first two digits

are only important to this table.

X.25 Name      Value     Explanation          ____________________________________

DTE Originated      0000 The other station disconnected

Number Busy         0100 The other station is busy

Congestion          0500 Retry Count Exceeded

Out of Order        0900 Network link not operating

Not Obtainable      0D00 No known path for address specified

Ship Absent         3900 No response from station

In the future these will be replaced with phrases with more

meaning.

Appendix A  contains a  complete list  of codes  used by the

ROSE X.25 Packet Switch.

"*** Reset *** nnnn"          "*** Reset *** nnnn"

This message  is sent when a RECONNECT command was issued or

the link  went through a level 2 "Link Reset", to notify you that

there may  have been  some data  lost.   For the complete list of

X.25 Cause and Diagnostic codes see Appendix A.

X.25 Name      Value          Explanation          _________________________________________

DTE_Orig       0092      The other user issued a REConnect

Congestion          0792      A Network Link issued a REConnect

Tips and Tricks          Tips and Tricks

Can't type full numeric digipeater field:

If you  own a  TNC from  the vendor that does not allow

numeric  fields   (TAPR  TNC-1  based  TNC's)  you  may

exchange any  1's for  I's and/or  0's for  O's and the

switch will  translate it  for you.   Don't worry about

incoming connect  requests as they don't place the same

limitation on received frames.

7

Appendix A

CCITT X.25 Cause Codes used by the ROSE X.25 Packet Switch             CCITT X.25 Cause Codes used by the ROSE X.25 Packet Switch

The clearing (disconnect) codes are comprised of two parts, the

first two digits are the X.25 Cause, indicating the general

reason for the failure and the second two digits are the X.25

Diagnostic to indicate the specific reason for the failure.

CCITT X.25 Name     Value     Explanation          _________________________________________

DTE Originated      00   The other station disconnected

Number Busy         01   The other station is busy

Invalid Facility         03   internal error

Network Congestion  05   Retry Count Exceeded

Out of Order        09   Network link not operating

Access Barred       0B * Not used

Not Obtainable      0D   No known path for address specified

Remote Procedure    11   internal error

Local Procedure          13   internal error

RPOA Out of Order   15 * RPOA Not operational

Reverse Charge      19 * Reverse Charging not subscribed to

Incompatible Dest.  21 * Incompatible Destination

Fast Select         29 * Fast Select Not subscribed to

Ship Absent         39   No response from station

Gateway Proc Error  C1 * Gateway Detected Procedure Error

Gateway Congestion  C5 * Gateway Congestion

*  Currently not used, should not be seen.

Table 1 - X.25 Clearing Cause Values

CCITT X.25 Name     Value     Explanation          _________________________________________

DTE Originated      00   The other station re-connected

Out of Order        01 *

Remote Procedure    03 *

Local Procedure          05 *

Network Congestion  07   Link Reset on Network Trunk

Remote Operational  09 *

Network Operational 0F *

Incompatible Dest.  11 *

Network Out of Order     1D *

Gateway Proc. Error C1 *

Gateway Congestion  C3 *

Gateway Operational C7 *

*  Currently not used, should not be seen.

Table 2 - X.25 Resetting Cause Values

8

Appendix A

Value          Explanation          __________________________

01 (01)        Invalid P(S) - Internal sequencing error

02 (02)        Invalid P(R) - Internal sequencing error

17 (11)        Invalid X.25 Packet for R1 State

19 (13)        Invalid X.25 Packet for R3 State

20 (14)        Invalid X.25 Packet for P1 State

21 (15)        Invalid X.25 Packet for P2 State

22 (16)        Invalid X.25 Packet for P3 State

23 (17)        Invalid X.25 Packet for P4 State

24 (18)        Invalid X.25 Packet for P5 State

25 (19)        Invalid X.25 Packet for P6 State

26 (1A)        Invalid X.25 Packet for P7 State

27 (1B)        Invalid X.25 Packet for D1 State

29 (1D)        Invalid X.25 Packet for D3 State

33 (21)        Unidentifiable Packet

36 (24)        Illegal Packet on unassigned logical channel

38 (26)        Packet too short

39 (27)        Packet too long

41 (29)        Restart packet on non-zero logical channel

43 (2B)        Unauthorized Interrupt Confirm Packet

44 (2C)        Unauthorized Interrupt Packet

71 (47)        No logical channel available

72 (48)        Call Collision

76 (4C)        Facility not provided when expected

120 (78)       Temporary Routing Problem

127 (7F)       Maintenance Action

146 (92)       Retry count exceeded for data packet transmission

Table 3 - X.25 Diagnostic Codes Used

9

Appendix B

Data Country Codes

Zone 2            ______

DCC  Country or Area            ____________________

202  Greece

204  Netherlands

206  Belgium

208  France

212  Monaco

214  Spain

216  Hungarian People's Republic

218  German Democratic Republic

220  Yugoslavia (Socialist Federated Republic of)

222  Italy

226  Romania  (Socialist Republic of)

228  Switzerland (Confederation of)

230  Czechoslovak Socialist Republic

232  Austria

234  United Kingdom of Great Britain and Northern Ireland

238  Denmark

240  Sweden

242  Norway

244  Finland

250  Union of Soviet Socialist Republics

260  Poland

262  Federal Republic of Germany

266  Gibraltar

268  Portugal

270  Luxembourg

272  Ireland

274  Iceland

276  Albania

278  Malta (Republic of)

280  Cyprus (Republic of)

284  Bulgaria (People's Republic of)

286  Turkey

Zone 3            ______

DCC  Country or Area            ____________________

302  Canada

308  St. Pierre and Miquelon

310  United States of America

311  United States of America

312  United States of America

313  United States of America

314  United States of America

315  United States of America

316  United States of America

330  Puerto Rico

332  Virgin Islands (USA)

334  Mexico

338  Jamaica

10

Zone 3 (Cont)            _____________

DCC  Country or Area            ____________________

340  French Antilles

342  Barbados

344  Antigua

346  Cayman Islands

348  British Virgin Islands

350  Bermuda

352  Grenada

354  Montserrat

356  St. Kitts

358  St. Lucia

360  St. Vincent

362  Netherlands Antilles

364  Bahamas (Commonwealth of the)

366  Dominica

368  Cuba

370  Dominican Republic

372  Haiti (Republic of)

374  Trinidad and Tobago

376  Turks and Caicos Islands

Zone 4            ______

DCC  Country or Area            ____________________

404  India (Republic of)

410  Pakistan (Islamic Republic of)

412  Afghanistan (Democratic Republic of)

413  Sri Lanka (Democratic Socialist Republic of)

414  Burma (Socialist Republic of the Union of)

415  Lebanon

416  Jordan (Hashemite Kingdom of)

417  Syrian Arab Republic

418  Iraq (Republic of)

419  Kuwait (State of)

420  Saudi Arabia (Kingdom of)

421  Yemen (Arab Republic)

422  Oman (Sultanate of)

423  Yemen (People's Democratic Republic of)

424  United Arab Emirates

425  Israel (State of)

426  Bahrain (State of)

427  Qatar (State of)

428  Mongolian People's Republic

429  Nepal

430  United Arab Emirates (Abu Dhabi)

431  United Arab Emirates (Dubai)

432  Iran (Islamic Republic of)

440  Japan

450  Korea (Republic of)

452  Viet Nam (Socialist Republic of)

454  Hong Kong

455  Macao

456  Democratic Kampuchea

457  Lao People's Democratic Republic

11

Zone 4 (Cont)            _____________

DCC  Country or Area            ____________________

460  China (People's Republic of)

470  Bangladesh (People's Republic of)

472  Maldives (Republic of)

Zone 5            ______

DCC  Country or Area            ____________________

502  Malaysia

505  Australia

510  Indonesia (Republic of)

515  Philippines (Republic of)

520  Thailand

525  Singapore (Republic of)

528  Brunei

530  New Zealand

535  Guam

536  Nauru (Republic of)

537  Papua New Guinea

539  Tonga (Kingdom of)

540  Solomon Islands

541  New Hebrides

542  Fiji

543  Wallis and Futuna Islands

544  American Samoa

545  Gibert and Ellice Islands

546  New Caledonia and Dependencies

547  French Polynesia

548  Cook Islands

549  Western Samoa

Zone 6            ______

DCC  Country or Area            ____________________

602  Egypt (Arab Republic of)

603  Algeria (Algerian Democratic and Popular Republic)

604  Morocco (Kingdom of)

605  Tunisia

606  Libya (Socialist People's Libyan Arab Jamahiriya)

607  Gambia (Republic of the)

608  Senegal (Republic of the)

609  Mauritania (Islamic Republic of)

610  Mali (Republic of)

611  Guinea (Revolutionary People's Republic of)

612  Ivory Coast (Republic of the)

613  Upper Volta (Republic of)

614  Niger (Republic of the)

615  Togolese Republic

616  Benin (People's Republic of)

617  Mauritius

618  Liberia (Republic of)

619  Sierra Leone

620  Ghana

621  Nigeria (Federal Republic of)

622  Chad (Republic of the)

12

Zone 6 (Cont)            _____________

DCC  Country or Area            ____________________

623  Central African Republic

624  Cameroon (United Republic of)

625  Cape Verde (Republic of)

626  Sao Tome and Principe (Democratic Republic of)

627  Equatorial Guinea (Republic of)

628  Gabon Republic

629  Congo (People's Republic of the)

630  Zaire (Republic of)

631  Angola (People's Republic of)

632  Guinea-Bissau (Republic of)

633  Seychelles

634  Sudan (Democratic Republic of the)

635  Rwanda (Republic of)

636  Ethiopia

637  Somali Democratic Republic

638  Republic of Djibouti

639  Kenya (Republic of)

640  Tanzania (United Republic of)

641  Uganda (Republic of)

642  Burundi (Republic of)

643  Mozambique (People's Republic of)

645  Zambia (Republic of)

646  Madagascar (Democratic Republic of)

647  Reunion (French Department of)

648  Zimbabwe

649  Namibia

650  Malawi

651  Lesotho  (Kingdom of)

652  Botswana (Republic of)

653  Swaziland (Kingdom of)

654  Comoros (Federal and Islamic Republic of the)

655  South Africa (Republic of)

Zone 7            ______

DCC  Country or Area            ____________________

702  Belize

704  Guatemala (Republic of)

706  El Salvador (Republic of)

708  Honduras (Republic of)

710  Nicaragua

712  Costa Rica

714  Panama (Republic of)

716  Peru

722  Argentine Republic

724  Brazil (Federl Republic of)

730  Chile

732  Colombia (Republic of)

734  Venezuela (Republic of)

736  Bolivia (Republic of)

738  Guyana

740  Ecuador

742  Guiana (French Department of)

13

Zone 7 (Cont)            _____________

DCC  Country or Area            ____________________

744  Paraguay (Republic of)

746  Suriname (Republic of)

748  Uruguay (Oriental Republic of)

14

The Radio Amateur Telecommunications Society                    The Radio Amateur Telecommunications Society

Membership Application                               Membership Application

Name:__________________________    Callsign:______________

Address:_______________________

State:_________________________    Zip:____________

Home Telephone:  (     ) -    OK in roster ? Y / N

OK on air ?    Y / N

Office Telephone:(     ) -    OK in roster ? Y / N

Ok on air ?    Y / N

Local/Home BBS:

BBS is what type (PRMBS/MBL/RLI/Other):

Local Network (ROSE/NetRom/TheNet/DIGI):

Local Network Address:

TNC/PAD:

Radio(s):

Computer(s):

Interests:

Comments:

Please enclose a check for $ 25.00 and send to:

The Radio Amateur Telecommunications Society

206 North Vivyen Street

Bergenfield, NJ  07621

For  further  information contact Gordon Beattie,  N2DSY  at

the KD6TH PRMBS or at 201-387-8896.