AMTOR/PACKET Link (APLINK) Version 3.81, June 10, 1989

Copyright (c) 1989 - Victor D. Poor, W5SMM

[NTS OPERATIONS SUPPLEMENT TO APLINK.DOC]

NATIONAL TRAFFIC SYSTEM FEATURES

APLINK, beginning with Version 3.8, includes features specifically

to support National Traffic System operations in the United States

and Canada.  Sysops for APLINK outside of these countries can

safely ignore these facilities.

The purpose of these features it to make selected APLINK

installations a clearing point for NTS traffic being passed between

active NTS stations.  When activated, these facilities will

restrict access for certain NTS traffic to designated NTS stations

and provide for automatic confirmation of access and forwarding

back to originating stations.

CONFIGURING THE SYSTEM FOR NTS OPERATION

To activate special handling of NTS traffic, the SYSOP must include

the following parameter in the APLINK.CNF file:

NTS=ON

If this parameter is not set then 'ST' messages will have no

restrictions placed on their access and no confirmation messages

will be generated.

The following discussion assumes that NTS=ON has been set.

DESIGNATING NTS STATIONS

The SYSOP must co-ordinate with NTS TCC Directors to designate

which stations have NTS access privileges.  This designation is

done with the USERS.EXE program, setting the NTS field for each NTS

station to YES.

ENTERING NTS TRAFFIC

Traffic to be cleared by designated NTS stations should be entered

into the system with the APLINK station's call or with the 'AT'

field not used.  For example, where WA8DRZ is the APLINK station,

any of the following commands will 'capture' a message for the NTS

users:

ST WA8DRZ

ST NTS

ST WA8DRZ AT WA8DRZ

ST NTS AT WA8DRZ

ST WA8DRZ AT 94062

ST NTSTX AT WA8DRZ

In other words, any 'ST' message reaching WA8DRZ with WA8DRZ in

either the 'TO' field or the 'AT' field or with a blank 'AT' field

is considered an NTS message with restricted access and to be

cleared by a participating NTS station.

Messages arriving at WA8DRZ, to use this example, that are

addressed such as following would be autoforwarded by the APLINK

station and would not be treated as restricted NTS messages:

ST 78240 AT NTSTX

ST WB7QWG AT WB7QWG

ST WA5QZI AT 78229

No restriction is placed on any station entering an 'ST' message -

restrictions only apply to access.

NTS MESSAGE FORMAT

NTS operators entering messages into the system to be cleared by

other NTS operators would be expected to use the following form

(lower case is the system response):

ST WA8DRZ

nr 5678 nts ga msg+?

NTS WISCONSIN

NR 45 R HXG W7ABC 8 TACOMA WASH 1245Z 23 MAY

ROSEY MAY JONES

321 GARDEN ST

GREEN BAY WISCONSIN

404-555-4321

HAPPY BIRTHDAY X SEE YOU SOON X LOVE

TONY

NNNN

nr 5678 filed ga+?

Note that the 'title' or 'subject' line is the NTS routing. The

first non-blank line following is the standard NTS header.

ACCESSING NTS MESSAGES

A list of NTS messages may be viewed by entering the command:

NTS

A list of all NTS messages that have not been accepted by an NTS

station will be displayed.  Any of the messages displayed may be

read by the standard APLINK command:

R <message number>

Following the display of the message the following prompt will be

displayed:

ACCEPT (YES/NO/CANCEL)+?

The system will not proceed until one of the three responses is

given.  A response of YES means that the NTS station accepts

responsibility for further delivery and APLINK will mark it as

forwarded and delete it from the NTS message list. A response of

NO leaves the status of the message unchanged and a response of

CANCEL provides the option of removing a message from the system

that is unsuitable for amateur handling.  In all three cases a

confirmation message will be generated as discussed below.

AUTOMATIC CONFIRMATIONS

Whenever a restricted NTS message is accessed a confirmation

message is automatically generated and sent to the station

originating the message into the APLINK system (not the station

whose call appears in the NTS header line).  The address to the

originating station will leave the 'AT' field blank unless the call

appears in the intercept file.

The confirmation will take the following form:

CNF

UR MSG NR 5678 ACCEPTED BY W2DEF 890523/1534Z

NR 45 R HXG W7ABC 8 TACOMA WASH 1245Z 23 MAY

NNNN

The word 'ACCEPTED' will alternately be 'READ' or 'CANCELLED'

depending on the accessing station's response to the above prompt.

ARCHIVING MESSAGES

APLINK will hold messages in the system for at least 24 hours after

they have been marked as forwarded.  A participating NTS station

may see a list of all messages including forwarded 'ST' messages

using the standard APLINK 'L' commands and may read any message

appearing on an 'L' command message list.

Sometime after 24 hours has passed from forwarding, APLINK will

remove the messages from the system and archive them. Only the

SYSOP can retrieve a message from the archive.

A message that remains unforwarded in the system for 21 days will

be removed and archived.