AA4RE BBS and the G8BPQ Switch.

Note!!!!!!  BPQ Versions 4.xx dropped support for the AA4RE system.

From version 3.23, the switch supports the AA4RE BBS.  This has been

done by extending the PK232 host mode support, originally written to

support the G8UFQ BBS.  The system allows the first Virtual TNC port to

support up to 8 users.  The configuration I used is detailed below:

In BPQCFG.TXT:

TYPE=PK232/AA4RE

APPLFLAGS=6

In PARMS.BB, the recommended port settings are below

port_type       = G8BPQ_NODE

com             = 2                ; use the first number in TNCPORTLIST

speed           = 4800             ; not actually used

max_connect     = 8                ; Up to 8

max_channel     = 9

max_pending     = 1

broadcast_path  = MAIL             ; is ignored - set in config.txt file

In PATHS.BB

PATH GB7DAD A GB7DAD FORCE

T 0000 2300

C SWITCH                                ; note initial C SWITCH

SC LRG7

R* to *

SC G4NAD-1

R* to *

Note that the remaining Virtual TNCs continue to run as normal.  If you

are running under Desqview, you can still run PAC2 (or even MBL/RLI!) in

other windows.  However, BB needs a LOT of store - you need to use all

the tricks described in the DV manual to get a big enough window to run

it effectively.