Special notes for those using the DRSI PC*PA cards.

---------------------------------------------------

**********************************************************************

Only applies if you are using the DRSI software.  If you are using the

G8BPQ node software, see G8BPQ.DOC

**********************************************************************

MBBIOS is not a requirement for these cards.  It will not be needed

unless you will also have convential TNCs.

During the initialization of your BBS , run TNCTSR-S.  It will go

resident.  I don't supply TNCTSR-S.  Its on the diskette that comes with

the card.  For most purposes, TNCTSR-S is quite sufficent but if you

have multiple cards, you might need TNCTSR-L.

BB will support multiple DRSI cards (up to 4).  To do more than one card

you have to have version 1.3 or higher of TNCTSR-S.  You get this

software from DRSI not me.

Remember that the MAX_CONNECT setting applies to all PC*PA ports in

total.

All the PC*PA ports should have identical MAX_CONNECT, MAX_CHAN,

MAX_PENDING, MONITOR, camd callsigns.  Only the settings from one port

will be used but you can't always figure which one.

Use COM=255 for PC*PA ports unless you have changed the DRSI software.

When using the terminal mode, you must explicitly define the port you

want to connect out on.  The port letter is ignored by the DRSI

software.  Your connects should look like this:

<esc>C 1:AA4RE