Editing a user to set parameters:

EU call

C x -- Sets the class.  x should be one of the following:

N = New

U = Unregistered user

O = Registered user

E = Expert user

B = BBS

R = Remote SYSOP

L = Local SYSOP

F x -- Set display format.  0, 1 are only allowed values for x

H x -- Sets the user's Home BBS to x

M x -- Set maximum packet size to x.  0 means port PACLEN

will be used.

N x -- Sets the user name to x

Q x -- Sets language to x

S x -- Set screen length to x

W x -- Sets password to x

Y x -- Set screen width to x

Z x -- Sets the user's zipcode to x

The following are switches that you invert by typing the proper letter:

A -- Is an advanced BBS (BIDS are ok)

B -- Is a BBS

D -- Delete this user.  Will happen after next start of BB

E -- Emergency mode user

L -- User is local

P -- Personal BBS

R -- Enable remote SYSOP function.  You can also include a number to

indicate a password.  See OPERATE.DOC on how the passwords work.

T -- Transparent.  If off then each packet will end in a carriage

return.  If on, then packets are usually full

X -- User is not permitted to connect

You should set all BBS to

C B        (BBS Class)

A          (Advanced BBS)

If a BBS is not BID equipped, use B for BBS instead of A.