Application Support for G8BPQ Switch

Until now, the switch has supported connects to two sets of application

programs. The first is the BBS, obtained by connecting to the BBS call/alias,

or connecting to the switch end entering BBS. The second is the SYSOP,

obtained by entering the 'C' command without a callsign from the switch. The

commands CONOK ON or HOSTOK ON had to be sent by the program, to indicate

that the TNC port was available. From Version 3.20, up to 8 applications

are supported. The first two are as above, the third is reserved for remote

SYSOP access, and the other 5 are available for your own applications. The

list of application names is given in the APPLICATIONS= param in the config

file, and is a string of up to 8 names of up to 12 chars each, separated by

commas. The names will be listed to the user, along with the normal commands

in the 'available commands' prompt, obtained by entering ?. The user enters

the Application name to connect to it, in the same way as he can enter BBS to

connect to the BBS. Note that, for the moment at least, these additional

applications do not have there own callsign/alias, so must be accessed via

the switch. (but you could rename the first application if you really want

to!).

It is possible to configure the port so that the user's command line is passed

to the application. You can also specify whether the user should receive

a '*** Connected to APPL' message, and whether the Application gets a

'*** CONNECTED to USER' message. The TNCPORTS are defined by sets of params

in the following format:

TNCPORT

COM=n		; COMBIOS port number

TYPE=TNC2	; From version 4.00 this is the only type supported

APPLMASK=nn	; Defaults to 1 (BBS)

APPLFLAGS=nn	; Defaults to 6

ENDPORT

;

APPLMASK is used in the same way as the APPL TNC command described

below. The value is in decimal.

APPLFLAGS control whether the user's command is passed to the application,

and also whether 'CONNECTED' messages are sent in each direction. Values

are:

1 - Send Command to application.

2 - Send 'Connected' to user.

4 - Send 'Connected' to application.

Values may be combined as required - the default is 6, which gives the

same result as the older versions.

There is also an equivalent APPLFLAG command available at TNC2 command level.

When using TNC2 mode, the program should send two commands to the virtual

TNC:-

APPL $NN

Where NN has a bit set corresponding to the appl. number. eg $01 for BBS,

$08 for 4th appl. More than one can be set, allowing the port to support

more than one application. This may be appropriate if the command line is

sent to the application, so it can work out what the user wants.

Note the the default is $01, so existing BBS applications will still work.

CONOK ON

To indicate port is ready to accept connects.

There is a trivial sample application program, SAMPLE.C  on the disk.

From version 4.00 onwards, a new application interface is provided. See file

BPQHOST.DOC for further details.

############################################################################################################################