The first stage of the Remote Sysop facility is now available. The system

also provides the same facilities locally. The program may be run as a

normal DOS program, either in a Desqview window or standalone, or in may

run as a TSR. The TSR mode would normally be used if you were running a

standalone node, or a BBS system that doesnt need DesqView (eg AA4RE).

In TSR mode, commands cannot be entered locally, and the SAVENODES command

is disabled.

Installation.

1. Add an extra port to TNCPORTLIST (unless you have a spare)

2. Add a suitible application name as the third APPLICATIONS Param, ie

APPLICATIONS=BBS,,*SYSOP

I suggest you use a name starting with a '*', as this will 'hide' the

command from the Valid Commands display.

3� Creat� � file� PASSWORD.BPQ, containing a string of up to 80 characters.

4. Run program SYSOP, with a single parameter of the TNCPORT specified above,

either on its own, or in a DesqView window. If a second parameter of

'RESIDENT' is specified, the program will become a TSR routine.

Commands may mow be entered directly on the PC, (unless in TSR mode) or

remotely by connecting to the Node, and entering the APPL name (eg *SYSOP).

When accessed remotely, a password prompt is sent, containing 5 random numbers

in the range 1 to the length of the PASSWORD.BPQ string. The first command

entered must be the corresponding five letters from the password string,

or the user will be disconnected.

Available Commands.

The following commands are supported:

BYE - Disconnect remote user (even if entered from console)

TXDELAY

MAXFRAME

FRACK

RESPTIME

PACLEN

RETRIES

QUALITY

PERSIST

TXTAIL

XMITOFF

DIGIFLAG

DIGIPORT

USERS

VALIDCALL

L3ONLY

BBSALIAS

Setting XMITOFF will supress all TX activity on a port (including CWID,

beacons, etc). I put it in so I could monitor the satellites without

causing QRM

Setting BBSALIAS will cause connects to the port call or port alias to go to

the BBS rather than the node.

The above display or update the corresponding parameter for a specified port,

eg   PACLEN 1 100

PERSIST 2 50

If the third parameter is omitted, the current value is displayed.

It is not currently possible to update VALIDCALLS, but you can display

the current setting.

*** Warning ***

At the moment the FRACK value is in 1/3sec units, and the TXDELAY is in

50ms units for HDLC cards, and 10ms units for KISS TNCs. I'll be sorting

this out when I get the time to rationalize the timer code, but in the

meantime, I suggest you make a note of the current values before you

start fiddling.

Also KISS users should note that the params are only sent to the TNC every

5 mins or so, so the updated values may not take effect for a while.

ROUTES

To display ROUTES table, enter ROUTES

To display entry for a particular neighbour, enter ROUTES Call Port,

eg   ROUTES G8BPQ-2 2

To update an entry, enter ROUTES Call Port Params

eg   ROUTES G8BPQ-2 2 100	- Set quality to 100

ROUTES G8BPQ-2 2 !         - Toggle 'Locked Route' flag

ROUTES G8BPQ-2 2 100 !     - Set quality and toggle 'locked' flag

SAVENODES

Not available in TSR mode.

Writes the ROUTES and NODES tables to the BPQNODES file.

The SAVENODES function may also be invoked directly from the DOS command

line - this function replaces BPQNODES.COM

ie.  SYSOP SAVENODES

Please let me know what other parameters you would like to be able to

update.

John Wiseman, G8BPQ

29.5.90

A version has now been produced to use with BPQ Host Mode - SYSOPH. It is

functionally equivalent to SYSOP, but the port number can be in the range

1 to 64.

20.2.91

#####################################################################################################